3GPP TSG RAN WG1 #87	R1-1612461

# Reno, USA, 14th – 18th November 2016

Agenda item:	7.1.2.3

Source: 	Samsung

Title: 	RACH preamble format considering beam correspondence

Document for:	Discussion and Decision

Introduction

During RAN1#86bis, RAN1 captured the following agreements regarding RACH preamble [1]:.

Agreements:

- NR supports multiple RACH preamble formats, including at least

  - RACH preamble format with longer preamble length

  - RACH preamble format with shorter preamble length

  - FFS how many signatures (e.g. number of RACH sequences, payload size, etc.)

- Multiple/repeated RACH preambles in a RACH resource is supported

  - FFS: How to support single-beam and/or multi-beam operation

  - FFS: Preamble could be the same or different

- Numerology for RACH preamble can be different depending on frequency ranges

  - FFS: How many numerologies will be supported per frequency range

- Numerology for RACH preamble can be different or the same from that for the other UL data/control channels

This contribution discusses the RACH preamble format considering beam correspondence

Discussion on RACH preamble format

The multi-beam based random access procedure with and without beam correspondence is described in [2]. Especially, regarding the RACH preamble transmission procedure, the RACH structure based on multiple beamforming can have different format depending on whether beam correspondence holds or not at TRP side.

When beam correspondence holds at TRP side, UE can transmit RACH preamble is transmitted using the selected preamble index and transmission power on RACH resource associated with TRP Rx beam side which has beam correspondence with TRP Tx beam. Fig 1 shows the relationship between RACH resource and the TRP Rx beam.

Figure 1. An example of the relationship between RACH resource and the Rx beam index at eNB

Figure 2 and Table 1 show the RACH preamble transmission for the TRP with beam correspondence.

Figure 2. The RACH transmission with beam correspondence

Figure 2 shows that TRP sweeps its Rx beam during RACH transmission while UE transmits RACH preamble with fixed UE Tx beam. One RACH resource for Rx beam consists of GP, CP and RACH symbol. Multiple UEs can be multiplexed by RACH resource associate with TRP Rx beam. However, in order to support beam division multiplexing access during one RACH occasion, the GP between each RACH resource should be inserted to avoid ISI caused by different propagation delay between TRP and each UE. For example, let’s assume that two UEs transmit RACH preamble on RACH resource associated with TRP Rx beam #0 and #1, respectively. A UE1 on TRP Rx beam #0 is located far from the TRP than a UE2 on TRP Rx beam #1. ISI can be avoided due to previous symbol while TRP detects RACH for UE 2 on TRP Rx beam #1 by CP. However, ISI might be occurred due to following symbol when TRP detects RACH for UE 1 on TRP Rx beam #0 if there is no GP in each RACH resource. Table 1 shows the multiple preamble formats to support various RACH coverages.

Table 1: Random access preamble parameters for the TRP with beam correspondence

| Preamble format |  |  |  | Note |
| --- | --- | --- | --- | --- |
| 0 |  |  |  | should be longer than round-trip delay is length for RACH sequence is the length of guard interval |

Observation 1: With TRP beam correspondence, one RACH resource can be consisted of CP, GP and RACH symbol.

Figure 3. An example of RACH resource without beam correspondence

Without beam correspondence, a UE should transmit multiple RACH symbols repeatedly on multiple RACH resources in one RACH occasion to cover all TRP Rx beams. UE uses one of UE Tx beams in one RACH occasion.  To allow more Rx beam sweeping at TRP side, it is desirable that consecutive RACH symbols are transmitted in one RACH occasion. Figure 4 shows the consecutive RACH transmissions without GP.

Figure 4. The RACH preamble transmission for the TRP without beam correspondence

TRP can receive RACH symbol from each UE with TRP Rx beam index sweeps and detect RACH symbol when TRP Rx beam index and UE Tx beam index is aligned. It is marked with grey color in figure 4. Even if multiple UEs are trying to transmit RACH preamble, orthogonality between multiple UEs with different Rx beam index can be maintained thanks to consecutive RACH symbols. However the number of available multiplexing UEs may be decreased comparing to the scenario with beam correspondence. In order to support flexible RACH coverage, TRP can use one RACH symbol or combine multiple RACH symbols. TRP should inform UE that how many repetitions would be needed for RACH transmission corresponding to the number of TRP Rx beam indices via SIB[4]. Thus, single preamble format for RACH symbol can be considered represented in Table 2.

Table 2: Random access preamble parameters without beam correspondence

| Preamble format |  |  |  | Note |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | A UE transmits consecutive ‘K’ RACH symbols during RACH occasion. The value of K should be indicated to UE via higher layer signalling |

Observation 2: Without TRP beam correspondence,  is K times longer than  for preamble format 0. K is indicated from higher layer signalling

Proposal 1: Study different RACH preamble format to support the cases with TRP with and without beam correspondence.

Proposal 2: Preamble formats to support various cell coverages should be further studied.

Conclusions

This contribution discusses the RACH preamble formats. The observations and proposals are as follows:

Observation 1: With TRP beam correspondence, one RACH resource can be consisted of CP, GP and RACH symbol.

Observation 2: Without TRP beam correspondence, the sequence length, , is K times longer than  for preamble format 0. K is indicated from higher layer signalling

Proposal 1: Study different RACH preamble format to support the cases with TRP with and without beam correspondence.

Proposal 2: Preamble formats to support various cell coverages should be further studied.

References

- Chairman’s Note of 3GPP TSG RAN WG1 #87.

- R1-1612466, “RA procedure with and without beam correspondence”, Samsung