3GPP TSG RAN WG1 Meeting #87								    R1-1612644

Reno, USA 14th - 18th November 2016

Agenda Item:	7.1.4.1

Source:	InterDigital Communications

Title:	Blocking probability of control channel for URLLC

Document for:	Discussion and Decision

# Introduction

In RAN1 #86 meeting [1] some agreements were made on Downlink Control Channel design in NR:

- NR should support at least the following

  - A DL control channel can be mapped on one or more NR-CCEs

    - This is at least for the case where the DL control region consists of one or a few OFDM symbol(s) of a slot or a mini-slot

    - A NR-CCE includes a positive integer number of PRBs (FFS: exact value)

      - FFS: whether a NR-CCE contains contiguous PRBs

      - FFS: whether multiple NR-CCEs may share one or more PRBs

  - FFS: whether NR-CCE is mapped on frequency-domain only or on both frequency and time-domain.

This agreement suggests having overlapping CCEs for further study, which can be also considered as non-orthogonal control channel designs.

In this contribution we first discuss the importance of lowering the blocking probability of Control Channel for URLLC. We then discuss a few design approachs based on non-orthogonal control channel design for URLLC which we first introduced in RAN1#86bis [2]. Simulation results will be then provided to show the advantage of using non-orthogonal control channel design over the conventional orthogonal designs for this usage scenario.

# Discussion

For the URLLC usage scenario, NR needs to adhere to strict latency and reliability requirements. To satisfy the low latency and high reliability requirement, the physical control channel should have very small blocking probability, which is the probability that no control channel is assigned to a user that needs service.

In previous designs for EDPCCH in LTE advanced, it was observed that having overlapping search spaces for different UEs makes it possible for the gNB to better avoid UE collision in the DL control channel and lower the blocking probability. However, the blocking probability of the legacy designs for DL control channel is still much higher than the strict requirements for URLLC applications. Indeed, with very tight limits on latency, there is no time for the second try in many URLLC applications. In this case, the limit on the probability of user blocking becomes comparable to the limit on the probability of unsuccessful data transmission, which is considered around 10-5 for URLLC. In this section, we discuss using overlapping NR-CCEs (along with having overlapping search spaces) to drastically lower the user blocking probability in DL control channel.

In RAN1#86bis it was agreed to study the case where multiple NR-CCEs may share one or more PRBs. Assuming that the control channel covers the first one or more OFDM symbols of a slot or mini-slot in one or more “control subbands”, an NR-REG comprises a subset of the corresponding OFDM symbols on a subset of frequency tones. That subset of frequency tones can be a physical resource block (PRB). For example, in the simplest case, an NR-REG can be the first OFDM symbol of a slot or mini-slot in one PRB. Figure 1 illustrates an example of two overlapping NR-CCEs which share one PRB, as indicated in the agreement.

Figure 1 Example of two overlapping NR-CCEs which share one PRB

An example of non-orthogonal control channel design using overlapping CCEs, where the CCEs can overlap in one or more REGs is shown in Figure 2. In this example, for each UE, a group of NR-REGs are assigned based on the corresponding signature of the UE. The signature is a sequence of 1s and 0s, of the length of the number of NR-REGs, showing the inclusion of NR-REGs in the correspong NR-CCE.

As an example, if there are 16 NR-REGs, each NR-CCE is composed of a group of NR-REGs, corresponding to the UE’s signature, which is a sequence of 1s and 0s of length 16. For example, the signature 1001000000000011 corresponds to the inclusion of NR-REGs with indices of {0, 3, 14, 15}. The number of 1s in the signature depends on the required coverage for the UE. An example of this approach is shown in Figure 1 as can be seen from the figure, the pair of signatures NR-CCE1 and NR-CCE2 only overlap on one NR-REG (i.e., REG 0). Similarly, the pair of signatures NR-CCE2 and NR-CCE3 only overlap on one NR-REG (i.e., REG 11).

To achieve better coverage, similar to LTE, several NR-CCEs can be assigned as an NR-PDCCH candidate for a UE. The number of NR-CCEs in an NR-PDCCH candidate (i.e., the aggregation level (AL)) is chosen based on the channel state information and the SNR requirement of the UE (similar to LTE and LTE advanced). Each UE can have several NR-PDCCH candidates (with possibly different aggregation levels). The set of NR-PDCCH candidates assigned to a UE is called the search space and the choice of a candidate among the candidates of a search space is done by the gNB (based on the required aggregation level and such a way that it avoids coincidence with NR-PDCCH of other UEs and reduces the overlap with the NR-PDCCH candidates chosen for other UEs). It should be noted that the chosen NR-PDCCH for two different UEs should not coincide, but can overlap (through the overlap of their corresponding NR-CCEs on some NR-REGs, or PRBs). The UE can detect its current NR-PDCCH by monitoring all its NR-PDCCH candidates and checking the CRC similar to the legacy systems. It should be noted that the seach space for different UEs can have overlap (i.e. two search spaces can have common candidates).

Figure 2 Example of non-orthogonal mapping of NR-CCEs to orthogonal NR-REGs

When we have overlap in the allocation of NR-REGs to NR-CCEs, we need to consider appropriate transmission schemes such that the partial collision of control data for different UEs (through the overlap in resource element usage) does not result in the loss of control signals. Similar to the signature-based non-orthogonal schemes that are proposed for data transmission in 5G NR, different methods can be used for this purpose.

One solution is to repeat the transmitted symbols, similar to the low-density spreading (LDS) method. This method can be used for signature-based non-orthogonal CCEs. In this solution, different NR-REGs corresponding to one NR-CCE are repetitions of each other.

Another approach is to induce inter-symbol dependencies by using mappings from the control data to the set of symbols that are sent over the corresponding REs (similar to constellation design for SCMA). This method can be applied to non-orthogonal NR-REGs as well as non-orthogonal NR-CCEs. In this method, control data is directly mapped to a set of symbols that are sent over the REs of the corresponding NR-REG (for the case of non-orthogonal NR-REGs) or the corresponding REs of different NR-REGs that are mapped to the same NR-CCE (for the case of non-orthogonal NR-CCEs).

# Simulation Results

In this section, we present simulation results for the blocking probability of NR-PDCCH, using the orthogonal (conventional) and non-orthogonal designs. We assumed a scenario where NR-REGs are are divided into blocks of 16 NR-REGs (comparable to the number of EREGs in one PRB in EDPCCH). For a block of 16 NR-REGs, if we consider the proposed method for non-orthogonal NR-CCEs, we can have 16 NR-CCEs where each pair of them overlap in at most one NR-REG. An example of mapping for the 16 NR-CCEs of size 4 to 16 NR-REGs with indices of 0, …, 15 can be as following:

NR-CCE0={0,1,2,3}, NR-CCE1={4,5,6,7}, NR-CCE2={8,9,10,11}, NR-CCE3={12,13,14,15},

NR-CCE4={0,4,8,12}, NR-CCE5={1,5,9,13}, NR-CCE6={2,6,10,14}, NR-CCE7={3,7,11,15},

NR-CCE8={0,5,10,15}, NR-CCE9={1,6,11,12}, NR-CCE10={2,7,8,13}, NR-CCE11={3,4,9,14},

NR-CCE12={0,7,10,13}, NR-CCE13={1,4,11,14}, NR-CCE14={2,5,8,15}, NR-CCE15={3,6,9,12}

Figure 3 and Figure 4 illustrates the user blocking probability as a function of number of active users, and packet arrival rate for both non-orthogonal and orthogonal control channel designs, respectively. For this evaluation, we assume an aggregation level of 4 and overlapping search spaces among the UEs where each UE is assigned 2 NR-PDCCH candidates. We further assume that the entire DL control channel region contains 128 NR-REGs (comparable to having 8 PRBs for the control region for EDPCCH). Also note that for Figure 4, we assume Poisson distribution for packet arrival.

Figure 3 Blocking probability as a function of number of active users for orthogonal and non-orthogonal DL control channel designs

Figure 4 Blocking probability as a function of packet arrival rate for orthogonal and non-orthogonal DL control channel designs

As can be seen from Figure 3, for the same number of active users, the blocking probability for non-orthogonal scheme with overlapping CCEs is an order of magnitude better than its orthogonal counterpart. Another interesting observation is that for certain blocking probability KPI, much higher number of UEs can be accommodated with the non-orthogonal scheme. For example at 10% blocking probability, the number of users that can be accommodated without collision for the non-orthogonal scheme would be 9 vs 4 for the orthogonal scheme which is almost double.

The same trend can be observed from Figure 4. In fact for the same packet arrival rate, the blocking probability for non-orthogonal scheme with overlapping CCEs is again improved by an order magnitude compared to the orthogonal scheme. Another interesting observation from Figure 4 is that for the same blocking probabilities, much higher packet arrival rates can be achieved by using the non-orthogonal scheme, compared to the conventional orthogonal scheme. This observation suggests that even lowering the blocking probability to  10-5 (comparable to the requirement for data error rate) is feasible for moderate URLLC packet arrival rates, if we use the non-orthogonal NR-PDCCH (which uses overlapping NR-CCEs).

Observation 1: Non-orthogonal DL control channel design, using overlapping CCEs, can drastically lower the probability of user blocking in NR-PDCCH.

Observation 2: Even lowering the blocking probability to 10-5 (comparable to the requirement for data error rate) is feasible for moderate URLLC packet arrival rates, if we use overlapping NR-CCEs.

Proposal 1: For URLLC, the probability of user blocking should be considered in the selection of NR-PDCCH design.

Proposal 2: NR downlink control channel design should consider overlapping CCEs, at least for the URLLC.

# Summary

This contribution discussed the importance of lowering the user blocking probability for the URLLC usage scenario. Non-orthogonal control channel designs using overlapping NR-CCEs were introduced and an example was compared to the orthogonal control channel design, in terms of the user blocking probability. The simulation results show that using overlapping NR-CCEs can drastically reduce the probability of blocking.

Observation 1: Non-orthogonal DL control channel design, using overlapping CCEs, can drastically lower the probability of user blocking in NR-PDCCH.

Observation 2: Even lowering the blocking probability to 10-5 (comparable to the requirement for data error rate) is feasible for moderate URLLC packet arrival rates, if we use overlapping NR-CCEs.

Proposal 1: For URLLC, the probability of user blocking should be considered in the selection of NR-PDCCH design.

Proposal 2: Non-orthogonal DL control channel design using overlapping CCEs should be studied as a candidate for NR-PDCCH, At least for the URLLC.

# References

[1] RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #86bis

[2] R1-1609913, “On non-orthogonal control channel design for NR”, InterDigital Communications, RAN1#86bis, October, 2016.