# 3GPP TSG-RAN1#87	R1-1612787

# 14th – 18th Nov 2016

# Reno, US

Agenda Item:	6.2.9.4

Source: 	Ericsson

Title:  	On new DCI format for the support of two NPDSCH HARQ processes

Document for:	Discussion

# Introduction

One of the objectives of the Release 14 NB-IoT enhancements work item is to reduce power consumption and latency. In a separate contribution [4] we have discussed the benefits of doing so by introducing two HARQ processes and compared this with a straight forward increase of the NPDSCH TBS. Our analysis showed that further increased NPDSCH TBS has several advantages over the introduction of two HARQ processes. Based on this we proposed that RAN1 should further increase the NPDSCH TBS instead of pursuing the specification of two HARQ processes.

In case RAN1, despite our recommendation, decides to move forth with the specification of two HARQ processes this contribution outlines a proposal to introduce a new DCI format dedicated for the scheduling of two HARQ processes.

# Background

In Release 13 the DCI format N1 conveyed in the NPDCCH is used for the scheduling of NPDSCH. To understand the impact on the scheduling information from scheduling a second NPDSCH block using a second HARQ process it is important to understand the current functionality and thereto related NPDSCH F1 transmission characteristics.

To start with Figure 1 and Table 1 describes a Release 13 NPDSCH scheduling cycle with the associated delays presented.

Figure 1 NPDSCH scheduling cycle for 15 kHz subcarrier spacing illustrating the Rel-13 scheduling parameters.

Table 1 Parameters associated with a NB-IoT DL data connection, including parameters for configuration of the NPDCCH UE specific search space and DCI Format N1 under the assumption that all subframes are NB-IoT subframes.

| Parameter | Range | Comment |
| --- | --- | --- |
| G | 1.5, 2, 4, 8, 16, 32, 48 | NPDCCH UE specific search space parameter npdcch-StartSF-USS |
| Rmax | 1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048 rep | NPDCCH UE specific search space parameter npdcch-NumRepetitions |
|  | - | Gives the locations of the NPDCCH potential starting positions |
| T = Rmax∙G | 8,..., 98304 ms | Search space periodicity. The scheduling gap between last SF of NPDCCH n and first SF of NPDCCH n+1 limits the range to start from 8. |
| - | 4 ms | Minimum gap between last SF of last NPDCCH candidate in USS n and first SF of first NPDCCH candidate in USS n+1. |
| - | 4 + 0,4,8,12,16,32,64,128 ms | Scheduling gap between last NPDCCH SF and first NPDSCH SF. |
| - | 12, 14, 16, 17 ms | Scheduling gap between last NPDSCH SF and first NPUSCH F2 SF. |
| - | 3 ms | Minimum scheduling gap between last NPUSCH F2 SF and first NPDCCH SF |

The parameters related to the search space periodicity T are transmitted in RRC signalling, and the minimum gaps are fixed in the specifications. The start and length of the NPDSCH block is however dynamically configured using the DCI format N1 information elements summarized in Table 2.

Table 2 DCI format N1 for scheduling of the NPDSCH.

| IE | Size [bits] |
| --- | --- |
| Flag for format N0/format N1 differentiation | 1 |
| NPDCCH order indicator | 1 |
| NPDSCH Scheduling delay | 3 |
| Resource assignment () | 3 |
| Modulation and coding scheme () | 4 |
| Repetition number | 4 |
| New data indicator | 1 |
| HARQ-ACK resource | 4 |
| DCI subframe repetition number | 2 |
| Total: | 23 |

Among the above parameters it is the DCI subframe repetition number and the scheduling delay parameter that indicates the start of a NPDSCH transmission. The DCI subframe repetition number indicates the end of the NPDCCH transmission while the scheduling delay can be chosen from the set of values {0,4,8,12,16,32,64,128}+4 ms for 128 or less NPDSCH repetitions.

The transmission time of NPDSCH block is further impacted by the configured number of repetitions set by the Repetition number information element and the number of assigned resource units via the Resource assignment parameter. The Repetition number (NREP) is selected from the set {1, 2, 4, 8, 16, 32, 64, 128, 192, 256, 384, 512, 768, 1924, 1536, 2048} while Resource assignment indicates the transmission of 1, 2, 3, 4, 5, 6, 8 or 10 subframes (NSF).

For a given configuration the transmission time of a NPDSCH block is calculated as  ms.

In addition to this it was specified in Release 13 that when 32 or more repetitions are used a configurable transmission gap shall periodically be applied during which the NPDSCH transmission is postponed.

# DCI Format NX

## Introduction of a new DCI Format NX to support two HARQ processes

The Release 13 DCI formats are optimized  to schedule a single HARQ processes. Hence, to facilitate efficient scheduling of two NPDSCH HARQ processes it is beneficial to redefine the content of the DCI format scheduling the two processes.

According to Release 13 the NPDCCH, after encoding at the eNB or after decoding at the UE, the CRC attached to the DCI is scrambled by the C-RNTI. By scrambling the C-RNTI with a specified bit map, e.g. the all 1’s code word, the content of the DCI message can be completely redefined to facilitate scheduling of two HARQ processes. The UE will then need to blindly detect when the Release 13 single HARQ format or the Release 14 two HARQ format is used. This will allow the eNB to dynamically switch between the legacy Format and the new proposed Format NX for two HARQ processes. To ease the burden on the UE RRC signalling during connection setup can inform the UE to expect the possible presence of the new DCI format when decoding the User specific Search Space (USS).

The new format will handle the cases:

- NPDSCH for both processes contains new transfer blocks (TB) with new data.

- NPDSCH for both processes resends TBs, i.e. resends data.

- For Process 1 the NPDSCH contains a new TB. For Process 2 the NPDSCH resends a TB.

- For Process 1 the NPDSCH resends a TB. For Process 2 the NPDSCH contains a new TB.

- For Process 1 the NPDSCH resends a TB. For Process 2 the buffer is empty, and the NPDSCH is not sent.

- For Process 1 the buffer is empty, and the NPDSCH is not sent. For Process 2 the NPUSCH resends a TB.

The only remaining case, where only one of the processes has new data while for the other process the buffer is empty and there is no data to resend, is handled by the single HARQ DCI Format N1.

Proposal 1: To facilitate two HARQ processes the introduction of a new DCI Format is facilitated by scrambling the C-RNTI with a predefined bit pattern.

## New DCI format NX for two NPDSCH HARQ processes

Table 3 summarizes the suggested definition of the new DCI format NX. The existing DCI Format N1 is also presented for reference. By securing that also the new format is limited to 23 bits, the same encoding, scheduling and link adaptation can be used as in Release 13, which is of considerable value.

The main changes are:

- The new data indicator is expanded to handle the 8 use-cases described in section 3.1.

- A new scheduling delay indicator is proposed to facilitate scheduling of a second UE in the gap between the two NPDSCH blocks. These values depend on the outcome of the discussion in [2][3].

- The NPDCCH order indicator is not needed and removed.

- The meaning of the resource assignment IE is adapted to the scheduling of two HARQ processes.

- The maximum number of NPDSCH repetitions is reduced to 8.

- The HARQ-ACK resource points at a bundled Ack/Nack as proposed in [5].

Table 3 Modification of DCI format N0 to support two HARQ processes.

| Rel-13 DCI format N1 |  | Rel-14 DCI format NX to cater for 2 HARQ NPDSCH |  |  |
| --- | --- | --- | --- | --- |
| IE | Size [bits] | IE | Size [bits] | Comment |
| Flag for format N0/format N1 differentiation | 1 | Flag for format N0/format N1 differentiation | 1 | Flag for Rel-14 NPUSCH/NDSCH DCI formats for 2 HARQ |
| NPDCCH order indicator | 1 | NPDCCH order indicator | 0 | Not used |
| NPDSCH Scheduling delay | 3 | NPDSCH Scheduling delay | 3 | Unmodified |
|  |  | NPDSCH Block 2 Scheduling delay relative block 1 | 1 | 8, 10 ms to allow for scheduling of a second UE in the gap. |
| Resource assignment () | 3 | Resource assignment | 3 | {ISF block 1, ISF block 2} = {10,10}, {10,8}, {10,6}, {10,5}, {10,4}, {10,3}, {10,2}, {10,1}. |
| Modulation and coding scheme () | 4 | Modulation and coding scheme | 4 | Unmodified, as it make sense to use same code rate for both blocks |
| Repetition number | 4 | Repetition number | 2 | It is sufficient to support 1, 2, 4, 8 repetitions. |
| New data indicator | 1 | New data indicator | 3 | New data indicator for two HARQ processes. - 000: Both processes new data - 001: Both processes resend data - 010: Process 1 new data Process 2 resend data - 011: Process 1 resend data Process 2 new data - 100: Process 1 resend data Process 2 no data - 011: Process 1 no data Process 2 resend data |
| HARQ-ACK resource | 4 | HARQ-ACK resource | 4 | Unmodified but schedules a joint bundled Ack/Nack [5]. |
| DCI subframe repetition number | 2 | DCI subframe repetition number | 2 | Unmodified |
| Total: | 23 | Total: | 23 |  |

Proposal 2: To support two HARQ processes a new DCI format is specified in accordance with Table 3.

# Summary

In this contribution we provide an overview of the Release 13 DCI Format N1 specification and proposes to introduce a new DCI format dedicated to dynamically schedule two HARQ processes :

Proposal 1: To facilitate two HARQ processes the introduction of a new DCI Format is facilitated by scrambling the C-RNTI with a predefined bit pattern.

Proposal 2: To support two HARQ processes a new DCI format is specified in accordance with Table 3.

# References

- RP-161901, “Revised work item proposal: Enhancements of NB-IoT,” RAN#73, source Huawei, HiSilicon, 19 – 22 September, 2016.

- R1-1610994, “WF on timing relationships of 2 HARQ processes for NB-IoT enhancement”, RAN1#86bis, source Huawei et al., 19 – 22 September, 2016.

- R1-1611019, “WF on timing relationships of 2 HARQ processes for Rel-14 NB-IoT”, RAN1#86bis, source Mediatek et al., 19 – 22 September, 2016.

- R1-1611127, “Power consumption and latency reduction by increasing max NPDSCH TBS or using two HARQ processes”, RAN1#87, source Ericsson

- R1-1612789, “On new NPUSCH Format 2 for supporting the bundling of ACKNACK of two NPDSCH HARQ processes”, RAN1#87, source Ericsson