TSG-RAN WG1 #87	R1-1612915

Reno, US, November 14– 18, 2016

Source:	Ericsson

Title:	On link-budget of NR PDCCH

Agenda Item:	7.1.4.1

Document for:	Discussion and Decision

# Introduction

In [1], we presented some high-level design principles of downlink control channel (PDCCH) for NR. Based on these principles, following agreements were made in RAN1#86bis regarding the NR-PDCCH design:

Agreements:

- From gNB perspective, DL control signaling can be located at the first OFDM symbol(s) in a slot and/or mini-slot

- FFS: From gNB perspective, DL control signaling can be located over the slot and/or mini-slot

Agreements:

- NR should support at least the following.

  - In frequency-domain, a PRB (or a multiple of PRBs) is the resource unit size (may or may not including DM-RS) for control channel

    - This is at least for the case where the DL control region consists of one or a few OFDM symbol(s) of a slot or a mini-slot

    - FFS: whether a PRB or a multiple PRBs is the resource unit size

      - FFS: If multiple PRBs is the resource unit size, the multiple PRBs are contiguous

  - FFS: whether the resource unit size for a DL control channel is called as NR-REG or not

Agreements:

- NR should support at least the following

  - A DL control channel can be mapped on one or more NR-CCEs

    - This is at least for the case where the DL control region consists of one or a few OFDM symbol(s) of a slot or a mini-slot

    - A NR-CCE includes a positive integer number of PRBs (FFS: exact value)

      - FFS: whether a NR-CCE contains contiguous PRBs

      - FFS: whether multiple NR-CCEs may share one or more PRBs

  - FFS: whether NR-CCE is mapped on frequency-domain only or on both frequency and time-domain.

In this paper, we analyze the coverage of NR PDCCH channel occupying only one OFDM symbol as compared to multi-symbol PDCCH for two different DCI sizes (24 bits + 64 bits) using localized mapping on frequency domain (i.e. NR-CCE contains contiguous PRBs).

# Discussion

## Evaluation Assumptions

Table 1 summarizes the simulation assumptions used.

| Parameter | Value |
| --- | --- |
| Numerology | 15kHz |
| FFT size | 2048 |
| Resource element group (REG) size | 12 subcarriers [2] |
| Modulation scheme | QPSK |
| Channel codec | LTE TBCC |
| PDCCH payload sizes | 24 bits and 64 bits |
| CRC size | 16 bits |
| Antenna ports | Tx: 2 Rx: 2 |
| Transmit diversity scheme | SFBC |
| RS overhead per PDCCH | 50% (3 RS per antenna per RB, every 4th subcarrier) |
| PDCCH frequency mapping | Fully localized (Contiguous PRB & CCE) |
| Aggregation level (AL) | 2 |
| Channel model | TDL-A with 100ns and 1000ns RMS delay spreads |

## Performance Results

Figure 1 and Figure 2 show the BLER performance of PDCCH in terms of SNR per subcarrier with 24bits and 64bits DCI payload sizes. ‘Red curves’ show the performance of multi-symbol PDCCH whereas the ‘orange curves’ show the performance of single-symbol PDCCH. We can observe that the link performance remains similar for either spreading PDCCH in time domain or frequency domain. As discussed in [3], choosing one OFDM symbol long PDCCH channel for NR decrease the scheduling complexity for semi-statically configured duration of downlink control search space.

- PDCCH payload size = 24 bits

- PDCCH payload size = 64 bits

Figure 1: PDCCH link performance with TDL-A channel with 100ns RMS delay spread

- PDCCH payload size = 24 bits

- PDCCH payload size = 64 bits

Figure 2: PDCCH link performance with TDL-A channel with 1000ns RMS delay spread

Observations:

- The link performance of both time spreading and frequency spreading is similar for a DCI size and a code rate.

# Conclusion

Aggregation in time is equivalent to aggregation in frequency assuming a constant power-spectral density.

# References

[1]      R1-1609619, “On downlink control channels”, TSG-RAN WG1#86bis, Portugal, October 10-14, 2016.

[2]      TSG-RAN WG1#86bis, Chairman’s notes.

[3]      R1-1612912, “On downlink control channel duration”, TSG-RAN WG1#87, US, November 14-18, 2016.