3GPP TSG RAN WG1 Meeting #87	 	R1-1612966

Reno, Nevada, USA, 14th - 18th November 2016

Source: 	Ericsson

Title:	SLSS/PSBCH and Transmitter Behavior under Congestion Control Restrictions

Agenda Item:	6.2.1.4.2

Document for:	Discussion and Decision

# Background

Congestion control prevents the radio channel from being congested by too many transmissions within a certain geographical area. This is achieved by placing limits on several transmitter parameters based on channel occupancy level at the moment. For example, draft ETSI EN 302 571 V2.0.0 (2016-03) [1] [2] for the 5.9GHz ITS band in Europe specifies regulatory limits on maximum Ton, minimum Toff and maximum Duty Cycle of UE transmitters based on the channel occupancy level measured as Channel Busy Ratio (CBR), where

- Ton is the current duration of a transmission,

- Toff is the current allowed minimum time between two consecutive transmissions,

- Duty Cycle is the ratio expressed as a percentage of the maximum transmitter “on” time on one carrier frequency, relative to 1 second. [2]

Even though it is not completely clear how DCC for LTE will be defined, it is already clear that LTE will need to support a congestion control mechanism in order to comply with European regulations. Such congestion control may include some parameters similar to the Ton, Toff and Duty Cycle discussed above.

During RAN1#86bis the following was additionally agreed:

Agreements:

- If the vehicle UE has selected the GNSS as syncRef,

  - For the OOC case:

    - The UE transmits SLSS and PSBCH every synchronization period when the SLSS resource is pre-configured and the UE is capable of SLSS/PSBCH transmission;

      - FFS whether any SLSS/PSBCH dropping behaviour is specified, e.g., for congestion control.

  - For the InC case:

    - Rel-12 D2D mechanism is reused, i.e.:

      - RSRP threshold or dedicated signaling to determine the SLSS transmission

  - When GNSS is the highest priority, the UE is not expected to read SLSS from other UEs.

- UE capability of SLSS transmission/reception will be discussed later.

In this contribution we discuss the impact of minimum Toff limit and maximum duty cycle limit on UE transmitter behavior and develop solutions from RAN1 perspective for fulfilling the corresponding requirements.

# Impact from regulatory Toff and duty cycle requirements on UE transmitter behavior

According to [2], (see also Appendix A,) the minimum Toff limit is 25 ms or higher and can be up to 1 s, e.g. when CBR > 62%.

Observations:

- According to draft ETSI EN 302 571 V2.0.0 (2016-03) [1] [2], the minimum Toff limit is 25ms or higher and can be up to 1 second depending on the current channel occupancy level.

As a radio layer requirement, the Toff limit is applicable to all transmissions of a UE operating in the ITS band. These include SA/Data and signaling transmissions, e.g. SLSS/PSBCH. The time interval between consecutive messages arriving from higher layers at the PDCP layer of a UE can be controlled by the higher layer congestion control function, as outlined in the companion paper [3]. However, in order to comply with the radio layer Toff requirement, which does not differentiate the type of transmissions, UE has also to coordinate transmissions of SA/Data and some signaling (SLSS/PSBCH) from RAN1 perspective.

Proposals:

- In order to comply with the radio layer Toff requirement, which does not differentiate the type of transmissions, UE has to coordinate transmissions of SA/Data and some signaling (SLSS/PSBCH) from RAN1 perspective.

## SLSS/PSBCH transmission:

The V2V SLSS/PSBCH transmission has a periodicity of 160ms. If the time between a data transmission and a SLSS/SPBCH transmission is too close, it may violate the minimum Toff limit. Due to the repetitiveness of the signaling transmissions, a transmitter UE may at times consider SLSS/PSBCH as of less importance than a data transmission. Hence, the UE shall be able to prioritize the data transmission over SLSS/PSBCH transmissions, in order to meet the Toff and duty cycle limits. Particularly, the following cases are considered:

1. If an upcoming SLSS/PSBCH transmission is going to occur within the Toff limit its data transmission, the UE shall skip the SLSS/PSBCH transmission.

2. If a data transmission is scheduled within the Toff limit before an upcoming SLSS/PSBCH transmission, the UE shall skip the SLSS/PSBCH transmission.

3. Before each SLSS/PSBCH transmission, the UE shall evaluate the duty cycle in the past 1 second taking into account the upcoming SLSS/PSBCH transmission. If the evaluation result exceeds the maximum duty cycle limit, the UE shall skip the SLSS/PSBCH transmission.

Figure 1 shows the cancellation of SLSS/PSBCH transmission due to the minimum Toff limit.

Figure 1: Prioritized data transmissions over SLSS/PSBCH transmissions within Toff limit

Proposals:

- The specifications should allow the UE to prioritize data transmissions over SLSS/PSBCH transmissions in order to meet the Toff and duty cycle limits, or in general any DCC-related requirement. More specifically,

  - if an upcoming SLSS/PSBCH transmission is going to occur within the Toff limit before a scheduled or reserved data transmission, the UE shall skip the SLSS/PSBCH transmission.

  - if a data transmission is scheduled or performed within the Toff limit before an upcoming SLSS/PSBCH transmission, the UE shall skip the SLSS/PSBCH transmission.

  - before each SLSS/PSBCH transmission, the UE shall evaluate the duty cycle in the past 1 second taking into account the upcoming SLSS/PSBCH transmission. If the evaluation result exceeds the maximum duty cycle limit, the UE shall skip the SLSS/PSBCH transmission.

# Conclusion

In this contribution, we discuss the UE transmitter behavior under congestion control in V2X sidelink.

Observations:

- According to draft ETSI EN 302 571 V2.0.0 (2016-03) [1] [2], the minimum Toff limit is 25ms or higher and can be up to 1 second depending on the current channel occupancy level.

Proposals:

- In order to comply with the radio layer Toff requirement, which does not differentiate the type of transmissions, UE has to coordinate transmissions of SA/Data and some signaling (SLSS/PSBCH) from RAN1 perspective.

Proposals:

- The specifications should allow the UE to prioritize data transmissions over SLSS/PSBCH transmissions in order to meet the Toff and duty cycle limits, or in general any DCC-related requirement. More specifically,

  - if an upcoming SLSS/PSBCH transmission is going to occur within the Toff limit before a scheduled or reserved data transmission, the UE shall skip the SLSS/PSBCH transmission.

  - if a data transmission is scheduled or performed within the Toff limit before an upcoming SLSS/PSBCH transmission, the UE shall skip the SLSS/PSBCH transmission.

  - before each SLSS/PSBCH transmission, the UE shall evaluate the duty cycle in the past 1 second taking into account the upcoming SLSS/PSBCH transmission. If the evaluation result exceeds the maximum duty cycle limit, the UE shall skip the SLSS/PSBCH transmission.

# Reference

- Draft ETSI EN 302 571 V2.0.0 (2016-03), Radiocommunications equipment operating in the 5 855 MHz to 5 925 MHz frequency band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU

- Revision proposal of Clause 4.2.10 of Draft ETSI EN 302 571 V2.0.0 (2016-03), URL: https://docbox.etsi.org/ERM/ERMTG37/05-CONTRIBUTIONS/2016//ERMTG37(16)000039_Revised_Clause_4_2_10_with_track_changes.docx (last accessed on 07 Aug. 2016)

- R1-1612934, Congestion Control Framework for V2V, Ericsson

- R1-1612951, Distributed Synchronization Procedure for V2X over PC5, Ericsson

# Appendix A

In the clause 4.2.10 on decentralized congestion control of [1][2] the following limits are specified:

The equipment testing for DCC conformance shall focus on the Ton, Toff and the duty cycle parameters where 0 < Ton ≤ Ton_max, Toff_min ≤ Toff ≤ 1000 and the values in Table 11 shall not be exceeded.

Table 11: Limits for the maximum utilization of a single channel by the equipment

| Parameters | Value |  |
| --- | --- | --- |
| Ton_max | 4 ms |  |
| Toff_min | 25 ms |  |
| duty cycle | 3 % |  |

When the CBR is ≤ 62%, Toff is equal to Toff_min. When the CBR is > 62 %, Toff time shall be determined with Equation 2.

Toff=Ton×4000×CBR-0.62CBR-1,		(2)

NOTE: The rationale behind Equation 2 is outlined in TS 103 175 [6], Equation 1 in Clause 7.2.