3GPP TSG RAN WG1 Meeting #87					R1-1613033

Reno, USA, 14th - 18th November 2016

Agenda Item:	7.1.5.1

Source: 	LG Electronics

Title: 	Channel coding scheme selection for eMBB

Document for:	Discussion and decision

- Introduction

In last RAN1#86bis meeting, there was discussion on channel coding selection of eMBB scenario and following conclusion was made [1].

- The channel coding scheme for eMBB data is LDPC, at least for information block size > X

- FFS until RAN1#87 one of Polar, LDPC, Turbo is supported for information block size of eMBB data <= X

  - The selection will focus on all categories of observation, including overall implementation complexity, regardless of the number of coding schemes in the resulting solution (except if other factors are generally roughly equal)

- The value of X is FFS until RAN1#87, 128 <= X <= 1024 bits, taking complexity into account

- The channel coding scheme(s) for URLLC, mMTC and control channels are FFS

In this contribution, we discuss channel coding selection for eMBB scenario.

Evaluation results of channel coding scheme for information block size < X

In this section, we show the performance evaluation results of channel coding scheme for several information block sizes less than X. Since 128<=X<=1024, we evaluate information block size of around 200 and 600. The evaluation assumptions are summarized in Table 1. We evaluate turbo code, LDPC, and polar code. For turbo code, we assume the turbo code in LTE specification [2]. For LDPC, we assume the LDPC proposed in [3]. For polar code, we assume the original polar code with frozen bit selection in [4] and QUP for rate matching.

Table 1. Evaluation assumptions for channel coding scheme evaluation

| Information block sizes (K) |  | 200, ~600 |
| --- | --- | --- |
| Modulation |  | QPSK |
| Coding rate |  | 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |
| Decoding algorithm | Turbo code | max-log-map with scaling factor 0.75, 8 iterations |
|  | LDPC | offset min-sum, 30 iterations, flooding |
|  | Polar code | CRC-aided list decoding with list size 8 |

We compare the performance of channel coding schemes in Figure 1.

(a) K= 200

- K= ~600

(c) K=~10000

As shown in Figure 1, For K=200, the turbo code shows the best performance in lower coding rate and comparable performance to LDPC and polar code in higher coding rate. The performance gain of turbo code in lower coding rate is up to 0.4 dB at a BLER of 0.1. For K=~600, the turbo code shows the best performance in lower coding rate and LDPC shows the best performance in higher coding rate. The performance gain of turbo code in lower coding rate is up to 0.3 dB at a BLER of 0.1. For K=~1000, LDPC and turbo shows comparable performance in lower coding rate and LDPC shows the best performance in higher coding rate. The performance gain of LDPC in higher coding rate is up to 0.2 dB.

Observation 1: For K=200, the turbo code shows the best performance in lower coding rate and comparable performance to LDPC and polar code in higher coding rate.

Observation 2: For K=~600, the turbo code shows the best performance in lower coding rate and LDPC shows the best performance in higher coding rate.

Observation 3: For K=~1000, LDPC and the turbo code show comparable performance in lower coding rate and LDPC shows the best performance in higher coding rate.

Turbo code may be beneficial if we consider NR UE supporting multiple standards. Current modem solution in UE (e.g., smart phone) supports multiple standards (e.g., GSM/WCDMA/LTE). When NR network would deploy in near future, it is highly probable that NR UE will also support multiple standards. Depending on the implementation, the legacy turbo decoder can be shared with NR turbo decoder, which means that complexity increase may be very small. Turbo code may be also beneficial considering operating mode in NR: non-standalone and standalone modes. Assuming NR UE supporting non-standalone mode, NR UE should have functionality to communicate with LTE protocol, which means that turbo code should be supported in non-standalone mode NR UE. Non-standalone mode can be interpreted as a special case of multiple standards.

Observation 4: Depending on the implementation of NR UE, complexity increase by turbo code support may be very small by sharing legacy turbo decoder.

Based on the observations, we propose that turbo code should be supported for information block size less than 600 bits.

Proposal 1: Turbo code should be supported for information block size less than 600 bits.

Conclusions

In this contribution, we discuss channel coding selection of NR eMBB scenario. We have following observations and proposal:

Observation 1: For K=200, the turbo code shows the best performance in lower coding rate and comparable performance to LDPC and polar code in higher coding rate.

Observation 2: For K=~600, the turbo code shows the best performance in lower coding rate and LDPC shows the best performance in higher coding rate.

Observation 3: For K=~1000, LDPC and the turbo code show comparable performance in lower coding rate and LDPC shows the best performance in higher coding rate.

Observation 4: Depending on the implementation of NR UE, complexity increase by turbo code support may be very small by sharing legacy turbo decoder.

Proposal 1: Turbo code should be supported for information block size less than 600 bits.

Reference

- [1] RAN1#86bis Chairman’s note.

- [2] TS 36.212 v13.2.0, “Multiplexing and channel coding (Release 13)”.

- [3] R1-161037, “LDPC rate compatible design overview,” Qualcomm, RAN1#86bis.

- [4] L, Zhang, Z. Zhang, X. Wang, Q, Yu, and Y. Chen, “On the puncturing patterns for punctured polar codes,” ISIT2014.