3GPP TSG-RAN1 Meeting #87	R1-1613004

# Reno, Nevada, United States, 14th Nov 2016 - 18th Nov 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0318 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on narrowband physical-layer cell identities definition in 36.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  | Date: |  |  | 2016-11-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To capature the following RAN1 agreement - Note that the PCID from NB-SSS and the LTE PCID indicate the same LTE CRS position |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding statement to capture the above mentioned RAN1 agreement, when operationModeInfo indicates inband-SamePCI or inband-DifferentPCI for a cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eNB may choose a improper narrowband cell ID that conflicts the LTE CRS position |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 10.2.7	Synchronization signals

There are 504 unique narrowband physical-layer cell identities indicated by the narrowband secondary synchronization signal.

If the higher layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI or guardband for a cell, the narrowband physical-layer cell identities shall be choosen which satisfies

#### < Unchanged parts are omitted >