3GPP TSG-RAN WG1 #87	R1-1612666

Reno, USA 14th - 18th November 2016

Source:	Ericsson

Title:	Support of Advanced CSI for CLASS B

Agenda Item:	6.2.2.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1#86, the following was agreed regarding advanced CSI reporting:

- Specify CSI feedback enhancement with the following advanced CSI feedback framework:

  - Reduced space (eigenvectors)/W1 is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Orthogonal basis (e.g. orthogonal DFT matrix)

    - Alt2. Non-orthogonal basis (e.g. Rel.13 Class A W1 for rank-1 and/or 2)

  - Reduced space representation/W2 is to further combine selected beams

  - Granularity of weighting(phase and/or amplitude) can be either wideband only or wideband/subband, and is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Phase and amplitude

    - Alt2. Phase-only weighting

  - How the enhanced framework can be applicable for Class A and/or Class B eMIMO-Types is FFS

  - FFS: How to handle the relationship between advanced CSI feedback and legacy CSI feedback framework

- Companies are encouraged to provide results comparing the above alternatives, considering a mix of smaller and larger numbers of ports within the following antenna port configurations

  - {4,8,12,16,20,24,28,32} ports

  - Focus on rank<=2 scenario MU-MIMO for evaluation

  - Feedback overhead needs to be taken into account

  - For {4,8,12,16, 20,24,28,32}-port scenario, companies are encouraged to compare their proposals to dual-stage codebook enhancement with increased number of beams in W1

As stated in the highlighted sentence in the agreement above, it was left for further study whether the enhanced CSI framework is applicable to both Class A and B, or, only Class A.  In this contribution, we discuss the applicability of advanced CSI for Class B type of reporting using beamformed CSI-RS.

# Use case for Class B Advanced CSI reporting

In the discussion around advanced CSI feedback in RAN1, most companies have assumed Class A type of behaviour with 16-32 non-precoded antenna ports and have designed their proposals with this setup in mind. However, there have been some proposals for advanced CSI feedback enhancements for Class B operation with beamformed CSI-RS as well. While the benefit of Class A advanced CSI reporting over Class A regular CSI reporting has been proven by a plurality of companies during the latest RAN1 meetings, the benefit of Class B advanced CSI reporting is less clear. And as specification of advanced CSI reporting for Class A has already been agreed to, introducing advanced CSI reporting also for Class B would have to be motivated by some additional benefit over Class A advanced CSI reporting.

Advanced CSI reporting for Class B should be motivated by some additional benefit over Class A advanced CSI reporting

The benefit over Class A does not necessarily have to be a benefit in system performance though, CSI enhancements for Class B could also be motivated by reduction in UE complexity compared to Class A operation, or, increased eNB flexibility. Although motivating enhancements based on such merits may require extra justification.

It is our view that the primary use case for advanced Class B CSI reporting is when it is combined with advanced Class A CSI reporting in a classic hybrid Class A/B fashion where non-precoded CSI-RS are transmitted at a larger periodicity and the reported W1 is used to determine the beamforming on the Class B CSI-RS. Thus, advanced CSI reporting for Class B should be tailored towards hybrid Class A/B operation, meaning that there should be a commonality between the Class A and Class B advanced CSI codebooks. A commonality between Class A/B codebook designs is further motivated by the few remaining RAN1 meetings for Rel. 14, there may not be enough time to agree on and specify two separate codebook designs.  We therefore make the following proposal:

Advanced Class B CSI reporting enhancements should be tailored towards hybrid Class A/B operation. There should be a commonality between Class A and Class B advanced CSI codebooks

# Class B Advanced CSI Codebook Design

In a contribution for RAN1#86bis [1] we presented a design for Class A advanced CSI codebook, summarized below for convenience.

We propose that an advanced CSI codebook for Class A should keep the familiar factorized codebook structure, where the precoding matrices W are decomposed into a wideband W1 matrix factor and subband W2 matrix factor as

W=W1W2.

The W1 codebook should comprise the following:

- Beam selection: Provide an unconstrained selection of Nbeams orthogonal (2D) DFT beams from a (rotated) DFT basis, to ensure that the selected beams correspond to the multi-path components of the channel.

- Beam power allocation: Setting the relative power level of the beams in the precoder so that the relative strength of the different multi-path components of the channel are taken into account.

The W1 matrix may thus be expressed as a beam selection matrix  BIS and a power scaling matrix P as

W1= BISP.

The W2 codebook should comprise co-phasing/combination between different beams on the same polarization and between different polarizations of the same beam. An enhanced W2 matrix may then be expressed as

W2=1ejα21⋮ejα2Nbeams1………1ejα2r⋮ejα2Nbeamsr,

where r is the transmission rank, Nbeams is the number of DFT beams in W1 per polarization and ejαkl is a co-phasing factor from a PSK alphabet intended for the k:th column of W1 and the l:th layer.

In hybrid Class A/B operation, the W1 matrix, W1= BISP, will be fed back in the Class A report and will be used to determine the precoding for the Class B CSI-RS.  For the Class B codebook design, we see the following alternatives:

- Use a dual-stage codebook structure with a wideband W1 and a subband W2, where a W1 matrix W1(B) consists of a power scaling matrix and a W2 matrix  W2(B) is identical to the Class A advanced CSI codebook. That is, the precoders in the Class B advanced CSI codebook may be expressed as

W(B)=W1(B)  W2(B)=PW2,

Where W1B=P,  W2B=W2,  and  P is the same power scaling matrix that is a W1-component in the Class A advanced CSI W1 codebook and W2 is the same as the Class A advanced CSI W2 matrix.

If the Class B advanced CSI codebook is structured in this way, the UE applies a wideband per-port power scaling (which translates to a beam power scaling) with the matrix P and linearly combines the ports into layers with the matrix W2. To create the beamformed Class B antenna ports, the eNB does not have to apply a power scaling P directly on the antenna ports, as would have been the case if the Class A W1matrix ,  W1= BISP,  would have been used directly to beamform the Class B antenna ports.

- Use a single-stage codebook and re-use the Class A W2 codebook directly, i.e.

W(B)=W2(B)=W2,

where W2 again is from the advanced Class A W2 codebook. Since a per-port power scaling is      not done by the UE with this codebook alternative, the beam power scaling must be accounted for by another mechanism (as observed in [1], using phase only beam combining does not result in good performance). The following options can be considered:

  - The eNB can include the power scaling in the CSI-RS beamforming by setting a different power level on the different CSI-RS ports. This could lead to poorer channel estimation performance on the weaker beams though, which may not be desirable.

  - The UE should assume the power scaling reported as part of the W1 report in the previous Class A report should be applied on top of Class B CSI-RS ports before applying Class B codebook, so that the beam power selection is semi-persistent. This option may not be desirable either since it creates an inter-dependency between the Class A and Class B CSI reporting, essentially limiting the eNB to directly following the W1 report by the UE. Since eNB flexibility is one of the arguments for introducing advanced CSI reporting for Class B, this speaks against this option.

  - The eNB signals the power scaling as part of the CSI request.  This option may not be desirable either due to the additional signaling overhead.

Since the additional overhead of alternative 1 over alternative 2 is small, we propose that alternative 1 should be adopted as the Class B advanced CSI codebook:

For Class B advanced CSI reporting, use a dual-stage codebook structure with a wideband W1 and a subband W2, where the W1 matrix consists of a power scaling matrix and the W2 matrix is identical to the Class A advanced CSI W2 matrix

# Evaluation results

In this section, we present evaluation results comparing Class A versus Hybrid Class A/B operation with the proposed advanced CSI codebooks. The simulations are performed using an 8x4 antenna array with 2x1 virtualization in the 3GPP 3D-UMi scenario. All systems perform MU-MIMO using SLNR precoding. For the systems with advanced CSI enhancements, the proposed Class A and Class B multi-beam codebooks are used, with 2 or 4 beams. For the hybrid systems, an 80ms periodicity is used for the Class A CSI-RS. The baseline system uses an extended Rel. 13 codebook with Config 2. Remaining simulation parameters are captured in the Appendix.

The results are presented Figure 1 below. When a multi-beam codebook consisting of 2 orthogonal DFT beams are used, the Hybrid Class A/B system performs slightly better than the Class A system and a cell edge gain of 7% is seen. However, when the multi-beam codebook consists of 4 orthogonal DFT beams, a cell edge loss of 9% is observed for the corresponding Hybrid Class A/B system. This makes sense, since each UE require 8 CSI-RS ports in this case, making the overhead reduction offered by hybrid CSI-RS smaller. Further, the more beams that are added to the precoder, the less certain it is that the beams selected in the Class A report will stay optimal during the entire 80ms interval until the next Class A report, which also may explain the losses observed for the hybrid system compared to the Class A system.

Based on these results, we observe that Hybrid Class A/B operation with advanced CSI may or may not be beneficial compared to standalone Class A operation, depending on the codebook configuration used. This further motivates the design criterion to reuse as much as possible of the Class A advanced codebook design for the Class B advanced CSI codebook.

Figure 1: Comparison between Class A and Hybrid Class A/B advanced CSI reporting

Hybrid Class A/B advanced CSI operation may or may not perform better than standalone Class A, depending on the codebook configuration

# Conclusions

In this contribution, we have discussed advanced CSI reporting for Class B eMIMO-Type and have made the following observations:

Observation 1	Advanced CSI reporting for Class B should be motivated by some additional benefit over Class A advanced CSI reporting

Observation 2	Hybrid Class A/B advanced CSI operation may or may not perform better than standalone Class A, depending on the codebook configuration

Based on these observations, we have made the following proposals:

Proposal 1	Advanced Class B CSI reporting enhancements should be tailored towards hybrid Class A/B operation. There should be a commonality between Class A and Class B advanced CSI codebooks

Proposal 2	For Class B advanced CSI reporting, use a dual-stage codebook structure with a wideband W1 and a subband W2, where the W1 matrix consists of a power scaling matrix and the W2 matrix is identical to the Class A advanced CSI W2 matrix

# References

R1-1609854, “Orthogonal vs. non-orthogonal basis for advanced CSI”, Ericsson, RAN1#86bis Lisbon

R1-1609855, “Phase and amplitude vs. phase only beam combining for advanced CSI reporting”, Ericsson, RAN1#86bis Lisbon

# Appendix

## Simulation parameters

| Simulation Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD |
| Antenna Configurations | 8x4 with 2x1 virt., UMi (130° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Advanced CSI codebook (when used) | Number of beams: 2 or 4 Beam space rotation hypotheses per dimension: 4 Beam power: 4 states Co-phasing: 8-PSK |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency Max 18 MU layers |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead accounted for. Channel estimation error modeled. |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |