3GPP TSG RAN WG1 Meeting #88                                      R1-1701595

Athens, Greece 13th - 17th February 2017

Source:	ZTE, ZTE Microelectronics

Title:	Analysis of URLLC reliability for HARQ

Agenda Item:	8.1.3.4.4

Document for:	Discussion & Decision

Introduction

In RAN1 NR_AH meeting, the following agreements on NR-PDCCH design for URLLC were made [1]:

Agreements:

- To ensure the reliability requirement of NR-PDCCH for URLLC, at least the following aspects should be supported

  - Defining a compact DCI format  targeting low BLER operation

  - The highest aggregation level should target a BLER of Y for this compact DCI format

    - FFS  Y, Y<1%

    - FFS highest  aggregation levels, e.g., 16,32

  - FFS other enhancements

Ultra reliable and low latency communication (URLLC) is identified as one of the usage scenarios in NR study item. In this contribution, it is discussed how to achieve reliability requirement of 99.999% within user plane latency of 1ms.

Analysis on reliability of URLLC in DL

In RAN1#87 meeting, it has been agreed that asynchronous and adaptive HARQ is supported on DL to meet URLLC requirements. Then the analysis in this contribution assumes at least 2 transmissions (including initial transmission and one retransmission) are supported to reach URLLC requirements on DL.

In order to achieve 99.999% reliability target within 1ms, the number of retransmission would be restricted by length of time interval and RTT (Round-Trip Time). Take total of 2 transmissions as an example, time interval is 0.25ms and mini-slot/slot with 7 symbols at 30 kHz SCS is used, in addition RTT is n+2 mini-slot/slot. Similar for maximum 4 transmissions with same RTT, mini-slot/slot with 7 symbols at 60 kHz SCS would be used.

Maximum 2 transmissions

Different HARQ retransmission times need different reliability requirements on each channel for one packet to be transmitted completely. Take one HARQ process of 2 transmissions as example, the probability for successful transmission within two transmissions is given by

P=PcPd1+1-PcPDTXPcPd1+Pc1-Pd1PNPcPd2     (1)

where

- The first term is the probability that the initial transmission is successfully received by UE. Pcis probability of successful PDCCH transmission and Pd1 is probability of successful transmission of  single data transmission without any HARQ combining at receiver.

- The second term is the probability that the second transmission is successfully received by UE, given the UE fails to detect the PDCCH for the initial transmission. Here PDTX=Prob{DTX or NACK is detected | UE sends DTX}.

- The third term is the probability that the second transmission is successfully received by UE, given the UE successfully detects the PDCCH for the initial transmission but fails to decode PDSCH for the initial transmission. Here PN=Prob{DTX or NACK is detected | UE sends NACK} and Pd2 is probability of successful decoding PDSCH with the re-transmission.

Firstly, it is easy to prove that  ∂P∂Pd1≥0, ∂P∂Pd2≥0, ∂P∂PDTX≥0 and  ∂P∂PN≥0 for the concerned ranges of these parameters. Then the largest  Pd1, Pd2, PDTX and PN (which are all 1’s) push the function value P upwards to P=2Pc-Pc2, which is just 99.99% and lower than the desired target of 99.999% with the current PDCCH reliability of Pc=99%. In fact, any value of Pc that is smaller than 1-1-PT=0.9968377 would have no chance to make P to reach PT=99.999%. So, at least the requirement on BLER of Y=0.1% should be targeted for the highest aggregation level with the compact DCI format of URLLC considering maximum 2 transmissions.

Secondly, for a given PDCCH reliability Pc,

- Because Pd1, Pd2 and PN are no larger than 1 in (1), P≤Pc+1-PcPcPDTX. Then any value of PDTX that is smaller than PT-Pc1-PcPc would not be able to make P≥PT. For example, PDTX≤0.99099 with Pc=0.999 or PDTX≤0.9001 with Pc=0.9999 cannot achieve P≥99.999%.

- Because PDTX, Pd2 and PN are no larger than 1 in (1), P≤PcPd1+1-PcPcPd1+Pc1-Pd1Pc. Then any value of Pd1 that is smaller than PT-Pc221-PcPc would not be able to make P≥PT. For example, Pd1≤0.9955 with Pc=0.999 or Pd1≤0.95 with Pc=0.9999 cannot achieve P≥99.999%.

- Because generally Pd1≤Pd2 in (1), P≤PcPd2+1-PcPcPd2+Pc1-Pd2PcPd2=2PcPd2-Pc2Pd22. It can be shown that any value of Pd2 that is smaller than 1-1-PTPc cannot make P≥PT. For example, Pd2≤0.9978 with Pc=0.999 or Pd2≤0.9969 with Pc=0.9999 cannot achieve P≥99.999%.

- It is often the case that PDTX≤PN. This is because PDTX and PNhave the same detection area in signal space while NACK has its signal constellation usually more far away from ACK than DTX. Then P≤PcPd1+1-PcPNPcPd1+Pc1-Pd1PNPc, which means any value of PN that is smaller than PT-PcPd1PcPc+Pd1-2PcPd1 cannot make P≥PT.

Observation 1: For a DL HARQ process of maximum 2 transmissions, its reliability target PT (overall success probability) is NOT achievable if any of following conditions is true:

- Pc≤1-1-PT, where Pc is the success probability of DL scheduling detection.

- Pd1≤PT-Pc221-PcPc, where Pd1 is the success probability of first detection of data packet.

- Pd2≤1-1-PTPc, where Pd2 is the success probability of data packet detection with combination of first and second HARQ transmissions.

- PDTX≤PT-Pc1-PcPc, where PDTX=Prob{DTX or NACK is detected | UE sends DTX}.

- PN≤PT-PcPd1PcPc+Pd1-2PcPd1, where PN=Prob{DTX or NACK is detected | UE sends NACK}.

Observation 2: For a DL HARQ process of maximum 2 transmissions, the legacy 99% reliability of PDCCH is not able to meet URLLC requirement of 99.999% and should be improved.

It should be noted that the above analysis only says the channel (PDCCH, PDSCH or ACK/NACK) reliability lower than certain threshold would not make the overall HARQ reliability to reach 99.999% or above. However, these thresholds give only the necessary conditions for P≥99.999% but not sufficient conditions, i.e., it is not guaranteed the overall HARQ reliability of 99.999% is achievable if all channel reliabilities exceed the mentioned thresholds. Further numerical trials and analysis are given in Table 2 and Table 3, where some numerical combinations of channel reliabilities are listed for Pc=0.999 and Pc=0.9999, respectively.

Table 2 Channel reliabilities with Pc=0.999

| PDTX | Pd1 | Pd2 | PN | P |
| --- | --- | --- | --- | --- |
| 0.999 | 0.999 | 0.999 | ≥0.99649 | ≥0.9999915 |
| 0.999 | 0.999 | 0.9999 | ≥0.99649 | ≥0.9999924 |
| 0.999 | 0.9999 | 0.9999 | ≥0.99199 | ≥0.9999970 |
| 0.9999 | 0.999 | 0.999 | ≥0.99649 | ≥0.9999924 |
| 0.9999 | 0.999 | 0.9999 | ≥0.99649 | ≥0.9999933 |
| 0.9999 | 0.9999 | 0.9999 | ≥0.99199 | ≥0.9999979 |

Table 3 Channel reliabilities with Pc=0.9999

| PDTX | Pd1 | Pd2 | PN | P |
| --- | --- | --- | --- | --- |
| 0.99 | 0.99 | 0.999 | 0.999 | 0.9999770143 |
| 0.99 | 0.99 | 0.9999 | 0.999 | 0.9999860035 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999950008 |
| 0.99 | 0.999 | 0.999 | 0.999 | 0.9999967925 |
| 0.99 | 0.999 | 0.9999 | 0.999 | 0.9999976914 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999985912 |
| 0.999 | 0.99 | 0.999 | 0.999 | 0.9999779052 |
| 0.999 | 0.99 | 0.9999 | 0.999 | 0.9999868944 |
| 0.999 | 0.99 | 0.9999 | 0.9999 | 0.9999958917 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.9999976915 |
| 0.999 | 0.999 | 0.9999 | 0.999 | 0.9999985904 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999994902 |

According to Table 2 and Table 3,

- When Pc=0.999, one possible reliability combination is PDTX=PN=Pd1=Pd2=0.999. It is shown in Figure A.1 that about 1.5dB should be enhanced on PUCCH to improve PDTX from 0.99 to 0.999.

- When Pc=0.9999, the possible reliability combination is PDTX=0.99 and PN=Pd1=Pd2=0.999, or PDTX=Pd1=0.99 and PN=Pd2=0.9999. It is also shown that there is no obvious gain to further improve PDTX from 0.99 and PN from 0.999 (when Pd1=0.999).

Observation 3: For a DL HARQ process of maximum 2 transmissions, the overall reliability target of 99.999% can be achieved by any of following combinations of channel reliabilities:

- Combination 1: Pc=PDTX=PN=Pd1=Pd2=0.999.

- Combination 2: Pc=0.9999, PN=Pd1=Pd2=0.999, PDTX=0.99.

- Combination 3: Pc=PN=Pd2=0.9999, PDTX=Pd1=0.99.

Observation 4: About 1.5dB enhancement on PUCCH is needed to improve PDTX from 0.99 to 0.999.

Maximum K transmissions

Based on above maximum 2 transmissions analysis, the probability for successful transmission can be extended for K transmissions for one HARQ process case. It is given by:

P=j=1KR(j), where Rj=k=0j-1Cj-1k1-Pc∙PDTXj-1-kPck+1Pd(k+1)i=1kPN∙1-Pd(i)

Where R(j) is the probability that the jth transmission is successfully received by UE.

For maximum 3 transmissions

Then the largest Pd1, Pd2, Pd3, PDTX and PN (which are all 1’s) push the function value P upwards to P=Pc3-3Pc2+3Pc, which is 99.9999% assuming Pc=99%. And the target of 99.999% can be reached even with the current PDCCH reliability of Pc=99%. Any value of Pc that is smaller than 0.97845565 would have no chance to make P to reach PT=99.999%. So, at least the requirement on BLER of Y=1% will be the targeting BLER for maximum 3 transmission and 1% would be similar to PDCCH for eMBB.

Further numerical trials and analysis are given in Table 4, Table 5 and Table 6, where some numerical combinations of channel reliabilities are listed for  Pc=0.99, Pc=0.999 and Pc=0.9999, respectively.

Table 4 Channel reliabilities with Pc=0.99

| Pd1 | Pd2 | Pd3 | PDTX | PN | P |
| --- | --- | --- | --- | --- | --- |
| 0.99 | 0.99 | 0.99 | 0.9999 | 0.9999 | 0.99999009 |
| 0.99 | 0.99 | 0.999 | 0.9999 | 0.9999 | 0.99999096 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.99999105 |
| 0.99 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999371 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999372 |
| 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999398 |

Table 5 Channel reliabilities with Pc=0.999

| Pd1 | Pd2 | Pd3 | PDTX | PN | P |
| --- | --- | --- | --- | --- | --- |
| 0.99 | 0.99 | 0.99 | 0.999 | 0.9999 | 0.99999665 |
| 0.99 | 0.99 | 0.99 | 0.9999 | 0.9999 | 0.99999756 |
| 0.99 | 0.99 | 0.999 | 0.999 | 0.9999 | 0.99999755 |
| 0.99 | 0.99 | 0.999 | 0.9999 | 0.9999 | 0.99999846 |
| 0.99 | 0.99 | 0.9999 | 0.999 | 0.9999 | 0.99999764 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.99999855 |
| 0.99 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.99999792 |
| 0.99 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999883 |
| 0.99 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.99999793 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999884 |
| 0.99 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.99999795 |
| 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999886 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.99999799 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.99999889 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.99999889 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999979 |
| 0.999 | 0.999 | 0.9999 | 0.999 | 0.999 | 0.99999799 |
| 0.999 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.99999889 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.99999889 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999979 |
| 0.999 | 0.9999 | 0.9999 | 0.999 | 0.999 | 0.99999799 |
| 0.999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.99999889 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.9999989 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999998 |

Table 6 Channel reliabilities with Pc=0.9999

| Pd1 | Pd2 | Pd3 | PDTX | PN | P |
| --- | --- | --- | --- | --- | --- |
| 0.99 | 0.99 | 0.99 | 0.99 | 0.9999 | 0.99999695 |
| 0.99 | 0.99 | 0.99 | 0.999 | 0.9999 | 0.99999786 |
| 0.99 | 0.99 | 0.99 | 0.9999 | 0.9999 | 0.99999795 |
| 0.99 | 0.99 | 0.999 | 0.99 | 0.9999 | 0.99999785 |
| 0.99 | 0.99 | 0.999 | 0.999 | 0.9999 | 0.99999876 |
| 0.99 | 0.99 | 0.999 | 0.9999 | 0.9999 | 0.99999885 |
| 0.99 | 0.99 | 0.9999 | 0.99 | 0.9999 | 0.99999794 |
| 0.99 | 0.99 | 0.9999 | 0.999 | 0.9999 | 0.99999885 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.99999894 |
| 0.99 | 0.999 | 0.999 | 0.99 | 0.9999 | 0.99999798 |
| 0.99 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.99999888 |
| 0.99 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999898 |
| 0.99 | 0.999 | 0.9999 | 0.99 | 0.9999 | 0.99999798 |
| 0.99 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.99999889 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999898 |
| 0.99 | 0.9999 | 0.9999 | 0.99 | 0.9999 | 0.99999799 |
| 0.99 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.9999989 |
| 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999899 |
| 0.999 | 0.999 | 0.999 | 0.99 | 0.999 | 0.999998 |
| 0.999 | 0.999 | 0.999 | 0.99 | 0.9999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999998 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.99999899 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999989 |
| 0.999 | 0.999 | 0.9999 | 0.99 | 0.999 | 0.999998 |
| 0.999 | 0.999 | 0.9999 | 0.99 | 0.9999 | 0.9999989 |
| 0.999 | 0.999 | 0.9999 | 0.999 | 0.999 | 0.9999989 |
| 0.999 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.9999998 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.99999899 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999989 |
| 0.999 | 0.9999 | 0.9999 | 0.99 | 0.999 | 0.999998 |
| 0.999 | 0.9999 | 0.9999 | 0.99 | 0.9999 | 0.9999989 |
| 0.999 | 0.9999 | 0.9999 | 0.999 | 0.999 | 0.9999989 |
| 0.999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.9999998 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.99999899 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999989 |
| 0.9999 | 0.9999 | 0.9999 | 0.99 | 0.999 | 0.9999989 |
| 0.9999 | 0.9999 | 0.9999 | 0.99 | 0.9999 | 0.99999899 |
| 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.999 | 0.9999998 |
| 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.99999989 |
| 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.99999989 |
| 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999998 |

According to Table 4, Table 5 and Table 6,

- When Pc=0.99, one possible reliability combination is Pd1=Pd2=Pd3=0.99, PDTX=PN=0.9999. Compared with maximum 2 transmission, reliability requirement on Pc can be reduced at the cost of increasing reliability requirement on PDTXand PN.

- When Pc=0.999, one possible reliability combination is PDTX=PN=Pd1=Pd2=Pd3=0.999 which is same reliability requirement on each channel with maximum 2 transmissions. In addition, reliability requirement on Pd1 can be reduced at the cost of increasing reliability requirement on PN and one possible reliability combination is PDTX=0.999, PN=0.9999, Pd1=Pd2=Pd3=0.99.

- When Pc=0.9999, the possible reliability combination is PDTX=0.99 and PN=Pd1=Pd2=0.999 which is same reliability requirement on each channel with maximum 2 transmissions. In addition, reliability requirement on data channel at retransmission like Pd2 can be reduced comparing with maximum 2 transmission and one possible reliability combination is PN=0.9999, PDTX=Pd1=Pd2=Pd3=0.99.

- In addition, when PN=0.9999 and Pd1=0.99, reliability requirement on Pc can be reduced at the cost of increasing reliability requirement on PDTX.

Observation 5: For a DL HARQ process of maximum 3 transmissions, the overall reliability target of 99.999% can be achieved by any of following combinations of channel reliabilities:

- Combination 1: Pc=Pd1=Pd2=Pd3=0.99, PDTX=PN=0.9999.

- Combination 2: Pc=PDTX=0.999, PN=0.9999, Pd1=Pd2=Pd3=0.99.

- Combination 3: Pc=PDTX=PN=Pd1=Pd2=Pd3=0.999.

- Combination 4: Pc=PN=0.9999, PDTX= Pd1=Pd2=Pd3=0.99.

- Combination 5: Pc=0.9999, PDTX=0.99, PN=Pd1=Pd2=Pd3=0.999.

For maximum 4 transmissions

Then the largest Pd1, Pd2, Pd3, Pd4, PDTX and PN (which are all 1’s) push the function value P upwards to P=4Pc-6Pc2+4Pc3-Pc4, which is 99.999999% with the current PDCCH reliability of Pc=99%. Any value of Pc that is smaller than 0.943765867 would have no chance to make P to reach PT=99.999%. So, BLER of Y=1% should be also targeted.

Further numerical trials and analysis are given in Table 7, Table 8 and Table 9, where some numerical combinations of channel reliabilities are listed for  Pc=0.99, Pc=0.999 and Pc=0.9999, respectively.

Table 7 Channel reliabilities with Pc=0.99

| Pd1 | Pd2 | Pd3 | Pd4 | PDTX | PN | P |
| --- | --- | --- | --- | --- | --- | --- |
| 0.99 | 0.99 | 0.99 | 0.99 | 0.9999 | 0.9999 | 0.99999781 |
| 0.99 | 0.99 | 0.99 | 0.999 | 0.9999 | 0.9999 | 0.99999782 |
| 0.99 | 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.99999782 |
| 0.99 | 0.99 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999786 |
| 0.99 | 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999786 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999786 |
| 0.99 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999792 |
| 0.99 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999792 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999792 |

Table 8 Channel reliabilities with Pc=0.999

| Pd1 | Pd2 | Pd3 | Pd4 | PDTX | PN | P |
| --- | --- | --- | --- | --- | --- | --- |
| 0.99 | 0.99 | 0.99 | 0.99 | 0.999 | 0.9999 | 0.99999796 |
| 0.99 | 0.99 | 0.99 | 0.99 | 0.9999 | 0.9999 | 0.99999887 |
| 0.99 | 0.99 | 0.99 | 0.999 | 0.999 | 0.9999 | 0.99999797 |
| 0.99 | 0.99 | 0.99 | 0.999 | 0.9999 | 0.9999 | 0.99999888 |
| 0.99 | 0.99 | 0.99 | 0.9999 | 0.999 | 0.9999 | 0.99999797 |
| 0.99 | 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.99999888 |
| 0.99 | 0.99 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.99999798 |
| 0.99 | 0.99 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999889 |
| 0.99 | 0.99 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.99999798 |
| 0.99 | 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999889 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.99999798 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999889 |
| 0.99 | 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.99999799 |
| 0.99 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999989 |
| 0.99 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.99999799 |
| 0.99 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999989 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.99999799 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999989 |
| 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.99999799 |
| 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.999998 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999998 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.999 | 0.999998 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999998 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.999 | 0.999998 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.9999989 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.9999989 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999998 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.999 | 0.999998 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.9999989 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.9999989 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999998 |

Table 9 Channel reliabilities with Pc=0.9999

| Pd1 | Pd2 | Pd3 | Pd4 | PDTX | PN | P |
| --- | --- | --- | --- | --- | --- | --- |
| 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.9999 | 0.99999797 |
| 0.99 | 0.99 | 0.99 | 0.99 | 0.999 | 0.9999 | 0.99999888 |
| 0.99 | 0.99 | 0.99 | 0.99 | 0.9999 | 0.9999 | 0.99999897 |
| 0.99 | 0.99 | 0.99 | 0.999 | 0.99 | 0.9999 | 0.99999798 |
| 0.99 | 0.99 | 0.99 | 0.999 | 0.999 | 0.9999 | 0.99999889 |
| 0.99 | 0.99 | 0.99 | 0.999 | 0.9999 | 0.9999 | 0.99999898 |
| 0.99 | 0.99 | 0.99 | 0.9999 | 0.99 | 0.9999 | 0.99999798 |
| 0.99 | 0.99 | 0.99 | 0.9999 | 0.999 | 0.9999 | 0.99999889 |
| 0.99 | 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.99999898 |
| 0.99 | 0.99 | 0.999 | 0.999 | 0.99 | 0.9999 | 0.99999798 |
| 0.99 | 0.99 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.99999889 |
| 0.99 | 0.99 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999898 |
| 0.99 | 0.99 | 0.999 | 0.9999 | 0.99 | 0.9999 | 0.99999798 |
| 0.99 | 0.99 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.99999889 |
| 0.99 | 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999898 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.99 | 0.9999 | 0.99999798 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.99999889 |
| 0.99 | 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999898 |
| 0.99 | 0.999 | 0.999 | 0.999 | 0.99 | 0.9999 | 0.99999799 |
| 0.99 | 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999989 |
| 0.99 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999899 |
| 0.99 | 0.999 | 0.999 | 0.9999 | 0.99 | 0.9999 | 0.99999799 |
| 0.99 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.9999989 |
| 0.99 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999899 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.99 | 0.9999 | 0.99999799 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.9999989 |
| 0.99 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999899 |
| 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.99 | 0.9999 | 0.99999799 |
| 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.9999989 |
| 0.99 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999899 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.99 | 0.999 | 0.999998 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.99 | 0.9999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999998 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.99999899 |
| 0.999 | 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99999989 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.99 | 0.999 | 0.999998 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.99 | 0.9999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.999 | 0.9999989 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.999 | 0.9999 | 0.9999998 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.99999899 |
| 0.999 | 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99999989 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99 | 0.999 | 0.999998 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.99 | 0.9999 | 0.9999989 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.999 | 0.9999989 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.9999998 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.99999899 |
| 0.999 | 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999989 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99 | 0.999 | 0.999998 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.99 | 0.9999 | 0.9999989 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.999 | 0.9999989 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.9999998 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.99999899 |
| 0.999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999989 |
| 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99 | 0.999 | 0.9999989 |
| 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99 | 0.9999 | 0.99999899 |
| 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.999 | 0.9999998 |
| 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.9999 | 0.99999989 |
| 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.999 | 0.99999989 |
| 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.99999998 |

According to Table 7, Table 8 and Table 9,

- Maximum 4 transmissions repeat observation 5 for maximum 3 transmissions.

- Reliability requirement on each channel for maximum 4 transmissions can’t be reduced further compared with maximum 3 transmissions.

Observation 6: Reliability requirement on each channel for maximum 4 transmissions can’t be reduced further compared with maximum 3 transmissions.

Observation 7: Value of Y can be determined by maximum times of transmissions. e.g. Y=0.1% with maximum 2 transmission, while Y=1% with maximum 3 transmission.

Proposal 1: To achieve best efficiency of URLLC services, it would be sufficient to support at most 3 transmissions for HARQ operation.

Proposal 2: The operating BLER for URLLC PDCCH should be no larger than 0.1% with maximum 2 transmissions. The operating BLER for URLLC PDCCH should be no larger than 1% with maximum 3 transmissions.

Improvement of URLLC PDCCH Reliability

According to the analysis in previous section, URLLC PDCCH reliability should be at least improved from legacy reliability 99% to 99.9% with maximum 2 transmissions. Figure A.2 shows that such improvement needs about 2dB enhancement on PDCCH.

Observation 8: About 2dB should be enhanced compared with legacy performance at same work point in order to achieve 99.9% reliability on PDCCH.

Some potential methods to improve PDCCH reliability are listed below.

- Higher Aggregation Levels

Higher aggregation levels for downlink control channel could be useful for low coding rate in LTE. It was agreed that the DL control channel will be mapped to one or more NR-CCEs in NR study item and aggregation levels could be introduced for NR PDCCH which is also applicable for URLLC. Then higher aggregation levels would also be used to improve reliability of URLLC PDCCH. According to [2], gain from 8 CCEs to 16 CCEs is about 2.5dB when achieving 99.9% reliability. Definition of CCE with higher aggregation level is a potential solution to improve the reliability of PDCCH.

- Compact DCI Formats

DCI is contained in downlink control channel for scheduling UL/DL data channels. For URLLC, some fields in eMBB DCI formats may be reduced or removed to get compact DCI, which is useful for improving PDCCH reliability in URLLC. According to [2], performance gain for reducing 20 bits is about 2dB. Compact DCI formats can also be considered as a potential solution to improve PDCCH reliability. Given DCI contents for NR are still in study, high level analysis is listed below.

Fields for flexible scheduling

According to the discussion on NR scheduling, both DL scheduling and UL scheduling may use dynamic scheduling delay which are indicated by DCI. For URLLC, minimum scheduling delay should be used in order to complete transmission as soon as possible. Thus fields for flexible scheduling could be removed for URLLC.

Fields for complexity transmission mode

Higher peak rate is an important target for eMBB in NR. Thus advanced MIMO schemes and higher-order modulation would be introduced for eMBB but increase size of DCI. For URLLC, because 99.999% reliability within user plane latency of 1ms is probably for X bytes [e.g. X=32, 50, 200], the peak rate requirement for eMBB may not be applicable to URLLC. So the fields such as MCS and MIMO related to eMBB application could be reduced.

- PSD boosting

PSD boosting with corresponding interference coordination is an implementation based way to improve the PDCCH reliability.

- Repetition

Time domain repetition is a common solution for reliability improvement that can also be used for downlink control channel. But different from existing system like eMTC/NB-IoT, large number of repetition times can’t be accepted by URLLC. Maximum number of repetition times including PDCCH and PDSCH/PUSCH for URLLC should be restricted by latency requirement and slot length for URLLC traffic transmission. In order to transmit successfully as soon as possible, repetition for PDCCH and repetition for PDSCH can be started at the same slot. If ACK is received by gNB, both PDCCH repetition and PDSCH repetition are terminated. If NACK is received by gNB, PDCCH repetition is terminated and PDSCH repetition is continued until ACK is received by gNB or 1ms boundary is extended.

Observation 9: Higher aggregation level and PSD boosting can be directly used to improve PDCCH reliability for URLLC. Compact DCI and repetition could be applied with specific considerations of URLLC features and requirements.

URLLC PDSCH Reliability

The PDSCH BLER for eMBB is generally targeted at 10%, since it don’t have hard limit on delay boundary. As it shown in the reliability analysis for URLLC, PDSCH for each transmission should not be lower than 1% level. For cases like 2-transmission, PDSCH BLER needs 0.1% level for BLER. Figure A.3 shows PDSCH can reach 1% at 2dB SNR and code rate of 0.356. Two more dB will further reduce BLER to 0.1%. Note those reasonable SNR range already require code rate to typical mother code rate of eMBB. In general, PDSCH for URLLC mainly use chase combine. With those considerations, the details for URLLC PDSCH will be much different to that for eMBB. Those include CQI and link adaptation.

Proposal 3: The operating BLER for URLLC PDSCH should not be larger than 1%. URLLC PDSCH should be able to cover BLER of 0.1%. CQI/TBS/MCS of URLLC should target on those BLER operating points.

Conclusion

This contribution concludes with following observations and proposals.

Observation 1: For a DL HARQ process of maximum 2 transmissions, its reliability target PT (overall success probability) is NOT achievable if any of following conditions is true:

- Pc≤1-1-PT, where Pc is the success probability of DL scheduling detection.

- Pd1≤PT-Pc221-PcPc, where Pd1 is the success probability of first detection of data packet.

- Pd2≤1-1-PTPc, where Pd2 is the success probability of data packet detection with combination of first and second HARQ transmissions.

- PDTX≤PT-Pc1-PcPc, where PDTX=Prob{DTX or NACK is detected | UE sends DTX}.

- PN≤PT-PcPd1PcPc+Pd1-2PcPd1, where PN=Prob{DTX or NACK is detected | UE sends NACK}.

Observation 2: For a DL HARQ process of maximum 2 transmissions, the legacy 99% reliability of PDCCH is not able to meet URLLC requirement of 99.999% and should be improved.

Observation 3: For a DL HARQ process of maximum 2 transmissions, the overall reliability target of 99.999% can be achieved by any of following combinations of channel reliabilities:

- Combination 1: Pc=PDTX=PN=Pd1=Pd2=0.999.

- Combination 2: Pc=0.9999, PN=Pd1=Pd2=0.999, PDTX=0.99.

- Combination 3: Pc=PN=Pd2=0.9999, PDTX=Pd1=0.99.

Observation 4: About 1.5dB enhancement on PUCCH is needed to improve PDTX from 0.99 to 0.999.

Observation 5: For a DL HARQ process of maximum 3 transmissions, the overall reliability target of 99.999% can be achieved by any of following combinations of channel reliabilities:

- Combination 1: Pc=Pd1=Pd2=Pd3=0.99, PDTX=PN=0.9999.

- Combination 2: Pc=PDTX=0.999, PN=0.9999, Pd1=Pd2=Pd3=0.99.

- Combination 3: Pc=PDTX=PN=Pd1=Pd2=Pd3=0.999.

- Combination 4: Pc=PN=0.9999, PDTX= Pd1=Pd2=Pd3=0.99.

- Combination 5: Pc=0.9999, PDTX=0.99, PN=Pd1=Pd2=Pd3=0.999.

Observation 6: Reliability requirement on each channel for maximum 4 transmissions can’t be reduced further compared with maximum 3 transmissions.

Observation 7: Value of Y can be determined by maximum times of transmissions. e.g. Y=0.1% with maximum 2 transmission, while Y=1% with maximum 3 transmission.

Observation 8: About 2dB should be enhanced compared with legacy performance at same work point in order to achieve 99.9% reliability on PDCCH.

Observation 9: Higher aggregation level and PSD boosting can be directly used to improve PDCCH reliability for URLLC. Compact DCI and repetition could be applied with specific considerations of URLLC features and requirements.

Proposal 1: To achieve best efficiency of URLLC services, it would be sufficient to support at most 3 transmissions for HARQ operation.

Proposal 2: The operating BLER for URLLC PDCCH should be no larger than 0.1% with maximum 2 transmissions. The operating BLER for URLLC PDCCH should be no larger than 1% with maximum 3 transmissions.

Proposal 3: The operating BLER for URLLC PDSCH should not be larger than 1%. URLLC PDSCH should be able to cover BLER of 0.1%. CQI/TBS/MCS of URLLC should target on those BLER operating points.

# References

- 3GPP, Draft Report of 3GPP TSG RAN WG1 NR_AH 1701, v0.1.0

- 3GPP, R1-1700265, Analysis of URLLC reliability in DL HARQ, ZTE, ZTE Microelectronics

# Annex

Simulation results for PUCCH/PDCCH/PDSCH reliability are shown in Figure A.1 to Figure A.3 respectively and simulation assumptions are listed in Table A.1 to Table A.3 respectively.

Figure A.1 PUCCH reliability

Figure A.2 PDCCH reliability

Figure A.3 PDSCH reliability

Table A.1 Simulation assumption for PUCCH

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 2GHz |
| System bandwidth | 10 MHz |
| TTI length | one symbol |
| Channel model | TDL-A |
| Delay spread | 100ns |
| UE speed | 3km/h |
| Antenna configuration | 1Tx(UE), 2 Rx(eNB) |
| CP length | Normal |
| Modulation mode | Sequence-based, BPSK/QPSK |
| Channel estimation | MMSE |

Table A.2 Simulation assumption for PDCCH

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 2GHz |
| System BW | 10MHz |
| Channel model | TDL-A |
| UE speed | 3kmph |
| Modulation mode | QPSK |
| Antenna configuration | 2Tx-2Rx |
| Channel estimation | MMSE |
| CP type | Normal CP |
| DCI Format | Format 1A |
| Aggregation Level | 8CCE |
| OFDM Symbols for PDCCH | 3 |

Table A.3 Simulation assumption for PDSCH

| Parameters | Assumptions |
| --- | --- |
| Carrier Frequency | 4GHz |
| User bandwidth | 5RBs for OFMDA |
| SC per RB | 12 |
| PHY Packet size | 32 Bytes (including 24bit CRC) |
| Modulation and coding rate | QPSK, |
| Code rate | 0.356 (256/(6*12*5*2)) |
| Sub-carrier spacing | 60 kHz |
| TTI length | 0.125 ms |
| OFDM symbols per TTI | 7 |
| OFDM symbols for data | 6 |
| Channel model | TDL-A, 3km/h |
| BS antenna configuration | 2Rx |
| UE antenna elements | 2Tx |
| HARQ | No |
| Channel estimation | Ideal |
| Receiver | MMSE MMSE-SIC is applied for collision cases |
| CQI feedback assumption | No |