3GPP TSG RAN WG1 Meeting #88	R1-1701673

Athens, Greece, 13th - 17th February 2017

Agenda Item:	8.1.6.1

Source:	Huawei, HiSilicon

Title:	Evaluation of duplexing flexibility in urban macro scenario

Document for:	Discussion and Decision

Introduction

In the RAN1# NR AH1701 meeting the following conclusion are achieved [1]:

Conclusion: - The WF in R1-1701329 is agreed - Companies are encouraged to perform evaluations under various RU percentage values - Note: the RU for a link direction (DL or UL) herein is defined as the amount of occupied resources for the given link direction divided by the total number of resources (irrespective of link directions) - Companies should also report assumptions regarding backhaul - In performing evaluations for flexible duplexing operation, companies should take into account additional overhead for the operation.

Duplexing flexibility allows flexible resource allocation among different transmission directions, for both paired and unpaired spectrum. In this contribution, some evaluation results of duplexing flexibility on both paired and unpaired spectrum in urban macro scenario are provided and discussed.

Discussion on key simulation assumptions

In this section, some simulation scenarios and assumptions are discussed. These key assumptions are important for identifying the technical feasibility and challenges of duplexing flexibility. For detailed system-level simulation parameters, the parameters in Appendix are used, which are aligned with RAN1# NR AH1701 agreements [2].

Network layout

In this contribution, the focus is on one operator scenario and the single-layer layout for urban macro is used. The detailed layout set is as follows:

- Macro layer: Hex. Grid

Further, for urban macro scenario where inter-site distance (ISD) is usually 500m or even larger, lower frequency band will be used. For this consideration, 2GHz and 4GHz are employed in the evaluation.

Both paired and unpaired spectrum are considered in this contribution, and 2GHz would be a good candidate to evaluate paired spectrum case. Therefore, two simulation cases are considered for urban macro single-layer deployment:

- 2GHz: Macro layer with paired spectrum

- 4GHz: Macro layer with unpaired spectrum

Downlink and uplink resource allocation

Duplexing flexibility allows flexible resource allocation among different transmission directions, for both paired and unpaired spectrum. For the evaluation, the downlink-uplink resource allocation on both paired spectrum and unpaired spectrum is summarized in Table 1.

In RAN1 # NR AH1701 meetings [2], it was agreed that FTP traffic model 3 with packet size 0.5M bytes can be used and the ratio of DL/UL traffic load are {1:1}, {2:1}, {4:1}.

Table 1 simulation setup parameters and resource allocations

(a) Paired spectrum

|  | # DL/UL | # Baseline (Traditional FDD) | # Duplexing flexibility |
| --- | --- | --- | --- |
| DL/UL subframe ratio | # DL carrier | # DL only transmission | - DL data transmission - SRS transmission |
|  | # UL carrier | # UL only transmission | # Flexible UL/DL subframe ratio allocation |
| DL/UL carrier bandwidth | # DL carrier | # 2GHz: 10MHz |  |
|  | UL carrier | 2GHz: 10MHz |  |

(b) Unpaired spectrum

|  | Baseline (Traditional TDD) | Duplexing flexibility |
| --- | --- | --- |
| DL/UL subframe ratio | semi-static subframe allocation - For DL:UL traffic ratio = 1:1 or 2:1, DL/UL subframe = 6:4 - For DL:UL traffic ratio = 4:1, DL/UL subframe = 4:1 | Flexible UL/DL subframe ratio allocation |
| DL/UL carrier bandwidth | 4GHz: 20MHz (DL+UL) |  |

SRS transmission in downlink dominant spectrum

In contribution [3], SRS transmission in the downlink dominant spectrum of paired spectrum to improve the MIMO performance is discussed. SRS configuration could be periodic or aperiodic and SRS could be transmitted in every slot. SRS transmission in a downlink dominant frequency/carrier is a promising choice to improve the performance of the DL MIMO on the paired spectrum.

One of the issues of the SRS transmission is the co-channel interference if the SRS resources configurations are different. For the MIMO performance improvement, all the cells that within one operator’s network can have the alignment SRS symbols, and then the interference issues can be avoided.

Another issue of the SRS transmission is the adjacent-channel interference due to the unsynchronized networks among different operators. With appropriate coordination scheme, SRS transmission configuration could be synchronized and coordinated among different operators to avoid the adjacent-channel interference, for example, strive for timing alignment among different operators, and then all the cells have the alignment SRS symbols among different operators. It should be noted that even with unsynchronized SRS transmission among different operators, note that SRS would be transmitted in 1 or 2 symbols among several subframes, so the UE-UE adjacent-channel interference only potentially affects a small percentage of DL time resources (e.g., for two unsynchronized SRS transmission operators, when one SRS symbol per 5ms, just 1.4% DL symbols are affected, and when two SRS symbols per 5ms, just 2.8% DL symbols are affected). Consider the large pathloss between UE-UE in Urban macro deployments, large UE-UE adjacent-channel interference caused by SRS transmission rarely occur. Thus, the impact of SRS transmission on the adjacent-channel DL transmission is expected to be marginal, as shown in section 3.1.1. The impact of TRP-TRP adjacent-channel interference on the SRS channel estimation is acceptable as long as there is sufficient improvement of DL MIMO performance, which are shown in section 3.1.1, both synchronized (w/o. adjacent-channel interference) and unsynchronized (w. adjacent-channel interference) SRS transmission among different operators are evaluated.

Another issue related to the SRS is the SRS transmission on both FDD UL and FDD DL carriers. Then there will be two carriers that a NR UE needs to transmit SRS. In this case, SRS switching could be used on the two carriers.

In the simulation, we select 2 subframes out of 10 subframes to transmit SRS, which is depicted as in Figure 1.

Figure 1. Example of SRS transmission on the downlink dominant spectrum

Modelling of cross-link interference mitigation

In duplexing flexibility, cross-link interference, e.g. TRP-to-TRP and UE-to-UE interference, exists in case that neighboring cells use different transmission directions on the same or partially-overlapping time-frequency resource regardless paired or unpaired spectrum. An example is shown in Figure 2.

Figure 2. Example of cross-link interference

In the simulation, the cross-link interference is modeled with appropriate propagation characteristics as given in [2]. It is noted that for urban macro scenario, 80% UEs are indoor UEs and 20% UEs are outdoor UEs, and the UE-to-UE cross-link refers to the link between UEs connected to different TRPs. In this case, it is very likely such cross-link is between UEs in different bulidings. In such cases, the  peneration loss would be large. The UE-to-UE cross link interference in  urban macro scenario is therefore not very severe. On ther other hand, all TRPs are outdoor, with large transmission powers. And the height of TRPs are higher than UE side, which results in higher probability of LOS channel betewen TRPs. All these factors lead to severe interference between TRP-to-TRP cross-link and will severely degrade the uplink performance. To assure the performance gain, cross-link interference mitigation schemes need to be considered, such as interference cancellation/suppression and interference coordination [4].

In NR, MIMO is a key feature to improve the system throughput. For cross link interference mitigation, beamforming and beam nulling are also promising. For TRP-to-TRP links, it is noted that the angle spread is small in both arrival angle and departure angle according to the channel model, so analog beam coordination and digital beamforming are effective to mitigate a number of TRP-to-TRP interference links simultaneously.

The detailed design of beam coordination is that:

- Step1: gNB gets the BS-BS coupling loss.

- Step2: gNB selects the best analog beam for each scheduled UE every 10ms.

- Step3: gNB exchanges the subframe type among neighbour cells via OTA/X2 every 10ms.And the macro cell exchange DL/UL traffic information among co-site cell.

- Step4: gNB calculates the interference power according to the BS-BS coupling loss and subframe type of neighbour cell. When the interference power above some threshold, the gNB uses dynamic analog beam and dynamic digital beamforming to avoid the cross link interference, otherwise gNB uses the best analog beam for scheduled UE.

In LTE studies such as NAICS and MUST etc., advanced receivers are studied and are used for interference suppression. In [4], it is observed that advanced receivers, e.g., IRC receiver can significantly reduce the cross link interference level with low complexity.

In this contribution, massive MIMO is used. For the TRP-to-TRP and UE-to-UE interference mitigation, analog beam coordination, digital beamforming and advanced IRC receiver are used to mitigate the influence of the cross-link interference to assure the performance.

Simulation results

This section shows the evaluation results for different scenarios based on the above simulation assumptions.

Duplexing flexibility on paired spectrum @ 2GHz

SRS transmission in downlink dominant spectrum

In this section, the evaluation results of SRS transmission in the downlink dominant spectrum [3] are presented in Table 2-1 and Table 2-2, as discussed in section 2.3. In the simulation, one SRS symbol is configured per 5ms and the overhead of both the SRS symbols and GP symbols are modelled. Table 2-1 is the DL performance gain of SRS transmission in the DL dominant spectrum. The baseline is codebook based DL transmission. Table 2-2 is the victim operator DL performance loss caused by the adjacent-channel SRS transmission.

Table 2-1 DL performance evaluation with SRS@ DL dominant spectrum

| Antenna configure | Feature | DL Throughput Gain |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5%-tile | 50%-tile | 95%-tile | Cell Average | Served/ offered packets | RU (%) |
| 4TXRU | SRS@ DL part carrier w/o. adjacent-channel interference | +17.78% | +22.55% | +41.74% | +31.95% | NA | NA |
|  | SRS@ DL part carrier w. adjacent-channel interference | +6.89% | +14.00% | +36.35% | +27.17% | NA | NA |
| 8TXRU | SRS@ DL part carrier w/o. adjacent-channel interference | +26.52% | +31.55% | +42.33% | +35.71% | NA | NA |
|  | SRS@ DL part carrier w. adjacent-channel interference | +12.58% | +21.46% | +35.48% | +32.36% | NA | NA |
| 16TXRU | SRS@ DL part carrier w/o. adjacent-channel interference | +35.65% | +62.69% | +92.64% | +73.46% | NA | NA |
|  | SRS@ DL part carrier w. adjacent-channel interference | +19.35% | +47.11%% | +78.92% | +69.64% | NA | NA |
| Note - Periodic SRS configuration, one SRS symbol per 5ms - The overhead of both the SRS symbols and GP symbols are modelled - Alignment SRS symbols of all the cells that within one operator’s network - Evaluation assumptions refer to the agreed in [2] except the following parameters: - The traffic model is full buffer - BS antenna tilt is 100 degree for baseline |  |  |  |  |  |  |  |

Table 2-2 Victim operator DL performance loss caused by adjacent-channel SRS

| Antenna configure | Cell Average Throughput Loss | Cell edge Throughput Loss |
| --- | --- | --- |
| 4TXRU | -1.26% | -2.70% |
| 8TXRU | -1.24% | -0.17% |
| 16TXRU | -1.95% | -1.42% |

It is observed from the simulation results that DL performance can be significantly improved due to the accurate downlink beamforming vector measurement, even with the TRP-TRP adjacent-channel interference, as illustrated in Table 2-1. For the downlink performance that affected by adjacent-channel SRS transmission, small performance degradation is observed, as illustrated in Table 2-2.

Observation 1:

SRS in the downlink dominant frequency carrier can significantly improve the downlink performance of MIMO of the paired spectrum in urban macro scenario. It achieves considerable throughput gain in downlink (e.g., cell average throughput up to 70%, cell edge throughput up to 35%).

Proposal 1: SRS transmission on a downlink dominant frequency carrier should be supported on a paired spectrum in duplexing flexibility.

Downlink data transmission in both paired spectrum

In this section, the evaluation results of duplexing flexibility for 2GHz paired spectrum with flexible UL/DL subframe ratio allocation in UL part carrier are presented in Table 3, as discussed in section 2.2. For duplexing flexibility, in UL carrier of the paired spectrum, the DL-UL subframe ratio is dynamically changed according to the DL and UL traffic load ratio. For some frames, if there is no UL traffic, the DL-UL subframe allocation ratio is set to be 9:1 at the UL carrier. Limited downlink transmission power in the UL carrier is also evaluated, e.g. 23dBm (same as UE maximum transmission power).

Table 3 Performance evaluation of duplexing flexibility on paired spectrum @ 2GHz

| Ratio of DL/UL traffic | Feature | DL UPT (Mbps) |  |  |  |  |  | UL UPT (Mbps) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5%-tile | 50%-tile | 95%-tile | Average | Served/ offered packets | RU (%) | 5%-tile | 50%-tile | 95%-tile | Average | Served/ offered packets | RU (%) | Total RU (%) |
| 4:1 | baseline | 4.64 | 22.60 | 57.97 | 26.17 | 0.98 | 24.87 | 1.51 | 22.47 | 58.82 | 24.87 | 0.97 | 5.55 | 30.42 |
|  | Duplexing flexibility(TX power 46dBm@UL carrier) | 7.75 (+67.03%) | 31.01 (+37.21%) | 74.07 (+27.77%) | 35.94 (+37.33) | 1.00 | 22.83 | 1.36 (-9.93%) | 21.28 (-5.30%) | 53.33 (-9.33%) | 23.71 (-4.66%) | 0.93 | 5.40 | 28.23 |
|  | Duplexing flexibility(TX power 23dBm@UL carrier) | 6.66 (+43.53%) | 26.32 (+16.46%) | 68.96 (+18.96%) | 31.14 (+18.99) | 0.99 | 25.17 | 1.47 (-2.65%) | 22.27 (-0.89%) | 53.33 (-9.33) | 24.47 (-1.61%) | 0.94 | 5.50 | 30.67 |
| Note - CIM (cross-link interference mitigation) scheme is MMSE-IRC receiver and semi-static beam coordination. Semi-static beam coordination is based on long term TRP-TRP measurement. Semi-static tilt coordination is used as semi-static beam coordination. - Evaluation assumptions refer to the agreed in [2] except the following parameters: - Traffic, the DL User arrival rate λ is 0.36 - BS antenna tilt is 100 degree for baseline - The DL transmission power on UL carrier is 46dBm or 23dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

It is observed from the simulation results that DL performance can be improved due to the increased resource for the downlink transmission, even with the limited transmission power. For the UL performance, small performance degradation is observed due to the fact that less resource can be used for UL compared to the baseline. It should be noted that massive MIMO with semi-static beam coordination and MMSE-IRC receiver are used in the evaluations for the cross-link interference mitigation.

With the limited transmission power, such as maximum UE transmission power, the cross-link interference power is not seriously, as illustrated in Figure 3. Most interference power is lower than the signal power (e.g. just 4% interference power is higher than signal power), and simple interference mitigation scheme, such as semi-static beam coordination or MMSE-IRC receiver can handle this.

Figure 3 Interference and Signal received power

Based on the above simulation results, we have the following observation.

Observation 2:

Downlink data transmission in both paired spectrum can significantly improve the downlink performance of the paired spectrum in urban macro scenario, even with the limited transmission power. Evaluation results shows that with cross-link interference mitigation, it achieves considerable gain over traditional FDD with symmetric DL and UL bandwidth in downlink (e.g., up to 37% for average UPT, up to 67% for 5%-tile UPT) and similar performance in uplink.

Proposal 2: Downlink data transmission on an uplink dominant frequency carrier should be supported on a paired spectrum in duplexing flexibility.

Duplexing flexibility on unpaired spectrum @ 4GHz

In this section, the evaluation results of duplexing flexibility for 4GHz unpaired spectrum case are presented in Table 4. The DL-UL subframe ratio of 6:4 and 4:1 are used in baseline, as discussed in section 2.2. The duplexing flexibility uses dynamic subframe ratio allocation, which is adapted according to the downlink and uplink traffic load ratio.

Table 4 Performance evaluation of duplexing flexibility on unpaired spectrum @ 4GHz

| Ratio of DL/UL traffic | Feature | DL UPT (Mbps) |  |  |  |  |  | UL UPT (Mbps) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5%-tile | 50%-tile | 95%-tile | Average | Served/ offered packets | RU (%) | 5%-tile | 50%-tile | 95%-tile | Average | Served/ offered packets | RU (%) | Total RU (%) |
| 1:1 | Baseline | 13.89 | 48.19 | 78.43 | 46.62 | 1.00 | 7.51 | 0.55 | 9.09 | 30.08 | 11.41 | 0.83 | 13.98 | 21.49 |
|  | Duplexing flexibility - w/o CIM | 10.50 (-24.41%) | 42.55 (-11.70%) | 83.33 (+6.25%) | 43.70 (-6.26%) | 1.00 | 11.77 | 0.41 (-25.91%) | 6.70 (-26.30%) | 33.90 (+12.71%) | 10.59 (-7.14%) | 0.75 | 17.92 | 29.69 |
|  | Duplexing flexibility - w/ CIM | 15.56 (+12.06%) | 50 (+3.75%) | 95.24 (+21.43%) | 51.99 (+11.52%) | 1.00 | 8.79 | 0.61 (+10.93%) | 10.26 (+12.82%) | 44.44 (+47.78%) | 14.78 (+29.53%) | 0.81 | 14.26 | 23.05 |
| 2:1 | Baseline | 13.79 | 48.19 | 78.43 | 46.83 | 1.00 | 7.57 | 0.60 | 10.20 | 30.53 | 11.86 | 0.85 | 7.69 | 15.26 |
|  | Duplexing flexibility - w/o CIM | 10.96 (-20.55%) | 47.06 (-2.35%) | 97.56 (+24.39%) | 49.02 (+4.66%) | 1.00 | 11.09 | 0.41 (-30.85%) | 6.39 (-37.38%) | 35.09 (+14.91%) | 10.65 (-10.15%) | 0.77 | 9.65 | 20.74 |
|  | Duplexing flexibility - w/ CIM | 17.09 (+23.93%) | 54.80 (+13.70%) | 102.56 (+30.77%) | 57.20 (+22.13%) | 1.00 | 8.44 | 0.70 (+18.15%) | 10.72 (+5.09%) | 44.94 (+47.19%) | 15.14 (+27.73%) | 0.84 | 7.72 | 16.16 |
| 4:1 | Baseline | 13.42 | 54.80 | 102.56 | 55.70 | 1.00 | 14.70 | 0.42 | 5.04 | 16.06 | 6.27 | 0.82 | 6.73 | 21.43 |
|  | Duplexing flexibility - w/o CIM | 10.61 (-20.94%) | 44.44 (-18.91%) | 102.56 (0.00%) | 49.18 (-11.71%) | 1.00 | 19.79 | 0.41 (-2.38%) | 3.86 (-23.41%) | 29.20 (+81.82%) | 7.87 (+25.52%) | 0.74 | 9.86 | 29.65 |
|  | Duplexing flexibility - w/ CIM | 15.04 (+12.07%) | 56.34 (+2.81%) | 108.11 (+5.41%) | 57.58 (+3.38%) | 1.00 | 15.81 | 0.53 (+26.19%) | 7.71 (+52.98%) | 40.40 (+151.56%) | 12.24 (+95.22%) | 0.85 | 7.77 | 23.58 |
| Note - CIM (cross-link interference mitigation) scheme is MMSE-IRC receiver and dynamic beam coordination. Dynamic beam coordination is based on long term TRP-TRP measurement and inter-TRP information exchange (include subframe type and DL/UL traffic information), the delay for the information exchange is 4ms and period for the information exchange is 10ms. Dynamic beam coordination includes analog beam coordination and digital beamforming. - Evaluation assumptions refer to the agreed [2], except the following parameters: - Traffic - {1:1}-the DL User arrival rate λ is 0.12 - {2:1}-the DL User arrival rate λ is 0.12 - {4:1}-the DL User arrival rate λ is 0.2 - BS antenna tilt is 100 degree for baseline |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

It is observed from the simulation that there are occasions when the network needs to serve DL packet only, or UL packet only. In this case, duplexing flexibility enjoys the flexibility of allocating DL/UL subframe ratio appropriately to match the instantaneous need of DL and UL traffic, e.g., to allocate a high DL ratio when there is dominant DL traffic. In this case, the DL radio resource is boosted compared to static DL/UL subframe ratio allocation, where the UL resource ratio is always reserved irrespective whether there is UL traffic or not. Such resource increase benefits the user perceived throughput. The cross-link interference mitigation schemes are described in section 2.4.

Based on the above simulation results, we have the following observation.

Observation 3:

Duplexing flexibility with cross-link interference mitigation (CIM) in unpaired spectrum can improve both the downlink and uplink performance in urban macro scenarios. Evaluation results show that with cross-link interference mitigation (CIM), duplexing flexibility achieves considerable gain over TDD with static subframe ratio allocation in uplink (e.g., up to 95% for average UPT, up to 26% for 5%-tile UPT ) and downlink (e.g., up to 22% for average UPT, up to 23% for 5%-tile UPT).

Observation 4:

Cross-link interference mitigation (CIM) can significantly improve both the downlink and uplink performance in urban macro scenarios. Evaluation results show that cross-link interference mitigation achieves considerable gain over without cross-link interference mitigation (CIM) in uplink (e.g., up to 55% for average UPT, up to 70% for 5%-tile UPT) and downlink (e.g., up to 18% for average UPT, up to 55% for 5%-tile UPT).

Proposal 3: Cross-link interference mitigation should be supported in duplexing flexibility.

Conclusions

In this contribution, we provided some evaluation results of duplexing flexibility in urban macro scenarios. The observations are summarized as follows.

Observation 1:

SRS in the downlink dominant frequency carrier can significantly improve the downlink performance of MIMO of the paired spectrum in urban macro scenario. It achieves considerable throughput gain in downlink (e.g., cell average throughput up to 70%, cell edge throughput up to 35%).

Observation 2:

Downlink data transmission in both paired spectrum can significantly improve the downlink performance of the paired spectrum in urban macro scenario, even with the limited transmission power. Evaluation results shows that with cross-link interference mitigation, it achieves considerable gain over traditional FDD with symmetric DL and UL bandwidth in downlink (e.g., up to 37% for average UPT, up to 67% for 5%-tile UPT) and similar performance in uplink.

Observation 3:

Duplexing flexibility with cross-link interference mitigation (CIM) in unpaired spectrum can improve both the downlink and uplink performance in urban macro scenarios. Evaluation results show that with cross-link interference mitigation (CIM), duplexing flexibility achieves considerable gain over TDD with static subframe ratio allocation in uplink (e.g., up to 95% for average UPT, up to 26% for 5%-tile UPT ) and downlink (e.g., up to 22% for average UPT, up to 23% for 5%-tile UPT).

Observation 4:

Cross-link interference mitigation (CIM) can significantly improve both the downlink and uplink performance in urban macro scenarios. Evaluation results show that cross-link interference mitigation achieves considerable gain over without cross-link interference mitigation (CIM) in uplink (e.g., up to 55% for average UPT, up to 70% for 5%-tile UPT) and downlink (e.g., up to 18% for average UPT, up to 55% for 5%-tile UPT).

Proposal 1: SRS transmission on a downlink dominant frequency carrier should be supported on a paired spectrum in duplexing flexibility.

Proposal 2: Downlink data transmission on an uplink dominant frequency carrier should be supported on a paired spectrum in duplexing flexibility.

Proposal 3: Cross-link interference mitigation should be supported in duplexing flexibility.

- References

[1] 3GPP, Chairman's Notes, RAN1 NR AH1701 meeting

[2] R1-1701329, “WF on flexible duplexing evaluation”, NTT DOCOMO, Huawei, HiSilicon, ZTE, ZTE Microelectronics, Ericsson, Intel, CATT, LG Electronics, Qualcomm, Gothenburg, Spokane, USA, 16 - 20 January 2017.

[3] R1-1611229, “On SRS transmission in flexible duplex”, Huawei, HiSilicon, Reno, USA, 14-18 November, 2016.

[4] R1-1701669, “On cross-link interference mitigation for duplexing flexibility”, Huawei, HiSilicon, Athens, Greece, 13th - 17th February 2017.

- Appendix

| Parameters | Urban Macro |
| --- | --- |
| Layout | Single layer: -Macro layer: Hex. Grid 19 site, 3sector per site |
| Inter-BS distance | 500m |
| Minimum BS-UE (2D) distance | 35m |
| Minimum UE-UE (2D) distance | 3m |
| Carrier frequency | 2GHz，4GHz |
| Aggregated system bandwidth | 2GHz: Up to 40 MHz (DL+UL) 4GHz: Up to 200 MHz (DL+UL) |
| Simulation bandwidth | 2GHz : 10MHz (DL)+ 10MHz(UL) (FDD) 4GHZ: 20MHz (TDD) |
| Channel model | Below 6GHz: Macro-to-UE: 3D UMa Micro-to-UE: 3D UMi Macro-to-Macro: 3D UMa (h_UE=25m) Macro-to-Micro: 3D UMa (h_UE=10m) Micro-to-Micro: 3D UMi (h_UE=10m) UE-to-UE: A.2.1.2 in TR36.843 |
| Penetration loss | Follow [2] |
| Fast fading channel model | Below 6GHz: Macro-to-UE: 3D UMa Micro-to-UE: 3D Umi Macro to Macro: 3D UMa O-to-O (h_UE=25m); ASA and ZSA statistics* updated to be the same as ASD and ZSD; ZoD offset = 0 Macro to Micro: 3D Uma O-to-O Micro to Micro: 3D Umi O-to-O (h_UE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 UE to UE: InH for indoor to indoor, and 3D Umi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. Dual mobility support. |
| BS Tx power | Below 6GHz: 49 dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 49 dBm |
| UE Tx power | Maximum 23 dBm |
| BS antenna configuration | Below 6GHz: Baseline: (M,N,P,Mg,Ng)=(8,8,2,1,1) (dH,dV)=(0.5,0.8)λ |
| BS antenna configuration |  |
| BS antenna height | # 25m |
| BS antenna element gain + connector loss | # 8dBi |
| BS antenna tilt | 100deg |
| BS receiver noise figure | Below 6GHz: 5 dB |
| UE antenna configuration |  |
| UE antenna elements | Below 6GHz: 2Tx and 2Rx |
| UE antenna | for outdoor UEs: 1 for indoor UEs: ~uniform(1, ) where ~uniform(4,8) |
| UE antenna gain | For below 6GHz: Follow the modeling of TR36.873 |
| UE receiver noise figure | Below 6GHz: 9 dB |
| Traffic model | FTP traffic model 3 with packet size 0.5Mbytes Paired spectrum: Ratio of DL/UL traffic = {4:1}-the DL packet arrival rate λ is 0.36 unaired spectrum: Ratio of DL/UL traffic = {1:1}-the DL User arrival rate λ is 0.12 {2:1}-the DL User arrival rate λ is 0.12 {4:1}-the DL User arrival rate λ is 0.2 |
| UE distribution | 10 users per macro TRP 80% indoor (3km/h) and 20% outdoor (30km/h) |
| UE receiver | advanced IRC |
| BS receiver | advanced IRC |
| UE association | based on RSRP measurement |
| Transmission mode | SU-MIMO with rank adaptation MU-MIMO |
| Delay for dynamic beam coordination | 4ms |
| Delay for semi-static beam coordination | 20ms |
| BS ACLR | 45dB(TR 36.104) |
| BS ACS | 46dB(TR 36.104) |
| UE ACLR | 30dB(TR 36.942) |
| UE ACS | 27dB(TR 36.101) |