3GPP TSG RAN WG1 Meeting #88	R1-1701705

Athens, Greece , 13th - 17th February 2017

Agenda Item:	8.1.4.2.2

Source:	Huawei, HiSilicon

Title:	Channel coding for very small control block lengths

Document for:	Discussion and Decision

# Introduction

In RAN1 NR Ad-Hoc meeting, it was agreed that [1]

Agreement:

- The channel coding working assumptions from RAN1#87 are agreed, with clarification that the mentioned DL control information means DCI (i.e. does not include PBCH, SIBs or PCFICH (if it exists for NR))

In this contribution, we discuss the complexity and the performance of the PC-Polar codes for very small information block.

Notations:

K: 	information block length

M: 	code length

N:   power-of-two mother code length, N = 2log2M

L: 	List Size

# Discussion

A channel coding scheme for NR has to be evaluated for the following major aspects:

- Complexity – it will have lower complexity than the counterparts used in LTE.

- Performance – it will have no worse performance than counterparts used in LTE.

- Integrity – a single coding scheme is preferred over a combination of multi-code schemes.

## Complexity

One motivation to investigate the possibility to use PC-Polar code for very small information blocks is to reduce the complexity.

Both LTE-RM and Simplex codes use a ML (maximum likelihood) decoding algorithm based on FHT (Fast Hadamard Transformation). This algorithm needs to compute 2K correlation values for the 2K code words. Thus, their computational complexities are as following:

- 5*25 for K≤6

- 5*25*2(K-6)=5*2(K-1) for K>6

In contrast, the complexity of a SCL (successive-cancellation-list) decoding algorithm is:

- O(ratio*L*N*log2(N)) + O(ratio*L*(N-1) ) + K* O(2*L*log2(2*L)) [3],

where ratio is the number of frozen header bits that a SCL decoder skips.

Figure 1 shows the comparison of the complexity between RM/Simplex ML decoder with SCL-2/8 decoder for K from 5 bits to 13 bits and N=20.

Figure 1: Complexity Comparison between polar SCL decoder and RM FHT decoder

Observation-1: The complexity of ML decoder for RM code and Simplex code is higher than SCL decoder for polar codes.

## Design Principle

We adopt the PC-Polar  codes design principle in [4] for such small information blocks, in which several parity-check sub-channels are selected to transmit the parity-check bits so that a SCL decoder can make use of them for path selection. To have a coding gain comparable with an ML decoding algorithm, the code construction (puncture-bit positions, shortened sub-channels, information/PC/frozen sub-channels) can be obtained through an exhaustive search for a given (K,M). For example, one possible searching procedure can be:

- To determine the rate matching pattern that includes repetition, puncture, shorten and combinations;

- To have a new generator G’ by repeating or removing certain columns and rows of generator matrix G.

- To flag certain rows of G’ as frozen sub-channels, PC-frozen sub-channels, and information sub-channels. To measure its polarization reliability metric.

- To set these PC-frozen bits by a forward-only PC function.

- To find its minimum code distance by enumerating 2K code words

By iteratively adjusting the parameters in (i)-(v), we measure the minimum coding distance of each resultant code construction to find the one with the largest minimum code distance.

Info length = 1 bit

The code construction is simply a repetition code. It is a special case of polar code, in which all sub-channels of its N-by-N generator matrix are allocated for frozen bits except the last one for an all-one row and then an arbitrary set of (N-M) coded bits are punctured.

Info length = 2/3/4/5 bits (code length≥2K-1)

Its power-of-two mother code length N is set to be N=2K. The last (all-one) row and last column of the N-by-N generator matrix G is shortened (removed) into an (N-1)-by-(N-1) intermediate generator matrix G’. The K sub-channels with a hamming weight of (K-1) are the information sub-channels, and the rest are frozen sub-channels. The code construction table can be found in the Appendix Table A.1. This intermediate polar code is repeated several times to reach the target code length M. In the case that M cannot be divided by (2K-1), the higher-end bits in the code word are punctured. These codes are equivalent or close to some of the best codes in terms of minimum code distance [5][6].

Info length ≥ 5 bits (code length≤32)

For these cases, parity check bits can be used to enhance the minimum code distance of Polar codes. As examples, we provided detailed code construction tables for M=32 and M=20 in Appendix Table A.2 and A.3.

Info length ≥ 12 bits (code length>32)

When the code length becomes longer, we may directly apply the construction method described in [3]. For very short information block length, we simulated the cases of M=48 and 12≤K≤22 and compared them with LTE-RM. For longer information block length, the performance comparison can be found in [7].

## Performance Evaluation

We compare PC polar code with the following coding schemes at different block length:

Table 1. Simulation assumptions for very short information block length

| Channel/Modulation | AWGN/QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Code length | 20, 32, 48 |  |  |  |
| Info block length | 1 | 2~4 | 5~22 | 1~22 |
| Coding scheme | Repetition | Simplex | LTE-RM | PC-Polar |
| Decoding algorithm | ML | ML | FHT (ML) | PC-SCL List ≤ 8 |

Simplex code is a dual of Hamming code that requests a code word length of 2K-1 and has a very limited granularity. We use repetition and puncturing scheme in the simulations to match the code length, as LTE PCFICH channel extends Simplex code (K=2, N=3) to 32 bits by repetition. We use ML (maximum likelihood) decoder for both LTE-RM and Simplex code and SCL 8 or less for PC polar codes.

BLER:

We simulate information block lengths from 1 bit to 13 bits using the codes provided in the Appendix Table A.1, A.2 and A.3. Note that channel coding of LTE PCFICH is based on Simplex code (K=2 and N=32 case).

Figure 2: M=20, K=1 to 13

Figure 3: M=32, K=1 to 11

Figure 4: M=48, K=12 to 22

Observation-2: PC-Polar with SCL-8 decoder can achieve similar or better performance than LTE-RM and Simplex code.

Figure 5 shows the comparison of the ML decoder (LTE-RM) and PC-SCL decoder with list size 2 and 4, for K=1,2,3,…,7 and M=32.

Figure 5: List 2 decoder achieves near-ML performance for K=6

Observation-3: PC-SCL-2 decoder can achieve almost the same performance as an ML decoder for LTE RM codes.

The channel coding for LTE control channel utilizes RM code for short block size up to 22 bits, TBCC for larger block size and Simplex for PCFICH. This multi-code solution naturally requires multiple sets of encoders/decoders, resulting in additional hardware area and energy consumption. Given that Polar codes have already been adopted for NR control channel, Polar coding scheme can be the single-code solution for NR control information of any length. Most of the hardware logic for longer block lengths Polar code can be reused for very short block lengths [1], and the implementation overhead introduced by very short codes is trivial.

Observation-4: Polar code can be a single-code solution for NR control channel including very short block length.

# Conclusion

In this contribution, we discuss the channel coding scheme for very small block lengths. We have the following observations:

Observation-1: The complexity of ML decoder for RM code and Simplex code is higher than SCL decoder for polar codes.

Observation-2: PC-Polar with SCL-8 decoder can achieve similar or better performance than LTE-RM and Simplex code.

Observation-3: PC-SCL-2 decoder can achieve almost the same performance as an ML decoder for LTE RM codes.

Observation-4: Polar code can be a single-code solution for NR control channel including very short block length.

Based on the analysis and evaluation, polar code should be adopted as the channel coding scheme for very small block lengths.

# References

Chairman’s Notes, 3GPP TSG-RAN1 Ad-Hoc meeting, Spokane, USA, January 16 – 20, 2017

Chairman’s Notes, 3GPP TSG-RAN1 #87 meeting, Reno, USA, November 10 - 14, 2016

R1-164040, “On latency and complexity”, Huawei, HiSilicon, 3GPP TSG-RAN1 #85 meeting, Nanjing, China, May 23 - 27, 2016

R1-1611254, “Details of the Polar code design,” Huawei, HiSilicon, 3GPP TSG-RAN1 #87 meeting, Reno, USA, November 10 - 14, 2016

J. Cordaro, T. Wagner, “Optimum (n,2) codes for small values of channel error probability”, IEEE Trans. Inf. Theory, 1967, 13 (2), pp 349-350.

Bounds on the minimum distance of linear block codes over GF(2), Available online: http://www.codetables.de/BKLC/

R1-1700089, “Performance of polar codes for control channels”, Huawei, HiSilicon, 3GPP TSG RAN1 #AH meeting, Spokane, USA, January 16 – 20, 2017

# Appendix

- The PC-Polar code construction table for very small block is given below. Figure 6 is an example of how a code construction is interpreted.

- Blue slot denotes a frozen bit position

- Green slot denotes an information bit position

- Red slot denotes a shortened bit position

- Yellow slot denotes a PC-frozen bit position

- The slot with number(s) denotes its associated parity check function number(s). For example, the information slot tagged “1,2” is checked by both the 1st and the 2nd PC-frozen bits; the information slot tagged “2” is only checked by the 2nd PC-frozen bit. Moreover, the PC-frozen slot tagged “1” is the check bit of the 1st parity function, and so on.

Figure 6: An example of code construction

- Specifically, the code constructions for K=2,3,4,5 and M≥2K-1 are given in Table A.1. The code constructions for M=32 and M=20 are given in Table A.2 and Table A.3, respectively.

Table A.1 Code construction for K=2,3,4,5 and M≥2K-1

Table A.2 Code construction for M=32

Table A.3 Code construction for M=20