3GPP TSG RAN WG1 Meeting #88	R1-1701862

Athens, Greece, 13th – 17th February 2017

| Agenda item: | 7.2.4.1.2 |
| --- | --- |
| Source: | Nokia, Alcatel-Lucent Shanghai Bell |
| Title: | UTDOA Positioning in NB-IoT |
| Document for: | Discussion and Decision |

# 1. Introduction

A new work item on “Enhancements of NB-IoT” was approved at RAN#72 meeting [1] with the following agreements in RAN#73 meeting for UTDOA positioning in NB-IoT [2].

- UTDOA positioning is supported under the following conditions:

- It uses an existing NB-IoT transmission

- It can be used by Rel-13 UEs

- Any signal used for positioning needs to have its accuracy, complexity, UE power consumption performance confirmed in RAN1

- Final approval of RAN3, RAN4 CRs relating to a particular method (OTDOA/UTDOA) are conditional on this RAN1 verification

In RAN1#86bis meeting, it was concluded that [3]

- Rel-13 NPRACH preamble is the best candidate and can be used as a signal for NB-IoT UTDOA in some scenarios.

- Whether it is feasible in a given network deployment depends on issues including network coordination, interference handling, power control, in-band emissions, capacity impacts, etc.

- Companies are encouraged to identify in RAN1#87 scenarios where it is and is not feasible.

In RAN1#87 meeting, there is no conclusion about whether NPRACH is feasible for UTDOA positioning in NB-IoT or not due to the diversity of the views about its feasibility and performance [4].

In this contribution, we share our views on UTDOA positioning in NB-IoT.

# 2. Discussion

UTDOA is the network-based location technology based on the uplink sounding reference signal (SRS) from the UE and the location measurement unit (LMU) at the network side. The main purpose of SRS is for uplink channel estimation in the existing LTE/LTE-A network. However, NB-IoT does not support the feature of uplink channel estimation and consequently has no corresponding uplink reference signal. Therefore, RAN1#86bis and RAN1#87 meetings discussed the feasibility of NPRACH as the alternative uplink reference signal for UTDOA positioning in NB-IoT. Due to the diversity of the views about its feasibility and performance, if using NPRACH to support UTDOA positioning in NB-IoT is still in RAN1 discussion [4].

In NB-IoT, the density of connected NB-IoT UEs may not be uniform, which leads to some cells having very high numbers of connected NB-IoT UEs, so that UTDOA positioning in NB-IoT needs to handle simultaneous positioning services for massive NB-IoT UEs and meanwhile guarantees enough hearability of uplink reference signal for time of arrival (ToA) measurement. It might be a big challenge for NPRACH resource allocation and corresponding ToA measurement. On the other hand, the lower transmission power at the UE side will further impact the ToA estimation at the network side due to the multipath transmission and building penetration loss, especially in urban scenarios.

Observation 1: NPRACH resource allocation and following ToA measurement might become challenging due to possible need for simultaneous positioning services for massive NB-IoT UEs and lower transmission power UEs.

In the UTDOA positioning with NPRACH as the positioning reference signal, the NPRACH from one UE for ToA measurement needs to be received by multiple LMUs with higher ToA measurement accuracy. To minimize the intra-cell/inter-cell interference, the time-frequency resources for this kind of NPRACH transmission of one UE are not expected to be utilized by other UEs which might result in stronger interference to these LMUs. Therefore, numbers of cells might be impacted in order to support the UTDOA positioning of one UE.

In the example given in Fig. 1, the network is split into positioning area, measurement area, protection area and unaffected area, where the LMUs in the positioning area and the measurement area need to measure the ToA of the NPRACH of one UE allocated in the positioning area; the other UEs in the positioning area, the measurement area and the protection area are not expected to transmit uplink signal and data for NPRACH hearability improvement; and only the UEs in the unaffected area would not be impacted. Therefore, in this example, all other UEs within 61 cells might be impacted. Suppose there are x UEs in this positioning area and each UE needs y time-frequency resources to transmit NPRACH during the considered period for the ToA measurement at the network side. Then the total number of used time-frequency resources for the UE positioning in one cell is 61*x*y during this period. Similarly, to support the UE positioning of 61 cells, the total number of needed time-frequency resources for NPRACH transmission would be 61*61*x*y during this period. Therefore, the resource usage to support UTDOA positioning of massive NB-IoT UE is unacceptable and the support of UTDOA positioning in NB-IoT is not practically feasible.

Fig. 1 UTDOA positioning

Moreover, due to the lower transmission power at the NB-IoT UE side as well as the single-tone transmission of NPRSCH, the implementation complexity of LMUs at the network side to support ToA measurement might be very high while the obtained positioning accuracy is possibly limited. Therefore, the support of UTDOA positioning in NB-IoT is not practically feasible with the considerations of NPRACH resource allocation, obtained positioning accuracy and network implementation complexity.

Observation 2: The support of UTDOA positioning in NB-IoT is not practically feasible.

Since it is expected to complete the work item for Rel-14 NB-IoT enhancement by March 2017, it is better to focus on topics which have been concluded to be feasible.

Proposal: Focus the remaining NB-IoT enhancement work on the objectives which have been concluded to be feasible.

# 3 Conclusions

In this contribution, we share our views on UTDOA positioning in NB-IoT with the following observations and proposal:

Observation 1: NPRACH resource allocation and following ToA measurement might become challenging due to possible need for simultaneous positioning services for massive NB-IoT UEs and lower transmission power UEs.

Observation 2: The support of UTDOA positioning in NB-IoT is not practically feasible.

Proposal: Focus the remaining NB-IoT enhancement work on the objectives which have been concluded to be feasible.

# References

- RP-161324, “New work item proposal: Enhancements of NB-IoT”, Vodafone, Huawei, HiSilicon, Ericsson, Qualcomm, 3GPP TSG RAN Meeting #72, Busan, Korea, 13th – 16th June, 2016.

- RP-162486, “Report of 3GPP TSG RAN meeting #73”, 3GPP TSG RAN meeting #74, Vienna, Austria, December 5 – 8, 2016.

- R1-1611081, “Final report of 3GPP TSG RAN WG1 #86bis”, 3GPP TSG RAN WG1 Meeting #87, Reno, USA, 14th – 18th November 2016.

- R1-16xxxx, “Draft report of 3GPP TSG RAN WG1 #87”, 3GPP TSG RAN WG1 Meeting #88, Athens, Greece, 13th – 17th February 2017.

- 3GPP TS 36.111, “3GPP; Technical Specification Group Radio Access Network; LMU performance specification; Network based positioning systems in E-UTRAN”.

- 3GPP TS 36.211, “3GPP; Technical Specification Group Radio Access Network; E-UTRA; Physical channels and modulation”.

- 3GPP TS 36.305, “3GPP; Technical Specification Group Radio Access Network; E-UTRAN;
Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN”.

- 3GPP TS 36.455, "3GPP; Technical Specification Group Radio Access Network; E-UTRA; LTE Positioning Protocol A (LPPa)".

- 3GPP TS 36.459, “3GPP; Technical Specification Group Radio Access Network; E-UTRA; SLm interface Application Protocol (SLmAP)”.

- Nokia, “LTE evolution for IoT connectivity”, whitepaper, July 2016.

- R1-166588, “UTDOA positioning in NB-IoT”, Nokia, Alcatel-Lucent Shanghai Bell, 3GPP RAN1#86 meeting.

- R1-166590, “Comparison of UTDOA and OTDOA in NB-IoT”, Nokia, Alcatel-Lucent Shanghai Bell, 3GPP RAN1#86 meeting.

- R1-1608880, “UTDOA positioning in NB-IoT”, Nokia, Alcatel-Lucent Shanghai Bell, 3GPP RAN1#86bis meeting.

- R1-1611305, “UTDOA positioning in NB-IoT”, Nokia, Alcatel-Lucent Shanghai Bell, 3GPP RAN1#87 meeting.

- R1-1611306, “Considerations on support of Rel-13 UEs for UTDOA”, Nokia, Alcatel-Lucent Shanghai Bell, 3GPP RAN1#87 meeting.

- RP-162257, “UTDOA positioning in NB-IoT”, Nokia, Alcatel-Lucent Shanghai Bell, 3GPP RAN#74 meeting.