3GPP TSG-RAN WG1 #88	                                  	                  R1-1702011

Athens, Greece, February 13th-17th 2017

Source:	Mitsubishi Electric

Title:	Transmit diversity for PUCCH in long duration

Agenda Item:	8.1.3.2.2

Document for: 	Discussion/Decision

# Introduction

At the latest RAN1 NR AH meeting the following was decided concerning the waveform and transmission scheme for long duration PUCCH:

- For PUCCH in long-duration, DFT-s-OFDM waveform is supported.

- For PUCCH in long-duration, transmit antenna diversity is supported.

Moreover, it was decided that:

- For PUCCH in long-duration,

  - Long UL-part of a slot can be used for transmission of PUCCH in long-duration.

    - i.e., PUCCH in long-duration is supported for both UL-only slot and a slot with the number of uplink symbols greater than X (X >= 2).

      - FFS exact value of X

  - In addition to simultaneous PUCCH-PUSCH transmission, UCI on PUSCH is supported.

  - Intra-slot frequency-hopping is supported

In this contribution, we show how Alamouti-based PAPR preserving transmit diversity can be achieved for DFTsOFDM without any restrictions for the number of symbols in the UL-only/UL-centric slot.

# Transmit diversity for DFTsOFDM long duration PUCCH

PUCCH coverage is a bottleneck for system deployments. In NR, UL control will carry higher payloads than in LTE, but at least the same coverage is desirable to be achieved. DFTsOFDM and transmit diversity support were decided for long PUCCH transmission at the last meeting, since low PAPR waveform and robust transmission are crucial for long PUCCH transmission.

Concerning the transmit diversity scheme for DFTsOFDM long PUCCH, several design constraints need to be met. Transmit diversity should conserve the low PAPR characteristics. The length (in DFTsOFDM symbols) of the long PUCCH is variable: 7 or 14 symbols for UL-only slots, up to 4/5 or 11/12 symbols for UL-centric slots (accounting for 1-2 DL control symbols and 1 gap symbol). Therefore, the transmit diversity scheme should remain contained within one DFTsOFDM symbol and not impose parity constraints on the slot length. Also, due to the higher maximum number of bits to be carried by the NR PUCCH format with respect to LTE, schemes requiring complex decoding (e.g. ML) should be avoided.

We will further investigate how several known transmit diversity techniques apply to DFTsOFDM for long PUCCH format.

## Alamouti-based transmit diversity for long PUCCH

Alamouti-based schemes achieve full diversity and are known to yield good results in combination with OFDM-based waveforms. STBC and SFBC schemes are classically implemented at subcarrier level in multicarrier systems. Nevertheless, directly applying the classical schemes has some drawbacks for long duration PUCCH with DFTsOFDM waveform:

- Applying classical STBC between successive DFTsOFDM symbols requires having an even number of symbols per slot, which is not the case in practice as we have shown in the previous subsection

- Applying classical SFBC between adjacent subcarriers within a DFTsOFDM symbol breaks the low PAPR characteristics of DFTsOFDM.

Modified Alamouti-based schemes overcoming these shortcomings exist.

Modified STBC schemes applying STBC precoding before DFT between successive modulation (e.g. QPSK) symbols were already investigated [2]. Since precoding is not applied at subcarrier level, low-complexity frequency-domain decoding (e.g. MMSE involving order-2 matrix inversions at the receiver side) is not possible. Moreover, for pre-DFT Alamouti precoding, some performance degradation may appear due to channel selectivity: for each transmit antenna, there may be a different interference level for the first symbol and the second symbol of a pair. For post-DFT STBC schemes, doubling the orphan symbol (for example by using a double subcarrier spacing onto this symbol) results in either inter-numerology interference affecting the users scheduled in adjacent frequency bands, or in scheduling constraints imposing the same numerology settings to all users scheduled in adjacent frequency bands.

Modified SFBC preserving the low PAPR [3] will be further detailed in the following. More detailed analysis can be found in [4].

### PAPR preserving SFBC

Classical SFBC schemes applying Alamouti precoding to adjacent subcarriers in DFTsOFDM are known for breaking the PAPR property. Nevertheless, PAPR-preserving SFBC has already been discussed in the literature [2]. Alamouti precoding is applied at subcarrier level, after DFT precoding, just as in classical SFBC, as it can be seen in Figure 1. Alamouti precoded symbols can be mapped onto non-adjacent subcarriers in such a way that PAPR is preserved. Figure 2 depicts the subcarrier pairing: Alamouti pairs are mapped onto subcarriers and . M is the number of allocated subcarriers (DFT precoder size) and p is an even integer, usually chosen as closest possible to M/2 to minimize the distance between subcarriers carrying Alamouti precoded pairs. This PAPR-preserving SFBC is called SC-SFBC [2].

In the frequency domain, this corresponds to sending on the second antenna port:

Where vector  represents a block of DFT precoded modulation symbols x which will be mapped as is onto the first antenna port. This is a DFTsOFDM signal. Since complex conjugation, time reversing, cyclic shifting and sign alternation applied in the frequency domain are PAPR preserving operations with respect to the time-domain signal, the resulting signal transmitted on the second antenna port has the same PAPR as the original signal. Indeed, as further explained in [4], this is equivalent to sending on the second antenna a DFTsOFDM signal corresponding to an equivalent constellation :

.

Figure 1 – PAPR preserving SFBC for DFTsOFDM long PUCCH

It is obvious that complex conjugation, cyclic shift or phase ramp applied in the time domain conserve the peak to average ratio of the input constellation and that the signal on the 2nd transmit antenna is a DFTsOFDM signal with the same PAPR as the one on the 1st transmit antenna. This is verified in Figure 3 where the CCDF of the instantaneous normalized power is plotted for DFTsOFDM with STBC, SFBC and the PAPR preserving SC-SFBC.

Figure 2 - Mapping of Alamouti pairs to subcarriers for PAPR preserving SFBC in DFTsOFDM

Figure 3 - PAPR of PAPR-preserving SFBC in DFTsOFDM

Therefore, PAPR preserving SFBC shows the following characteristics:

- Full-diversity achieving

- Applicable to any number of symbols per slot

- PAPR conserving

- Low-complexity frequency-domain MMSE detection based on order-2 matrix inversions.

## Other open loop transmit diversity schemes in DFTsOFDM

Data block x(i)

Figure 4 - Open loop transmit antenna selection (OL-TAS) and cyclic delay diversity (CDD)

Figure 5 – SORTD with DFTsOFDM

Open loop transmit antenna selection (OL-TAS) and cyclic delay diversity (CDD) represented in Figure 4 are also known transmit diversity schemes that preserve the PAPR of DFTsOFDM.

CDD transforms the spatial diversity into frequency diversity and recovers the extra diversity through coding. For single layer transmission this is achieved by sending onto a second antenna port a cyclically delayed copy of the signal sent on a first antenna port, which is equivalent to SIMO transmission over a channel with modified impulse response. Cyclic delay operation can be implemented as a phase ramp applied in the frequency domain, similar to a precoding operation. This operation is DM-RS transparent if the delay is short enough. Nevertheless, as it will be shown in the following, CDD has limited gain in already frequency selective environments, such as PUCCH transmission supporting frequency hopping.

OL-TAS performs switching between transmit antennas during a codeword transmission. Sufficient DM-RS need to be inserted in order to estimate channels from all the transmit antenna ports in order to support switching during a codeword transmission, which leads to denser DM-RS pattern in the time domain than Alamouti-type precoding or CDD, at least for low/medium-speed scenarios (DM-RS overhead is NTx times higher for OL-TAS in order to yield similar channel estimation reliability). OL-TAS is sensitive to high speed and is generally outperformed by Alamouti-based schemes and CDD [4]. We will not further investigate this scheme here.

With spatial orthogonal resource transmit diversity (SORTD) used in LTE-A, as depicted in Figure 5, one orthogonal resource, e.g. one orthogonal sequence (OCC), is used for each transmit antenna. Compared to other schemes, SORTD requires two OCC resources per UE instead of one. Thus, the multiplexing capacity is reduced from 5 to 2.5 UEs per PUCCH PRB. Even though its performance is lower than that of Alamouti schemes [2], this scheme was preferred in LTE-A for specification effort reduction, since not much specification effort was needed to add this scheme to pre-existing LTE specifications. This is not a valid argument any longer and performance should be the prime argument, since NR specifications need to be written from scratch. We will not further investigate this scheme here.

# Initial Evaluation

We present initial simulation results comparing the performance of PAPR-preserving SC-SFBC, SFBC and CDD for DFTsOFDM long PUCCH transmission. In this initial evaluations, we use the following simulation parameters

| Assumptions , | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Slot length | 14 DFTsOFDM symbols with NCP, 1ms |
| System bandwidth | 20MHz |
| Data bandwidth | 1PRBs (12 subcarriers) with frequency hopping |
| Numerology | Subcarrier spacing ∆f=15kHz, NIFFT=2048, Fs=30.72MHz |
| UE antenna model | 2 Tx |
| TRP antenna model | 2 Rx |
| Phase noise model | - |
| Channel coding | LTE Turbo code |
| HPA | Polynomial model for UL [5], IBO=-8dB |
| MCS | QPSK1/2 |
| Channel estimation | Realistic, based on LTE-type pilots |
| Channel model | TDL-A 300ns |
| Receiver type | L-MMSE |
| CDD delay | 64 samples |

As shown in [4], for higher bands transmit diversity schemes can be combined with analog beamforming as long as we are able to define two DM-RS antenna ports serving as support for 2Tx transmit diversity.

SFBC is only given as a FER performance reference and is not proposed to be used for DFTsOFDM long PUCCH as it engenders around 1dB PAPR loss at CCDF target 10-4.

Pre-DFT STBC schemes are not investigated here: although diversity achieving, a more complex receiver is needed than for SFBC/CDD type schemes.

Figure 6 presents comparative performance of SC-SFBC, SFBC, and SIMO transmission for long PUCCH with DFTsOFDM.

Full diversity Alamouti schemes are more robust than CDD, even with the current sub-optimal L-MMSE decoder.

CDD is easy to implement and can be transparent to the receiver in the case of short delay CDD. Nevertheless, CDD has very variable performance depending on the applied delay, and an optimized delay (depending on the channel profile, allocation size, modcod) cannot be applied in an open loop scenario. Also, since CDD recovers frequency diversity through coding, CDD performance depends on the coding rate and tends to degrade and in scenarios where enough diversity is already present (frequency selective channels/frequency hopping transmission where the extra diversity obtained through CDD is limited, high speed where the extra diversity wrt to already present time diversity has limited gain, etc).

Even in highly selective environments, where SC-SFBC may slightly suffer from precoding onto distant subcarriers susceptible of having different channel realizations, The PAPR preserving SC-SFBC matches the FER performance of SFBC. Since PUCCH allocations are rather short, there is no loss due to precoding on distant subcarriers.

The PAPR preserving SC-SFBC matches the FER performance of SFBC without any PAPR loss and is thus preferred to SFBC. SC-SFBC outperforms SFBC when taking into account FER performance and PAPR performance.

Figure 6 - Fc=4GHz, TDL-A 300ns, 3kmph, real channel estimation, 1RB with frequency hopping, LTE-type pilots

# Conclusion

Observation 1: PAPR preserving SFBC shows the following characteristics:

- Full-diversity achieving

- Applicable to any number of symbols per slot

- PAPR conserving

- Low-complexity frequency-domain MMSE detection based on order-2 matrix inversions

Observation 2: Full diversity Alamouti schemes are more robust than short delay CDD

Observation 3: The PAPR preserving SC-SFBC matches the FER performance of SFBC without any PAPR loss and is thus preferred to SFBC. SC-SFBC outperforms SFBC when taking into account FER performance and PAPR performance.

Proposal 1: Support Alamouti-based transmit diversity schemes for long PUCCH with DFTsOFDM.

Proposal 2: Consider PAPR preserving SC-SFBC as transmit diversity scheme for long PUCCH with DFTsOFDM.

# References

### Chairman’s notes, RAN1 NR AH 01, Spokane, Washington, January 2017.

### R1-105709, Comparison of 2-Tx transmit diversity schemes for PUCCH Format 3, RAN1#62b.

### C. Ciochina, D. Castelain, D. Mottier and H. Sari, “A Space-Frequency Block Code for Single-Carrier FDMA,” Electronics Letters 44(11):690 - 691 February 2008.

### R1-1701925 PAPR preserving SFBC for DFTsOFDM, Mitsubishi Electric, RAN1#88, Athens, Feb. 2017.

### R1-166004, “Response LS on realistic power amplifier model for NR waveform evaluation”, RAN4 LS, RAN1#85, Nanjing, China, May 2016.