3GPP TSG RAN WG1 Meeting #88                                                               	         R1-1702252

Athens, Greece, 13th – 17th February 2017

Agenda item:	8.1.8

Source: 	Intel Corporation

Title: 	Remaining LTE-NR coexistence issues and handling

Document for:	Discussion and Decision

- Introduction

In the last RAN WG1 NR Ad-Hoc Meeting, the following agreements were made regarding the LTE-NR coexistence [1].

Agreements: - NR supports efficient adjacent channel co-existence with LTE-TDD using UL-DL configurations 0,1,2,3,4,5 in unpaired spectrum - FFS detailed mechanism - NR supports efficient adjacent channel co-existence with LTE-TDD using all the special subframe configurations in unpaired spectrum - Notes: - The above bullets does not necessarily imply that two or more frame structures are to be defined for NR - The wording “efficient” in the above two bullets does not imply exact alignment of configurations - RAN1 has agreed the following - Design at least one semi-statically assigned DL/UL transmission direction configuration for NR that avoids DL/UL interference with at least one LTE TDD DL/UL configuration and special subframe configuration Agreements: - Supporting NR DL in MBSFN subframes of LTE - FFS details

In this contribution, various remaining LTE-NR coexistence issues will be discussed. In particular, the discussions in this contributions relate to the scenarios when the LTE/NR coexist in the adjacent channels and when the NR utilizes LTE MBSFN subframes for DL transmission.

- Remaining LTE-NR coexistence issues

  - Adjacent channel coexistence

Per the agreement made in the last meeting, NR supports ‘efficient’ adjacent channel coexistence with LTE-TDD in unpaired spectrum. In this subsection, we try to address how the adjacent channel coexistence could be ‘efficient’ for collocated and non-collocated cases.

Collocated LTE-NR base stations:

It is reasonable to assume that single RF front-end will be shared between eNB and gNB, which mandates that the transmission directions of the adjacent channels must be perfectly aligned. Although it is unlikely, even if the collocated eNB and gNB use different RF front-end, the effect of self-interference would not allow the adjacent carriers to operate in different directions. In this regard, the most straightforward way to support efficient adjacent channel co-existence between LTE-TDD and NR would be to make DL/UL directions perfectly aligned.

However, if some LTE UL subframes on all the adjacent LTE carriers are unused due to, e.g., no UL data, the eNB may instantly inform the collocated gNB such that those unused UL subframes can be flexibly used for either UL or DL. This operation is illustrated in Figure 1 below. When NR transmits DL on the unused LTE UL subframes on the adjacent carriers, gNB may perform power control to reduce the cross-link interference leakage to adjacent channels for other neighboring base stations. The UL subframes that can be dynamically switched can be further restricted to a certain set, called ‘flexible subframes’, in consideration of other neighboring base stations operating on the same band.

Figure 1. Dynamic switching of flexible UL subframes if adjacent carriers are not used by collocated eNB

On the other hand, even if some DL subframes are unused by collocated eNB on the adjacent carriers, NR cannot arbitrarily use it for UL transmission because CRS and some control messages are still needed to be transmitted, i.e., the RF needs to be in the mode of transmission.

Proposal 1: If ‘flexible’ LTE UL subframes on all the adjacent LTE carriers are unused, the collocated gNB can dynamically decide to use those subframes for either DL or UL.

Non-collocated LTE-NR base stations belonging to the same network operator:

In non-collocated scenarios, the transmission direction alignment between adjacent carriers is not a critical issue as neither the non-collocated base stations share the same RF nor there exists self-interference. In some situations, however, it may be still beneficial to align the transmission direction of adjacent channels between non-collocated base stations, if it happens to be that LTE UE in DL reception is close by the NR UE in UL transmission and, vice versa. However, this additional optimization from the perspective of cross-link interference leakage to adjacent channels can restrict ‘efficient’ operation of gNB from traffic adaptation perspective. By giving some flexibility, the gNB can better adapt to varying DL/UL traffic demand. For instance, the NR static DL/UL frame structure is designed to be constituted of DL, UL, S (special), and F (flexible) subframe types. The S refers to subframes in line with LTE TDD special subframes. The new subframe type F is intended to be flexibly used for either DL or UL up to gNB decision. The overall NR static DL/UL frame structure for coexistence scenarios can be, for example, chosen to have maximum commonality with the LTE TDD configurations used by the operator.

Figure 2. An example NR static DL/UL configuration with ‘flexible’ subframes

As in the above figure, if the MNO uses TDD UL/DL configurations 1 and 2 most of the times, then one NR static DL/UL configuration may be set with the maximum commonality between the two configurations while leaving the subframes having non-coinciding directions as F subframes. In utilizing the F subframes, the gNB may schedule cell-center UEs with reduced power to minimize the potential impact to neighbouring base stations operating on the same channel or adjacent channels.

Proposal 2: In the case of non-collocated scenarios, the NR static DL/UL frame structure can be constituted of DL, UL, S (special), and F (flexible) subframe types, where the subframe type F is intended to be flexibly used for either DL or UL.

Non-collocated LTE-NR base stations between different network operators:

In some frequency band allocation, it is also possible that the adjacent channels are assigned to different operators and one operator have upgraded its network to NR through LTE spectrum refarming while the other operator is still operating with LTE. In this inter-operator scenario, enforcing to strictly abide by the agreed TDD UL/DL pattern seems reasonable as the information exchange between BSs belonging to different operators would be difficult and will experience long delay. Therefore, in such scenarios, the NR network should follow the configured TDD pattern, which can be specified in the NR specification.

  - NR DL in LTE MBSFN subframes

In the last meeting, it was agreed to ‘support NR DL transmissions in MBSFN subframes of LTE.’ In LTE, certain subframes are not to be configured for MBSFN subframe until Release 13. For instance, in FDD, subframe 0, 4, 5, and 9 cannot be used for MBSFN due to the overlap with synchronization signals and/or paging occasions. Similarly, in TDD, subframe 0, 1, 2, 5 and 6 cannot be configured for MBSFN subframe.

Figure 3. Potential MBSFN subframes in LTE FDD (until Release 13)

Such restrictions on the configurable subframes for MBSFN subframe by LTE, in return, implies that NR design should be able to configure its periodic transmissions or possible periodic transmission occasions on whichever subframes configured by LTE for MBSFN subframes.

- Synchronization signals: In LTE, the synchronization signals are transmitted in the fixed time/frequency resources. The NR synchronization signal transmission could be an issue if their occasions overlap with subframes that cannot be configured for MBSFN subframe. In this regard, the NR synchronization signals should be designed flexible enough such that it can be transmitted in any configured MBSFN subframes. For instance, the NR synchronization signals can be transmitted in non-fixed slot by embedding the slot information in the sequence itself. Alternatively, the NR frame boundary can have an offset with respect to the LTE frame boundary to avoid such overlap.

- Paging occasions: In LTE, the paging instance, i.e., radio frame and subframe indices, are derived as a function of UE DRX cycle and UE ID. Depending on the network configuration in SIB2, possible paging occasions can be subframe 9, subframe 4 or 9, or subframe 0, 4, 5, or 9 for FDD. Similarly, the possible paging occasions are defined for TDD as well. From this, it can be seen that the LTE paging occasions are, although configurable, not fully flexible. Therefore, unless a particular assumption is made such that the shared LTE carrier via MBSFN can only be used as an NR SCell, NR paging design should also take into account the LTE-NR coexistence scenario. In other words, the NR paging should be designed flexible enough such that the paging occasions can be configured to whichever subframes configured for MBSFN subframes by LTE.

- MIB/SIB transmissions: In LTE, MIB is transmitted every frame on subframe 0. Similarly, SIB1 is transmitted in subframe 5 in radio frame satisfying SFN mod 2 = 0. Other SIBs are scheduled by SIB1. Unless a particular assumption can be made such that NR UEs do not camp on LTE-NR shared carrier, the NR system information transmission design should allow that it can be transmitted and received in whichever subframes configured for MBSFN subframes by LTE.

Based on the preceding discussion, we drive the following proposals and observation.

Proposal 3: NR synchronization signals should be designed flexible enough such that it can be transmitted in any configured MBSFN subframes.

Observation 1: If the shared LTE carrier via MBSFN can only be used as an NR SCell, no considerations are needed for the paging occasions and system information transmissions on the shared carrier due to the coexistence.

Proposal 4: If the shared LTE carrier via MBSFN can be configured as an NR PCell, NR paging and system information transmissions should be designed flexible enough to be configurable to whichever subframes set for MBSFN subframes by LTE.

  - Restriction on the NR slot types

It was agreed for NR to ‘study flexible/dynamic TDD, including both downlink and uplink transmissions in the same subframe interval.’ Following this agreement, indeed any combination of DL/UL multiplexing within an NR slot can be conceived. The following figure illustrates probable NR DL/UL slot structure. Again, with the additional multiplexing of URLLC or special purpose reference signal transmissions, e.g., for interference measurement, there could be numerous possible constructions of DL/UL slot structures.

Figure 4. Possible NR DL/UL slot structure

Note that in sharing the spectrum between LTE and NR, there are cases that certain subframes are supposed to be used for either DL or UL only. Those cases are, for instance,

- Sharing the same carrier in UL frequency band.

- Sharing the same carrier in DL frequency band, e.g., via LTE MBSFN configuration.

- Adjacent channel coexistence in unpaired spectrum.

If a certain shared LTE subframe is supposed to be used for a particular direction, NR should operate accordingly. This requirement may be enforced due to the cross-link interference issue and/or the regulations on use of FDD bands, e.g., DL band is not allowed to be used for UL transmission. In this regard, NR design should provide DL-only and UL-only slots. In the case of UL slots, in particular, it should not be assumed that UL slots always start with DL control channel transmission. This requires that UL data needs to be scheduled in the earlier DL slots (in the case of unpaired spectrum without cross-carrier scheduling) or by another DL carrier.

Proposal 5: NR design should provide DL-only and UL-only slots. In particular, it should not be assumed that UL slots always start with DL control channel transmission.

- Conclusions

In this contribution, various remaining LTE-NR coexistence issues were discussed and the following proposals and observations were derived.

Proposal 1: If ‘flexible’ LTE UL subframes on all the adjacent LTE carriers are unused, the collocated gNB can dynamically decide to use those subframes for either DL or UL.

Proposal 2: In the case of non-collocated scenarios, the NR static DL/UL frame structure can be constituted of DL, UL, S (special), and F (flexible) subframe types, where the subframe type F is intended to be flexibly used for either DL or UL.

Proposal 3: NR synchronization signals should be designed flexible enough such that it can be transmitted in any configured MBSFN subframes.

Observation 1: If the shared LTE carrier via MBSFN can only be used as an NR SCell, no considerations are needed for the paging occasions and system information transmissions on the shared carrier due to the coexistence.

Proposal 4: If the shared LTE carrier via MBSFN can be configured as an NR PCell, NR paging and system information transmissions should be designed flexible enough to be configurable to whichever subframes set for MBSFN subframes by LTE.

Proposal 5: NR design should provide DL-only and UL-only slots. In particular, it should not be assumed that UL slots always start with DL control channel transmission.

References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting NR Ad-Hoc, January, 2017