3GPP TSG RAN WG1 Meeting #88 	                           		  R1-1702323

Athens, Greece, 13th - 17th February 2017

Agenda Item: 	8.1.2.1.5

Source:	InterDigital Communications

Title:	Reciprocity-based UL Beamformed Transmit Diversity

Document for:	Discussion and Decision

# Introduction

In RAN1 meeting NR-AH [1], discussion on UL transmission schemes continued and the following agreements were reached;

- Support at least the following UL transmission schemes for data in NR

  - Scheme A: Codebook based UL transmission

    - Support frequency selective precoding for CP-OFDM when the number of transmission port is greater than X (FFS: Value of X).

    - Study codebook design including single-stage and multi-stage, e.g., W1W2 structure, codebook

    - Study the following DL signaling, e.g.,

      - One level DCI

      - Two level DCI

      - MAC CE

      - DCI associated with PDSCH (like UCI associated with PUSCH in LTE)

  - Scheme B: Non-codebook based UL transmission

    - Support frequency selective precoding for CP-OFDM when the number of transmission port is greater than Y (FFS: Value of Y).

    - Support the indication of DL measurement RS for UE to calculate candidate precoder

    - Study the mechanisms for UL precoder determination, e.g. precoded SRS based, non-precoded SRS based, hybrid precoded and non-precoded SRS based

  - Diversity-based transmission schemes

    - FFS: Whether the scheme has specification impact or not

  - FFS: Merging of the schemes

- Support rank determination by gNB

- Support PRB bundling for CP-OFDM

  - Study configurability of PRG size for CP-OFDM

  - Study the PRG size

- FFS: Single port transmission is supported for the UE capable for multiple antenna port transmission.

Another use case of reciprocity-based UL precoding is the case of transmit diversity (TX-D) mode where by its open loop nature, it does not benefit from any form of spatial feedback. Potential use of partial reciprocity for UL transmission has been discussed in an earlier contribution [2]. In this contribution, we present our views and our preliminary investigations on the reciprocity-based beamforming for UL transmit diversity.

# UL Beamformed Transmit Diversity

Transmit diversity is an open loop transmission mode where the only feedback available to the UE is to assist the adaptive modulation coding aspect of the transmission. In this mode, given UE mobility state or low SNR condition, spatial feedback would be either outdated or very inaccurate. In LTE, transmit diversity is implemented by a fixed precoding that is defined based on the SFBC transmit diversity scheme. Table 1 shows the Antenna/RE mapping of s1 and s2 symbols in a 2 TX SFBC-based transmit diversity transmission [3]. Therefore, regardless of the relative spatial position of the UE with respect to the base station, the transmit power is applied uniformly across the antennas.

Table 1 – 2 TX SFBC Antenna/RE Symbol Mapping

| Antenna RE | 1 | 2 |
| --- | --- | --- |
| f1 | s1 | s2 |
| f2 | -s2* | s1* |

The performance of the uplink transmit diversity may be improved by creating a properly directed beam pattern  for transmission of the TX-D coded blocks. The overall transmitter operation can be stated as follows:

y=HW1W2s

where W1 and W2 are  the directional beam manifold and the SFBC TX-D precoder, respectively. Hence, after applying the fixed SFBC precoder W2, the directional beam weigth W1 is applied to direct the transmission to a better spatial direction. Figure 1 captures the basic concept of the beamformed transmit diversity implementation. As demonstrated, rather than transmission of the TX-D coded blocks in almost all directions, we could correct the radiation pattern by applying a rotation.

Figure 1 – Operation of the beamformed transmit diversity scheme

The beamformer W1 can be determined based on the estimated uplink channel HUL. Assuming channel reciprocity, the uplink channel estimate can be derived from the downlink channel estimate HDL. In a TDD system, the channel reciprocity may be based on a direct or a statistical estimation of the channel. However in a FDD system, it could be only based on a statistical estimation of the channel.

Having HUL, the beamformer W1 can be determined as a unitary vector that attempts to maximize the transmission of the energy towards the base station.

# Preliminary Evaluation

A 2×2 transmit diversity system is considered. We assumed LTE Turbo Code and packet size of 200 bits. Figure 2 compares the raw BER, coded BER and BLER performance results of a basic SFBC against a beamformed SFBC transmission schemes. It is assumed that the UL channel is estimated by relying on the channel reciprocity. Then, the W1 is determined as the precoder/beamformer matrix that simply maximizes the transmitted energy towards the base station. As demonstrated in Figure 2, the beamformed SFBC transmit diversity scheme outperforms the basic SFBC transmission by more than 1 dB. Moreover, an examination of the presented results indicate that the diversity degree of the beamformed SFBC is maintained as the original SFBC transmission.

Observation 1 – Beamformed SFBC transmit diversity scheme can provide a superior performance compared to the basic SFBC scheme.

Observation 2 – The diversity degree of the transmission of the beamformed SFBC is the same as the basic SFBC transmission.

Proposal 1 – RAN1 considers further study on the beamformed transmit diversity scheme.

Figure 2 – BER and BLER evaluation of the beamformed TX-D

# Summary

This contribution presented and discussed the concept of beamformed transmit diversity transmission. The preliminary investigation indicate a notable performance improvement over the basic transmit diversity scheme.

Observation 1 – Beamformed SFBC transmit diversity scheme can provide a superior performance compared to the basic SFBC scheme.

Observation 2 – The diversity degree of the transmission of the beamformed SFBC is the same as the basic SFBC transmission.

Proposal 1 – RAN1 considers further study on the beamformed transmit diversity scheme.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting NR-AH, Spokane, USA, January 2017

- R1-1609903, “Partial Reciprocity Based UL MIMO Schemes”, InterDigital Communications, RAN WG1 #86b, October 2016, Lisbon, Portugal

- 3GPP TS 36211, “Physical Channels and Modulation”.