3GPP TSG RAN WG1 Meeting #88			    			            	R1-1702342

Athens, Greece 13th - 17th February 2017

# Source:	ETRI

# Title:		Delivery of remaining system information

# Agenda item:	8.1.1.2.2

# Document for:	Discussion/Decision

Introduction

In this contribution, we discuss on delivery of remaining system information including remaining minimum SI (system information) and other SI.

Delivery of System Information

Remaining Minimum System Information

The following agreements on the transmission of minimum system information were made in RAN1 #87 meeting [1]:

Agreements: - Consider followings for minimum system information transmission: - NR-PBCH is a non-scheduled broadcast channel carrying at least a part of minimum system information with fixed payload size and periodicity predefined in the specification depending on carrier frequency range - Alt. 1: NR-PBCH carries a part of minimum system information - Alt 1-1: Remaining minimum system information is transmitted via other channel at least partially indicated by NR-PBCH - Alt 1-2: Remaining minimum system information is transmitted via other channel not indicated in NR-PBCH - Alt. 2: NR-PBCH carries all of minimum system information Agreements: - For study of the cases where NR-PBCH carries a part of minimum system information, consider the following alternatives (or combinations) for the minimum system information other than those included in NR-PBCH : - Alt. 1: NR defines the additional channel as secondary broadcast channel - Secondary broadcast channel may be different design from NR-PBCH, e.g. payload size, resource mapping, periodicity and etc. - FFS on transmission: beam-specific, cell-specific, and/or TRP-specific, etc. - Alt. 2: The remaining information is transmitted in shared downlink channel similar to ,e.g. NR-PDSCH - FFS on transmission: UE-specific, UE group-specific, beam-specific, cell-specific, and/or TRP-specific, etc. - Note: This does not preclude defining of other mechanisms transmitting Other SI

The number of bits for minimum SI will be at least around 250 bits for L2/L3 operation according to the RAN2 agreements [2]. The additional bits for L1 parameters will be added based on a discussion in RAN1. Considering the number of bits for minimum SI, it is infeasible to transmit all of minimum SI through NR-PBCH only. Therefore, we need to define additional channel to transmit remaining minimum SI. It will be beneficial to indicate the additional channel by NR-PBCH for flexibility. According to the previous agreement in RAN1 #87, there are two alternatives for the additional channel. One is the secondary broadcast channel (NR-SPBCH). The other is the shared downlink channel similar to NR-PDSCH.

In case of NR-SPBCH, it seems to be difficult to support flexible payload size. Therefore it might need to design NR-SPBCH considering worst case scenario. It might not also be suitable for forward compatibility. On the other hand, transmitting remaining minimum SI through NR-PDSCH will be able to support flexible payload size and forward compatibility since the NR-PDSCH can be scheduled through NR-PDCCH similar as LTE. However, under the multi-beam operation, the beam sweeping will be required for NR-PDCCH and/or NR-PDSCH. It might increase the complexity of system design and overhead. Thus, this approach is not suitable option for multi-beam operation especially in high frequency bands. Therefore, both approaches should be supported and the selection between both can be configured through NR-PBCH based on the deployment scenarios as shown in Figure 1.

Figure 1. Example for supporting both alternatives

Proposal 1: Both alternatives should be supported for transmission of remaining minimum SI. It should be configured through NR-PBCH.

Other System Information

Based on RAN2 agreements, entire system information is divided into minimum SI and other SI, and the other SI may either be broadcasted, or provisioned via on-demand basis. According to LS from RAN2 [3], RAN2 also has discussed on-demand other SI request mechanism and solution based on MSG1 and MSG3 have been proposed to request one or more System Information Blocks (SIBs).

With regards to the MSG1 based solution, UEs may use specific PRACH preamble/resource(s) to identify the on-demand SI. In this solution, multiple UEs may simultaneously use the same PRACH preamble/resource(s) for on-demand SI requests. If it is happened during initial random access procedure (i.e., contention based random access), it will increase latency of random access procedure since the gNB might be able to detect the preamble from the combined transmission from multiple UEs, but may not be able to distinguish the UEs as stated in the response of the LS [3]. In addition, the use of PRACH transmission for additional purpose may increase the random access load. In order to address such issues, it may need to restrict MSG1 based solution to RRC_IDLE UEs only for requesting other SI without uplink synchronization. It would be also beneficial to define periodical request and/or reception mechanisms for other SI by the MSG1 based solution. Even though we do not have any strong preference and clear solution for requesting other SI for the present, it should be further studied in RAN1 perspective.

Proposal 2: Detail mechanism for requesting other SI should be studied further in RAN1 perspective.

Summary

In this contribution, we made the following observation and proposals.

Proposal 1: Both alternatives should be supported for transmission of remaining minimum SI.

Proposal 2: Detail mechanism for requesting other SI should be studied further in RAN1 perspective.

References

- Chairman’s notes, RAN1#87, November 2016.

- R1-1701519, “Response LS on minimum system information”, RAN WG2

- R1-1701518, “LS on PRACH preambles for on demand SI requests”, RAN WG2