3GPP TSG RAN WG1 Meeting #88		R1-1702358

Athens, Greece 13th - 17th February 2017

Source:	Kyocera

Title:	Remaining issue of frequency hopping for resource allocation

Agenda Item:	7.2.3.1.1

Document for:	Discussion/Decision

Introduction

In RANP#72, Further Enhanced MTC for LTE WI was approved [1] that included a task to support the higher data rate than 1Mbps for voice capable wearable devices and health monitoring devices. In this contribution, we discuss remaining issue of frequency hopping with larger maximum bandwidth for higher data rates.

- In the RAN1 meeting the following was agreed [2]:

Agreements: - The PDSCH Allocation for FeMTC UEs supporting larger BW is limited to resource blocks that are part of narrow bands defined in LTE Release 13 for UEs configured with 5 MHz max PDSCH channel bandwidth - 24 PRBs is the maximum number of PRBs that can be allocated - The PDSCH Allocation for FeMTC UEs supporting larger BW is limited to resource blocks that are part of narrow bands defined in LTE Release 13 for UEs configured with 20MHz max PDSCH channel bandwidth - 96 PRBs is the maximum number of PRBs that can be allocated - The PUSCH allocation for FeMTC UEs with larger BW can include at least certain PRBs that are not part of narrow bands defined in LTE Release 13 - This includes at least the central PRB in case of odd system bandwidth - FFS: PRBs at the band edges that do not belong to any narrowband Agreement: - For Rel-14 BL/CE UE configured with max 5 MHz PDSCH channel bandwidth, contiguous and non-contiguous resource allocation is supported Agreement: - PDSCH/PUSCH frequency hopping is supported for BL/CE UE operating 5 MHz maximum PDSCH/PUSCH channel bandwidth. - Reuse Rel-13 frequency hopping RRC parameters. - FFS how to handle center PRB in odd system bandwidths - FFS whether wrap-around can occur - It is up to eNB implementation to ensure that frequency hopping error cases with wrap-around (e.g. for PUSCH or for PDSCH with UE bandwidth smaller than system bandwidth) do not occur. - This does not preclude specifying a UE behavior for these cases. - PDSCH/PUSCH frequency hopping can be dynamically enabled and disabled via DCI in CE Mode A.

Reaming issues of frequency hopping

Frequency hopping for higher data rates is agreed and Rel-13 frequency hopping RRC parameters are reused. RRC parameter has hopping offset and hopping interval. If frequency hopping flag in DCI is ON, hopping is applied as configured by the RRC parameters. Resource allocation of frequency hopping is based on the initial resource allocation indicated by DCI.

Regarding PDSCH frequency hopping, there is no issue except for wrap around if the Rel-13 frequency hopping follows legacy RRC parameter, because the resource allocation of PDSCH is only located within the Rel-13 NBs. On the other hand, for PUSCH, supporting the at least center PRB which is not part of Rel-13 was agreed. However, how to deal with the center RBs which are not part of NBs was not yet decided. PRB which is not part of NBs depends on the system bandwidth shown in the Table 1 below and Table 2 shows how the situation would be divided into 3 cases.

Table 1 Number of PRB which is not part of Rel-13 resource allocation

| System bandwidth | Total number of 6-PRB narrowbands | Number of PRBs which is not part of NBs in Rel-13 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge (high) | Center | Edge (low) |
| 1.4 MHz, 6 PRBs | 1 NB = 6 PRBs | 0 | 0 | 0 |
| 3 MHz, 15 PRBs | 2 NBs = 12 PRBs | 1 | 1 | 1 |
| 5 MHz, 25 PRBs | 4 NBs = 24 PRBs | 0 | 1 | 0 |
| 10 MHz, 50 PRBs | 8 NBs = 48 PRBs | 1 | 0 | 1 |
| 15 MHz, 75 PRBs | 12 NBs = 72 PRBs | 1 | 1 | 1 |
| 20 MHz, 100 PRBs | 16 NBs = 96 PRBs | 2 | 0 | 2 |

Table 2 Cases of number of PRB which is not part of NBs

between initial resource allocation and resource allocation after frequency hopping

|  | Include or neighbor to PRB(s) which is not part of NBs |  |
| --- | --- | --- |
|  | Initial | Frequency hopping |
| Case 1 | 1(/2) | 1(/2) |
| Case 2 | 1(/2) | 0 |
| Case 3 | 0 | 1(/2) |

Case 1 means the initial resource allocation includes the PRB or the neighboring PRB which is not part of Rel-13 NBs, and resource allocation after frequency hopping also includes the PRB or the neighboring PRB which is not part of Rel-13 NBs. In this case, UE has opportunity to utilize both PRBs with no additional indication. However, the resource allocation after frequency hopping is slightly changed from resource allocation indicated by DCI.

- PRB(s) which is not part of Rel-13 NBs should be used for resource allocation after frequency hopping, if initial resource allocation includes the PRB(s) which is not part of Rel-13 NBs.

Regarding the case 2 and case 3, only one of the initial resource allocation or resource allocation after frequency hopping does not have PRBs which are not part of Rel-13 NBs. Therefore, how to deal with the PRB which is not part of Re-13 NBs should be decided. We suggest the following two methods:

- PRB expansion within Rel-13 NBs

- Adapt the number of PRBs depending on the existence of included or the neighboring PRB which is not part of NBs

1. PRB Expansion within Rel-13 NBs

As shown in the below Figure 1, initial resource allocation has PRBs which is not part of Rel-13 NBs, and resource allocation after frequency hopping expands a PRB within NBs instead of PRBs which is not part of Rel-13 NBs. This method can keep the number of PRBs between initial resource allocation and resource allocation after frequency hopping even if there is no PRB which is not part of Rel-13 NBs. However, overlapping with Rel-13 eMTC UE and other Rel-14 FeMTC UEs might occur within the expanded PRB. To avoid overlapping, such resource allocation should be taken into account when UL grant is transmitted. This method can keep the number of PRBs for FeMTC, while resource allocation for Rel-13 FeMTC UE and other Rel-14 FeMTC UEs would be slightly restricted in order to avoid overlapping.

Figure 1 Example of PRB expansion within Rel-13NBs.

2. Adapt the number of PRBs depending on the existence of included or the neighboring PRB which is not part of NBs

Second method adapts the PRB which is not part of NBs, if one of resource allocation does not include the PRB which is not part of NBs as shown in the Figure 2. For example, initial resource allocation has 8 PRBs within NB and 1 PRB (center (/edge)) which is not part of Rel-13 NBs, while resource allocation after frequency hopping has only 8 PRBs within NB. In this case, overlapping does not happen since the usage of the PRBs within the NBs does not change during hopping. On the other hand, we should consider how to decide the TBS format/size because the number of PRBs is different between initial and with frequency hopping. One of the simplest method is UE only use the number of PRB of initial resource allocation and the same TBS is used for transmission after frequency hopping even if the number of PRB changes. This method may have potential issue because the coding rate has to be adjusted for the subsequent transmission with a different number of PRBs. However, this issue can be solved by the eNB scheduler (for example, adjust the MCS index). In addition, eNB should be careful about the number of allocated PRBs, because number of allocated PRBs should be 2a * 3b * 5c, for both the initial and the after frequency hopping resource allocation.

Figure 2 Example of adapting the number of PRBs

Comparing the two methods, the PRB expansion within Rel-13 NBs might decrease the system efficiency due to scheduling restriction for collision avoidance. In addition the second method allows adjusting the coding rate without impacting the legacy eMTC UEs. Therefore, we prefer the second method to be adopted for the Rel-14. From other perspective, it’s not decided yet whether to support the edge PRBs which is not part of NBs. However, second method also can be applicable to the case of supporting edge PRBs for FeMTC transmission.

- Adapt the number of PRBs depending on the existence of included or the neighboring PRB which is not part of NBs, if the number of PRBs between initial resource allocation and frequency hopping resource allocation is different.

Conclusions

In this contribution, we provided our views remaining issue of UL frequency hopping. We have following proposals:

- PRB(s) which is not part of Rel-13 NBs should be used for resource allocation after frequency hopping, if initial resource allocation includes the PRB(s) which is not part of Rel-13 NBs.

- Adapt the number of PRBs depending on the existence of included or the neighboring PRB which is not part of NBs, if the number of PRBs between initial resource allocation and frequency hopping resource allocation is different.

- References

- RP-161321, “New WI proposal on Further Enhanced MTC”, Ericsson

- R1-17xxxxx, “Final Report of 3GPP TSG RAN1 #87 v1.0.0”, ETS