3GPP TSG RAN WG1 Meeting #88							R1-1702781

Athens, Greece 13th - 17th February 2017

Source:	NTT DOCOMO, INC.

Title:	Views on sTTI scheduling

Agenda Item:	7.2.5.2.2.1

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss sTTI scheduling scheme.

- Single-level DCI vs two-level DCI

At the RAN1#86bis meeting, following agreement was achieved [1]:

Agreement - Select a sTTI scheduling scheme among the following candidates for each sTTI length - Single level DCI - RRC configuration of sPDCCH search space and/or sPDCCH frequency region - UE-specific information in sDCI related to sPDSCH/sPUSCH - Two level DCI - RRC configuration may or may not at least partially indicate sPDCCH frequency region/search space for some of the variants described below - variant 1 - Slow DCI: non UE-specific information in PDCCH - Fast DCI: UE-specific information in sDCI - variant 2 - Slow DCI: UE-specific information in PDCCH - Fast DCI: UE-specific information in sDCI - variant 3 - Slow DCI: UE-specific information in PDCCH and/or sPDCCH - Fast DCI: UE-specific information in sDCI - Note: the sTTI scheduling scheme may be the same or different for different sTTI length - FFS how to reduce the payload of sDCI/DCI messages for sTTI operation - FFS support of multi-sTTI scheduling - Additional L1 signaling related to sTTI operation can be considered

There was extensive discussion on whether to support two-level DCI, and which variant of two-level DCI is the most feasible for shortened TTI operation if supported. The main motivation of two-level DCI for shortened TTI operation is to reduce DL control channel overhead. On the other hand, following concerns are still not resolved:

Combination of slow-DCI and fast-DCI to get scheduling information makes sTTI operation complicated

In some of the variants, UE needs to decode both slow-DCI and fast-DCI correctly to receive sPDSCH or to transmit sPUSCH. Miss detection of either one causes failure of data scheduling. In general, PDCCH decoding error probability is adjusted to be sufficiently low (e.g., 1%). If the combination of slow-DCI and fast-DCI is required to achieve full scheduling information, each of the DCI shall be decodable with better performance compared to single-DCI. It may require lower coding rate (or higher aggregation level) for each of the DCIs. Furthermore, in all the variants of two-level DCI, slow-DCI is common for multiple sTTIs and/or for multiple UEs. In this case, the reliability of slow-DCI is more important and critical over the whole performance.

Slow-DCI in the UE-specific search space may not reduce the control overhead and increases BD number

In variants 2 and 3, both slow-DCI and fast DCI contains UE-specific information. The fast DCI can update/overwrite the scheduling information in slow DCI. The benefit of above two-level DCI is clear if the eNB schedules multiple sTTIs for the UE in the given subframe where slow-DCI is effective. However, it is questionable that scheduling of multiple sTTIs for one UE in the given subframe is common use-case. From spectral efficiency point of view, 1ms TTI scheduling is always better than multiple sTTI scheduling. In addition, variant 3 allows the slow DCI to be transmitted in the UE-specific search space, which increases the BD number.

Combination of slow-DCI and fast-DCI would not fit with HARQ operation in shortened TTI

Assuming that DL data is scheduled on a certain sTTI, the DL data may be failed to decode, and the re-transmission becomes necessary. From eNB point of view, at a timing where the slow-DCI is transmitted for a given subframe, it may not be sure whether the HARQ-ACK feedback from the UE for DL data of an sTTI in the previous subframe is NACK or ACK. Therefore, eNB has to continue transmission of slow-DCI at the current subframe, as long as there is a possibility of scheduling re-transmission in the current subframe. This drawback is more visible if the slow-DCI is common for multiple UEs and for multiple sTTIs, as in variant 1 and variant 3. In summary, the combination of slow-DCI and fast-DCI would not fit with fast HARQ feedback for the shortened TTI operation.

Fig. 1	HARQ operation in shortened TTI with the two-level DCI.

Due to the above three reasons, we prefer to support single-level DCI. Use of single-level DCI is not harmful and works well. Besides, as long as dynamic switching between sTTI and 1ms TTI is enabled, there is no strong reason to use multi-sTTI scheduling.

Proposal 1:

- Single-level DCI is supported as it works well without harmful overhead increase.

  - Overhead reduction of the single-level DCI can be considered further.

- No strong motivation of supporting multi-sTTI scheduling is identified.

- Complementary control signalling to enhance single-level DCI operation

As a possible approach to utilize multiple DCI steps, complementary control signaling to enhance single-level DCI operation can be considered. Focusing on blind decoding complexity of shortened TTI operation, it is possible to control sPDCCH blind decoding of single-level sDCI by using complementary control signaling which is transmitted with different periodicity from the scheduling sDCI. Therefore, the combination of this complementary control signaling and scheduling sDCI can be interpreted as two-level DCI.

The complementary control siganlling can be DCI, transmitted per subframe or per multiple subframes. Then, the control signaling indicates whether the UE is allowed not to monitor sPDCCH over the given time duration. Note that the control signaling does not indicate whether the UE shall monitor sPDCCH over the given time duration; if this is the case, once the UE miss detects the control signalling, the UE cannot receive sPDCCH during the time duration. On the other hand, in case if the signaling indicates allowing not to monitor the sPDCCH, once the UE miss detects the signalling, the UE monitors sPDCCH, even if the eNB does not transmit sPDCCH. As such, this complementary control siganlling can offer the benefit of reducing blind decoding complexity and power saving, while its miss-detection does not cause harmful impact to the sTTI operation.

Proposal 2:

- A complementary DCI can be introduced on top of single-level sDCI, to reduce UE’s blind detection complexity.

  - Once the signaling indicates allowing not to monitor sPDCCH over the given time duration, the UE can skip sPDCCH blind decodes.

  - Miss detection of the signaling does not have harmful impact as the UE just continue monitoring sPDCCH.

- Conclusion

In this contribution, we discussed sTTI scheduling schemes and sPDCCH design and proposed the following:

Proposal 1:

- Single-level DCI is supported as it works well without harmful overhead increase.

  - Overhead reduction of the single-level DCI can be considered further.

- No strong motivation of supporting multi-sTTI scheduling is identified.

Proposal 2:

- A complementary DCI can be introduced on top of single-level sDCI, to reduce UE’s blind detection complexity.

  - Once the signaling indicates allowing not to monitor sPDCCH over the given time duration, the UE can skip sPDCCH blind decodes.

  - Miss detection of the signaling does not have harmful impact as the UE just continue monitoring sPDCCH.

References

- 3GPP RAN1#86bis Chairman’s note.