3GPP TSG RAN WG1 Meeting #88                                    R1-1702845

Athens, Greece 13th – 17th February 2017

Source:	NTT DOCOMO

Title:	Phase-2 SLS MIMO Calibration Results

Agenda Item:	8.1.2.5

Document for: 	Discussion

- Introduction

At the RAN1 #86 meeting [1], it was agreed to conduct MIMO calibration.

Agreements:

- For the purpose of calibration, companies are encouraged to provide baseline results for NR MIMO in the following meetings, including link-level and system level simulations

  - Discuss further on baseline simulation assumptions and metrics for calibration in email discussion after RAN1#86

  - Thereafter, email reflector is used to collect results (no online time for calibration discussions, similar to channel model calibration)

In the email discussion of [86-20], detailed evaluation methodologies, performance metrics and simulation assumptions were discussed and agreed. Related agreements were summarized in [2]:

- Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (aim to finish it in RAN1#86bis)

- Phase 2: Start discussion on whether and how to establish the baseline.  Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (aim to finish it in RAN1#87)

- Phase 3: Calibration can be used to check the UE movement/rotation/blockage (aim to finish it after RAN1#87)

To reduce gaps between submitted results, Phase 1 calibration target was further broken down into the following three targets in email discussion [3]:

For system level calibration, SNR CDF curves are generated with no interference in this step:

- Single TX/RX subelement - without TX/RX beamforming

- The first sub-element in a single fixed panel is used with single-pol.

- With analog TX/RX beamforming, using a single digital TX/RX port

- The first TXU/RXU in a single fixed panel is used with single-pol

b’. With fixed analog TX/RX beamforming, using a single digital TX/RX port

- The first TXU/RXU in a single fixed panel is used with single-pol

- With analog TX/RX beamforming, and with SVD-based digital TX/RX beamforming

- The UE panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels.

Based on the agreed evaluation assumptions, companies still have their own choices of implementation details. In order to achieve better alignments in Phase 1 results and avoid more diverse results for Phase 2, companies are encouraged to provide their implementation details.

In this contribution, we discuss our implementation details and provide updated results for Phase 1 MIMO calibration. In addition, Phase 2 implementation and results are also addressed.

- Clarification on Phase 1 calibration assumption

Although many assumption items have been discussed in [86-20] email discussion [2] and a way forward in RAN1 #86bis [4], there are still different understandings/interpretations of the evaluation assumptions. In addition, for those detailed items which are not mentioned in the evaluation assumption list, companies have different implementations. In this section some more detailed description used in the simulation are provided. The following description is based on the agreement: MIMO calibration only focuses on DL [2].

SVD-based digital beamforming

Digital beamforming based on SVD is agreed to be involved in calibration. In our opinion, for low complexity and less ambiguity, SVD-based digital beamforming is suggested to be realized as the following description:

- Get the channel matrix (all TRP TXRU ports to all TXRU ports of a selected UE panel after analog beamforming)

- SVD on the channel matrix for U and V

- The Hermitian transpose of V and U will be precoders for Tx and Rx respectively

- Multiply transposed U and V on original channel matrix G=UHHVH to realize digital beamforming

Fast fading related

Fast fading may affect channel matrix as TTI varies. In order to reduce the effect of randomness, all parameters are advised to be measured in TTI 0 and used for further processing.

UE distribution per TRP

UE number TRP is clarified as 10 per TRP. Here, it is clarified in further that in the contribution, UE distribution is realized by distributing 10 UEs in TRP-related geographic area without considering the association between distributed UEs and TRPs.

- System Level Performance for Phase 1 broken down cases

Considering that there are too many new items including UE beamforming, UE panel selection and digital beamforming based on SVD, in this calibration, Phase 1 calibration target was further broken down into three steps [3] to help reduce gaps between curves step by step. In addition, evaluation metric is determined as SNR to replace the SINR metric used in the previous calibration in order to eliminate the randomness of interference. The beam sweeping ranges are also specified in [3] in the form of concrete degree values in order to reduce gaps resulted from different beam angle settings. They are listed in Appendix: simulation assumption.

TX and RX analog beam pair is selected to achieve maximum RSRP. In some cases, each UE has two panels but only one will be selected according to previous agreement, for maximizing RSRP.

As mentioned in Introduction, the following four cases are examined:

- Single TX/RX subelement - without TX/RX beamforming

      - The first sub-element in a single fixed panel is used with single-pol

- With analog TX/RX beamforming, using a single digital TX/RX port

      - The first TXU/RXU in a single fixed panel is used with single-pol

b’. With fixed analog TX/RX beamforming, using a single digital TX/RX port

      - The first TXU/RXU in a single fixed panel is used with single-pol

- With analog TX/RX beamforming, and with SVD-based digital TX/RX beamforming

      - The UE panel with the best receive SNR is chosen for output metric. i.e. no combining between panels.

Based on the above mentioned evaluation assumption, the following figures are achieved wideband SNR CDF plots in UMa, Indoor Hotspot and Dense Urban Macro layer scenarios, respectively.

Figure 1. CDF of wideband SNR for four cases in UMa scenario (upper left), Indoor scenario: Opt.1 and 12 TRPs (upper right) and Dense Urban Macro Layer (bottom)

- Clarification on Phase 2 calibration

Compared to Phase 1 calibration, the following items requires further clarification:

Modeling for serving and interfering TRP

Serving TRP is selected in order to maximize RSRP with only analog beamforming. The beam pair of TRP and UE is also determined for maximum RSRP after analog beamforming. At this stage, no digital beamforming is taken into consideration.

Non-serving TRPs will provide interference with its own ACTUAL transmission analog beam AND digital precoder rather than modelling a random beam as used in Phase 1.

- System level performance for Phase 2 calibration

The same beam sweeping ranges in Phase 1 calibration is applied in Phase 2 as suggestion from [87-30] email discussion summary [6]. Based on the above mentioned evaluation assumption, the following figures are achieved spectral efficiency CDF plots in Urban Macro (UMa), Indoor Hotspot (InH) and Dense Urban Macro Layer (DUMa) scenarios respectively. The overhead value set in simulation is 0.3762.

Figure 2. CDF of Spectral Efficiency for UMa, Indoor Hotspot and Dense Urban Macro Layer

- Summary

In this contribution, we present the SNR performance for MIMO calibration Phase 1 broken down cases and spectral efficiency in Phase 2 in Dense Urban Macro Layer.

# References

- 3GPP, “Draft Report of 3GPP TSG RAN WG1 #86 (v0.2.0),” August 2016.

- 3GPP, R1-1608661, ZTE, “Summary of [86-20] email discussion on MIMO calibration for NR”, October, 2016.

- 3GPP, R1-1611423, ZTE, “Summary of [86b-19] email discussion on Phase 2 MIMO calibration for NR”, November, 2016

- 3GPP, R1-1610890, ZTE, “Clarification on assumptions for Phase 1 link-level calibration for NR-MIMO”, October, 2016.

- 3GPP, R1-1610072, NTT DOCOMO, “Phase-1 MIMO Calibration Results”, October, 2016.

- 3GPP, R1-1700395, ZTE, “Summary of email discussion [87-30] on NR-MIMO calibration”, Janurary, 2017

# Appendix

Table A: Evaluation Assumptions for MIMO Calibration in UMa Scenario

| Parameters | Values |
| --- | --- |
| Layout | Single Layer Macro layer: Hex. Grid |
| Inter-BS distance | 500 m |
| Carrier frequency | 30 GHz |
| Bandwidth | 40 MHz |
| Subcarrier spacing | 60 KHz |
| Channel model | 3D UMa in TR38.900 |
| BS Tx power | 43 dBm |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH, dV, dHg,dVg) = (0.5, 0.5, 4.0, 2.0) |
| BS TXRU mapping | A single TXRU per panel per polarization is used. 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x 2. |
| BS Antenna height | 25 m |
| BS receiver noise figure | 7 dB |
| UE antenna configurations | (M,N,P,Mg,Ng) = (2,4,2,1,2) (dH, dV) = (0.5, 0.5) Polarization degrees: 0 and 90 |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE receiver noise figure | 10 dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP for full buffer traffic |
| Beam selection method | Select the best beam pair at the same time based on the criteria of maximizing receive power after beamforming. BS: Azimuth angle [-5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16]; Zenith angle [5*pi/8 7*pi/8]; UE: Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8]; Zenith angle [pi/4 3*pi/4]; No UE panel combination: only one UE panel will be used each time, selected for RSRP maximization. In fixed beamforming case, angles underlined are applied. |
| Interfering beam selection | Non-serving TRPs use its own transmission analog beam and digital precoder to cause interference |
| Scheduling algorithm | Round Robin |
| Evaluation Metrics | Phase 1 broken down cases: CDF of SNR Phase 2: CDF of spectral efficiency |

Table B: Evaluation Assumptions for MIMO Calibration in Indoor Scenario

| Parameters | Values |
| --- | --- |
| Layout | Single Layer, 12 TRPs |
| Inter-BS distance | 20 m |
| Carrier frequency | 30 GHz |
| Bandwidth | 40 MHz |
| Subcarrier spacing | 60 KHz |
| Channel model | Indoor in TR38.900 |
| BS Tx power | 23 dBm |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,8,2,1,1) (dH, dV) = (0.5, 0.5) |
| BS TXRU mapping | A single TXRU per panel per polarization is used. 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x 2. |
| BS Antenna height | 3 m |
| BS receiver noise figure | 7 dB |
| UE antenna configurations | (M,N,P,Mg,Ng) = (2,4,2,1,2) (dH, dV, dHg, dVg) = (0.5, 0.5, 0, 0) |
| UE antenna height | 1.5 m |
| UE receiver noise figure | 10 dB |
| UE distribution | 100% Indoor in houses: 3km/h 10 users per TRP for full buffer traffic |
| Beam selection method | Select the best beam pair at the same time based on the criteria of maximizing receive power after beamforming. BS: Azimuth angle [pi/16 3*pi/16 5*pi/16 7*pi/16 9*pi/16 11*pi/16 13*pi/16 15*pi/16]; Zenith angle [pi/8 3*pi/8 5*pi/8 7*pi/8]; UE: Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8]; Zenith angle [pi/4 3*pi/4]; No UE panel combination: only one UE panel will be used each time, selected for RSRP maximization. In fixed beamforming case, angles underlined are applied. |
| Interfering beam selection | Non-serving TRPs use its own transmission analog beam and digital precoder to cause interference |
| Scheduling algorithm | Round Robin |
| Evaluation Metrics | Phase 1 break down cases: CDF of SNR Phase 2: CDF of spectral efficiency |

Table C: Evaluation Assumptions for MIMO Calibration in Dense Urban Scenario Macro Layer

| Parameters | Values |
| --- | --- |
| Layout | Single Layer: Macro layer: Hex. Grid |
| Inter-BS distance | 200 m |
| Carrier frequency | 4 GHz |
| Bandwidth | 20 MHz |
| Subcarrier spacing | 15 KHz |
| Channel model | 3D UMa in TR38.900 |
| BS Tx power | 44 dBm |
| BS antenna configurations | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH, dV) = (0.5, 0.8) |
| BS TXRU mapping | A single TXRU per panel per polarization is used. 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x 2. |
| BS Antenna height | 25 m |
| BS receiver noise figure | 5 dB |
| UE antenna configurations | (M,N,P,Mg,Ng) = (1,1,2,1,1) |
| UE antenna element radiation pattern | Omnidirectional |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE receiver noise figure | 9 dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP for full buffer traffic |
| Beam selection method | Select the best beam pair at the same time based on the criteria of maximizing receive power after beamforming. BS: Zenith angle [9*pi/16 11*pi/16 13*pi/16]; No beam sweeping in Azimuth In fixed beamforming case, angles underlined are applied. |
| Interfering beam selection | Non-serving TRPs use its own transmission analog beam and digital precoder to cause interference |
| Scheduling algorithm | Round Robin |
| Evaluation Metrics | Phase 1 broken down cases: CDF of SNR Phase 2: CDF of spectral efficiency |