3GPP TSG RAN WG1 Meeting #88		R1-1702863

Athens, Greece February 13 – 17, 2017

Agenda Item:		7.2.1.1

Source:			Samsung

Title:			DCI design for support of SL SPS

Document for:		Discussion and Decision

- Introduction

In RAN1#87 meeting, following agreements are achieved for support of multiple SL SPS configurations [1]:

Agreements:

- If the DCI for SL SPS or dynamic scheduling is transmitted in subframe n and the offset value (in two-bit field) is set to m, the time location of initial transmission is the first subframe included in the resource pool that occurs at or after subframe n+4+m.

  - Offset value is one of {0, 1, 2, 3}.

  - Applicable to both SL SPS and dynamic scheduling.

  - Subframe n can be any of DL subframes or special subframes in TDD Uu carrier.

Agreements:

- Confirm the following working assumptions:

  - A V2X UL SPS RNTI different from the legacy SPS C-RNTI is defined.

In this contribution, we will discuss the remaining issues on DCI design for support of multiple SL SPS configurations.

- Discussion

2.1 Search space for SPS DCI

Carriers for SL V2X operation are configured through v2x-InterFreqInfoList-r14 IE included within SIB21 or RRCConnectionReconfiguration message [2]. For carrier aggregation, it is defined that DCI on a scheduling cell may or may not have CIF field depending on whether the IE cif-Presence-r10 is set to TRUE or FALSE(or absence)[2].  Since the IE cif-Presence-r10 is included within the messages for SCell configuration, if UE does not work under carrier aggregation mode, DCI of the scheduling cell does not contain CIF field; even though UE has configured SCell, DCI on one serving cell may not have CIF filed either if the cell is not configured for cross-carrier DL/UL scheduling[3]. If SL carrier is cross-carrier scheduled by a serving cell on which DCI does not have CIF field, then SPS DCI is mapped to common or UE specific search space for the scheduling cell.

On the contrary, if DCI on the scheduling cell has CIF field, then SPS DCI is mapped to a particular UE specific search space corresponding to its carrier indicator value. Note that if UE is configured with a SCell with the same carrier indicator value, SPS DCI for the SL carrier and DCI for the Scell are mapped to the same search space, as shown in Fig. 1.

Figure 1 Search Space for serving cell and SL carrier when CIF is present on scheduling cell

Observation 1

- SL SPS DCI is mapped to common or UE specific search space when CIF field is not present on the scheduling cell;

- SL SPS DCI is mapped to UE specific search space corresponding to the carrier indicator value of the SL carrier if CIF field is present on the scheduling cell;

  - DCI for SL carrier and another SCell may map to same search space;

2.2 size of SPS DCI

SPS DCI sizes for sub-channel number 1/3/5/10/15/20 are given in Table 1, and the sizes of DCI 0 for different bandwidth are given in Table 2. According to the analysis in section 2.1, SL carrier is possible to be cross-carrier scheduled by a serving cell of which DCI has no CIF field. In this case, if number of sub-channels on the SL carrier is 20, then for both FDD and TDD the sizes of DCI 0 for bandwidth of 1.4MHz and 3MHz are smaller than SPS DCI. Even number of sub-channel on the SL carrier is 15, for both FDD and TDD the size of DCI 0 for 1.4MHz is also smaller than SPS DCI. In case of SL carrier is cross-carrier scheduled by a serving cell on which DCI has CIF field, and sub-channel number on the SL carrier is 20, the size of FDD DCI 0 for 1.4MHz is smaller than SPS DCI.

Table 1 SPS DCI size for sub-channel number 1/3/5/10/15/20 respectively

| Fields | #bits |
| --- | --- |
| Carrier indicator | 3 |
| Lowest index of the sub-channel allocation | 0/2/3/4/4/5 |
| Frequency resource location | 0/3/4/6/7/8 |
| Time gap between initial transmission and retransmission | 4 |
| SL index | 0(FDD) 2(TDD) |
| SL SPS configuration index | 3 |
| Activation/release indication | 1 |
| Total | 11/16/18/21/22/24 (FDD) 13/18/20/23/24/26(TDD) |

Table 2 DCI 0 size for different bandwidth

| Size of DCI 0 | 1.4MHz | 3MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHZ |
| --- | --- | --- | --- | --- | --- | --- |
| FDD, w/o CIF | 21 | 22 | 25 | 27 | 27 | 28 |
| FDD, W/ CIF | 23 | 25 | 27 | 29 | 30 | 31 |
| TDD, w/o CIF | 23 | 25 | 27 | 29 | 30 | 31 |
| TDD, w/ CIF | 27 | 28 | 30 | 33 | 33 | 34 |

Observation 2

- Size of DCI 0 is smaller than SPS DCI in following cases:

|  | Scheduling cell |  |  |  | SL carrier |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Duplex mode | CIF | Bandwidth | Size of DCI0 | Sub-channel number | Size of SPS DCI |
| Case 1 | FDD | w/o | 1.4MHz | 21 | 20 | 24 |
| Case 2 | FDD | w/o | 1.4MHz | 21 | 15 | 22 |
| Case 3 | FDD | w/o | 3MHz | 22 | 20 | 24 |
| Case 4 | FDD | w/ | 1.4MHz | 23 | 20 | 24 |
| Case 5 | TDD | w/o | 1.4MHz | 23 | 20 | 26 |
| Case 6 | TDD | w/o | 1.4MHz | 23 | 15 | 24 |
| Case 7 | TDD | w/o | 3MHz | 25 | 20 | 26 |

From the table above we can find that the majority of cases happen when there is no CIF on the scheduling cell, we believe it is natural to specify that CIF should always be present on a serving cell used for cross-carrier sidelink scheduling.

Proposal 1

- CIF is always present on the serving cell used for sidelink cross-carrier scheduling;

As to the extreme case 4, we prefer to address it with a simple way, e.g. truncating the SL SPS configuration index field of SPS DCI to 2 bits (i.e. restrict the number of supported SPS configuration to 4).

Proposal 2

- If a SL carrier is cross-carrier scheduled by a FDD serving cell with a bandwidth of 1.4MHz, truncating the SL SPS configuration index field of SPS DCI to 2 bits;

3. Conclusion

The following proposals and observation are made to support multiple SPS configurations on SL:

Observation 1

- SL SPS DCI is mapped to common or UE specific search space when CIF field is not present on the scheduling cell;

- SL SPS DCI is mapped to UE specific search space corresponding to the carrier indicator value of the SL carrier if CIF field is present on the scheduling cell;

  - DCI for SL carrier and another SCell may map to same search space;

Observation 2

- Size of DCI 0 is smaller than SPS DCI in following cases:

|  | Scheduling cell |  |  |  | SL carrier |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Duplex mode | CIF | Bandwidth | Size of DCI0 | Sub-channel number | Size of SPS DCI |
| Case 1 | FDD | w/o | 1.4MHz | 21 | 20 | 24 |
| Case 2 | FDD | w/o | 1.4MHz | 21 | 15 | 22 |
| Case 3 | FDD | w/o | 3MHz | 22 | 20 | 24 |
| Case 4 | FDD | w/ | 1.4MHz | 23 | 20 | 24 |
| Case 5 | TDD | w/o | 1.4MHz | 23 | 20 | 26 |
| Case 6 | TDD | w/o | 1.4MHz | 23 | 15 | 24 |
| Case 7 | TDD | w/o | 3MHz | 25 | 20 | 26 |

Proposal 1

- CIF is always present on the serving cell used for sidelink cross-carrier scheduling;

Proposal 2

- If a SL carrier is cross-carrier scheduled by a FDD serving cell with a bandwidth of 1.4MHz, truncating the SL SPS configuration index field of SPS DCI to 2 bits;

4. References

- Chairman’s notes, 3GPP TSG RAN WG1 Meeting #87;

- 3GPP TS 36.331 V14.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA);Radio Resource Control (RRC);Protocol specification”;

- 3GPP TS 36.213 V12.5.0, “Evolved Universal Terrestrial Radio Access (E-UTRA);Physical layer procedures”;