3GPP TSG RAN WG1 Meeting #88 	R1-1702945

# Athens, Greece 13th - 17th February 2017

Agenda item:	8.1.2.3.5

Source: 	Samsung

Title: 	Discussions on aperiodic CSI reporting for NR

Document for:	Discussion and Decision

Introduction

In RAN1 NR Ad-Hoc, it is agreed to support aperiodic CSI reporting with periodic CSI-RS, semi-persistent CSI-RS and aperiodic CSI-RS as follows [3]:

Agreements:

- For periodic CSI-RS,

  - Semi-persistent CSI reporting is activated/deactivated by MAC CE and/or DCI

  - Aperiodic CSI reporting is triggered by DCI

    - FFS: Necessity of additional signaling with MAC CE

- For semi-persistent CSI-RS,

  - Periodic CSI reporting is not supported.

  - Semi-persistent CSI reporting is activated/deactivated by MAC CE and/or DCI

    - Semi-persistent CSI-RS is activated/deactivated by MAC CE and/or DCI

    - FFS: Relationship of signaling between CSI reporting and CSI-RS transmission

  - Aperiodic CSI reporting is triggered by DCI

    - Semi-persistent CSI-RS is activated/deactivated by MAC CE and/or DCI

    - FFS: Necessity of additional signaling with MAC CE

- For aperiodic CSI-RS,

  - Periodic [and semi-persistent] CSI reporting is not supported

  - Aperiodic CSI reporting is triggered by DCI

    - Aperiodic CSI-RS is triggered by DCI and/or MAC CE

    - FFS: Relationship of signaling between CSI reporting and CSI-RS transmission, e.g., common DCI signaling between CSI reporting and CSI-RS transmission

    - FFS: Necessity of additional signaling with MAC CE

- Note that further down selection can be done later between MAC CE and DCI in above bullets

- Note that it is possible to dynamically trigger RS and reports through links in the measurement setting

In this contribution, we discuss about aperiodic CSI reporting (A-CSI) design considering various design aspects.

Discussion on aperiodic CSI reporting for NR

In Rel-13 LTE, A-CSI is provided. While A-CSI reporting provides more concrete information via PUSCH which has larger container, periodic CSI reporting (P-CSI) provides compact information via PUCCH which is more robust. Considering such benefits and drawbacks, it would be preferred to support both in NR with streamlining to avoid unnecessary overlap which increases the number of redundant features and extra signalling. In order to streamline the specification, following design aspects should be considered with the main objective of A-CSI:

- CSI content

- CSI reporting procedure

- Signalling design for aperiodic CSI trigger.

In this contribution, dual-level CSI-RS structure is considered. A first type, termed level-1 CSI-RS (for non-UE-specific RS use cases) which is composed of K ≥ 1 CSI-RS resources. Each of the K static macro-beams represented by each of the K CSI-RS resources is termed the “coverage beam”. As evident, level-1 CSI-RS are being targeted for the non-UE-specific RS use case for CSI measurement and provides aggregate role of Rel-13 non-precoded (NP) CSI-RS and cell-specific beamformed (BF) CSI-RS. The second level of CSI-RS is analogous to Rel-13 UE-specific BF CSI-RS. Sharing the same characteristics, level-2 CSI-RS can be dynamically beamformed and composed of smaller number of ports than those for the level-1 CSI-RS. More details are given in a companion contribution [5].

  - CSI content

Two types of CSI (Type I and Type II CSI) with different spatial resolutions are agreed in RAN1#86bis [2]. Between the two types of CSI reporting, Type I feedback supports low spatial resolution feedback with dual stage codebook. For low spatial resolution feedback, implicit CSI which is inherited from HSDPA to Rel-8 LTE is suitable. This feedback paradigm fits quite well for all practical purposes and has advantages on testability (i.e. RAN4 PMI test inherited from HSDPA) and relatively low feedback overhead.

In contrast to Type I feedback, explicit CSI should be considered for Type II feedback. Considering evolution of LTE MIMO (dynamic switching between SU-/MU-MIMO by the introduction of DMRS-based transmissions, introduction of 12-/16-CSI-RS ports), the adequacy of implicit feedback paradigm was being questioned as per following aspects:

- Scalability: As implicit CSI requires a codebook design (designed for a small subset of particular array geometries), it is unclear how this is scalable for larger number of antenna ports. When we support larger number of antennas, possible candidate numbers of CSI-RS ports and its 2D antenna configurations should be considered. For example, combinations of CSI-RS ports and antenna combinations are provided in Table 1 when we consider maximum 20, 24, 28 and 32 CSI-RS ports in Rel-14 eFD-MIMO. In order to cope with the difficulty, parameterized codebook and Codebook-Config are introduced in Rel-13 FD-MIMO, but, it will not be sufficient when we support larger number of antenna ports for NR.

- SU-MU mismatch: As the system becomes more and more MU-centric due to the limit of UE hardware size and enhanced antenna capability of gNB on spatial separation, an inherent mismatch between CSI feedback (assuming SU) and gNB MU transmission becomes a limiting factor. This mismatch will result in an irreducible throughput loss regardless of the CSI resolution. In order to reduce such mismatch, the gNB typically employs any MU-CSI prediction algorithm from received SU CSI in conjunction with MU precoding algorithm such as ZF-BF or SLNR-BF. It is clear, however, that this scheme is sub-optimal. While ZF-BF or SLNR is a good criterion for MU precoding, it works quite well with uncompressed (albeit quantized) DL channel estimates rather than their coarse estimates with typical constraints from implicit feedback codebooks (e.g. constant modulus imposed to avoid performance penalty for SU transmission).

Table 1 Number of possible CSI-RS ports and its possible 2D antenna configuration

| Number of aggregated CSI-RS ports | Number of aggregated CSI-RS ports per polarization | Available antenna array geometry, (N1, N2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 10 | (1,10) | (2,5) | (5,2) | (10,1) |  |  |
| 24 | 12 | (1,12) | (2,6) | (3,4) | (4,3) | (6,2) | (12,1) |
| 28 | 14 | (1,14) | (2,7) | (7,2) | (14,1) |  |  |
| 32 | 16 | (1,16) | (2,8) | (4,4) | (8,2) | (16,1) |  |

In RAN1#86bis, however, the maximum number of CSI-RS ports is agreed as 32 antenna ports. This implies that an MU-centric design becomes more important (since the primary source of cell throughput gain for large number of transmit antennas comes from MU transmission). It is also expected that the drawbacks mentioned above become more severe. In order to better facilitates MU-MIMO multiplexing and circumvents the SU-MU mismatch inherent in implicit feedback paradigm, explicit CSI can be considered. Some examples of explicit CSI are direct (quantized) channel feedback, covariance matrix feedback, or eigenvector feedback via an UL control.

For the explicit CSI, linear combination codebook based feedback should be supported in NR. For linear combination codebook approach, it should be noted that support of linear combination codebook based advanced CSI is agreed in Rel-14 eFD-MIMO. While “explicit” feedback has been not supported in LTE, linear combination codebook based feedback has been verified through various simulations from many companies. For system performance aspects, linear combination codebook based explicit feedback provides more performance benefits than implicit CSI. In our companion contribution [7], initial simulations results are provided to compare the performance of implicit and explicit CSI. In the evaluation result, linear combination based explicit CSI can reduce performance gap with ideal CSI while the performance gap of implicit CSI is still large.

For the explicit CSI, CQI and RI may not be needed. When UE reports implicit CSI, acquiring direct channel information is impossible for gNB (except DL channel reciprocity in TDD). However, the situation is different when UE reports explicit feedback. Since UE reports explicit information on channel, gNB is able to find the optimized rank, modulation, channel coding rate and PMI. Moreover, it will be more accurate considering additional available information in gNB (e.g. MU-MIMO scheduling, coordination with other cell). However, additional interference feedback may be considered since gNB cannot directly measure inter-cell interference in UE’s position.

Based on the above discussion, proposals can be summarized as follows:

Proposals:

- Support implicit CSI for Type I feedback.

- Support explicit CSI for Type II feedback.

  - Support at least linear combination codebook based feedback for explicit CSI.

- Support both Type I and Type II feedback for aperiodic CSI reporting in NR.

  - CSI reporting procedure

Based on the CSI content discussed in the above, following design aspects should be considered to design CSI reporting procedure:

- Support of CSI-RS transmission types

- Reporting instances and PUCCH format for P-CSI

- Support of CSI reporting modes.

Support of CSI-RS transmission types

A streamlined CSI reporting framework can be obtained by correlating CSI reporting with CSI-RS type. While P-CSI is targeted for link maintenance rather than providing higher resolution CSI, A-CSI is to acquire detailed and accurate CSI. In order to provide such information, measurement on level-1 CSI-RS resources is essential. In this case, level-1 CSI-RS doesn’t have to be periodic since the A-CSI trigger is performed only when needed which reduces DL overhead (hence increases cell throughput) and potential inter-cell interference.

Support of CSI reporting modes

In legacy LTE, there are multiple CQI feedback types and PMI feedback types. Table 2 summarizes the comparison of supported CQI (wideband CQI, UE selected subband CQI and high layer-configured subband CQI) and PMI (no PMI, single PMI and multiple PMI) feedback types.

Table 2 Summary on CQI and PMI feedback types

| CQI feedback type | Features | PMI feedback type | Features |
| --- | --- | --- | --- |
| Wideband CQI | - Only one CQI feedback per codeword assuming wideband transmission - PMI assumption can be one or multiple - Supported in both P-CSI and A-CSI | No PMI | - CQI feedback assuming SFBC/FSTD - When eMIMO type is class B, codebook subset restriction based CQI can be provided - Supported in both P-CSI and A-CSI |
| UE selected subband CQI | - Multiple CQI feedback per codeword - UE selects and reports its CSI for preferred subband - Supported in both P-CSI and A-CSI | Single PMI | - UE selects only one PMI regardless of configured CQI feedback type - Supported in both P-CSI and A-CSI |
| High layer-configured subband CQI | - Multiple CQI feedback per codeword - UE selects and reports its CSI for preferred subband - Supported only in A-CSI | Multiple PMI | - UE selects multiple PMIs regardless of configured CQI feedback type - Supported only in A-CSI |

Among the described CQI feedback types, support of UE selected subband CQI for NR would be redundant feature. In order to report UE selected subband CQI, UE selects one of subband among subband group and calculates its CQI. This subband group is so called as ‘bandwidth parts’. This operation may reduce CSI reporting overhead, but limits the benefits of subband reporting. In contrast to high layer-configured subband CQI in which UE reports PMI/CQI for whole subbands, PMI/CQI is available only for the reported subbands. It is difficult to match gNB’s scheduling requirements and reported subbands and therefore there would be limited gain. Therefore, streamlining UE selected subband CQI for NR would be desirable. It should be noted that UE requires additional overhead to report the position of selected subband among the subband group in UE selected subband CQI.

In A-CSI, supporting both wideband CQI and high layer-configured subband CQI would be beneficial. Each of CQI feedback type has its own benefits. For example, when CSI is reliable, having detailed information for each subband can enhance system performance despite of its reporting overhead. However, when CSI is not reliable (e.g. high mobility or high inter-cell interference), reporting subband CSI would be meaningless. In that sense, supporting both wideband CQI and higher layer-configured subband CQI for NR is preferred.

Support of PMI feedback type is already well streamlined in Rel-13 and does not require any additional streamlining. In addition to the support of PMI feedback type, design of PMI feedback type configuration can be considered for NR. In Rel-13, for CRS based TMs, PMI feedback type configuration when UE reports its CSI relies on the configured TM (for example, TM3 only supports No PMI). The precoding that is predefined in the specification and indicated by configuration of TM is applied on top of the CRS. Therefore, a UE needs to receive only TM configuration and its CQI feedback type. Compared to CRS based TMs, independent RRC parameter (P-CSI) or explicit mode configuration (A-CSI) to turn on/off PMI/RI reporting is provided for DMRS based TMs. In NR, modular configuration on transmission scheme and CSI reporting is benenficial [4]. gNB can indicate its precoded channel without relation to CSI.

Based on the above discussion, proposals can be summarized as follows:

Proposals:

- Aperiodic CSI reporting procedure for NR should be designed by considering following proposals:

  - Aperiodic CSI supports both level-1 and level-2 CSI-RS with large CSI-RS ports.

  - Both wideband and subband CQI feedback are supported.

  - For PMI feedback type, no PMI/single PMI/multiple PMI are considered.

  - Signalling design for aperiodic CSI trigger

For the trigger of A-CSI, DCI based triggering is agreed in RAN1 NR Ad-Hoc [3]. Remaining part for the A-CSI triggering design is the necessity of additional signalling via MAC CE (MAC Control Element). Resource activation via MAC CE is introduced in eFD-MIMO to support dynamic activation and deactivation of resource candidates for multi-shot CSI-RS (which is similar to semi-persistent CSI-RS). The main motivation of additional signalling via MAC CE is to provide better error protection even though it requires higher latency than DCI. When misdetection of activation and deactivation occurs, issues such as collision of RS transmission can be problematic since it supports multiple RS transmission with periodicity and subframe offset. However, situation is different for A-CSI compared to multi-shot CSI-RS. Since A-CSI supports one shot transmission of A-CSI report, the impact of misdetection on A-CSI is relatively lower than S-CSI or semi-persistent CSI-RS.

Proposal:

- For Aperiodic CSI reporting, additional signalling with MAC CE is not needed.

Exemplary operation

An exemplary operation utilizing the above features is illustrated in Figure 1. A simple LTE subframe structure is assumed. Within one CSI process, a UE is configured with P-CSI linked to a level-2 CSI-RS [6] and A-CSI linked to 2-resource level-1 CSI-RS (two 16-port CSI-RS resources, forming a 32-port partial-port CSI-RS). Upon receiving an UL grant accompanied with one of the two resources associated with level-1 CSI-RS, the UE reports A-CSI containing explicit channel feedback. Unlike implicit feedback, the gNB can assemble these two A-CSI reports without any reconstruction issue. Assembling these two reports simply amounts to concatenating two sets of disjoint 16-port quantized channel into a 32-port quantized channel. With a 32-port DL channel estimate, the gNB performs UE-specific beamforming on CSI-RS. Configured with a 4-port level-2 CSI-RS, the UE reports P-CSI (consisting of wideband CQI, PMI, and RI) to maintain the link. Since the number of ports is small, the link can be well maintained with wideband PMI.

Figure 1 Joint use of aperiodic advanced CSI (using K=2 level-1 CSI-RS) and periodic legacy CSI (using UE-specific CSI-RS)

Conclusions

In this contribution, aperiodic CSI reporting design for NR is considered considering two design aspects: CSI contents and CSI reporting procedure. Building on the latest MIMO LTE features via streamlining and adding differentiating features, our proposals on CSI reporting can be summarized as follows:

- Support implicit CSI for Type I feedback.

- Support explicit CSI for Type II feedback.

  - Support at least linear combination codebook based feedback for explicit CSI.

- Support both Type I and Type II feedback for aperiodic CSI reporting in NR.

- Aperiodic CSI reporting procedure for NR should be designed by considering following proposals:

  - Aperiodic CSI supports both level-1 and level-2 CSI-RS with large CSI-RS ports.

  - Both wideband and subband CQI feedback are supported.

  - For PMI feedback type, no PMI/single PMI/multiple PMI are considered.

- For Aperiodic CSI reporting, additional signalling with MAC CE is not needed.

# References

- RP-160671, “New SID Proposal: Study on New Radio Access Technology”, NTT DOCOMO.

- 3GPP TSG RAN1#86bis, Chairman’s Note

- 3GPP TSG RAN1 NR Ad-Hoc, Chairman’s Note

- R1-17xxxxx, “CSI acquisition for DL NR MIMO”, Samsung

- R1-17xxxxx, “Discussions on CSI-RS design for NR MIMO”, Samsung

- R1-17xxxxx, “Discussions on periodic and semi-persistent CSI reporting for NR”, Samsung

- R1-17xxxxx, “Simulation results for explicit (Type II) CSI reporting for NR MIMO”, Samsung

- R1-17xxxxx, “Type I CSI reporting for NR”, Samsung

- R1-17xxxxx, “Type II CSI reporting for NR”, Samsung.