3GPP TSG RAN WG1 Meeting #88	R1-1703349

Athens, Greece, 13th - 17th February 2017

Agenda Item:	8.1.2.5

Source:	Huawei, HiSilicon

Title:	Discussion on link-level calibration for NR-MIMO

Document for:	Discussion and decision

Introduction

At RAN1#86 meeting, it was agreed that

- For the purpose of calibration, companies are encouraged to provide baseline results for NR MIMO in the following meetings, including link-level and system level simulations

  - Discuss further on baseline simulation assumptions and metrics for calibration in email discussion after RAN1#86

Further, in a follow up email discussion [86-20] MIMO calibration for NR, it was agreed that the phased approach will be used for calibration, which include 3 phases:

- Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (aim to finish it in RAN1#86bis)

- Phase 2:  Start discussion on whether and how to establish the baseline.  Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (aim to finish it in RAN1#87)

- Phase 3: Calibration can be used to check the UE movement/rotation/blockage (aim to finish it after RAN1#87)

Further, at RAN1#86bis meeting, it was agreed that

- Companies are encouraged to provide/update results for Phase 1 NR MIMO calibration in RAN1#87, including link level and system level simulations

  - Extend the email discussion for further clarification on simulation assumptions and metrics, if needed

- Including discussions on simulation assumptions and metrics for  Phase 2 NR MIMO calibration, including link level and system level simulations

In RAN1 NR AH meeting, the summary of email discussion [87-30] on NR-MIMO calibration was agreed in [1], where further clarification on phase2 calibration assumptions was also included. This contribution provides Phase 2 link level simulation results for MIMO calibration together with detailed evaluation parameters.

Simulation assumptions

The link level simulation assumptions of phase 2 calibration are described in this section. A summary is provided in Annex.

Channel model

The carrier frequencies under calibration are 30GHz and 4GHz with delay spread = 100ns and UE speed = 3km/h.  For 30GHz, the subcarrier spacing is 60kHz; for 4GHz, the subcarrier spacing is 15kHz.

The link-level channel model CDL-A and CDL-B in [2] are used as the channel model. The angle translation operation which has been detailed in 7.7.5.1 is applied, where the desired mean angles are uniformly distributed within the following intervals:

-  AoD in [-60°, 60°];

-  ZoD in [90°, 135°];

-  AoA in [-180°, 180°];

-  ZoA in [45°, 90°].

Antenna configuration

The antenna configuration is featured by the parameters (M, N, P, Mg, Ng), (dV, dH) and (dgV, dgH) as defined in [7].

For 30GHz, BS side antenna configuration is (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dV, dH) = (0.5, 0.5) λ and (dg,V, dg,H)=(2.0, 4.0) λ; UE side antenna configuration is (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2), (dV, dH) = (0.5, 0.5) λ and (dg,V, dg,H)=(0, 0) λ, where the panel orientation is Θmg,ng=90° and Ω0,1= Ω0,0+180°. Directional antenna is supposed at both BS and UE sides and the recommendation in [4] is used: for BS (Table A.2.1-6), θ3dB and φ3dB  are both 65° and maximum gain is 8dBi with SLAV=30 and Am=30; for UE (Table A.2.1-8), θ3dB and φ3dB  are both 90° and maximum gain is 5dBi with SLAV=25 and Am=25.

For 4GHz, BS side antenna configuration is (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ; UE side antenna configuration is (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1). As proposed in [6], BS antenna is supposed directional with radiation pattern according to [5] and UE antenna is Omni-directional.

The BS array orientation is azimuth 0°, downtilt 90° with respect to z-axis and the polarization angles are -45° and +45°. The UE array orientation is configured as follows: ΩUT,α is uniformly distributed in [0°,360°], ΩUT,β is 90°, ΩUT,γ is 0°, with polarization angles 0° and +90°.

TXRU mapping and beam sweeping

For 30GHz, a single TXRU per panel per polarization is assumed and 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal vectors. For 4GHz, TXRU virtualization is applied only in vertical dimension.

For beam selection, select the best beam pair among the limited set of DFT beams based on the criteria of maximizing receive power after beamforming. The limited beam direction set is as follows:

- Beam directions for BS@30GHz:

- Azimuth angle [-7*pi/16 -5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16 7*pi/16];

- Zenith angle  [pi/8  3*pi/8  5*pi/8  7*pi/8]

- Beam directions for BS@4GHz:

- Zenith angle  [pi/8  3*pi/8  5*pi/8  7*pi/8]

- Beam directions for UE @30GHz：

- Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8];

- Zenith angle [pi/4 3*pi/4];

Data allocation, RS pattern and digital beamforming

LTE frame structure with 12 REs-by-14 symbols is used: the first two symbols (24 REs) are assumed for control channel and one-port DMRS (12 REs), i.e., antenna port 7 for Rank=1 transmission as agreed in [3]. Meanwhile 8 PRBs, i.e., transmission block size (TBS) of 256, are allocated for data transmission and QPSK is used for modulation for MCS=1.

The RS pattern is shown in Figure 1.

Figure 1: RS pattern in calibration

For digital beamforming, SVD is supposed without delay that is based on ideal CSI feedback at the beginning of the slot per PRB. The same SVD beamforming is then used for the rest of the PRB. At the UE side, DMRS-based channel estimation is performed.

Metric

BLER with beamforming as a function of SNR ranging from -20 dB to 0dB is adopted as the metric of link level calibration.

Simulation results

In this section, the link level calibration results of phase 2 are presented, based on the assumptions described above and as shown in the Annex.

30GHz LLS calibration results

The calibration results for 30GHz have been presented in Figure 2.

Figure 2: BLER of LTE MCS 1 in CDL-A and CDL-B Channel 30GHz

4GHz LLS calibration results

The calibration results for 4GHz have been presented in Figure 3.

Figure 3: BLER of LTE MCS 1 in CDL-A and CDL-B Channel 4GHz

Conclusion

Phase 2 link level calibration results for NR-MIMO calibration have been presented in this contribution, based on simulation assumptions listed in section 2 and Annex.

- Reference

- R1-1701517, “Summary of email discussion [87-30] on NR-MIMO calibration”, ZTE, ZTE Microelectronics

- 3GPP TR 38.900, Channel Model for frequency spectrum above 6 GHz

- R1-1613741, “Way forward on Phase 1 and Phase 2 MIMO calibration for NR”, ZTE et al.

- 3GPP TR 38.802, Study on New Radio (NR) Access Technology – Physical Layer Aspects

- 3GPP TR 36.873, Study on 3D channel model for LTE

- R1-1608661, “Summary of [86-20] email discussion on MIMO calibration for NR”, ZTE

- R1-1700395, “Summary of 87-30 email discussion on NR MIMO calibration”, ZTE

Annex: Simulation Assumptions

Table 1. Parameter setting for LLS calibration phase 2

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier spacing | 15kHz | 60KHz |
| Delay spreading | 100 ns |  |
| Data allocation | 8RBs |  |
| Channel model | CDL-A, CDL-B |  |
| UE speed | 3km/h, direction uniformly distributed in x-y plane |  |
| desired mean angle | AoD in [-60°, 60°],ZoD in [90°,135°], AoA in [-180°, 180°], ZoA in [45°, 90°] |  |
| BS antenna configurations | (M, N, P, Mg, Ng)= (4, 4, 2, 1, 1) (dV, dH) = (0.5, 0.5)λ | (M, N, P, Mg, Ng)= (4, 8, 2, 2, 2) (dV, dH) = (0.5, 0.5)λ, (dg,V, dg,H) = (2.0, 4.0)λ |
| BS antenna pattern | θ3dB=φ3dB=65° Max Gain = 8dBi SLAV=Am=30 polarization angles are -45° and +45° |  |
| BS array orientation | azimuth 0°, mechanic downtilt 0° (zenith 90°) |  |
| UE antenna configurations | (M, N, P, Mg, Ng)= (1,1, 2, 1, 1) (dV, dH) = (0.5, 0.5)λ | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2) (dV, dH) = (0.5, 0.5)λ (dgV, dgH) = (0, 0)λ *The panel with the best receive SNR is chosen for output metric. |
| UE array orientation | ΩUT,α is uniformly distributed on [0°,360°], ΩUT,β is 90°, ΩUT,γ is 0°; | ΩUT,α is uniformly distributed on [0°,360°], ΩUT,β is 90°, ΩUT,γ is 0° panel orientation : Θmg,ng=90°; Ω0,1=Ω0,0+180°; |
| UE antenna pattern | omnidirectional | θ3dB=φ3dB=90° Max Gain = 5dBi SLAV=Am=25 polarization angles are 0° and +90° |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming |  |
| MCS | LTE MCS=1 |  |
| MIMO mode | SU-MIMO with Rank=1 |  |
| UE receiver type | MMSE-IRC |  |
| Metric | BLER with beamforming as a function of transmission SNR ranging from -20dB to 0dB |  |