3GPP TSG RAN WG1 Meeting #88                                                              R1-1703535

Athens, Greece, 13th –17th February 2017

Source:	ZTE, ZTE Microelectronics

Title:	Evaluation assumptions for Phase 2 NR MIMO link level calibration

Agenda Item:	8.1.2.5

Document for: 	Discussion and Decision

Introduction

In RAN1 NR Ad-Hoc meeting, the following agreements about the roadmap of NR-MIMO calibration have been reached [1].

Agreements:

- Set-up email discussions to collect phase1~3 results separately – Chuangxin (ZTE)

  - For updating phase-1 results: Until Feb. 6

    - Companies are encouraged to update results (particularly for the micro layer of dense urban scenario)

    - If further clarification of simulation assumptions is needed, it can also be discussed under this email thread

  - For phase-2 results: Until Mar. 31

  - For phase-3 results: Until Apr. 28

- Separate the excel sheet(s) for link level and system level.

- Email discussion moderator to summarize the above results and submit separate tdocs.

- Add in TR 38.802 A.1 link level evaluation assumptions/results for MIMO calibration

  - Summarize evaluation assumptions based on email discussion

  - Capture the reference tdoc # containing the link level evaluation assumption

  - Capture the reference tdoc # containing the excel sheet(s)

- Add in TR 38.802 A.2 system level evaluation assumptions/results for MIMO calibration

  - Summarize evaluation assumptions based on email discussion

  - Capture the reference tdoc # containing the system level evaluation assumption

  - Capture the reference tdoc # containing the excel sheet(s)

- FFS: how to capture the phase-2 and phase-3 results

This contribution summarizes the agreements for simulation assumptions of Phase 2 link level calibration as one draft to be captured in TR 38.802 A.1 link level evaluation assumptions/results for MIMO calibration.

Evaluation assumptions on NR-MIMO Phase 2 link level calibration

This section describes the agreements for simulation assumptions used for evaluating Phase 2 link level calibration [2]. Some further clarifications can be found in [3].

Table 1 Simulation assumptions for Phase 2 link-level calibration

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 4 GHz | 30 GHz |
| Subcarrier Spacing | 15kHz | 60kHz |
| Data allocation | 8 RBs - Note: Error free PDCCH decoding is assumed. - First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel - One-port DMRS (12 REs) |  |
| Channel Model | CDL-A and CDL-B - delay spread =100ns - UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly. |  |
| TXRU mapping to antenna elements | One TXRU per vertical dimension per polarization | One TXRU per panel per polarization |
| TXRU mapping weights | TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT, i.e., 1D sub-array partition model defined in TR36.897 | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming1). |  |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,4,2,1,1). (dV,dH) = (0.5, 0.5)λ | (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ |
| BS antenna element radiation pattern | According to TR36.873 | See Table A.2.1-6 in TR 38.802 |
| UE antenna configurations | (M,N,P,Mg,Ng) = (1,1,2,1,1) | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. 2) Θmg,ng=90; Ω0,1=Ω0,0+180; |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |  |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |  |
| UE antenna element radiation pattern | Omnidirectional | See Table A.2.1-8 in TR 38.802 |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD |  |
| MIMO mode | SU-MIMO with rank=1 |  |
| UE receiver type | MMSE-IRC |  |
| MCS | LTE MCS = 1 |  |
| DMRS channel estimation | 2DMMSE Based on LTE antenna port 7 |  |
| Metrics | BLER w/ beamforming as a function of transmission SNR ranging from -20 dB to 0dB |  |

- Note1: the DFT beam candidate in this beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows:

, for i=1,…,rN

where r =1 (which is analogous to oversampling factor of 1) , N denotes the number of vertical/horizontal antennas.

- Note2: The polarization angles are 0 and 90 and the panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels for above 6GHz. (Ωmg,ng, Θmg,ng)  is for orientation of the panel (mg, ng), 0≤mg<Mg, 0≤ng<Ng,  where the orientation of the first panel (Ω0,0, Θ0,0) is the same as UE orientation, Ωmg,ng is the array bearing angle and Θmg,ng is the array downtilt angle defined in [TR 36.873].

- References

- R1-1701516, WF on MIMO Calibration, NTT DOCOMO, ZTE, ZTE Microelectronics, LG Electronics, Samsung, Xinwei, Spreadtrum, ETRI

- R1-1613741, Way forward on Phase 1 and Phase 2 MIMO calibration for NR, ZTE, ZTE Microelectronics, Ericsson, LG Electronics, Spreadtrum Communications

- R1-1701517, Summary of email discussion [87-30] on NR-MIMO calibration, ZTE, ZTE Microelectronics