R1-1704065

update of R1-1703757

3GPP TSG RAN WG1 Meeting #88                                    	update of R1-1702028

Athens, Greece 13th - 17th February 2017

Source:	CATT

Title:	Some corrections on 36.213 PC5 V2V CRs

Agenda Item:	7.1.3

Document for:	Discussion and Decision

Introduction

In this contribution, some descriptions of CR 36.213 are discussed, and the corresponding modifications are proposed.

Discussion and modifications

  - Discussion on priority setting

Parameter priority-r14 has not been defined by higher layer, and the UE shall set the “Priority” field according to the highest priority determined among those priority(s) determined by higher layer corresponding to the transport block. See below text proposal:

----------------------------------------------------TP on CR 36.213 (start)---------------------------------------------------- - 14.2.1 UE procedure for transmitting the PSCCH For sidelink transmission mode 3, - The UE shall set the contents of the SCI format 1 as follows: - the UE shall set the Modulation and coding scheme field according to the Modulation and coding scheme indicated by the higher layer parameter mcs-r14 if the parameter is configured by higher layers. - the UE shall set the “Priority” field according to the highest priority among those priority(s) determined by higher layers corresponding to the transport block. For sidelink transmission mode 4, - the UE shall set the contents of the SCI format 1 as follows: - the UE shall set the Modulation and coding scheme field according to the Modulation and coding scheme indicated by the higher layer parameter mcs-r14 if the parameter is configured by higher layers. - the UE shall set the “Priority” field according to the highest priority among those priority(s) determined by higher layers corresponding to the transport block. ----------------------------------------------------TP on CR 36.213 (end)----------------------------------------------------

  - Discussion on MCS setting

The modulation and coding scheme of UE should be set according to the value determined by higher layers for both mode3 and mode4. See below text proposal:

----------------------------------------------------TP on CR 36.213 (start)---------------------------------------------------- - 14.2.1 UE procedure for transmitting the PSCCH For sidelink transmission mode 3, - The UE shall set the contents of the SCI format 1 as follows: - the UE shall set the Modulation and coding scheme field according to the Modulation and coding scheme determined by higher layers. For sidelink transmission mode 4, - the UE shall set the contents of the SCI format 1 as follows: - the UE shall set the Modulation and coding scheme field according to the Modulation and coding scheme determined by higher layers. ----------------------------------------------------TP on CR 36.213 (end)----------------------------------------------------

  - Discussion on the resources indicated by physical layer

The descriptions of the resources indicated by PHY layer to MAC layer by 36.213, but 36.321 are not consistent with each other. According to 36.321 CR R2-169121 [1], the MAC just select the resources from the set of resources provided by PHY layer, hence in 36.213 there is no need to mention that some resources may be excluded by lower layers. A  simpler way is to just inform the availabe resource set to higher layers. See below text proposal:

----------------------------------------------------TP on CR 36.213 (start)---------------------------------------------------- #### 14.1.1.6 UE procedure for determining the subset of resources to be informed to higher layers in PSSCH resource selection in sidelink transmission mode 4 When requested by higher layers in subframe n, the UE shall determine the set of resources to be informed to higher layers sfor PSSCH transmission according to the following steps. Higher layers determine the parameters the number of sub-channels to be used for the PSSCH transmission in a subframe, the resource reservation interval determined by higher layers, the priority to be transmitted in the associated SCI format 1 by the UE. is determined according to subclause 14.1.1.4B. 1) A candidate single-subframe resource for PSSCH transmission is defined as a set of contiguous sub-channels with sub-channel x+j in subframe where . The UE shall assume that any set of contiguous sub-channels included in the corresponding PSSCH resource pool (described in 14.1.5) within the time interval corresponds to one candidate single-subframe resource, where selections of and are up to UE implementations under and . UE selection of shall fulfill the latency requirement. The total number of the candidate single-subframe resources is denoted by. 2) The UE shall monitor subframes n-1001, n-1000, n-999, …, n-2 except for those in which its transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and S-RSSI measured in these subframes. 3) The parameter is set to the value indicated by the i-th SL-ThresPSSCH-RSRP field in SL-ThresPSSCH-RSRP-List-r14 where . 4) The set is initialized to the union of all the candidate single-subframe resources. The set is initialized to an empty set. 5) The UE shall exclude any candidate single-subframe resource from the set if it meets all the following conditions: - the UE receives an SCI format 1 in subframe , and “Resource reservation” field and “Priority” field in the received SCI format 1 indicate the values and , respectively. - PSSCH-RSRP measurement according to the received SCI format 1 is higher than . - the same SCI format 1 which is assumed to be received in subframe will determine according to 14.1.1.4C the set of resource blocks and subframes which overlaps with for j=0, 1, …, . 6) If the number of candidate single-subframe resources remaining in the set is smaller than , then Step 4 is repeated and Step 4 is repeated with increased by 3 dB. 7) For a candidate single-subframe resource remaining in the set, the metric is defined as the linear average of S-RSSI measured in sub-channels x+k for in the monitored subframes in Step 2 that can be expressed by for a non-negative integer j. 8) The UE moves the candidate single-subframe resource with the smallest metric from the set to . This step is repeated until the number of candidate single-subframe resources in the set becomes greater than or equal to . The UE shall inform the set to the higher layers. ----------------------------------------------------TP on CR 36.213 (end)----------------------------------------------------

Conclusion

In this contribution, we discuss and propose some modifications of CR 36.213 as follows:

(1) It is clarified that the “Priority” field is set according to the value provided by higher layer corresponding to the transport block.

(2) It is clarified that the modulation and coding scheme of UE should be set according to the value determined by higher layers for mode3 and mode4.

(3) It is proposed in 14.1.1.6 that physical layer shall inform the set  to higher layers.

References

  - 36321_CR0976R2_(Rel-14)_R2-169121 “Miscellaneous V2V corrections”, Ericsson, RAN2#96, November 2016