3GPP TSG RAN WG1 89 Meeting	R1-1706918

Hangzhou, China, May 15th - 19th 2017

Agenda Item:	7.1.3.3.6

Source:	Huawei, HiSilicon

Title:	Discussion on UL multiplexing of grant-based eMBB and grant-free URLLC

Document for:	Discussion and decision

Introduction

It is agreed in RAN1#86 that [1]

- At least the following potential options should be considered

  - At least for shorter transmission UL, semi-static resource sharing between URLLC and eMBB

    - FDM and/or TDM manner

      - UL grant-free transmission for URLLC

      - Other schemes are not precluded

  - Dynamic resource sharing between URLLC and eMBB

    - For DL, mechanisms to schedule a transmission where the resources of it can overlap with resources of ongoing/scheduled longer transmission at least from network perspective

      - FFS: A similar or same mechanism applicability to UL

      - Preemption or superposition

      - Other schemes are not precluded

    - Scheduling based approaches (e.g., by adapting transmission duration or by using different subbands) to allow multiplexing of different durations of transmission

    - UL grant-free transmission for URLLC

  - Other mechanisms are not precluded.

Furthermore,  the following agreements were reached for UL grant-free transmission [1]:

- Continue study at least the following:

  - Retransmission/repetition and potential combining, e.g. HARQ

RAN1#86bis has agreed the following [2]

- Slot aggregation is supported

- Data transmission can be scheduled to span one or multiple slots

and  consideration for further tradeoffs for meeting URLLC requirements for the following [2]

  - Semi-static resource allocation for UL data transmission.

  - Dynamic indication of available resource (e.g., by broadcast DCI) for UL data transmission.

  - Normal SR-based transmission

  - Other solutions are not precluded

and  the following was agreed regarding slot duration [2]

- For SCS of up to 60kHz with NCP, y = 7 and 14

  - FFS: whether/which to down select for certain SCS(s)

- For SCS of higher than 60kHz with NCP, y = 14

In RAN1 NR-Adhoc meeting, the following agreements were reached for an UL transmission scheme without grant

  - at least semi-static resource (re-)configuration is supported

    - FFS: The resource configuration includes at least physical resource in time and frequency domain and RS parameters

    - Higher-layer signaling could be similar to Rel-8 LTE SPS

    - FFS: MCS

  - RS is transmitted together with data

    - channel structure of grant-based data transmission can be starting point

and  for an UL transmission scheme with/without grant

  - K repetitions including initial transmission (with the same or different RV and FFS with different MCS) (K>=1) for the same transport block are supported,

      - FFS the way K is determined

    - FFS: hopping mechanisms over the transmissions

Based on these agreements, this paper focuses on UL URLLC multiplexing considerations in NR.

Coexistence of eMBB/URLLC in UL

NR may support diverse kinds of traffic in a common carrier with same or different numerology. Different traffic types, e.g., eMBB and URLLC have different KPI requirements and URLLC requires much shorter latency than eMBB. To satisfy the URLLC latency, shorter transmission interval can be adopted by using larger SCS in a separate BW part than eMBB which may use smaller SCS such as 30kHz or 15kHz. For smaller SCS, URLLC transmission can be based on mini-slot or symbols-based duration. Multiplexing solutions of eMBB and URLLC in shared resources in UL can be different depending on whether grant-based or grant-free transmission is used for URLLC.

Figure 1: Separate BW parts are configured for eMBB and URLLC.

Here, we assume eMBB adopts grant-based transmission and consists of significantly larger packets than URLLC, i.e., scheduling interval of eMBB is longer than URLLC transmission interval.

UL Resource sharing by dynamic indication to eMBB UEs

As URLLC traffic can be sporadic and transmitted in a shorter interval, e.g., mini-slot than that used for eMBB transmission, which can be slot-based, stopping an on-going grant-based eMBB transmission at the mini-slot granularity may not be feasible; this would result in excessive signaling overhead. In some cases, eMBB UEs may have to monitor DL control signaling to pause transmission at symbol(s) level, as different URLLC traffic can start transmission at different symbols.

Unlike DL multiplexing, where NW knows coexistence scenario and controls resource assignment, for UL resource sharing, the eMBB UE will have to reliably receive the indication and respond accordingly to ensure the reliability requirements of URLLC traffic. Hence, control signaling to adjust eMBB transmission in UL has to be very reliable and would cause lot of overhead, as it would need to be sent quite frequently for high URLLC payload. Blind detection and energy consumption at the eMBB UE side will increase significantly that may not be desirable. Our companion paper discusses grant-based coexistence in more detail [3]. On the other hand, grant-free transmission is more desirable to meet URLLC latency requirements, as it would not be bottlenecked by the delay of SR transmission, receiving and decoding the grant. As SR resources are reserved, frequent SR transmission opportunities may not be possible as well.

Observation 1: Reliable indication for UL pre-emption would cause lot of overhead, increased energy consumption and blind detection at the eMBB UE side, where indication monitoring would have to be done at a symbol level granularity.

Below, we discuss coexistence of grant-free URLLC and grant-based eMBB transmissions.

Semi-static resource sharing between grant-based eMBB and grant-free URLLC

URLLC UEs transmit in semi-statically configured resources and resources can be shared among different URLLC UEs for transmission. As UL eMBB transmission may not be dynamically punctured, in the carrier BW, semi-static resource sharing can be adopted between eMBB and URLLC, where certain BW part is configured for grant-free URLLC transmission. As URLLC traffic can be aperiodic, eMBB traffic could be scheduled in the URLLC band to improve resource utilization efficiency, depending on URLLC load statistics and reliability requirements. As a result, NW assigns resources to eMBB transmission maintaining controlled collision between eMBB and URLLC traffic by exploiting the grant-free resources configuration for URLLC transmission. Figure 2 shows an example of resource configuration where some part of URLLC region is reserved for grant-free transmission only, and some part is used as co-existence region where eMBB traffic can be scheduled. Hence, three possible scenarios are identified in the URLLC only region:

1) eMBB packets do not collide with URLLC;

2) URLLC traffic only in some reserved resources;

3) Collision of URLLC and eMBB packets may be observed.

If eMBB UEs are scheduled in a region where collision with URLLC may happen, power control mechanisms can be adopted for eMBB and/or URLLC transmissions. For example, eMBB UEs may be configured to adjust power to a certain level if they are scheduled in coexistence region.

Figure 2: URLLC region consists of a reserved region and a co-existence region. Co-existence region may observe some collision between eMBB and URLLC transmission.

Simulation results

Below, we show LLS results for URLLC and eMBB with controlled collision in coexistence region. We assume URLLC packet has four transmissions and some of its transmission may collide with eMBB data. Partial overlap can occur in time/frequency/power domain. For 60kHz SCS and 7-symbols slot, we assume one URLLC packet occupies 5RB in each transmission and eMBB packet occupies 10RB. We evaluate performance for a scenario where time/frequency resource is shared among 4 URLLC UEs and URLLC packets may observe eMBB interference in one or two transmissions out of four, i.e., partial overlap in time/frequency resources and there are some reserved areas where URLLC do not observe collision with eMBB (by pre-configured resource assignment). 5RB of eMBB data may collide with URLLC. Another option we explore is that eMBB transmit power can be controlled over the suspected collision region. In this example, 5RB of eMBB data may reduce power to 80%, other 5RBs do not observe power reduction. Advanced receiver is assumed for collision handling and interference cancellation. Detail simulation parameters are provided in Appendix. In Figure 3, we show URLLC BLER performance with 4 transmissions and URLLC UEs are decoded first treating eMBB as interference.

We observe that URLLC performance degrades very little compared to no collision, when one of its transmission (i.e., 25%) overlaps with eMBB which has power reduced to 80%. If 25% overlap is used with same power or 50% overlap is used with 80% power, performance is still reasonable, with less than 0.5dB loss.

In Figure 4, we show eMBB performance where 5 out of 10RB data may observe power reduction. We observe that partial power reduction causes small performance loss, with less than 0.5dB. Hence, allowing eMBB data to use coexistence region in a controlled manner improves the capacity of the coexistence region.

Observation 2: Resource efficiency can be improved by multiplexing eMBB with grant-free URLLC in UL.

- Collision can be controlled by semi-static resource assignment, e.g., partial overlap in resources (time/frequency/power) assigned to eMBB and URLLC transmission such that performance of each will not be degraded much.

Figure 3: URLLC Performance with controlled collision with eMBB in coexistence region

Figure 4: eMBB Performance in coexistence region, with partial power control.

Proposal 1: Grant-based eMBB transmission can be scheduled in part of the resources configured for URLLC grant-free transmission.

- This can be achieved by network implementation; no specific standardization work is expected.

Conclusion

In this contribution, we provide our views on the UL URLLC multiplexing design. We have the following proposal.

Observation 1: Reliable indication for UL pre-emption would cause lot of overhead, increased energy consumption and blind detection at the eMBB UE side, where indication monitoring would have to be done at a symbol level granularity.

Observation 2: Resource efficiency can be improved by multiplexing eMBB with grant-free URLLC in UL.

- Collision can be controlled by semi-static resource assignment, e.g., partial overlap in resources (time/frequency/power) assigned to eMBB and URLLC transmission such that performance of each will not be degraded much.

Proposal 1: Grant-based eMBB transmission can be scheduled in part of the resources configured for URLLC grant-free transmission.

- This can be achieved by network implementation; no specific standardization work is expected.

- References

- [1] RAN1 Chairman’s notes, RAN1#86, Gothenburg, Sweden, Aug 22-26, 2016.

- [2] RAN1 Chairman’s notes, RAN1#86bis, Lisbon, Portugal, Oct 10-14, 2016

- [3] R1-1708125, “Discussion on UL multiplexing of grant-based eMBB and URLLC” Huawei, HiSilicon, RAN1 89, May, 2017.

- Appendix

Table A-1: Simulation parameters used in LLS evaluation.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| User bandwidth | 5 RB (URLLC), 10RB (eMBB) |
| Modulation and coding | ½, QPSK(URLLC), ½ 16 QAM, 64 QAM (eMBB) |
| URLLC re-transmission scheme | IR, Number of transmissions = 4 |
| Number of URLLC UE collision | 4 |
| Channel model | TDLA, 3km/h |
| SNR range | -10 dB to 10 dB |
| Subcarrier spacing | 60KHz |
| TTI length | 0.125 ms |
| OFDM symbols per TTI | 7 |
| OFDM symbols for reference signals | 1 |
| BS Antenna configuration | 4 Rx |
| UE antenna elements | 1 Tx |
| Multiple access scheme | OFDMA |
| Channel estimation | Ideal |

BW partitioning between reserved and coexistence region is logical, i.e., resources may not be grouped in contiguous manner always.