3GPP TSG RAN WG1 RAN1 #89 Meeting                                              	          R1-1706968 Hangzhou, China, 15th-19th May 2017

Agenda Item:	7.1.4.4

Source:	Huawei, HiSilicon

Title:	Channel coding chain for control channel

Document for:	Discussion and Decision

# Introduction

In this contribution, we briefly discuss the channel coding chain design for NR eMBB control channel.

Notations:

K:	information bits length (including CRC bits)

M:	code block length

N:	mother code block length, equal to min2log2M,Nmax

R: 	code rate

I: 	Information bits set

F: 	Frozen bits set

P:	Shortening/Puncturing bits set

A: 	assistant bits set

Q:	ordered nested sequence

W:	weight sequence

PCF: PC frozen bits set

BIV:  bit reversal

# Channel coding chain for NR control channel

Summarizing the design described in [1] [2], Figure 1 illustrates the channel coding chain for NR control channel, which consists of channel coding and rate matching:

Fig. 1 Channel coding chain for control channel

  - Channel coding

As shown above, the channel coding consists of assistant bits placement and Polar encoding.

## Assistant Bit Placement

Assistant bit placement design shall be predefined according to the application scenario, including  assistant bits number, assistant bits type, the principle for placement and the check function design.

Polar codes with PC-CA design has been proposed in [1] to maximize the benefit from assistant bits, including assistant bits type/ number and check function design. The detailed steps about how to select the assistant bits set A and their position can be found in the Appendix of [1].

## Polar encoding

Polar encoding contains three parts: 1) bits position set selection, 2) encoding vector preparation and 3) kernel encoding.

Bits position set selection

The bits position set mainly includes information set I, assistant bits set A, and shortening/puncturing set P.

An ordered nested sequence Q0Nmax-1 of bit positions (index sequence) is generated according to the reliability of each bit position and stored [3]. Nmax is the maximum mother code length of Polar codes [4]. The reliability of each bit position is measured through a weight sequence W0Nmax-1, calculated as follows

Wi= j=0n-1bj*2j*14

where n = log2 (Nmax),  is decimal expression of the index. Then sorting of W0Nmax-1 such that WQ0≤WQ1≤WQ2≤…≤WQNmax-1 , and saving of the corresponding index sequence as Q0Nmax-1.

For mother code length N=min2log2M,Nmax [3], an ordered sequence Q0N-1 is generated by picking up all the indices which are smaller than N while sequentially reading Q0Nmax-1.

The main steps used to decide the bits position set I, A and P are listed below:

- shortening/puncturing set P：

A rate-matching scheme with fine granularity has been introduced in [2].

- Information bits set I

Select I according to sub-channel reliability and parameters, e.g., wmin introduced in [1];

- Assistant bits set A

Detail rules for selecting assistant bits set A can be refereed to [1].

Encoding vector preparation

Here, we can prepare the encoding vector u0, u1, u2,…, uN-1 as follows:

Put information bits on bit positions indicated by I and mark the shortening/puncturing bit positions according to P. Calculate the assistant bits and put them on the bit positions according to A. Assistant bits calculation is shown in the appendix of [1].

Kernel encoding

Kernel is constructed by recursively applying a linear polarization transform expressed as 2-by-2 matrix 1011 [5]. Repeating the use of this polarization transform n = log2(N) times, results in an N-by-N matrix Fn=1011n, where n denotes the n-fold Kronecker product.

The encoded bits x0, x1, x2,…, xN-1 are obtained by

x0  x1  x2  …  xN-1=u0  u1  u2  …  uN-1∙Fn

  - Rate matching

The rate matching is defined per encoded block and consists of bit collection and bit selection as shown in Fig. 2 (similar to section 5.1.4 in [6]).

Fig. 2 Rate matching

## Bit collection

Bit collection generates a virtual circular buffer and prepares encoded bits for bit selection operation.

For example, the encoded bits x0, x1, x2,…, xN-1 are input to the virtual circular buffer in a natural order, i.e., the output of length N is generated as follows:

wk=xk   for k = 0,1,…,N-1

Fig. 3 Virtual circular buffer

## Bit selection and pruning

Denoting by M the rate matching output sequence length, the rate matching output bit sequence is e0, e1, e2,…, eM-1. Bits are selected from the virtual circular buffer to form the final bit sequence to be transmitted.

# Conclusion

In this contribution, we discussed the channel coding chain designs for NR control channel and have the following proposal.

Proposal 1: The channel coding chain shall consist of assistant bits placement, Polar encoding and rate matching operations.

- Assistant bits placement shall maximize the benefit of assistant bits.

- Polar encoding operation shall include bits position set selection, encoding vector preparation and kernel encoding.

- Rate matching operation shall include bit collection, bit selection and pruning.

# References

- R1-1706965. “Polar Coding Design”, Huawei, HiSilicon, 3GPP TSG RAN1 Meeting #89, Hangzhou, China, 15th –19th May 2017.

- R1-1706967, “Rate matching for polar codes”, Huawei, HiSilicon, 3GPP TSG RAN1 Meeting #89, Hangzhou, China, 15th –19th May 2017.

- R1-1700088, “Summary of polar code design for control channels”, Huawei, HiSilicon, 3GPP TSG RAN1 Ad-Hoc Meeting, Spokane, USA, 16th –20th January 2017.

- R1-1701703, “Maximum mother code size of polar codes”, Huawei, HiSilicon, 3GPP TSG RAN1 Meeting #88, Athens, Greece, 13rd –17th February 2017.

- Final Report of 3GPP TSG RAN WG1 #AH1_NR v1.0.0

- Evolved Universal Terrestrial Radio Access (E-UTRA), Multiplexing and channel coding, 3GPP TS 36.212.