3GPP TSG RAN WG1 Meeting #89	R1-1707004

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	6.2.3.2.2

Source:	Huawei, HiSilicon

Title:	Evaluation results of transmit diversity solutions for R15 sidelink

Document for:	Discussion and decision

# Introduction

At the RAN1 #88bis meeting, the following agreement [1] for Transmit Diversity was reached:

- At least the following candidate TxD schemes for PSSCH transmission to be evaluated:

  - Small delay CDD

  - STBC (including half symbol STBC proposal in R1-1705002)

  - SFBC

  - PVS in time domain

- At least the following candidate TxD schemes for PSCCH transmission to be evaluated:

  - Small delay CDD

- Note: other schemes are not precluded provided that they fulfill objective 2 of the WID

Details of the simulated TxD schemes are described in [2]. In this contribution, initial evaluation results of these TxD schemes are presented.

# Evaluation results

In this section, the effect of TxD schemes on cubic metric (CM) is analyzed. In addition, the impact on Rel-14 UE is evaluated in the following scenarios:

- A  Rel-14 UE interferer

- A  Rel-15 UE interferer using TxD

## Cubic metric

Different TxD schemes are described in [2]. Some TxD schemes such as SFBC may break the single carrier property and significantly increase CM. The CM performance of different TxD schemes is evaluated in this section.

The CM is defined as [3]

CM=(20logrmsvnorm3n-1.52)/K

where 20logrmsvnorm3n is the raw cubic metric, vnormn=v(n)rmsv(n) , v(n) is a random sequence, rms denotes the root-mean-square, and K is an empirical constant and taken as 1.56 here.

Table 1 gives the computed CM values at each antenna for various TxD schemes with QPSK and 16QAM modulations. The values are obtained based on the assumption that PSSCH and PSCCH are FDMed in the same subframe, which breaks single carrier property, and that the PSD of PSCCH is 3dB higher than PSSCH. It shows that the CM value of antenna #2 with SFBC increases by 0.6-0.7 dB for QPSK and 0.3-0.4 dB for 16QAM, respectively. The other TxD schemes do not increase CM compared with the single antenna scheme.

Table 1 Computed CM Values

| TxD schemes | Antenna | QPSK |  | 16QAM |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 12PRBs (dB) | 20PRBs (dB) | 6PRBs (dB) | 10PRBs (dB) |
| Single antenna | Ant #1 | 1.53 | 1.34 | 2.63 | 2.39 |
| SFBC | Ant #1 | 1.53 | 1.34 | 2.63 | 2.39 |
|  | Ant #2 | 2.17 | 2.05 | 2.93 | 2.79 |
| STBC | Ant #1/Ant#2 | 1.53 | 1.34 | 2.63 | 2.39 |
| Small Delay CDD | Ant #1/Ant#2 | 1.53 | 1.34 | 2.63 | 2.39 |
| PVST | Ant #1/Ant#2 | 1.53 | 1.34 | 2.63 | 2.39 |

Observation 1: SFBC increases the CM by about 0.7 dB for QPSK and by about 0.4 dB for 16-QAM. Other TxD schemes do not affect the CM

## Performance of TxD schemes in AWGN scenario

The performance of different TxD schemes in AWGN scenario is evaluated in this section. For comparison, the performance of single antenna is also shown in the figures. The details of the simulation setting are shown in the Appendix.

### PSCCH

Figure 1 compares the performance of single transmitter and small delay CDD with 1us delay. It shows that small delay CDD scheme has performance gain at low speed scenario because of diversity gain in time domain. For high to extreme high speed scenarios, small delay CDD does not provide additional gain and has similar performance as single antenna transmission.

Observation 2: For PSCCH, small delay CDD can provide performance gain at low speed scenario.

| - 30km/h | - 280km/h |
| --- | --- |
| - 500km/h |  |

Figure 1. TxD performance of PSCCH

### PSSCH

Figure 2 compares the performance of different TxD schemes and single transmitter scheme with QPSK modulation. At low speed, all TxD schemes have better performance than the single transmitter case because of the diversity gain. At high and extreme high speeds, SFBC, STBC and small delay CDD schemes have similar performance and perform better than the single transmitter case. PVST underperforrms the other schemes because there is no channel estimation interpolation between slots.

Figure 3 compares the performance of different TxD schemes and single transmitter scheme with 16QAM modulation. It shows that SFBC and STBC have better performance for different speed cases. Small delay CDD is worse than SFBC and STBC for the high speed case.

The results show that SFBC and STBC are robust in all scenarios. Furthermore, considering the higher CM of SFBC, STBC is recommended for PSSCH.

Observation 3: for PSSCH, STBC and SFBC are robust in all considered scenarios. PVST has worse performance in high speed scenario. Small delay CDD has worse performance in high speed scenario combined with 16QAM modulation.

Observation 4: SFBC has similar performance than STBC at high and extreme high speed scenarios.

| - 30km/h | - 280km |
| --- | --- |
| (c) 500km |  |

Figure 2. TxD performance of PSSCH with QPSK modulation*

| (a) 30km | (b) 280km |
| --- | --- |

Figure 3. TxD performance of PSSCH with 16QAM modulation**

* Note: QPSK with coding rate 0.25 evaluated for 500km/h

** Note: 16QAM for 500km/h is not evaluated because only low MCSs can be used at 500km/h.

## Performance of TxD schemes in interference scenario

The impact of interference on Rel-14 UE is evaluated in this section. The interfering UE is either a Rel-14 UE (with single transmitter) or Rel-15 UE (with Small Delay CDD scheme for PSCCH and STBC scheme for PSSCH) respectively. The details of the simulation setting are shown in the Appendix.

Figure 4 and figure 5 show the impact of interference on Rel-14 UEs PSCCH and PSSCH performance, respectively. For PSCCH, interferers from Rel-15 UEs lead to worse performance than interferer from Rel-14 UEs. The performance gap between Rel-14 interferer and Rel-15 interferer is about 1-2 dB for a BLER of 1%. For PSSCH, the performance is the same whether the interferer is a Rel-14 or Rel-15 UE.

Observation 5:

- For PSCCH: A Rel-14 UE interfered by a Rel-15 UE has a 1-2dB performance loss than when interfered by a Rel-15 UE

- For PSSCH: the performance of a Rel-14 UE is the same whether interfered by a Rel-14 or Rel-15 UE

Figure 4. Impact of interference on Rel-14 UE’s PSCCH performance

Figure 5. Impact of interference on Rel-14 UE’s PSSCH performance

# Conclusions

The performance of various TxD schemes is evaluated in the scenario of AWGN and interference respectively. In addition, the CM is also simulated. Based on the results, the following observations and proposals are given.

Observation 1: SFBC increases the CM by about 0.7 dB for QPSK and by about 0.4 dB for 16-QAM. Other TxD schemes do not affect the CM

Observation 2: For PSCCH, small delay CDD can provide performance gain at low speed scenario.

Observation 3: for PSSCH, STBC and SFBC are robust in all considered scenarios. PVST has worse performance in high speed scenario. Small delay CDD has worse performance in high speed scenario combined with 16QAM modulation.

Observation 4: SFBC has similar performance than STBC at high and extreme high speed scenarios.

Observation 5:

- For PSCCH: A Rel-14 UE interfered by a Rel-15 UE has a 1-2dB performance loss than when interfered by a Rel-15 UE

- For PSSCH: the performance of a Rel-14 UE is the same whether interfered by a Rel-14 or Rel-15 UE

# References

RAN1 Chairman’s notes, 3GPP RAN1 #88bis, April 3 – 7, 2017.

R1-1707003, Transmit diversity solutions for R15 sidelink, Huawei, HiSilicon,

Lianjun Deng, etc., “Performance comparison of open-loop transmit diversity schemes for DFT-precoded OFDMA in uplink frequency-selective fading channels”, ISPACS 2010.

# Appendix A: Simulation setting

Table 2 Parameter setting of link level simulation

| Parameter | Value |
| --- | --- |
| General parameters: |  |
| Carrier frequency | 6G Hz |
| System bandwidth | 10 MHz |
| Channel Model | ITU-R UMi NLOS |
| Vehicle speed (absolute) | 15 km/h, 140km/h, 250km/h |
| CFO | 1800 Hz |
| Timing error | 2 us |
| Tx-Rx antenna configuration | 2x2 |
| TxD schemes | - SFBC - STBC - Small Delay CDD (with 1us delay) - PVST |
| Receiver type | MMSE |
| Frequency offset estimation | Half-sequence correlation in time domain |
| Interference scenarios: |  |
| Interference type | {AWGN, Rel-14 UE, Rel-15 UE} |
| SNR | 25dB |
| SINR | -8:24 dB |
| PSSCH parameters: |  |
| TBS | 300Bytes, 190Bytes |
| Modulation and Coding rate | 15km/h, 140km/h: {QPSK 0.5 and 16QAM 0.5} 250km/h: {QPSK 0.25 and 16QAM 0.25} |
| PSCCH parameters: |  |
| TBS | 48bits (including 16bits CRC) |
| Modulation | QPSK |
| PRB | 2 |

# Appendix B: Simulation results for 190Bytes

| - 30km/h | - 280km |
| --- | --- |
| (c) 500km |  |

Figure 6 TxD performance of PSSCH with QPSK modulation

| (a) 30km | (b) 280km |
| --- | --- |

Figure 7 TxD performance of PSSCH with 16QAM modulation

Figure 8 Impact of interference on Rel-14 UE’s PSSCH performance