3GPP TSG RAN WG1 Meeting #89 Meeting	R1-1707007

Hangzhou, China, 15th-19th May 2017

Agenda Item:	7.1.4.4

Source:	Huawei, HiSilicon

Title:	Channel coding for URLLC

Document for:	Discussion and Decision

# Introduction

Channel coding for URLLC scenario has been discussed from RAN1#85 meeting and several contributions have provided performance evaluation of the different channel coding candidate schemes. However the RAN1 discussions on channel coding have prioritized the eMBB scenario and the RAN1 work has progressed with the selection of the channel coding schemes for data and control channels for eMBB. The requirements for URLLC are different from eMBB and further study is needed to select the channel coding scheme for URLLC.

URLLC (Ultra-Reliable and Low Latency Communications), one of NR (new radio) scenarios, requests a channel coding scheme for a reliable transmission in a short latency [1][2].

- Transmission reliability: to reach an error-floor-free BLER as low as 10-5 with or without HARQ support during 1-ms period

- Latency: one packet within 1 ms latency

Although detailed specifications of URLLC applications are under discussions, some of its potential key implications over channel coding scheme would be:

- Information packets less than 200 bytes

- Code rates no higher than 1/3

- Error-floor below the BLER of 10-5

- Several-OFDM duration decoding latency

In this contribution, to compare different channel coding scheme candidates for uRLLC scenario, we mainly focus on the investigation on low latency design and high reliability property.

# Discussion and Performance Evaluation

## BLER performance

The BLER performance comparison between Polar and LDPC codes has been thoroughly investigated in the previous contributions [3]. Here, in this contribution, we only provide more results on different fading channels and modulation orders, which can be found in the Appendix. Simulation assumptions are also in the appendix, where LDPC scheme 1 comes from [4] and LDPC scheme 2 comes from [5]. For Polar codes, we use the design described in [6].

As we can see from the performance comparison under fading channel, Polar always outperforms LDPC.  This is because the potential length of URLLC is short and the coding rate is low. LDPC needs more iterations to get convergence for short information length and low coding rate leading to high computational complexity, high power consumption and low energy efficiency. The high computational complexity also leads to longer decoding latencies which is inappropriate under the URLLC scenario.

## Error Floor

To meet the reliability requirement of URLLC scenario, a channel coding scheme with high coding gain and without error floor should be chosen. In this section, we mainly focus on the error floor performance of the candidate channel coding schemes.

It is well known and observed that LDPC codes have error floor even above BLER of 10-5 in [7] and [8]. 107 blocks are simulated to verify this observation; meanwhile, Polar codes are also simulated.

- K=100

- K=200

Figure 1 Error floor verification

Clearly in Figure 1, for K=100 and R=1/3, LDPC 1 shows error floor at above 10-5, and for K=200 and R=1/6, both LDPC schemes have error floors. As proven in [9], Polar codes have no error floor, which is also verified in Figure 1 and [3]. Both LDPC schemes show error floor above BLER of 10-5 for some cases, which makes them not applicable for uRLLC.

Observation-1:  Polar codes have no error floor, while both LDPC schemes in [4] and [5] show error floor above BLER of 10-5.

For LDPC codes, error-floor occurs for short information. In [10], it was mentioned that for QC-LDPC for a given coding rate, the minimum distance cannot be increased by increasing the code length and thus, the girth as for random constructions. So it is impossible to construct a QC-LDPC matrix with both good waterfall region and low error-floor. On the other hand, during the evaluation, we implement the LOMS algorithm in float-point way. However, when it comes to fix-point, the error-floor will be even higher.

Note that only BP-like algorithm can be used because of the high computational complexity of ML-like for LDPC. Different from list decoder for Polar, BP-like algorithm (Belief Propagation) used by LDPC code is a sub-optimal decoding algorithm and the performance gap to ML-like (say OSD algorithm with high order) algorithm is huge [11]. Furthermore, from the engineering point, only Min-Sum like algorithm is implementable which has another performance gap to BP-like algorithm.

## Decoder power

Since the decoding power consumption is highly related with the computational complexity, the comparison of decoder complexity between Polar and LDPC can reflect the difference in the energy efficiency. Here, we evaluate the computation complexity of LDPC [4] and Polar [6]. For LDPC, LOMS (Layered Normalized Min-Sum) algorithm with iteration 20 is taken into consideration, and the total computational complexity can be expressed as [12]:

I*Z*2dvNb+2dc-1Mb+2Mb

where, I stands for the iteration number, Z the circulating size, dv the degree of the row of PCM and dc the degree of the column of PCM, Nb the number of columns of protomatrix, Mb the number of parity bit columns of PCM.

For Polar, SCL-8 is adopted, and the computational complexity can be expressed as [13]
R*L*N*log2N+L*N-1+K*2*L*log2(2*L)

where R stands for percentage of frozen bits skipped before the first information bits over the total number of bits, N mother code length, K information block length, L list size.

For the sake of fair comparison, we set information as 100, 400 and 1000, and the coding rate 1/3.

The computational complexity is shown as below:

Figure 2	the decoding operations needed by Polar and LDPC

As we can see, LDPC takes more than 2.5 times complexity than that of Polar.

Observation-2:  Polar codes have lower decoding complexity compare to LDPC codes in uRLLC scenario for low coding rate and short block length.

## Decoding Latency

The following table provides estimated decoding latency for uRLLC codewords with a Polar decoder [6]. The maximum mother code length is limited to 4096 for the latency estimations. Codeword segmentation and parallel decoding for Polar codes can also be considered to reduce further the latency [14].

Table 1  uRLLC Decoding Latency Estimates

| Codeword Length (N) | 512 | 1024 | 2048 | 1024 | 2048 | 2048 | 4096 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Code Rate (R) | 1/3 | 1/6 | 1/12 | 1/3 | 1/6 | 1/3 | 1/3 |
| Info Bits (K) | 100 | 100 | 100 | 200 | 200 | 600 | 1000 |
| Latency (µs) @ 1GHz | 0.51 | 0.89 | 1.52 | 1.19 | 1.78 | 2.45 | 4.22 |

The decoding latency estimates were obtained with functional simulations of a PC-CA SCL8 Polar decoder implementation. The results show that Polar codes can meet the latency requirement even for a system with 60kHz sub-carrier spacing and self-contained slot structure.

Observation-3:  Polar codes meet uRLLC decoding latency requirements.

# IR-HARQ Scheme

Polar IR-HARQ scheme proposed in [2] to improve the coding gain of polar codes attributed to further polarization, outperforms LDPC codes for every (re-)transmissions in AWGN and fading channels, as shown in Figure 3 (refer from [2]).

(a)	AWGN channel

(b)	Fading channel

Figure 3 IR-HARQ performance of Polar and LDPC codes

Because the Arikan polarization kernel is recursive, a short polar code can be extended to a longer one, which generates incremental coded bits for IR-HARQ. Figure 4 illustrates a generic diagram of this polar IR-HARQ scheme. Its coding gain is attributed to the fact that the overall polarization reliability is enhanced when the code length is increased. It is well-known that short polar code can be recursively extended to the longer one. Then, a polar decoder is able to recursively combine several short code words into a longer one at the input LLR stage. In each re-transmission, the encoder generates a longer codeword with incremental redundancy codeword via encoding partial information bits with further step of polarization, and only transmits the incremental redundancy part. At the decoder side, the received LLRs of all transmissions are combined and decoded as a long codeword.

Figure 4 Overview of IR-HARQ scheme of polar codes

Since a punctured polar code can be considered as coding on a reduced polar kernel, the punctured bits can be recovered or extended in retransmissions, which enlarges the polar kernel. There is no limitation on the extended code length, which means that arbitrary number of incremental coded bits can be generated in retransmission, which supports the adaptive IR-HARQ for polar codes.

Observation-4:  Polar codes support adaptive IR-HARQ and performance is better than LDPC codes.

# Conclusion

In this contribution, we discuss the channel coding scheme candidates for uRLLC scenario. Polar and LDPC are compared in terms of BLER performance, error floor and decoding complexity (both computation and implementation). Polar codes show better performance than LDPC. Moreover, we show that Polar codes support IR-HARQ very well, which meet the requirement of uRLLC.

Observation-1:  Polar codes have no error floor, while both LDPC schemes in [4] and [5] show error floor above BLER of 10-5.

Observation-2:  Polar codes have low decoding complexity compare to LDPC codes in uRLLC scenario for low coding rate and short block length.

Observation-3:  Polar codes meet uRLLC decoding latency requirements.

Observation-4:  Polar codes support adaptive IR-HARQ and performance is better than LDPC codes.

Further evaluation on Polar and LDPC for URLLC scenario is needed, and we have the following proposal.

Proposal: Channel coding for URLLC shall be further studied.

# References

[1] R1-1704249, Channel coding for PBCH, Huawei, HiSilicon.

[2] R1-1611255, HARQ scheme for polar codes, Huawei, HiSilicon.

[3] R1-1611692, Channel coding schemes for URLLC scenario, Huawei, HiSilicon

[4] R1-1705627, LDPC code design for larger lift sizes, Qualcomm Incorporated.

[5] R1-1705420, Performance evaluation of LDPC code, Samsung.

[6] R1-1706965. “Polar Coding Design”, Huawei, HiSilicon.

[7] T. Richardson, Error floors of LDPC codes. Proceedings of the annual Allerton conference on communication control and computing. Vol. 41. No. 3. The University; 2003.

[8] Lin, Shu, and D. J. Costello. Error control coding. Pearson Education India, 2004.

[9] M. Mondelli, S. H. Hassani, R. Urbanke, Unified Scaling of Polar Codes: Error Exponent, Scaling Exponent, Moderate Deviations, and Error Floors, IEEE transactions on information theory, 2016.

[10] Marc P.C. Fossorier, Quasi-Cyclic Low-Density Parity-Check Codes From Circulant Permutation, Matrices, IEEE Transactions on Information Theory, vol. 50, No. 8, Aug. 2004

[11]	R1-1612277, Near optimal performance for LDPC with OSD/List decoding, Nokia

[12]	R1-165360, Selection of the eMBB channel coding scheme, Nokia

[13]	R1-164040, On latency and complexity, Huawei, HiSilicon

[14] R1-1608865, Design aspects of Polar Code and LDPC for NR, Huawei, HiSilicon

# Appendix

We compare the coding gains among LDPC and Polar codes, in the following simulation assumptions. Both AWGN and fading channels are considered in the simulations.

Table 2	Simulation Assumptions for URLLC scenario

| Channel | TDL-A 100ns, TDL-C 300ns |  |  |
| --- | --- | --- | --- |
| Modulation | QPSK |  |  |
| Coding Scheme | LDPC 1 [4] | LDPC 2 [5] | PC-CA Polar [6] |
| Code rate | 1/12, 1/6, 1/3 |  |  |
| Decoding algorithm | LNMS (20) CRC24 | LOMS（20） CRC24 | List-8 T=8 CRC27 |
| Info. block length (bits w/o CRC) | 40, 200, 600, 1000 |  |  |

Figure 5   BLER of candidate coding schemes under TDL-A100 channel (QPSK)

Figure 6   BLER of candidate coding schemes under TDL-A100 channel (16QAM)

Figure 7   BLER of candidate coding schemes under TDL-C300 channel (QPSK)

Figure 8   BLER of candidate coding schemes under TDL-C300 channel (16QAM)

Figure 9   BLER of candidate coding schemes with 2Tx 2Rx

Figure 5~9 show the BLER performance comparison between Polar and LDPC under fading channels. Polar always shows better performance compared to both LDPC schemes.