3GPP TSG RAN WG1 Meeting #89		R1-1707231

Hangzhou, P.R. China 15th – 19th May 2017

Source:	vivo

Title:	 CCE size and bundling size for NR-PDCCH

Agenda Item:	7.1.3.1.1

Document for:	Discussion and Decision

- Introduction & Background

CCE size and bundling size were considered in the last RAN1 meeting as follows,

Working assumption:

  - A NR-CCE is defined as 6 REGs

    - Candidate bundle sizes for distributed REG-to-CCE mapping: 2 or 3 REGs if NR-CCE is defined as 6 REGs

  - FFS: impact of the NR-CCE definition on CORESET size, CCE aggregation levels, data resource allocation granularity, etc.

In this contribution, further investigation on CCE size and bundling size are provided. For CCE size, the average REG usage for a PDCCH transmission is evaluated according to the SINR distribution from link level simulation. And for bundling size, the performance comparing different AL are provided.

- Discussion

  - CCE Size

The following assumptions and metrics are considered,

- Average number of used REGs for a PDCCH

  - In order to fairly compare different CCE size, the average number of used REGs for a PDCCH is considered. The average number of used REGs for a PDCCH is derived from averaging the number of used REGs for different AL (aggregation level) weighted by the probability this AL is being used.

  - The probability a certain AL is used is derived from the SINR distribution in a cell. Here ideal AL adaptation according to SINR is assumed.

  - The system-level results for SINR distribution is provided by Phase 1 NR MIMO SLS calibration [3]. The mean value of wideband SINR for beamforming is used in the calculation (see Annex B). Without loss of generality, Macro-DenseUrban-4G is assumed.

- DCI size

  - From [2], it is estimated that the DCI size may vary from 51 to 77 bits. NR DCI shall support UE-specific configured feature targeting different scenarios, its presence and absence is configurable. The most common used DCI fields are considered in the simulation. Multi-slots scheduling, additional DMRS pattern, aperiodic SRS/CSI-RS request, CIF, QCL and PDSCH mapping are not counted in this comparison.

  - In summary, 51-bit is assumed as DCI size in the simulation

- Single-port precoding cycling is considered in the simulation. The DMRS for PDCCH is provided in Annex A.

- Distributed mapping is considered for CCE-to-REG mapping

- 3 Km/h, TDL-C, 300ns

- Annex A provides the other simulation assumptions

Figure 1. LLS curve for CCE 4 and CCE 6 (AL = 1,2,4,8, 1-port PC, RS with length-2 OCC)

According to the link level curve in Figure 1, a further analysis on the average number of used REGs for a PDCCH is provided as follows,

Table 1. Average number of used REGs without beamforming

| W/o BF | CCE = 4 REGs |  | CCE = 6 REGs |  |
| --- | --- | --- | --- | --- |
|  | Required SNR | Percentage | Required SNR | Percentage |
| AL=1 | 14.8183 | 7% | 8.5441 | 21% |
| AL=2 | 4.2078 | 36% | 2.1003 | 34% |
| AL=4 | -0.3706 | 31% | -2.2602 | 33% |
| AL=8 | -3.9075 | 21% | -5.4094 | 10.5% |
| Failed |  | 5% |  | 1.5% |
| Average # of REGs | 14.84 REGs (100%) |  | 18.30 REGs (123%) |  |

Table 2. Average number of used REGs with beamforming

| W/ BF | CCE = 4 REGs |  | CCE = 6 REGs |  |
| --- | --- | --- | --- | --- |
|  | Required SNR | Percentage | Required SNR | Percentage |
| AL=1 | 14.8183 | 66% | 8.5441 | 92% |
| AL=2 | 4.2078 | 32.5% | 2.1003 | 6.5% |
| AL=4 | -0.3706 | 1.0% | -2.2602 | 0.3% |
| AL=8 | -3.9075 | 0.5% | -5.4094 | 0.2% |
| Failed |  | 0 |  | 0 |
| Average # of REGs | 5.56 REGs (100%) |  | 6.468 REGs (116%) |  |

From the results, it is shown that the control resource overhead for CCE = 6 is much higher than CCE = 4.

- For beamforming PDCCH case, CCE=6 costs 16% additional overhead than CCE=4.

- For non-beamforming PDCCH case, CCE=6 costs 23% additional overhead than CCE=4.

Proposal 1: Reconsider the CCE size of NR PDCCH. From resource utilization perpestive, we propose that a NR-CCE is defined as 4 REGs.

  - Bundling Size

The following assumptions are considered,

- DCI size = 76-bits (including 16-bit CRC)

- Single-port precoding cycling is considered in the simulation. The DMRS for PDCCH is provided in Annex A.

- Distributed mapping is considered for CCE-to-REG mapping

- 3 Km/h, TDL-C, 300ns

- Annex A provides the other simulation assumptions

Figure 2. Performance for NR-PDCCH with different ALs and bundling sizes

It can be observed from Figure 2 that for the precoding cycling scheme, distributed resource allocation with different aggregation levels,

- For AL=1, bundling size =1 outperforms others

- For AL=2, bundling size =2 outperforms others

- For AL=3, bundling size =2 outperforms others

- For AL=4, bundling size =3 outperforms others

Proposal 2 : For the precoding cycling scheme, different REG-bundling sizes for different aggregation levels shall be considered.

- Conclusion

This contribution discusses some considerations on NR CCE size and bundling size. In summary, it is proposed that,

Proposal 1: Reconsider the CCE size of NR PDCCH. From resource utilization perpestive, we propose that a NR-CCE is defined as 4 REGs.

Proposal 2 : For the precoding cycling scheme, different REG-bundling sizes for different aggregation levels shall be considered.

References

- RAN WG1#88bis Chairman’s Notes.

- R1-1707234, “DCI content size and compression”, vivo, RAN1#89, Hangzhou, P.R. China 15th - 19th May 2017

- Phase 1 SLS Calibration for NR MIMO, v54

Annex A. Simulation assumptions

Table A-1 Link-level simulation assumptions

| Parameter | Assumption |
| --- | --- |
| Bandwidth | 20 MHz |
| Channel model | 3 Km/h, TDL-C, 300ns |
| Channel estimation | Ideal MMSE |
| Resource allocation | One OFDM symbol is reserved for CCE mapping |
|  | One PRB for one REG |
| Control Channel Payload | 35-bit / 60-bit payload, 16-bit CRC |
| Coding scheme | Polar Code |
| Modulation Scheme | QPSK, Max-log-likehood algorithm |
| Subcarrier spacing | 15kHz |
| Symbol length | Normal CP 160Ts + 2048 Ts, Ts =1/30.72us |
| Antenna Model | 2 Tx, 2 Rx |
| Transmission scheme | 1-port Precoder cycling, OCC = [1 -1], RS pattern in Figure A1-1 |
| CCE size | 4 / 6 RBs, 1/3 RS overhead |

Figure A-1. RS pattern for PDCCH

Annex B-1 cdfplot of Wideband SINR with beamforming

With Beamforming

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -2.78 | 20 | 11.75 | 40 | 16.13 | 60 | 20.34 | 80 | 26.08 |
| 1 | 3.59 | 21 | 12.00 | 41 | 16.34 | 61 | 20.56 | 81 | 26.38 |
| 2 | 5.12 | 22 | 12.26 | 42 | 16.54 | 62 | 20.82 | 82 | 26.72 |
| 3 | 5.85 | 23 | 12.49 | 43 | 16.74 | 63 | 21.05 | 83 | 27.08 |
| 4 | 6.55 | 24 | 12.72 | 44 | 16.95 | 64 | 21.32 | 84 | 27.57 |
| 5 | 7.10 | 25 | 12.92 | 45 | 17.13 | 65 | 21.56 | 85 | 27.94 |
| 6 | 7.64 | 26 | 13.13 | 46 | 17.32 | 66 | 21.80 | 86 | 28.42 |
| 7 | 8.07 | 27 | 13.33 | 47 | 17.53 | 67 | 22.05 | 87 | 28.86 |
| 8 | 8.45 | 28 | 13.53 | 48 | 17.74 | 68 | 22.29 | 88 | 29.42 |
| 9 | 8.74 | 29 | 13.76 | 49 | 17.93 | 69 | 22.54 | 89 | 29.95 |
| 10 | 9.07 | 30 | 14.00 | 50 | 18.12 | 70 | 22.81 | 90 | 30.51 |
| 11 | 9.43 | 31 | 14.20 | 51 | 18.34 | 71 | 23.11 | 91 | 31.12 |
| 12 | 9.71 | 32 | 14.41 | 52 | 18.57 | 72 | 23.44 | 92 | 31.71 |
| 13 | 10.00 | 33 | 14.61 | 53 | 18.79 | 73 | 23.79 | 93 | 32.36 |
| 14 | 10.27 | 34 | 14.87 | 54 | 19.02 | 74 | 24.12 | 94 | 33.12 |
| 15 | 10.53 | 35 | 15.09 | 55 | 19.24 | 75 | 24.40 | 95 | 33.84 |
| 16 | 10.81 | 36 | 15.30 | 56 | 19.47 | 76 | 24.73 | 96 | 34.69 |
| 17 | 11.06 | 37 | 15.50 | 57 | 19.69 | 77 | 25.05 | 97 | 35.95 |
| 18 | 11.26 | 38 | 15.71 | 58 | 19.90 | 78 | 25.37 | 98 | 37.79 |
| 19 | 11.50 | 39 | 15.91 | 59 | 20.12 | 79 | 25.73 | 99 | 40.68 |
|  |  |  |  |  |  |  |  | 100 | 48.95 |

Figure B-1. SLS calibration results, mean wideband SINR with beamforming (dB)

Annex B-2 cdfplot of Wideband SINR without beamforming

Without Beamforming

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -9.28 | 20 | -1.10 | 40 | 1.42 | 60 | 4.39 | 80 | 8.89 |
| 1 | -5.77 | 21 | -0.95 | 41 | 1.57 | 61 | 4.59 | 81 | 9.20 |
| 2 | -4.94 | 22 | -0.83 | 42 | 1.71 | 62 | 4.78 | 82 | 9.58 |
| 3 | -4.43 | 23 | -0.70 | 43 | 1.84 | 63 | 4.95 | 83 | 9.95 |
| 4 | -4.09 | 24 | -0.58 | 44 | 1.98 | 64 | 5.16 | 84 | 10.34 |
| 5 | -3.79 | 25 | -0.45 | 45 | 2.13 | 65 | 5.33 | 85 | 10.66 |
| 6 | -3.52 | 26 | -0.32 | 46 | 2.26 | 66 | 5.56 | 86 | 11.02 |
| 7 | -3.26 | 27 | -0.18 | 47 | 2.39 | 67 | 5.77 | 87 | 11.44 |
| 8 | -3.04 | 28 | -0.04 | 48 | 2.54 | 68 | 5.96 | 88 | 11.88 |
| 9 | -2.84 | 29 | 0.07 | 49 | 2.67 | 69 | 6.16 | 89 | 12.37 |
| 10 | -2.65 | 30 | 0.20 | 50 | 2.83 | 70 | 6.38 | 90 | 12.86 |
| 11 | -2.48 | 31 | 0.32 | 51 | 2.98 | 71 | 6.59 | 91 | 13.34 |
| 12 | -2.31 | 32 | 0.43 | 52 | 3.14 | 72 | 6.83 | 92 | 13.93 |
| 13 | -2.12 | 33 | 0.54 | 53 | 3.28 | 73 | 7.07 | 93 | 14.60 |
| 14 | -1.97 | 34 | 0.67 | 54 | 3.45 | 74 | 7.30 | 94 | 15.33 |
| 15 | -1.81 | 35 | 0.78 | 55 | 3.58 | 75 | 7.53 | 95 | 16.07 |
| 16 | -1.65 | 36 | 0.89 | 56 | 3.73 | 76 | 7.77 | 96 | 17.09 |
| 17 | -1.52 | 37 | 1.03 | 57 | 3.92 | 77 | 8.06 | 97 | 18.10 |
| 18 | -1.38 | 38 | 1.17 | 58 | 4.08 | 78 | 8.36 | 98 | 19.50 |
| 19 | -1.24 | 39 | 1.30 | 59 | 4.21 | 79 | 8.65 | 99 | 21.45 |
|  |  |  |  |  |  |  |  | 100 | 27.64 |

Figure B-2. SLS calibration results, mean wideband SINR without beamforming (dB)