3GPP TSG RAN WG1 Meeting #89		R1-1707232

Hangzhou, P.R. China 15th – 19th May 2017

Source:	vivo

Title:	 CORESET and BD for NR-PDCCH

Agenda Item:	7.1.3.1.2

Document for:	Discussion and Decision

- Introduction & Background

The mapping rules for REG were discussed in RAN1#88bis meeting. Time-first and frequency first mapping for CCE-to-REG has its own merits to be adopted as candidate. However, blind decoding for these two schemes need to be clarified [1]. This contribution addresses this issue.

- Discussions

  - CORESET duration

Regarding what information should be carrier on the ‘group common PDCCH’, some FFS were made after RAN1 NR adhoc meeting [2].

Agreements:

- FFS: ‘Control resource set duration’

  - FFS: Indicates the duration of the control resource set(s)

  - FFS: Can help the UE skip some of the semi-statically configured blind decodings. If not received, the UE performs all blind decodings.

Considering the benifits for reducing the UE bllind decoding, it is preferred to introduce the ‘CORESET duration’ from the network to instruct UE blind decoding. The agreeemnts made here gives preference to be included in ‘group common PDCCH’, however, we are also open to be included in other way, e.g., RRC and/or MAC signalling, which are more reliablely to be received but less frequently to be updated.

Proposal 1 : UE is indicated by the network of ‘CORESET duration’. FFS how to indicate ‘CORESET duration’ to UE.

  - Blind decoding by considering CORESET duration

The following notations are used

N: ‘CORESET duration’,

M1: index of first OFDM symbol of CORESET,

M2: index of last OFDM symbol of CORESET,

Figure 1 shows the UE operations when time-first and frequency-first CCE-to-REG mapping are configured respectively.

- For a CORESET configured with time-first mapping,

  - UE is not expected to perform blind decoding operations in the CORESET when N < M2

- For a CORESET configured with frequency-first mapping,

  - UE is not expected to perform blind decoding operations for the CORESET when N < M1

  - UE is not expected to perform blind decoding from symbol N+1 to M2 in the CORESET when M1<=N< M2

Proposal 2 : Define UE blind decoding behaviour to differnet CCE-to-REG mapping scheme.

Figure 1. Indication for reducing number of blind decoding operations

  - Reducing blind decoding by considering other indications

Indication for scaling number of PDCCH decoding operations seems promising. A simple approach for such indication is on/off with 1-bit. This gives guidance to the UE whether performing PDCCH decoding is needed or not.

Proposal 3: FFS other indications beside ‘CORESET duration’ for the purpose of reducing the UE blind decoding.

- Conclusion

This contribution discusses the blind decoding by consideration the CORESET design. It is proposed as follows,

Proposal 1 : UE is indicated by the network of ‘CORESET duration’. FFS how to indicate ‘CORESET duration’ to UE.

Proposal 2 : Define UE blind decoding behaviour to differnet CCE-to-REG mapping scheme.

- Denote N = ‘CORESET duration’, M1 = index of first OFDM symbol of CORESET, M2 = index of last OFDM symbol of CORESET

- For a CORESET configured with time-first mapping,

  - UE is not expected to perform blind decoding operations in the CORESET when N < M2

- For a CORESET configured with frequency-first mapping,

  - UE is not expected to perform blind decoding operations for the CORESET when N < M1

  - UE is not expected to perform blind decoding from symbol N+1 to M2 in the CORESET when M1<=N< M2

Proposal 3: FFS other indications beside ‘CORESET duration’ for the purpose of reducing the UE blind decoding.

References

- R1-1706692, “WF on blind decoding on CORESET”, vivo, Samsung, ZTE, ZTE Microelectronics, InterDigital, RAN1#88bis, Spokane, USA, 3rd - 7th April 2017

- R1-1701521, “Summary of offline discussion on group common PDCCH – part 2”, Ericsson, RAN1#88bis, Spokane, USA, 3rd - 7th April 2017