3GPP TSG RAN WG1 Meeting #89		                                                      R1-1707464

Hangzhou, P.R. China 15th – 19th May 2017

Source:	CATT

Title:	Further details on NR 4-step RA Procedure

Agenda Item:	7.1.1.4.2

Document for:	Discussion and Decision

# Introduction

NR shall support a 4-step RA procedure similarly to LTE. During the NR SI phase, a basic RA framework was agreed targeting both single- and multi-beam operation. Some key agreements relevant to the discussion in this paper include

- Regardless of whether TX/RX reciprocity is available or not at gNB at least for multiple beams operation, the following RACH procedure is considered for at least UE in idle mode

  - Association between one or multiple occasions for SS block and a subset of RACH resources and/or subset of preamble indices is informed to UE by broadcast system information or known to UE or FFS dedicated signaling

    - FFS gNB can configure an association between CSI-RS for L3 mobility and a subset of RACH resources and/or a subset of preamble indices, for determining Msg2 DL Tx beam

  - At gNB, the DL TX beam for message 2 can be obtained based on the detected RACH preamble/resource and the corresponding association

- At least for the case without gNB Tx/Rx beam correspondence, gNB can configure an association between DL signal/channel, and a subset of RACH resources and/or a subset of preamble indices, for determining Msg2 DL Tx beam.

  - Based on the DL measurement and the corresponding association, UE selects the subset of RACH resources and/or the subset of RACH preamble indices

- During a RACH transmission occasion of single or multiple/repeated preamble(s), as informed by broadcast system information, UE uses the same UE TX beam.

- Msg1 definition:

  - a random access preamble format consists of one or multiple random access preamble(s),

  - a random access preamble consists of one preamble sequence plus CP, and

  - one preamble sequence consists of one or multiple RACH OFDM symbol(s)

- UE transmits PRACH according to the configured random access preamble format

- For 4-step RACH procedure, a RACH transmission occasion is defined as the time-frequency resource on which a PRACH message 1 is transmitted using the configured PRACH preamble format with a single particular TX beam

- A single RAR reception is assumed at a UE within a given RAR window

In this contribution we discuss several outstanding details needed to finalize the basic RA functionality in Rel-15.

# Discussion

In NR a preamble format may consist of one or multiple RA preambles. Figure 1 depicts a scenario where the network configures a preamble format made up of multiple RA preambles, where each constituent preamble consists of N (N ≥ 1) symbols. Performance evaluation of different preamble formats is presented in a separate contribution [1]. The RACH occasion in this example takes up one slot duration. If there is no analog beam correspondence at the receiver side, the TRP may sweep its analog RX beams across the N symbols of each preamble contained in the preamble format.

Figure 1. PRACH occasion for a multi-preamble format and the associated RAR window

One outstanding issue is the timing of the RAR window corresponding to a preamble (Msg1) transmission. In LTE, the RAR window starts 2 subframes (2ms) after the subframe containing the end of the preamble. Furthermore, the RAR window length is configurable from 2 to 10 subframes allowing for some flexibility in provisioning gNB resources to handle the computationally intensive RACH processing load. Moreover, the lower subcarrier spacing employed for LTE PRACH could result in different FFT implementations between PUSCH/PUCCH and PRACH. In contrast, NR systems should support advanced receiver circuitry capable of faster data processing on the order of at least 4 times LTE subcarrier spacing, particularly for above 6GHz deployment scenarios. Secondly, one NR proposal is to adopt the same subcarrier spacing for NR-PUSCH/PUCCH/PRACH simplifying the UL receiver circuitry. Finally, URLLC services would require across the board reduction in processing times to meet tighter latency budget requirements.  Therefore, a corresponding reduction in RA processing time is a reasonable expectation for NR, although the reduction may not be linearly scaled with the numerology.

Proposal: NR processing time reduction in comparison to LTE should be applicable to all channels including PUSCH, PUCCH, and PRACH.

In LTE the UE monitors for a PDCCH scheduling a RAR, where the CRC of the DCI is scrambled by an RA-RNTI. The RA-RNTI is a function of the first subframe index containing the PRACH (t_id) and the PRACH frequency subband (f_id). For LTE non-BL/CE UEs, the RA-RNTI is given by

RA-RNTI= 1 + t_id + 10*f_id

In NR as long as there is a one-to-one association between a PRACH time-frequency resource and a DL TX beam, the same design principle can be reused. It is up to the network to ensure unambiguous association between DL beams and PRACH configurations. The timing reference for computing the RA-RNTI could be based on either a subframe granularity (as in LTE) or based on the same time granularity used for configuring RACH time-domain resources. In the latter case, the timing reference could follow the slot timing within a radio frame with respect to the reference numerology.

Proposal:

- Indication of the RAR corresponding to a transmitted PRACH is a function of the time and frequency location of the PRACH.

- The timing reference for the RA-RNTI is based on slot granularity within a radio frame with respect to the reference numerology.

## Other details for RA procedure in Idle Mode

The necessity of multiple Msg1 transmissions within a RAR window was discussed at the RAN1 #88 and RAN1#88bis without agreement.. A related discussion involved the necessity of a UE monitoring for multiple RARs within a configured RAR window. It was agreed that:

Agreements:

- For contention-free random access, the following options are under evaluation

  - Option 1: Transmission of only a single Msg.1 before the end of a monitored RAR window

  - Option 2: A UE can be configured to transmit multiple simultaneous Msg.1

    - Note: multiple simultaneous Msg.1 transmissions use different frequency resources and/or use the same frequency resource with different preamble indices

  - Option 3: A UE can be configured to transmit multiple Msg.1 over multiple RACH transmission occasions in the time domain before the end of a monitored RAR window

- Following is baseline UE behavior

  - UE assumes single RAR reception at a UE within a given RAR window

- NR random access design should not preclude UE reception of multiple RAR within a given RAR window, if need arises

In a previous contribution [2] we provided our views on multiple Msg1 transmissions and multiple RAR receptions within a RAR window. Here, we provide further analysis on these proposals.

The main motivation for multiple Msg1 transmissions is that a UE without DL/UL beam correspondence may need to sweep its UL TX beams to find an UL TX/RX beam pair for a successful RA procedure. The associated latency due to UL beam sweeping may not meet a target latency budget. For example, when handover is initiated a UE may not have UL beam alignment to the target gNB as discussed in [3].

One solution to reduce the overall RA latency is to allow a UE transmit multiple preambles without necessarily waiting to receive a RAR after the first such transmission. This implies that the network configures a burst of RACH occasions, where the burst is linked to the same RAR window. An illustration is shown in Figure 2, where the RACH occasions are not necessarily contiguous.

Figure 2. Configuration of a set of RACH occasions and a single extended RAR window dimensioned to support PRACH processing for Msg1 transmissions across the same set

The RAR window is extended to provision for gNB PRACH processing time from the first to last RACH occasion. Some possible issues with this scheme include:

- It is not clear whether all Msg1 transmissions are part of the same RA procedure. If part of the same RA procedure, it may imply that all UEs – even those with beam correspondence – are mandated to transmit multiple preambles leading to significant resource overhead and inefficient device power utilization. Alternatively, a UE may select a subset of the RACH occasions to transmit, while monitoring for a corresponding RAR message over an extended RAR window.

- It complicates the RA procedure by specifying different solutions depending on the degree of beam correspondence at the UE.

It should be noted that the network has flexible mechanisms to support all user categories (full, partial or no beam correspondence). Rather than an extended RAR window as shown in Figure 2, an alternative solution is that the network may provision PRACH resources that accomplish the same goal but using independent RA procedures, i.e. preserving the existing linkage between a RACH occasion and RAR window.. As shown in Figure 3 a UE initiates multiple independent RA procedures by transmitting Msg1 using different TX beams in each configured RACH occasion. Furthermore, the UE monitors separate and possibly overlapping RAR windows as shown in Figure 3. The main benefit of this approach lies in the fact that since the network is unaware of a UE’s beam correspondence, at least for initial access, it decouples a UE’s autonomous use of multiple TX beams from the RA procedure. It is up to UE implementation to independently select which subset of RACH occasions for beam-switched RA procedures. Secondly, as each RA procedure is independent, the preamble transmission counter is initialized for each RA procedure. This re-initialization ensures that a minimum UL power is used for each TX beam minimizing the inter-cell interference.

Observation: It is up to the network to configure PRACH occasions that could facilitate TX beam switching at a UE without beam correspondence.

Figure 3. Configuration of multiple PRACH occasions and corresponding RAR windows to enable UE TX beam sweeping

Option 1 should be agreeable as the baseline since it is sufficient at least for UEs with beam correspondence or with a single UL TX beam. For UEs without beam correspondence it is first necessary to evaluate how much of a problem a misalignment in the received DL beam and selected UL TX beam would be before introducing additional specification. Note that additional specifications would also affect the MAC layer as it requires a relaxation of the LTE MAC principle, wherein only one RA procedure is ongoing at a given time instance.

Proposal:

- Baseline UE procedure is the transmission of a single Msg.1 before the end of a monitored RAR window

- First evaluate the performance impact of beam non-correspondence at the UE for Msg1 detection before considering multiple simultaneous RA procedures.

Other use cases for multiple RAR receptions were discussed in [4]. In one scenario two UEs select the same DL TX beam and also select the same preamble from the subset of preambles configured for contention-based RA at a specific RACH occasion. If the TRP performs RX beam sweeping and detects the said preamble in different RX beams, the network cannot determine if it is the same UE or different UEs. One possibility is for the network to transmit multiple RAR messages within the RAR window. Note that transmitting multiple RARs within the RAR window is not helpful even if the RAR contains an UL RX beam identifier because a UE cannot determine which UL RX beam is aligned to its UL TX beam. Therefore, each UE would have to perform multiple Msg3 transmissions and monitor for multiple contention-resolution messages. Whilst this is a possible solution, the likelihood of this event – two or more UEs attempt RA in the same RACH occasion based on the same DL TX beam and select the same preamble – may be small enough that further optimization is unnecessary. Furthermore, this case is somewhat equivalent to the well known LTE contention case, where at most 1 UE will declare a successful RA procedure after the contention resolution message is received.

A related scenario was also presented in [4], where two or more TRPs receive the preamble and due to non-ideal backhaul there is no coordination of which TRP should send the RAR. Again, there is no need for optimization as it is sufficient for the UE to detect one RAR.

Observation: we have not currently identified a need for multiple RARs within a RAR window due to colliding UEs in UL RX beam sweeping or Msg1 reception at loosely coordinated TRPs.

## Non-contention based RA procedure

It is assumed that NR would support the same contention-free scenarios as in LTE, namely, handover, DL data arrival, positioning and obtaining timing advance alignment for a secondary TAG. For scenarios such as DL data arrival when the UE is not UL time aligned, it is assumed that the DL beam pair link is valid and known to both the gNB and UE. Therefore, the gNB can provide a dedicated RACH resource for Msg1 transmission.

For the handover case, RAN2 agreed to support reporting of individual beam measurement i.e. that network can configure the UE to report the N best beams [5]. A UE may be configured to measure on one or more SS blocks or other RS in a neighboring cell. If one of the neighboring cell SS-block measurements triggers a handover request, the source gNB could signal a preferred beam index in a handover request to the target gNB. The target gNB in turn could provide a beam-specific dedicated RACH resource (including preamble) in the handover command. Alternatively, the target gNB could provide a set of dedicated resources e.g. one for each SS-block in the handover command. The UE then transmits Msg1 using the dedicated preamble corresponding to the preferred DL beam in the target cell.

Proposal: for contention-free random access, a UE is configured with a dedicated RACH resource (including preamble) corresponding to a preferred DL beam or with a set of dedicated RACH resources corresponding to a set of SS blocks.

## Power ramping procedure

It was agreed at the RAN1#88bis meeting to support power ramping as follows

Agreements:

For NR RA Msg. 1 retransmission at least for multi-beam operation:

- NR supports power ramping.

  - If the UE conducts beam switching, working assumption that one of the alternatives below will be selected (configurability between multiple alternatives may be considered if clear benefit is shown):

    - Alt 1: the counter of power ramping is re-set.

    - Alt 2: the counter of power ramping remains unchanged.

    - Alt 3: the counter of power ramping keeps increasing.

    - Alt 4: as proposed on slide 4 and illustrated on slide 5 in R1-1706613

    - Other alternatives or combinations of the above are not precluded.

  - If UE doesn’t change beam, the counter of power ramping keeps increasing.

  - Note: UE may derive the uplink transmit power using the most recent estimate of path loss.

  - The detail of power ramping step size is FFS.

- Whether UE performs UL Beam switching during retransmissions is up to UE implementation

  - Note: which beam UE switches to is up to UE implementation

In LTE, a UE assumes a PRACH (Msg1) transmission failure if it does not detect a RAR within a specified RAR window. Such an event occurs either because of a preamble collision or insufficient UL transmission power. Accordingly, the UE increments its preamble transmission counter, effectively ramping up its transmission power by a configured step size, and re-transmits the preamble after a random backoff time. When a UE does not have beam correspondence or the beam coherence time has elapsed after a previous gNB-UE beam alignment, the UE may decide to switch its beams before attempting a new Msg1 transmission. Incrementing the preamble transmission counter without beam alignment may cause increased inter-cell interference with no guarantee of success as the UL TX-RX beams may still be misaligned. Therefore, this option should be ruled out. Resetting the preamble transmission counter ensures minimum inter-cell interference but may also increase latency for an UL TX beam sweeping procedure. Alternatively, the UE may maintain the preamble transmission counter at its current state.

Proposal: If a UE switches its UL TX beam during a RA procedure, decide between resetting and keeping the current value of the preamble transmission counter.

## Association between CSI-RS for L3 mobility and RACH resources

In RAN1#88bis, the following agreement was made that

  - Association between one or multiple occasions for SS block and a subset of RACH resources and/or subset of preamble indices is informed to UE by broadcast system information or known to UE or FFS dedicated signaling

    - FFS gNB can configure an association between CSI-RS for L3 mobility and a subset of RACH resources and/or a subset of preamble indices, for determining Msg2 DL Tx beam

RA procedure for UE In Idle mode

For UE in idle mode, the association between one or multiple occasions for SS block and a subset of RACH resources and/or subset of preamble indices is informed to UE by broadcast system information or known to UE or FFS dedicated signaling. In addition, CSI-RS for RRM for L3 mobility is configured using dedicated RRC signaling for supporting UE in CONNECTED mode, but not UE in idle mode. It is thus clear that there is no motivation for gNB to configure an association between CSI-RS for L3 mobility and a subset of RACH resources and/or a subset of preamble indices.

RA procedure for UE in CONNECTED mode

For UE in CONNECTED mode, a UE can be configured with CSI-RS resources for L3 mobility. The UE will conduct CSI-RS RSRP measurements on the configured CSI-RS resources. The RA procedure may be carried out in the following scenarios:

Contention Free RA. After receiving UE measurement report on CSI-RS RSRP measurement results with the indication of the associated CSI-RS resource back to the gNB, the gNB will select the dedicated RACH resource (time/frequency resource as well as the preamble index) and provide the information to the UE for the transmission of the PRACH preamble. The gNB should have all of the information needed to select the right DL Tx beam for Msg 2 when receiving the PRACH preamble based on its internal known association of the CSI-RS resource and DL Tx beam. Thus, it seems no particular benefits for gNB to specifically “configure an association between CSI-RS for L3 mobility and a subset of RACH resources and/or a subset of preamble indices, for determining Msg2 DL Tx beam”.

Contention Based RA. For contention based RA, the UE may not send UE measurement report on CSI-RS RSRP measurement back to the gNB. In this case, the gNB may not know whether the UE transmits the PRACH transmission based on CSI-RS RSRP measurements or based on SS-block RSRP measurements. In this case, it is desirable for the gNB to know whether the UE transmits the PRACH transmission based on SS-block RSRP, or CSI-RS RSRP, and if based on CSI-RSRP, based on which CSI-RS resource. The information will help the gNB to select the proper DL Tx beam for Msg 2. Therefore, it will be beneficial to configure the association between CSI-RS for L3 mobility and a subset of preamble indices. On the other hand, it seems no particular advantage to separately allocate a subset of RACH (time/frequency) resources for the association. It will increase the complexity of the PRACH resource configuration and reduce the RACH resource sharing without particular benefits.

Proposal: Further investigate the benefits to configure an association between CSI-RS for L3 mobility and a subset of preamble indices for supporting Contention Based RA for UE in RRC_CONNECTED mode.

# Conclusion

This contribution discussed several open issues for the NR 4-step RA procedure. UL waveform selection for RA Msg1 and Msg3 was also addressed. Our proposals are summarized as follows:

- NR processing time reduction in comparison to LTE should be applicable to all channels including PUSCH, PUCCH, and PRACH.

- Indication of the RAR corresponding to a transmitted PRACH is a function of the time and frequency location of the PRACH.

- The timing reference for the RA-RNTI is based on slot granularity within a radio frame with respect to the reference numerology.

- Observation: It is up to the network to configure PRACH occasions that could facilitate TX beam switching at a UE without beam correspondence.

- Regarding multiple Msg1 transmissions

  - Baseline UE procedure is the transmission of a single Msg.1 before the end of a monitored RAR window

  - First evaluate the performance impact of beam non-correspondence at the UE for Msg1 detection before considering multiple simultaneous RA procedures

- Observation: we have not currently identified a need for multiple RARs within a RAR window due to colliding UEs in UL RX beam sweeping or Msg1 reception at loosely coordinated TRPs.

- For contention-free random access, a UE is configured with a dedicated RACH resource (including preamble) corresponding to a preferred DL beam or with a set of dedicated RACH resources corresponding to a set of SS blocks.

- If a UE switches its UL TX beam during a RA procedure, decide between resetting or keeping the current value of the preamble transmission counter.

- Further investigate the benefits to configure an association between CSI-RS for L3 mobility and a subset of preamble indices for supporting Contention Based RA for UE in RRC_CONNECTED mode.

# References

  - R1-1706183, “On NR RACH Preamble Design”, CATT, RAN1 #89, May 15-19, 2017

  - R1-1702066, “Further details on NR 4-step RA Procedure”, CATT, RAN1 #88bis, April 3 – 7, 2017

  - R1-1703399, “On multiple Msg.1 transmissions”, ZTE, ZTE Microelectronics, Lenovo, Sony, Nokia, ASB,  RAN1 #88, Feb 13 – 17, 2017

  - R1-17001366, “WF on Multiple RAR”, Samsung, ZTE, NTT DOCOMO, RAN1 AH_NR, Jan 16 – 20, 2017

  - Chairman’s Notes, RAN2 #97, Feb 13 – 17, 2017