3GPP TSG RAN WG1 Meeting #89                                            						R1-1707465

Hangzhou, China,	15th - 19th May 2017

Agenda item:	7.1.1.5.1

Source: 	CATT

Title: 	NR Measurements and Mobility Management in IDLE and CONNECTED state

Document for:		Discussion and Decision

Introduction

In RAN1#88bis, RRM measurements for L3 mobility were agreed as follows,

# Agreements:

- For CONNECTED mode RRM measurement for L3 mobility based on CSI-RS, NR supports following targets for CSI-RS design and configuration:

- Design:

- Reuse CSI-RS design for beam management as baseline

- Identify additional requirements on the CSI-RS to support L3 mobility

- Configuration:

- Support measurement of a large number of beams

- Minimize configuration overhead, reporting overhead, and UE complexity

- FFS: Detailed signaling or format

# Agreements:

- For Connected mode, CSI-RS are supported to be configured using at least dedicated RRC signaling for DL based RRM measurement for L3 mobility.

  - Note that signalling other than dedicated RRC signalling is not precluded

# Agreements:

- Clarify previous RAN1 agreements on the RSRP definition for DL RRM measurements for L3 mobility as follows

  - Define SS block RSRP and CSI-RS RSRP as

    - SS block RSRP : measured RSRP from SSS

      - FFS additional use of PBCH-DMRS for measurement

    - CSI-RS RSRP : measured RSRP from CSI-RS in connected mode

# Agreements:

- The time synchronization reference for a CSI-RS for L3 mobility is the frame/slot/symbol timing of a cell.

  - Note: The frame/slot/symbol timing of the cell can be obtained from an SS block

  - FFS: Note: timing synchronization between CSI-RS and SS block of the cell is assured. Timing synchronization refers to frame/slot/symbol timing.

- NR cell ID for time reference of CSI-RS(s) is informed to the UE

In this contribution, we discuss the signals for RRM Measurements to support L3 mobility of IDLE and CONNECTED mode UEs.

RRM Measurements and Mobility Management

RRM measurements are used for the selection of connection point and mobility management when UE accesses the NR system.  Two types of RRM measurements had been used for access point selection and mobility management as follows,

- Long-term pathloss measurements – Long-term pathloss measurements are the measurements with L1 filters and L3 filters to extract the pathloss of the radio channel by filtering out the effects of fast and slow/shadow fading.  The measurements are used as the indication of the access point coverage area.  The long-term pathloss measurements are used as the reference for determining the camping cell or mobility management, such as cell selection/ re-selection or handover.

- Short-term channel quality measurements - Short-term channel quality measurements are the measurements to with L1 filters to filter out the fast fading effect of radio channel.  The measurements are used to indicate the link quality of each access point/beam.  The short-term channel quality measurements are used as the reference for determining the link quality of the access point/beam to enable dynamic switching of connecting point/beam for spatial diversity gain.

The NR network is covered with cells with each cell consisting of multiple TRPs and/or multiple beams as shown in Figure 1.   Both types of RRM measurements are used for initial access and mobility management in NR.    UE would perform long-term pathloss measurements for the selection/re-selection of camping cell and handover.   The short-term channel quality measurements are used as the reference for dynamic selection of serving TRP and/or beam for spatial diversity gain in beam management.

Figure 1: Illustration of NR network with multiple TRPs and multi-beam configuration

The initial access procedure starts with the UE acquiring the NR system DL transmission timing and the cell ID by detection of the NR-PSS and NR-SSS.  UE might detect more than one set of DL NR-PSS and NR-SSS in multi-beam configuration.  UE will select the strongest detected synchronization signals as the camping cell and use its DL timing as the reference timing.   UE will try to decode the NR-PBCH and performs the RRM measurements.   In RAN1#88bis, the delivery of remaining system information is as follows,

- NR-PDSCH carrying the remaining minimum system information is scheduled using NR-PDCCH.

- NR-PBCH provides configuration information for the NR-PDCCH scheduling the NR-PDSCH carrying the remaining minimum system information

  - FFS if a part of configuration information can be derived by specification

It was agreed in RAN1#88bis that self-contained DMRS of NR-PBCH is supported as the reference signals for demodulation.   DM RS of NR PBCH would be used for RRM measurements.

IDLE Mode UE measurements

For IDLE mode UEs, UE needs to perform RRM measurements based on the Cell ID derived from the detected NR-PSS/NR-SSS.  UE would need to retrieve system information from NR-PBCH of the detected cell for cell selection/re-selection.   DM RS for the channel tracking and channel estimation is required to be allocated with the cell physical channel carrying the system information for cell selection and re-selection.  In RAN1#86, it was agreed that all QCL assumptions that a UE is allowed to make among antenna ports should be identified and explicitly specified.

Proposal 1:  NR-PSS/NR-SSS should be QCL’ed with DM RS of Physical Channel carrying essential system information in order for UE to perform channel estimation for demodulation and RRM measurements.

RRM measurements are used for the selection/re-selection of camping cell for UE in the IDLE state.   The camping cell is selected by UE when UE is under its coverage area.  The determination of the cell coverage area is based on the long-term pathloss measurements.   The signals for IDLE mode UE long-term pathloss measurements are the cell-level signals, such as the NR-PSS/NR-SSS and DM RS of broadcast channel.  If a cell is consists of multiple TRPs or multi-beam configuration as shown in Figure 1, IDLE mode UE does not need to detect individual TRP or beam within the cell for the long-term pathloss measurements.

Proposal 2:  RRM measurement for IDLE mode UE camping cell selection is based on the long-term pathloss measurements.   Signals used for RRM measurements include NR-PSS/NR-SSS and DM RS of NR-PBCH.

IDLE mode UE needs to perform RRM measurements of individual TRP or beam for the selection of access point within a cell when it starts accessing the network with multiple TRPs or multi-beam configuration as shown in Figure 1.  In the initial access procedure, RRM measurements for the selection of access point (TRP or beam) within a cell are short-term channel quality measurements.  UE needs to detect the RS signals of individual TRP/beam before performing RRM measurement of individual TRP/beam.    It was analyzed in [8] that the time index of SS block along could not clear identify each beam in multiple TRP deployment scenario with each TRP configured with multi-beam as shown in Figure 2.  Additional signal, such as TSS as beam-specific SS, is proposed for UE autonomous detection of individual beam and obtaining the beam-ID.   TSS is also used as the reference signal for channel tracking of the specific TRP or beam.  After selecting one beam from the detected TRPs/beams, UE starts decoding beam-specific NR-PDCCH of individual TRP/beam.  UEs would use TSS and beam-specific DM RS of NR-PDCCH for channel tracking of the selected TRP/beam and RRM measurements.

Proposal 3:  RRM measurement for IDLE mode UE access point selection during the initial access is based on the short-term channel quality measurements.   Signals used for RRM measurements include TSS and beam-specific DM RS of NR-PDCCH.

CONNECTED mode UE measurements

For CONNECTED mode UEs, the RRM measurements for L3 mobility would be determined by the UE connectivity. In particular, the configuration of NR access nodes and cells could be multi-dimension as shown in Figure 2.   A NR cell is configured with multiple TRPs and each TRP could be configured with multiple beams.   The RRM measurements for CONNECTED mode UEs would be subjected to the UE connection to a TRP within a cell or a beam in the multi-beam configuration within a TRP as shown in Figure 1.   In order to have the connectivity to a TRP in a cell or a beam of the multi-beam configuration in a TRP, TSS as beam specific RS is needed for initial acquisition and the channel estimation and channel state measurement.   TSS should be used for the RRM measurements of L3 mobility.

Figure 2:  Illustration of UE multi-connectivities in a NR cell with multi-TRP and multi-beam configuration

It was agreed that the remaining minimum system information (SIBs) not carried by NR-PBCH, the SIBs would be carried by NR-PDSCH.  The resource allocation of the NR-PDSCH is scheduled by DCI of NR-PDCCH, which the resource is allocated by NR-PBCH.  TSS is used for identification of each beam in addition to the time index of SS-block.  TSS would also be used as the reference signal for channel tracking of a specific beam.   For UE mobility in the CONNECTED state within a cell, the RRM measurements are the short-term channel quality measurements to allow the gNB to dynamically select the best beam for spatial diversity gain.   Beam-specific DM RS of the DL control channel and DL shared channel carrying system information could be used along with TSS for the short-term channel quality RRM measurements for intra-cell mobility.   In RAN1#88, it was agreed that CSI-RS can be used in addition to IDLE mode RS for CONNECTED mode RRM measurement for L3 mobility.  Each CSI-RS resource is associated with a TRP or a beam within a cell.   CSI-RS is also considered as the RS for beam management.   CSI-RS is a beam specific RS for UE to identify.   The characteristic of CSI-RS for beam managements and RRM measurements for L3 mobility is similar to that of TSS.    Thus, CSI-RS could be considered as TSS as the candidate for beam-specific RRM measurement.

Proposal 4: CSI-RS could be considered as TSS as the candidate for beam-specific RRM measurement.

Proposal 5:  RRM measurements for CONNECED mode UE intra-cell mobility are short-term channel quality measurements.   DL signals for short-term channel quality measurements include TSS (CSI-RS).

When multi-beam is configured in a NR system, each beam will be configured with its own beam-specific RS, such as CSI-RS, in order for UE autonomously detection of the beam and measurement of channel state information of each beam.   The beam-specific RS is also used for the channel tracking and channel estimation of each beam in the multi-beam configuration.  The RRM measurements would be based on beam-specific RS.  DL broadcast channel, DL control channel, and DL shared channel are all beam formed in the multi-beam configuration.  The DM RS of the DL control channel and DL shared channel needs to be QCL’ed with the beam-specific RS for multi-beam operation.   The RRM measurements of multi-beam operation would be based on beam-specific RS and DM RS of DL control channel and DL shared channel.

For inter-cell mobility for CONNECTED mode UEs, the RRM measurements are the long-term pathloss measurements to determine the best camping cell.  The camping cell is selected by UE when UE is under its coverage area. CONNECTED mode UEs continues search for NR-PSS/NR-SSS of neighboring cells.   When the signal strength of the neighboring cell is detected above the configured threshold, UEs will use the cell associated signals NR-PSS/NR-SSS and associated cell specific signals DM RS of NR-PBCH for the RRM measurements.  Thus, NR-PSS/NR-SSS and the NR-PBCH are the signals for the long-term pathloss RRM measurements for CONNECTED mode UE inter-cell mobility.

Proposal 6:  RRM measurements for CONNECTED mode UE inter-cell mobility are the long-term pathloss measurements.   NR-PSS/NR-SSS and the NR-PBCH are the signals for the long-term pathloss RRM measurements.

Conclusion

This paper discusses the RRM measurements for IDLE and CONNECTED mode UEs.  We propose the following,

- Proposal 1:  NR-PSS/NR-SSS should be QCL’ed with DM RS of Physical Channel carrying essential system information in order for UE to perform channel estimation for demodulation and RRM measurements.

- Proposal 2:  RRM measurement for IDLE mode UE camping cell selection is based on the long-term pathloss measurements.   Signals used for RRM measurements include NR-PSS/NR-SSS and DM RS of NR-PBCH.

- Proposal 3:  RRM measurement for IDLE mode UE access point selection during the initial access is based on the short-term channel quality measurements.   Signals used for RRM measurements include TSS and beam-specific DM RS of NR-PDCCH.

- Proposal 4: CSI-RS could be considered as TSS as the candidate for beam-specific RRM measurement.

- Proposal 5:  RRM measurements for CONNECED mode UE intra-cell mobility are short-term channel quality measurements.   DL signals for short-term channel quality measurements include TSS (CSI-RS).

- Proposal 6:  RRM measurements for CONNECTED mode UE inter-cell mobility are the long-term pathloss measurements.   NR-PSS/NR-SSS and the NR-PBCH are the signals for the long-term pathloss RRM measurements.

Reference

- TR38.913v0.2.1, “Study on Scenarios and Requirements for Next Generation Access Technologies”, CMCC

- R1-1706544, “Way Forward on CSI-RS Design and Configuration for RRM measurement”, 	NTT DOCOMO, HW, ZTE, Ericsson, MTK,

- R1-1706533, “WF on DL RS for L3 mobility”, 	 Huawei, HiSilicon, ATT, LGE, Ericsson, DOCOMO, Intel, MTK

- R1-1706487, “WF on DL RSRP for L3 mobility”,  LG Electronics, ERICSSON, NTT DOCOMO

- R1-1706813, “WF on CSI-RS configuration for L3 mobility”, Intel

- R1-1706417,”WF on remaining system information delivery”, Samsung, Ericsson, Nokia, Qualcomm, NTT Docomo

- R1-1704542, “NR measurements and mobility management in IDLE and CONNECTED state ”, CATT

- R1-1707459, “SS-block time index indication”, CATT