3GPP TSG RAN WG1 #89	R1-1708044

# Hangzhou, China, 15th – 19th May 2017

Agenda item:	7.1.4.1.1.1

Source: 	Samsung

Title: 	CRC attachment for data channel

Document for:	Discussion and Proposal

Introduction

In RAN1#88bis, it was agreed for CRC attachment that

Agreement:

- Number of bits for TB-level CRC is: LTB,CRC =24 bits, at least for TBs larger than a threshold (e.g. around 512 bits)

- FFS the value of LTB,CRC for TBs smaller than the threshold, and the value of the threshold (0 is not precluded)

- If a TB is segmented into 2 or more CBs after code block (CB) segmentation,

  - CB-level CRC is applied, i.e., CRC bits are attached to each code block individually (as in LTE)

  - Number bits for CB-level CRC is: 0 < LCB,CRC <= 24 bits

    - Exact value(s) LCB,CRC are to be agreed after base graph(s) are agreed, taking into account inherent LDPC PC capability

- FFS whether for a code block group (CBG) containing 2 or more CBs but not all CBs of the TB, any additional CRC bits are attached to the CBG

  - To be decide after decision on the value(s) of LCB,CRC

In this contribution, we propose a procedure for decision CRC attachment.

FAR According to CRC Attachment

For our convenience, we define the following two options for CRC attachment:

- Option 1: No CBG-level CRC bits and only TB-/CB-level CRC bits (0 < LCB,CRC <= 24, LTB,CRC = 24).

- Option 2: Option 1 + LCBG,CRC CBG-level CRC bits attached to CBG (0 < LCB,CRC, LCBG,CRC <= 24, LTB,CRC = 24)

In addition, we assume that a CBG consists of g CBs. Clearly, each CB-level FAR for both options can be regarded as 2-LCB,CRC and the worst case CBG-level FAR for option 1 is also FAR1=2-LCB,CRC. Here, the worst case means the event that (g-1) CBs are correctly decoded by channel code and only 1 erroneous CB is false-alarmed. Therefore, the worst case CBG-level FAR for option 2 is FAR2=2-(LCB,CRC+LCBG,CRC). Consequently, if we need to support CBG-level FAR lower than 2-24, CBG-level CRC bits should be added.

Observation 1: If we need to support CBG-level FAR lower than 2-24, CBG-level CRC bits should be needed.

There may be several options to support the target FAR=2-A since it is enough to satisfy LCB,CRC+LCBG,CRC=A. However, the minimum value of LCB,CRC should be first decided to guarantee the FAR for TB consisting of 1 CB. Therefore, we propose the following procedure to make a decision on CRC attachment:

1) Decide the target CBG-level FAR, 2-A

2) Decide the minimum FAR for one CB, 2-LCB,CRC

3) Decide the number of CBG-level CRC bits as (A-LCB,CRC).

Proposal 1: Based on the following procedure, make a decision on CRC attachment:

1) Decide the target CBG-level FAR, 2-A.

2) Decide the minimum FAR for one CB, 2-LCB,CRC.

3) Decide the number of CBG-level CRC bits as (A-LCB,CRC).

Observations and Proposals

In this contribution, we present the following observation and proposal:

Observation 1: If we need to support CBG-level FAR lower than 2-24, CBG-level CRC bits should be needed.

Proposal 1: Based on the following procedure, make a decision on CRC attachment:

1) Decide the target CBG-level FAR, 2-A.

2) Decide the minimum FAR for one CB, 2-LCB,CRC.

3) Decide the number of CBG-level CRC bits as (A-LCB,CRC).

- References

- Chairman’s note, 3GPP TSG RAN WG1 #88bis, Spokane, WA, 3-7 April 2017.