3GPP TSG-RAN WG1 #89 	R1-1708276

May 15th – 19th, 2017

Hangzhou, P. R. China

Agenda item:	7.1.8

Source: 	Apple Inc.

Title: 	Uplink sharing in NSA mode

Document for:	Discussion/Decision

# Introduction

It was decided in RAN plenary that co-channel coexistence will be supported in Rel. 15 [1]:

-	NR-LTE co-existence mechanisms [RAN1, RAN2, RAN4];

-	Support co-existence of LTE UL and NR UL within the bandwidth of an LTE component carrier and co-existence of LTE DL and NR DL within the bandwidth of an LTE component carrier, and identify and specify at least one NR band/LTE-NR band combination for this operation.

-	Minimize impact to NR physical layer design to enable this co-existence.

-	No impact to the ability of legacy LTE devices to operate on the LTE carrier co-existing with NR

-	No implication that UE has to support simultaneous connection of NR and LTE in the bandwidth of an LTE component carrier

A possible interpretation of the above description is that the coexistence within the same carrier is from the network-perspective, i.e.  the LTE UL and NR UL can be from different users. From a single UE-perspective, it is not clear whether the spec will allow a UE to support two active uplinks in the same carrier, although the last bullet above indicates it is not a required feature.

On the other hand, the non-standalone (NSA) dual connectivity mode will be prioritized as the first phase of Rel. 15 for the purpose of accelerated NR deployment [4], where E-UTRA-NR DC UE connects via EPC where the E-UTRA is the master (Option 3/3a/3x in TR 38.801 section 10.1.2). In this case, the UE will need to maintain two active uplinks corresponding to the two RAT’s, respectively.

It was also agreed in the Rel. 14 NR study item phase [2] that, at least for collocated LTE and NR base stations, a dual connectivity UE with both LTE and NR uplink in the same uplink carrier can be supported.

- LTE-NR co-existence should support the following UL sharing scenarios:

  - Collocated LTE and NR base stations with network operating UL on frequency F1 where LTE UL and NR UL share UL subframes of LTE

    - Detailed sharing on the UL is FFS

      - Note: this is not intended to have impact on legacy LTE UEs

    - LTE DL on a paired frequency F3

    - NR DL transmission on frequency F2 (different than LTE DL frequency)

    - NR UE operates in either of the following cases based on a common NR design:

      - Standalone NR: UE accesses standalone NR carrier on F2. The UE may not be connected to an LTE carrier (some UE may not even support LTE).

        - FFS whether NR UL frequency F1 is signaled in NR broadcast system information or derived from MIB/PBCH, or implicitly from NR DL frequency F2

      - Dual connectivity of LTE and NR: UE accesses LTE PCell (with LTE UL on F1), then is configured by dual connectivity to also operate NR on F1 (UL) and F2 (DL).

        - NR DL and UL frequencies (and/or NR band number) are signaled by RRC

  - Non-collocated LTE and NR base stations is FFS

These indicates that at least in certain deployment scenarios (e.g. UL co-channel coexistence is required due to spectrum availability constraints), some UEs need to maintain two active uplinks in the same uplink carrier for LTE and NR, respectively.

Further, depending on the band combination, sending two concurrent uplinks in two different bands can produce undesirable inter-modulations that degrade UE’s sensitivity. This creates more challenges in UE RF implementations. Therefore, it is also desirable to allow LTE uplink and NR uplink sharing the same uplink carrier from a single NSA-mode UE perspective.

In this contribution, we briefly discuss the possibilities of uplink sharing from a single LTE/NR dual connectivity UE perspective. Specifically, in section 2, we further discuss the benefits of channelizing NR and LTE uplink through 1 carrier from UE RF emission perspective. In section 3, we discuss options of supporting both LTE and NR uplink in the same carrier.

# Dual Connectivity UE RF Considerations

It was agreed in RAN4#82bis that LTE and NR carrier aggregation band combinations with LTE_4DL/1UL CC (B1,3,7,20) + NR_1DL/1UL CC (3.4-3.8GHz) is to be supported [7]. With this setup, simultaneous LTE uplink and NR uplink can create various intermodulation products that fall in to the LTE FDD downlink frequencies and cause serious receiver sensitivity degradation. To give a few examples:

- The IM5 produced at the NR transmitter from the 2nd harmonic of Band 3 uplink (1710-1785 MHz) and NR uplink (3.3-3.8 GHz) could well fall into the Band 7 downlink frequencies (2620-2690 MHz) under the above 4DL/1UL CC (B1,3,7,20) + NR_1DL/1UL CC (3.4-3.8GHz) simultaneous dual uplink operation.

- Similarly, a simultaneous 4th order harmonic of uplink transmission at Band 20 and NR uplink transmission can also create IM5 intermodulation product interfering Band 7 downlink reception and thus desense the Band 7 receiver.

Therefore, in order not to cause serious receiver sensitivity degradations on any agreed LTE/NR band combinations in RAN4#82bis or in any future RAN4 meetings, it is imperative that some other coexistence mechanisms to be specified in Rel-15 NR work item phase under the scope of NR/LTE coexistence frame work, such as uplink sharing between LTE and NR from a UE perspective to avoid simultaneous LTE and NR uplink transmissions.

Observation 1: depending on the band combinations, concurrent NR and LTE uplink transmission from a single UE in different bands could results in DL sensitivity degradation.

# Uplink sharing for UE with NSA mode

In this section, we discuss a few possibilities of LTE uplink and NR uplink sharing from a single UE perspective, which can also be applied to the NSA NR UE with co-channel uplink coexistence deployment.

## Physical layer multiplexing with two uplink waveforms

For NR UE operating in NSA mode, the control plane messages go through LTE protocol stack while user plane data go through NR protocol stack. The simplest way to multiplex the two uplinks in the same carrier is to multiplex at physical layer, while keeping all the higher protocol layers intact and independent, as shown in Figure 1.

Figure 1 Physical layer multiplexing of NR and LTE uplinks for NSA UE

Assuming a unified 15kHz SCS for both LTE and NR uplinks, the physical layer multiplexing can potentially be implemented in either TDM or FDM approach, which are further explained in section 3.1.1 and section 3.1.2, respectively.

### TDM two uplink waveforms

One option for the LTE/NR dual connectivity UE to operate with a single uplink carrier is to separate the LTE and NR uplink in different sub-frames, as shown in Figure 2.

Figure 2 Physical layer TDM of NR and LTE uplink waveforms from the same UE

Figure 2 shows an example where a single LTE uplink carrier accommodates a legacy LTE-only device (UE 1), and a dual connectivity device (UE 2) operating in NSA mode. The two uplink RAT’s of UE 2 are time division multiplexed in different sub-frames.

Since NR uplink can only be transmitted in a subset of the sub-frames, ACK aggregation across different NR DL slots is needed. On the LTE side, due to the same reason, scheduler needs to skip certain DL sub-frames accordingly. Therefore, the innate low latency benefits from NR design can’t be fully leveraged here.

### FDM two uplink waveforms

Another option for the dual connectivity UE to operate with a single uplink carrier is to send the LTE and NR uplink in different RB’s within the same sub-frames, as shown in Figure 2.

Figure 3  Physical layer FDM of NR and LTE uplink waveforms from the same UE

In this option, UE 2 is the dual connectivity UE operating in NSA mode, and can be allocated dedicated uplink resources for both RATs in each sub-frames within same carrier. The main advantage of the FDM approach is that both NR and LTE can potentially be transmitted in every sub-frame, so there is no latency hit.

On the other hand, the FDM approach could result in multi-cluster uplink transmission, which requires higher MPR and impacts link budget.

Obviously, both the TDM and FDM approach requires tight interworking between the LTE and NR schedulers at the eNB and gNB, respectively. On the other hand, it is expected that most of the NSA deployment will have co-located gNB and eNB from the same operator, which facilitates the tight interworking between the schedulers.

## MAC layer multiplexing with single uplink waveform

To fully exploit the low latency advantage associated with the FDD uplink carrier and NR design, as well as to avoid the higher MPR from the FDM approach described in section 3.1.2, the uplink sharing can also be implemented by multiplexing the two streams at NR-MAC layer and mapped to a single uplink waveform at physical layer, as shown in Figure 4.

The different logical channels from LTE RLC and NR RLC layers are multiplexed/de-multiplexed within the NR-MAC, and mapped to the NR transport channels. A single RAT NR uplink waveform is transmitted by the NSA mode UE over physical layer. This way, it is expected that the only changes in the protocol stack are localized to the NR MAC.

Among the three options listed above, we have slight preference to the MAC layer multiplexing approach due to its capability of enabling low latency feature of NR, and that extra specification impact is localized to the NR MAC design.

Figure 4 MAC layer multiplexing of NR and LTE uplink waveforms from the same UE

Proposal 1: NR should support a mode of operation where a dual connectivity UE can transmit both LTE uplink payload and NR uplink payload within the same uplink carrier.

Proposal 2: NR should study the different approaches of LTE/NR uplink sharing within the same carrier from a single UE, including MAC layer multiplexing with single NR uplink waveform, and physical layer multiplexing (TDM or FDM) of two uplink waveforms.

# Conclusions

In this contribution, we briefly discuss the motivations and possibilities of LTE/NR dual connectivity uplink from a single UE perspective. We have the following observation:

Observation 1: depending on the band combinations, concurrent NR and LTE uplink transmission from a single UE in different bands could results in DL sensitivity degradation.

We also discuss the benefits of channelizing NR and LTE uplink through 1 carrier from UE RF perspective. We list three possible options of supporting both LTE and NR uplink in the same carrier. Among the three options listed, we have slight preference to the MAC layer multiplexing approach due to its capability of enabling low latency feature of NR, and that extra specification impact is localized to the NR MAC design.

Specifically, we propose the following:

Proposal 1: NR should support a mode of operation where a dual connectivity UE can transmit both LTE uplink payload and NR uplink payload within the same uplink carrier.

Proposal 2: NR should study the different approaches of LTE/NR uplink sharing within the same carrier from a single UE, including MAC layer multiplexing with single NR waveform, and physical layer multiplexing (TDM or FDM) of two waveforms.

# References

- RP-170847, “New WID on New Radio Access Technology”, 3GPP TSG RAN meeting #75, Dubrovnik, Croatia, March 6th-9th, 2017.

- 3GPP RAN1 NR Ad-Hoc Chairman Notes, “Chairman’s Notes RAN1 NR Ad-Hoc final”, Spokane, USA, January 16th-20th, 2017.

- 3GPP RAN1 #88Bis Chairman Notes, “Chairman’s Notes RAN1_88bis_final”, April 3rd – 7th, 2017,  Spokane, USA.

- RP-170741, “Way Forward on the overall 5G-NR eMBB workplan”, RAN#75, Dubrovnik, Croatia, March 6-9, 2017

- R4-1704411, “Proposal of frequency ranges for LTE-NR coexistence with UL sharing”, RAN4#82bis, Spokane, US, April 3-7, 2017

- R4-1704410, “WF on 3.3-4.2 GHz and 4.4-4.99 GHz NR spectrum”, RAN4#82bis, Spokane, US, April 3-7, 2017

- R4-1703999, “European LTE-NR band combinations, and coexistence analysis”, RAN4#82bis, Spokane, US, April 3-7, 2017