3GPP TSG RAN WG1 Meeting #89                       	R1-1708332

Hangzhou, China, 15th – 19th May 2017

Agenda item:		7.1.2.1.6

Source: 		InterDigital Inc.

Title: 			PRG Size Indication in NR

Document for:		Discussion/Decision

# Introduction

In RAN1 #88b Meeting [1], WG1 continued the discussion on PRB bundling size for MIMO transmission, and the following agreements were reached;

- For PRB bundling of data channel

  - Support common design for PRB bundling for different scenarios, e.g.  channel reciprocal or non-reciprocal, different Tx/Rx beamforming, etc.:

    - Specify common PRB bundling size set for all scenarios

      - FFS: Different PRB bundling size sets for different BWs

    - Specify common indication procedure for all scenarios

  - The following PRB bundling sizes are studied:

    - Specified value(s) X:

      - FFS X

      - FFS whether more than one value is needed.

      - Strive for value aligned with resource allocation granularity

    - Contiguous allocated PRBs when at least Y contiguous PRBs are allocated. FFS: values of Y;

    - Scheduled BW dependent, FFS the relationship

    - Values equal or larger than scheduled BW

    - FFS different transmission schemes may be related to different subsets of PRB bundling configurations

    - Others are not precluded;

  - Support UE specific PRB bundling size indication:

    - FFS: RRC configured with a subset, DCI dynamically indicated

      - DCI overhead should be considered;

        - MAC CE can be considered if the number of subset elements are large, details FFS

      - FFS: the presence of DCI field related to PRB bundling is configured by RRC;

    - FFS: Implicit signaling to inform PRB bundling size can be considered

  - FFS UE feedback assisted PRB bundling size

In this contribution, we provide our views for an efficient definition, and design of RB bundling size for MIMO transmission.

# PRG Size Considerations for MIMO

In NR, the performance of MIMO transmission may be improved by considering more flexibility in choosing the precoding resolution. As shown in Table 1, the selection of RBG and PRG sizes in LTE are very restricted, and the precoding operation is always applied on specific sizes of RBG and PRG.

While such an approach offers some benefits in terms of feedback overhead, per recent agreement [1], companies have expressed the desire to improve the PRB bundling size definition by allowing both smaller and larger sizes to enable better exploitation of frequency selectivity of the channel as well as potential improvement for channel estimation.

Table 1

| System BW (MHz) | RBG Size | PRG Size |
| --- | --- | --- |
| 1.4 | 1 | 1 |
| 3 | 2 | 2 |
| 5 | 2 | 2 |
| 10 | 3 | 3 |
| 15 | 4 | 2 |
| 20 | 4 | 2 |

Moreover, for uplink transmission, there are additional arguments in favor of introducing new set of choices for UL RBG sizes;

- Given the UE limited power, the possibility of a better channel estimation by the gNB for a cell-edge UE is an important aspect for a robust coverage.

- Achieving a superior UL performance through a better channel estimation may allow some UEs to reduce their transmit power and result in a reduction of overall inter-cell interference.

- In low-latency UL transmission, where mini-slots may be used extensively, a wideband transmission is expected. Therefore, such UL transmissions can significantly benefit from an enhanced channel estimation at the gNB.

For the largest system bandwidth, an RBG size definition similar to LTE calls for an RBG size of equal to 4 RBs. In such case, if a higher scheduling granularity is needed, resource allocation Type 1 may be used to improve the scheduling resolution to 2RBs even for very wideband transmission.

As it has been studied by several companies, the performance of a MIMO system may be further improved by considering larger PRG sizes to improve the accuracy of channel estimation. However, as shown by several studies, due to limited coherence bandwidth of the channel, the options for larger PRG size would not be extensive. Figure 1 shows correlation coefficient across subcarriers for different channel scenarios.

Figure 1 Subcarrier correlation over different channel scenarios

# PRG Size Determination

For CP-OFDM transmission, configurability of the PRG size should be supported to enable the MIMO transmission to adapt to the frequency selectivity of channel and facilitate channel estimation. The PRG size configurability feature can be supported by explicit signaling to the UE. However, to eliminate any additional feedback overhead resulting from explicitly signaling, an implicit approach for PRG size determination would be preferable.

Assuming a contiguous scheduling assignment, the LTE RBG/PRG size table shown in Table 1 can be considered as a starting point for the design. An immediate improvement may be achieved simply by increasing the PRG size corresponding to the larger RBGs without changing other elements. In other words, we can devise a rule such that for a given system bandwidth, any scheduled allocation greater than a threshold would automatically indicate a predefined size for PRG. Therefore, by decoding the RA information, a UE would also be able to detect the PRG size.

As shown in Table 2, a substitute approach to the LTE RBG/PRG table can be considered. As such, for a given RBG size configuration, the PRG size can be determined from the size of the contiguous parts of the scheduled transmission. For example, for a transmission scheduled with contiguous parts of wider than ≥x RBGs, the PRG size may be determined as wide as the size of the contiguous part of the transmission, or by another pre-defined rule.

The following table shows an exemplary case where the PRG size is increased from 2 to 4 RBs if the scheduled transmission has contiguous parts of wider than x RBG;

| System BW (MHz) | RBG Size | PRG Size |
| --- | --- | --- |
| 1.4 | 1 | 1 |
| 3 | 2 | 2 |
| 5 | 2 | 2 |
| 10 | 3 | 3 |
| 15 | 4 | 2 |
| 15 (If contiguous parts are wider than x RBGs) | 4 | 4 |
| 20 | 4 | 2 |
| 20 (If contiguous parts are wider than x RBGs) | 4 | 4 |

Table 2

Proposal 1 – RAN1 studies implicit methods for PRG size determination, such as scheduling size dependent PRG size.

# Summary

In this contribution, we provide our views on the requirements and benefits of flexible RB bundling size in support of frequency selective precoding for MIMO transmission.

Proposal 1 – To minimize control overhead, RAN1 considers implicit methods for PRG size determination, such as scheduling size dependent PRG size.

# References

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #88b, Spokane, USA, April 2017