3GPP TSG RAN WG1#89					          	                 	R1-1708501

Hangzhou, P.R. China, 15th – 19th May 2017

Agenda item:		7.1.3.1.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	PDCCH support for multi-beam system

Document for:		Discussion and Decision

# 1	Introduction

Rel-14 NR study item [1] has been closed and a new Rel-15 WI dealing with New Radio Access Technology [2] has been approved. The work item should specify the NR functionalities for enhanced mobile broadband (eMBB) and ultra-reliable low-latency-communication (URLLC) as defined in [3]. The NR under this work item should consider frequency ranges up to 52.6 GHz.

The beamforming architecture used in the gNB needs to be taken into account also in the PDCCH design. This is due to the fact that not all the beam directions can be served simultaneously. This contribution relates to PDCCH transmission/reception in the case when cell is operating according to a multi-beam configuration.

# 2	Discussion

2.1. 	Beam sweeping

The beam sweeping operation needs to be supported for PDCCH at least in the following scenarios:

- PDCCH transmission is triggering a common channel transmission/reception in the multi-beam scenario, for example paging.

- PDCCH transmission is triggering a shared channel transmission/reception (PDSCH, PUSCH) for an UE in the multi-beam scenario where the gNB has uncertainy related to the UE location within the cell.

There are two main options for the PDCCH beam sweeping:

- Option 1: Pre-defined resource elements of the synchronization signal block are made available for the sweeping PDCCH. For example, each SS block could contain four NR-CCEs (4*6 REGs) and these resources could be used as the sweeping PDCCH (common search space).

- Option 2: A separate PDCCH sweep block is provided for NR-PDCCH independently from the SS block. For example in the case of paging, a sweeping PDCCH could be used for scheduling PCH on NR-PDSCH after the scheduling sweep [4]. The principle of the PDCCH sweep block is shown in Figure 1. It is mapped to predefined PDSCH resources of the DL slot, or bi-directional DL slot. In the current example, each PDCCH sweep block can support four different set of beams. gNB can allocate one ore more PDCCH sweep blocks with given periodicity. The actual data transmission triggered by the PDCCH sweep block can be based on slots or mini-slots.

Generally speaking, there seems to be a need for both options. Option 1 can be seen as a simple and straightforward solution with limited flexibility in terms periodicity and the number of CCEs supported. Option 2 can be seen as an optimized solution for different use cases, having full flexibility.

Proposal 1: Support sweeping PDCCH as part of syhronization signal block

Proposal 2: Support a separate PDCCH block sweep with NR-PDCCH independently from SS block.

As discussed in [4], each scheduling block in the PDCCH sweep could have association to an SS-block in the cell and hence the UE could determine the scheduling block and its transmission timing based on the SS-block timing in the SS sweep, and the given PDCCH block sweep configuration. Following this principle, the UE can minimize the PDCCH blind detection burden since it doesn’t need to monitor each symbol of the PDCCH sweep block.

On the other hand, in principle there does not need to be dependency between SS-block and scheduling block and it should be discussed whether any advantages can be achieved using this approach. Following this principle, the UE needs to monitor each PDCCH symbol of the PDCCH sweep block.

Proposal 3: PDCCH monitoring on the separate PDCCH block sweep supports association to an SS-block in the cell

Figure 1. DL slot structure with PDCCH sweep block

2.2. Mini-slot based operation in the multi-beam scenario

The following was agreed in TSG RAN WG1 #AH1_NR meeting:

- Take into account following targets/use-cases to design mini-slots:

  - […]

  - Support of finer TDM granularity of scheduling for the same/different UEs within a slot

    - Especially if TRxP uses beam-sweeping (e.g., above 6GHz).

  - […]

Mini-slot based scheduling is a way to provide finer TDM granularity of scheduling within a slot. There are two main approaches for mini-slot based scheduling shown in Figure 2 and Figure 3.

- Scheduling is made from DL control symbols located at the beginning of each DL slot (K0>0).

- Scheduling is made from PDCCH located at the beginning of each DL mini-slot (K0=0).

It can be assumed that each CP-OFDM symbol of the slot can be processed using different Tx/Rx beams at gNB/UE.

Figure 2. DL slot structure with control symbols located at the beginning.

Figure 3. DL slot structure with control symbols located in each mini-slot

It can be noted that both scheduling options are supported already by previous decisions made by RAN1:

- It was agreed in RAN1 Ad-hoc that “Timing between DL assignment and corresponding DL data transmission is indicated by a field in the DCI from a set of values. The set of values is configured by higher layer”. Furthermore it was agreed in RAN1 #88bis that “Specification supports data channel having minimum duration of 1 OFDM symbol of the data and starting at any OFDM symbol to below-6GHz, in addition to above-6GHz”. These agreements indicate that operation mode according to principle shown in Figure 2 is supported already.

- It was agreed in RAN1 #88bis that “Specification supports occasion of "DL control channel monitoring" per 1 symbol with respect to the numerology of the DL control channel”. This agreement indicates that operation mode according to principle shown in Figure 3 is supported already.

Based on the discussion above, it can be noted that there exist two main principles for mini-slot based scheduling in the multi-beam scenario. Both of them are supported by current agreements, and the selection can be seen as a gNB implementation issue.

Observation 1: gNB can select the operation mode for the mini-slot based scheduling in the multi-beam scenario from the following

- Scheduling is made from DL control symbols located at the beginning of each DL slot (K0>0).

- Scheduling is made from PDCCH located at the beginning of each DL mini-slot (K0=0).

# 3	Conclusions

In this contribution we discussed the PDCCH transmission in the case when cell is operating according to a multi-beam configuration. Based on the results and discussion we make the following proposals and an observation:

Proposal 1: Support sweeping PDCCH as part of syhronization signal block

Proposal 2: Support a separate PDCCH block sweep with NR-PDCCH independently from SS block.

Proposal 3: PDCCH monitoring on the separate PDCCH block sweep supports association to an SS-block in the cell

Observation 1: gNB can select the operation mode for the mini-slot based scheduling in the multi-beam scenario from the following

- Scheduling is made from DL control symbols located at the beginning of each DL slot (K0>0).

- Scheduling is made from PDCCH located at the beginning of each DL mini-slot (K0=0).

# References

- RP-160671, “Study on New Radio Access Technology”, NTT DoCoMo

- RP-170847, “New WID on New Radio Access Technology”, NTT DoCoMo

- TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, 3GPPRAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #87, 3GPP

- R1-1705842, “Paging in multi-beam system”, Nokia, Alcatel-Lucent Shanghai Bell