3GPP TSG RAN WG1 #89	R1-1708582

May 15th -19th, 2017

Hangzhou, China

Agenda item:	7.1.2.1.4

Source: 	Qualcomm Incorporated

Title: 	Non-codebook based UL transmission

Document for:	Discussion/Decision

# Introduction

In RAN1 #88, the following agreements were made regarding UL MIMO transmission:

- Scheme A: Codebook based UL transmission

  - For the previous agreement “Support frequency selective precoding for CP-OFDM when the number of transmission port(s) is equal to or greater than X (FFS: Value of X).”, FFS the value X and the interpretation of transmission port(s)

- Scheme B: Non-codebook based UL transmission

  - For the previous agreement “Support frequency selective precoding for CP-OFDM when the number of transmission port(s) is equal to or greater than Y (FFS: Value of Y).”, FFS the value Y and the interpretation of transmission port(s)

In this contribution, we discuss open issues related to non-codebook based UL transmission scheme. We provide more system-level simulation results for different granularity of the sub-band used for frequency selective precoding, for different number of transmission ports at the UE.  The impact of wideband vs. sub-band scheduling on the frequency selective precoding gains is also considered.

# Discussion

## Frequency selective precoding

In [2], we presented link-level simulation results which show that, compared to wideband precoding, frequency selective precoding provides

- No gain when the number of UL transmission ports is 2.

- No gain when the number of UL transmission ports is 4 and the MSE of UL channel estimation is not high enough.

In [3], we studied the potential benefit of subband precoding with wideband scheduling based on the proportional fair metric.  We showed the mean cell-spectral efficiency and 5th percentile UE-spectral efficiency for following two cases:

- Ideal channel estimation for demodulation, wideband scheduling (SU and MU-MIMO)

- Realistic channel estimation for demodulation, wideband scheduling (SU and MU-MIMO)

We observe that

- With ideal channel estimation, subband precoding provides no gain for 2 TX ports, negligible gain for 4 TX ports, and some gain for 8 TX.

- With realistic channel estimation and demodulation, subband precoding does not provide any performance improvement when the number of transmission ports at the UE is 4 or lower.

In this contribution, we show results with subband scheduling based on the proportional fair metric.  We show results for following case:

- Realistic channel estimation for demodulation, sub-band scheduling (SU-MIMO)

The simulation assumptions are same as those in [2].  The simulation assumes that the 20 MHz simulation bandwidth is divided into 52 RBs, after excluding the guard band. Various subband precoding granularities were considered from 4 RBs (13 subbands) and 52 RB (wideband).   The detailed simulation assumptions are summarized in the Appendix.

The procedure used for determining the UL precoder is as described in [2] – the UE estimates the downlink channel, selects uplink precoding along the singular vectors and signals them using precoded SRS. The gNB estimates the proposed precoders and performs scheduling to down-select the ports and compute MCS. The scheduling is performed based on the proportional fair metric.

Figure 1 shows results for SU-MIMO for sub-band scheduling with realistic demodulation channel estimation error.  In the plots, the X-axis shows the number of ports at the gNB and the UE respectively.  For each case, two bars are shown – one corresponding to subband precoding with 4-RB granularity (marked as “SB”) and the other corresponding to 52-RB granularity, i.e., wideband precoding (marked as “WB”).  The percentage change in the performance metric for subband precoding relative to wideband precoding is also shown.

Figure 1: SU-MIMO (Realistic Channel Estimation for Demod), sub-band scheduling
Mean cell spectral efficiency and 5th %ile UE spectral efficiency

In terms of the impact of subband precoding, both for the mean and the tail spectral efficiency, 4-RB subband precoding does not provide much benefit over wideband precoding when the number of ports at the UE is 2 or 4.  In fact, the performance is worse than wideband precoding in many cases. Subband precoding provides some benefit only with 8 ports at the UE.  In Figure 2, we provide a full picture of spectral efficiency versus various subband precoding granularities.

Figure 2: Spectral efficiency versus various subband precoding granularities

Observation: 	UL frequency selective precoding does not provide any significant performance improvement when the number of transmission ports at the UE is 4 or lower.

Proposal: 	Frequency selective precoding for CP-OFDM is not supported when the number of transmission port(s) is less than or equal to 4.

# Conclusions

To summarize, we present system-level simulation results for SU-MIMO with sub-band scheduling with various UL subband precoding granularity.  We observe:

Observation: 	UL frequency selective precoding does not provide any significant performance improvement when the number of transmission ports at the UE is 4 or lower.

We propose:

Proposal: 	Frequency selective precoding for CP-OFDM is not supported when the number of transmission port(s) is less than or equal to 4.

# Appendix

Table 1: System simulation assumptions.

| Parameters | Dense urban |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | Macro layer: 200m |
| Carrier frequency | Macro layer: 4 GHz |
| Aggregated system bandwidth | 80MHz (DL+UL) |
| Simulation bandwidth | 20MHz per CC |
| Channel model | 3D UMa |
| Tx power | BS: 44 dBm PA scaled with simulation BW UE: 23dBm |
| UE power control | Open loop, P0 = 20 dB above thermal, α = 0.8 |
| BS antenna configuration | {64, 128} Tx /Rx antenna elements (X-pol) {(8, 4, 2), (8, 8, 2)} |
| BS antenna pattern | Follow the modeling of TR36.873 |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5 dB |
| UE antenna elements | {2, 4, 8} Tx /Rx antenna elements |
| UE antenna height | Follow TR36.873 |
| UE antenna gain | Follow TR36.873 |
| UE receiver noise figure | 9 dB |
| Traffic model | Full buffer |
| Traffic load (Resource utilization) | 100% |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) Uniform/macro TRP (10 users per TRP for full buffer traffic) |
| UE receiver | MMSE-IRC as the baseline receiver |
| Channel estimation | Ideal for scheduling, beam selection; Realistic at demodulation |

# References

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting #88, February 2017.

- R1-1702600, “Non-codebook based UL transmission,” Qualcomm Incorporated.

- R1-1705577, “Non-codebook based UL transmission,” Qualcomm Incorporated.