3GPP TSG RAN WG1 #89	R1-1708792

May 15th – 19th, 2017

Hangzhou, China

Agenda item:	6.2.4.3

Source: 	Qualcomm Incorporated

Title: 	Enhancement to CSI feedback for NCJT

Document for:	Discussion and Decision

# 1	Introduction

In RAN #75, non-coherent joint transmission (NCJT) is approved in WID of feCoMP [2].

In the scope of NCJT, the following are included

- Specify enhancements to support non-coherent joint transmission (JT) schemes [RAN1]

  - Support of a new QCL assumption for DM-RS antenna ports

  - Support of control signalling enhancements to assist QCL,PDSCH REs mapping and resource allocation

  - Support possible CSI feedback enhancement

In this paper, we focus on CSI support enhancement.

# 2	Discussion

In [1], the following categories of NCJT are identified.

- Mapping of codewords (CWs) to TPs:

Case 1: Different CWs are transmitted from different TPs. Each TP perform adaptive precoding independently

Case 2a: The same CW is transmitted from different TPs with spatial diversity (e.g. SFBC) / spatial multiplexing

Case 2b: The same CW is transmitted from different TPs using SFN

- Resource allocations on different TPs:

Scheme 1: The resource allocations from different TPs for a UE are fully overlapped

Scheme 2: The resource allocations from different TPs for a UE are partially overlapped

Scheme 3: The resource allocations from different TPs for a UE are not overlapped

Out of the three cases of codeword mapping, case 1 has the best gain from simulation studies.

Out of the three resource allocation schemes, scheme 3 is not realistic as it implies the restriction that a UE cannot be assigned full bandwidth from both TPs. Scheme 2 can be considered a super set of scheme 3. However, overlapping parts and non-overlapping parts of the codeword from each TP typically have different interference, which will lead to complicated CSI reporting and MCS selection design. Consider the limited availability of study time, we propose to focus on case 1 and scheme 1 for NCJT.

Proposal 1.  NCJT in feCoMP only support different CWs from different TPs with independent precoding, fully overlapped resource allocation from different TPs.

Proper CSI support is critical in achieving the performance gain from NCJT. At the same time, we want to support enough flexibility at the eNB scheduler. Therefore, the CSI reporting for NCJT needs to support joint transmission from the multiple TPs, together with single TP only transmission. A natural design is to have three CSI processes defined when there are two TPs performing NCJT:

-	CSI process 1: CSI assuming TP1 is serving alone.

-	CSI process 2: CSI assuming TP2 is serving alone.

-	CSI process 3: CSI assuming TP1 and TP2 are jointly serving.

Note that the CSI process 3 needs to provide the PMI/RI/CQI for both TPs for NCJT. If the rank combinations of the NCJT transmission from the two TPs are expanded, the RI in the CSI may not be enough to cover all the combinations, and a generalized RI report might be required. More precisely, instead of reporting the total rank from the two TPs, the UE needs to report the rank combination from the two TPs.

Proposal 2.  Use a multiple CSI processes configured to report CSIs conditioned on two TPs serving or single TP serving.

# 3	Conclusions

For the CSI enhancement for NCJT,  we propose the following:

Proposal 1.  NCJT in feCoMP only support different CWs from different TPs with independent precoding, fully overlapped resource allocation from different TPs.

Proposal 2.  Use a multiple CSI processes configured to report CSIs conditioned on two TPs serving or single TP serving.

# 4			References

[1]. TR 36.741 v2.0.1 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE”, 3GPP

[2]. RP-170750, “New WID: Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE”, Intel, ZTE