3GPP TSG-RAN WG1 Meeting #89	R1-1709196

Hangzhou, China, 15th – 19th May 2017

Agenda item:	6.2.5.1

Source: 	Qualcomm Incorporated

Title: 	Evaluation results for 1024QAM

Document for:     Discussion/Decision

# Introduction

In RAN#75 a new work item [1] “Enhancements for high capacity stationary wireless link and introduction of 1024 QAM” was approved. One of the objectives of the work item is to evaluate and, if benefits are shown, define 1024 QAM modulation for 1024 QAM.

The following simulation assumptions were agreed in RAN1#88b:

| Channel model | AWGN, TDL with delay spread of {10, 100}ns |
| --- | --- |
| Doppler | 5Hz |
| Bandwidth | 20 MHz |
| Tx EVM | Between 0-[3]% |
| Rx EVM | Between 0-[3]% |
| Number of tx/rx antennas | 2T2R, 2T4R, 2T8R, 8T8R (optional) |
| Transmission modes | TM3 for open loop TM4 for closed loop TM9/10 |
| Format of reported results | - Crossover SNR between 256QAM and 1024QAM - Throughput gain at [30]dB and [35]dB SNR |
| Modulation mapping | Gray mapping (described in R1-1705007) |
| Link adaptation scheme | AMC (companies to provide details on the selected scheme) |
| Channel estimation | Realistic |
| Antenna correlation (Tx and Rx) | Uncorrelated |

In this contribution we present evaluation results for 1024 QAM based on these simulation assumptions.

# 1024 QAM modulation and MCS/TBS table

1024QAM gray coding is an extension of the current 256QAM, for which the octuplet of bits bi,bi+1,bi+2,bi+3,bi+4,bi+5,bi+6,bi+7, bi+8, bi+9 is mapped to:

x=16821-2bi16-1-2bi+28-1-2bi+44-1-2bi+62-1-2bi+8

+ j16821-2bi+116-1-2bi+38-1-2bi+54-1-2bi+72-1-2bi+9

Table, shows the gray coding of 1024QAM for either I or Q dimension.

Table 1 Gray coding of 1024QAM in one dimension

| Gray code bi,bi+2,bi+4,bi+6,bi+8 | Lattice point (ignoring the scaling factor of 1/682 ) | Gray code bi,bi+2,bi+4,bi+6,bi+8 | Lattice point (ignoring the scaling factor of 1/682 ) |
| --- | --- | --- | --- |
| 01111 | 31 | 11111 | -31 |
| 01110 | 29 | 11110 | -29 |
| 01100 | 27 | 11100 | -27 |
| 01101 | 25 | 11101 | -25 |
| 01001 | 23 | 11001 | 23 |
| 01000 | 21 | 11000 | -21 |
| 01010 | 19 | 11010 | -19 |
| 01011 | 17 | 11011 | -17 |
| 00011 | 15 | 10011 | -15 |
| 00010 | 13 | 10010 | -13 |
| 00000 | 11 | 10000 | -11 |
| 00001 | 9 | 10001 | -9 |
| 00101 | 7 | 10101 | -7 |
| 00100 | 5 | 10100 | -5 |
| 00110 | 3 | 10110 | -3 |
| 00111 | 1 | 10111 | -1 |

Figure 1 Constellation for 1024 QAM

A plot of the constellation points for 1024QAM s shown in Figure 1.

### CQI table for 1024QAM

The CQI table for 1024QAM mode of operation is obtained by modifying the current CQI table. Two entries of the new CQI table are added for 1024QAM modulation and 2 legacy QPSK entries are removed. The CQI indices are ordered according to their spectral efficiencies. Table 2 indicates the CQI table applied for 1024QAM.

Table 2 Proposed 1024QAM CQI table

| CQI index | Modulation | Code rate × 1024 | Efficiency | MCS |
| --- | --- | --- | --- | --- |
| 0 | Out of range |  |  | 0 |
| 1 | QPSK | 449 | 0.8770 | 2 |
| 2 | 16QAM | 378 | 1.4766 | 4 |
| 3 | 16QAM | 490 | 1.9141 | 6 |
| 4 | 16QAM | 616 | 2.4063 | 8 |
| 5 | 64QAM | 466 | 2.7305 | 11 |
| 6 | 64QAM | 567 | 3.3223 | 13 |
| 7 | 64QAM | 666 | 3.9023 | 15 |
| 8 | 64QAM | 772 | 4.5234 | 18 |
| 9 | 64QAM | 873 | 5.1152 | 20 |
| 10 | 256QAM | 711 | 5.5547 | 21 |
| 11 | 256QAM | 797 | 6.2266 | 22 |
| 12 | 256QAM | 885 | 6.9141 | 23 |
| 13 | 256QAM | 948 | 7.4063 | 24 |
| 14 | 1024QAM | 817 | 7.9837 | 25 |
| 15 | 1024QAM | 928 | 9.0646 | 26 |

### MCS/TBS table for 1024QAM

The proposed MCS table, shown in Table 3, has two explicit entries for 1024QAM. One implicit entry is also added to correspond to 1024QAM retransmissions. Three entries corresponding to QPSK have been removed.

Table 3 Placeholder 1024QAM MCS table

| MCS | Modulation (Rel-14) | TBS for 100 RBs (Rel-14) | New Modulation (proposed) | TBS for 100 RBs (proposed) |
| --- | --- | --- | --- | --- |
| 0 | QPSK | 2792 | QPSK | 10296 |
| 1 | QPSK | 4584 | QPSK | 14112 |
| 2 | QPSK | 7224 | 16QAM | 17568 |
| 3 | QPSK | 10296 | 16QAM | 19848 |
| 4 | QPSK | 14112 | 16QAM | 22920 |
| 5 | 16QAM | 17568 | 16QAM | 25456 |
| 6 | 16QAM | 19848 | 16QAM | 28336 |
| 7 | 16QAM | 22920 | 16QAM | 30576 |
| 8 | 16QAM | 25456 | 64QAM | 32856 |
| 9 | 16QAM | 28336 | 64QAM | 36696 |
| 10 | 16QAM | 30576 | 64QAM | 39232 |
| 11 | 64QAM | 32856 | 64QAM | 43816 |
| 12 | 64QAM | 36696 | 64QAM | 46888 |
| 13 | 64QAM | 39232 | 64QAM | 51024 |
| 14 | 64QAM | 43816 | 64QAM | 55056 |
| 15 | 64QAM | 46888 | 64QAM | 57336 |
| 16 | 64QAM | 51024 | 64QAM | 61664 |
| 17 | 64QAM | 55056 | 64QAM | 63776 |
| 18 | 64QAM | 57336 | 256QAM | 66592 |
| 19 | 64QAM | 61664 | 256QAM | 71112 |
| 20 | 64QAM | 63776 | 256QAM | 73712 |
| 21 | 256QAM | 66592 | 256QAM | 78704 |
| 22 | 256QAM | 71112 | 256QAM | 81176 |
| 23 | 256QAM | 73712 | 256QAM | 84760 |
| 24 | 256QAM | 78704 | 256QAM | 97896 |
| 25 | 256QAM | 81176 | 1024QAM | 105528 |
| 26 | 256QAM | 84760 | 1024QAM | 119816 |
| 27 | 256QAM | 97896 | QPSK | n/a (HARQ repeat) |
| 28 | QPSK | n/a (HARQ repeat) | 16QAM | n/a (HARQ repeat) |
| 29 | 16QAM | n/a (HARQ repeat) | 64QAM | n/a (HARQ repeat) |
| 30 | 64QAM | n/a (HARQ repeat) | 256QAM | n/a (HARQ repeat) |
| 31 | 256QAM | n/a (HARQ repeat) | 1024QAM | n/a (HARQ repeat) |

# Simulation results

We evaluated the performance of 1024 QAM under the evaluation assumptions agreed in RAN1#88b. In particular, we evaluated the cases in Table 1 (note that these are a subset of the agreed simulation assumptions).

Table 4 Simulation assumptions

| Parameters | Value |
| --- | --- |
| Channel model | TDL A/B/D/E with very short (10ns) and normal (100ns) delay spread For Rician channels the LOS is generated as completely correlated (LOS channel matrix is all 1’s, rank 1) or orthogonal (LOS channel matrix is Hadamard, rank 2) |
| Doppler | 5Hz |
| Bandwidth | 20 MHz |
| Tx EVM | Between 0-3% |
| Rx EVM | Between 0-2% |
| Number of tx/rx antennas | 2T2R, 2T4R |
| Transmission modes | TM4 |
| Format of reported results | - Crossover SNR between 256QAM and 1024QAM - Throughput gain at 30dB and 35dB SNR |
| Link adaptation scheme | Open loop with 10% BLER |

The complete set of results are captured in the appendix of this contribution. For the throughput values at 30 and 35 dB SNR, the first entry corresponds to 256QAM and the second entry to 1024QAM.

Based on the results presented in the appendix we make the following observations (note that the maximum gain is ~22%):

Observations:

- The gains provided by 1024QAM are higher in scenarios with LOS component (e.g. TDL-D/E).

- For TDL-A/B and 2T2R configurations, 1024QAM does not offer any gain until ~36dB SNR.

- For TDL-A/B and 2T4R configurations, 1024QAM offers throughput gains:

  - At 30dB SNR, in the range of ~0-4%

  - At 35dB SNR, in the range of ~10-22%

  - The crossover SNR is between ~28dB-32dB depending on the EVM values

- For TDL-D/E correlated and 2T2R configurations, 1024QAM offers throughput gains:

  - At 30dB SNR, in the range of ~0-6%

  - At 35dB SNR, in the range of ~10-17%

  - The crossover SNR is between 28-31dB depending on the EVM values

- For TDL-D/E correlated and 2T4R configurations, 1024QAM offers throughput gains:

  - At 30dB and 35dB SNR, in the range of ~10-17%

  - The crossover SNR is between ~22-24dB depending on the EVM values

- For TDL-D/E Hadamard and 2T2R configurations, 1024QAM offers throughput gains:

  - At 30dB SNR, in the range of 0-20% depending on the EVM value

  - At 35dB SNR, in the range of 20% except for very high EVM values

- For TDL-D/E Hadamard and 2T4R configurations, 1024QAM offers throughput gains:

  - At 30dB SNR, in the range of 10-20%

  - At 35dB SNR, in the range of 20%, except for very high EVM values

Proposal 1: Capture the results in the appendix and observations in the TR.

# Summary of observations and proposals

In this contribution we provided evaluation results for 1024 QAM. The complete (tabular) set of results is provided in the appendix. Based on these results, we make the following observations and proposals:

Observations:

- The gains provided by 1024QAM are higher in scenarios with LOS component (e.g. TDL-D/E).

- For TDL-A/B and 2T2R configurations, 1024QAM does not offer any gain until ~36dB SNR.

- For TDL-A/B and 2T4R configurations, 1024QAM offers throughput gains:

  - At 30dB SNR, in the range of ~0-4%

  - At 35dB SNR, in the range of ~10-22%

  - The crossover SNR is between ~28dB-32dB depending on the EVM values

- For TDL-D/E correlated and 2T2R configurations, 1024QAM offers throughput gains:

  - At 30dB SNR, in the range of ~0-6%

  - At 35dB SNR, in the range of ~10-17%

  - The crossover SNR is between 28-31dB depending on the EVM values

- For TDL-D/E correlated and 2T4R configurations, 1024QAM offers throughput gains:

  - At 30dB and 35dB SNR, in the range of ~10-17%

  - The crossover SNR is between ~22-24dB depending on the EVM values

- For TDL-D/E Hadamard and 2T2R configurations, 1024QAM offers throughput gains:

  - At 30dB SNR, in the range of 0-20% depending on the EVM value

  - At 35dB SNR, in the range of 20% except for very high EVM values

- For TDL-D/E Hadamard and 2T4R configurations, 1024QAM offers throughput gains:

  - At 30dB SNR, in the range of 10-20%

  - At 35dB SNR, in the range of 20%, except for very high EVM values

Proposal 1: Capture the results in the appendix and observations in the TR.

# References

[1] RP-170838: New WI proposal: Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE, Huawei, HiSilicon, Qualcomm Incorporated, China Telecom

# Appendix: Complete set of results

TDL-A Nominal delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 36.67dB | 161.79/151.91 (-6.0%) | 188.96/182.78 (-3.0%) | 37.13dB | 160.55/151.91 (-5.0%) | 186.49/181.55 (-3.0%) | -dB | 158.08/149.44 (-5.0%) | 181.55/177.84 (-2.0%) | -dB | 154.38/149.44 (-3.0%) | 175.37/170.43 (-3.0%) |
| Rx EVM=0.5% | 37.21dB | 161.12/151.24 (-6.0%) | 187.31/181.14 (-3.0%) | 37.73dB | 159.89/151.11 (-5.0%) | 184.84/179.9 (-3.0%) | -dB | 157.42/149.04 (-5.0%) | 180.31/175.38 (-3.0%) | -dB | 153.84/148.77 (-3.0%) | 173.73/169.61 (-2.0%) |
| Rx EVM=1.0% | 39.42dB | 159.21/149.33 (-6.0%) | 182.79/176.61 (-3.0%) | 40.31dB | 157.97/148.81 (-6.0%) | 180.56/175.13 (-3.0%) | -dB | 155.5/147.89 (-5.0%) | 176.36/169.94 (-4.0%) | -dB | 152.31/146.86 (-4.0%) | 169.69/166.98 (-2.0%) |
| Rx EVM=1.5% | -dB | 156.32/146.44 (-6.0%) | 175.66/169.92 (-3.0%) | -dB | 155.08/145.35 (-6.0%) | 173.99/167.45 (-4.0%) | -dB | 152.61/146.16 (-4.0%) | 169.12/166.65 (-1.0%) | -dB | 150.0/143.97 (-4.0%) | 165.42/161.71 (-2.0%) |
| Rx EVM=2.0% | -dB | 152.68/141.63 (-7.0%) | 168.75/162.36 (-4.0%) | -dB | 151.45/141.57 (-7.0%) | 167.01/161.34 (-3.0%) | -dB | 148.98/142.21 (-5.0%) | 164.54/160.62 (-2.0%) | -dB | 146.51/137.99 (-6.0%) | 160.84/156.4 (-3.0%) |

TDL-A Nominal delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 29.0dB | 193.9/201.31 (4.0%) | 193.9/235.89 (22.0%) | 29.33dB | 193.9/201.31 (4.0%) | 193.9/234.65 (21.0%) | 30.2dB | 193.9/192.66 (-1.0%) | 193.9/230.95 (19.0%) | 33.0dB | 192.66/186.49 (-3.0%) | 193.9/202.54 (4.0%) |
| Rx EVM=0.5% | 29.09dB | 193.9/200.51 (3.0%) | 193.9/235.06 (21.0%) | 29.43dB | 193.9/200.11 (3.0%) | 193.9/233.42 (20.0%) | 30.32dB | 193.76/191.86 (-1.0%) | 193.9/228.89 (18.0%) | 33.22dB | 192.53/185.69 (-4.0%) | 193.9/200.9 (4.0%) |
| Rx EVM=1.0% | 29.36dB | 193.9/198.21 (2.0%) | 193.9/233.17 (20.0%) | 29.73dB | 193.9/196.67 (1.0%) | 193.9/230.45 (19.0%) | 30.68dB | 193.38/189.57 (-2.0%) | 193.9/221.82 (14.0%) | 33.97dB | 192.14/183.39 (-5.0%) | 193.9/196.86 (2.0%) |
| Rx EVM=1.5% | 29.86dB | 193.9/194.75 (0.0%) | 193.9/230.44 (19.0%) | 30.27dB | 193.9/191.47 (-1.0%) | 193.9/225.43 (16.0%) | 31.38dB | 192.8/186.1 (-3.0%) | 193.9/208.21 (7.0%) | 35.59dB | 191.57/179.93 (-6.0%) | 193.9/192.59 (-1.0%) |
| Rx EVM=2.0% | 30.66dB | 193.31/190.38 (-2.0%) | 193.9/217.05 (12.0%) | 31.18dB | 193.31/187.85 (-3.0%) | 193.9/211.83 (9.0%) | 32.59dB | 192.07/182.91 (-5.0%) | 193.9/201.37 (4.0%) | -dB | 189.67/176.15 (-7.0%) | 193.9/188.73 (-3.0%) |

TDL-A Very short delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 38.4dB | 155.61/143.26 (-8.0%) | 179.08/171.67 (-4.0%) | 39.0dB | 154.38/143.26 (-7.0%) | 177.84/167.96 (-6.0%) | 39.0dB | 151.91/142.03 (-7.0%) | 174.14/164.26 (-6.0%) | 39.0dB | 149.44/139.56 (-7.0%) | 169.2/160.55 (-5.0%) |
| Rx EVM=0.5% | 39.23dB | 155.08/142.6 (-8.0%) | 177.84/170.43 (-4.0%) | 39.96dB | 153.84/142.46 (-7.0%) | 176.2/167.55 (-5.0%) | 39.96dB | 151.37/141.23 (-7.0%) | 172.49/163.84 (-5.0%) | 39.96dB | 148.9/139.02 (-7.0%) | 168.37/159.73 (-5.0%) |
| Rx EVM=1.0% | 43.75dB | 153.55/140.68 (-8.0%) | 174.39/166.98 (-4.0%) | -dB | 152.31/140.17 (-8.0%) | 172.16/165.74 (-4.0%) | -dB | 149.84/138.93 (-7.0%) | 168.46/162.28 (-4.0%) | -dB | 147.37/137.49 (-7.0%) | 165.99/157.34 (-5.0%) |
| Rx EVM=1.5% | -dB | 151.24/137.79 (-9.0%) | 168.69/160.84 (-5.0%) | -dB | 150.0/136.7 (-9.0%) | 167.45/159.61 (-5.0%) | -dB | 147.53/135.47 (-8.0%) | 164.62/156.7 (-5.0%) | -dB | 145.06/135.18 (-7.0%) | 161.28/153.07 (-5.0%) |
| Rx EVM=2.0% | -dB | 147.15/135.33 (-8.0%) | 162.58/153.21 (-6.0%) | -dB | 146.51/134.09 (-8.0%) | 162.07/151.24 (-7.0%) | -dB | 144.04/132.86 (-8.0%) | 160.11/149.72 (-6.0%) | -dB | 141.57/131.69 (-7.0%) | 155.9/148.49 (-5.0%) |

TDL-A Very short delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 28.67dB | 192.66/197.6 (3.0%) | 193.9/232.18 (20.0%) | 29.25dB | 192.66/196.37 (2.0%) | 193.9/227.24 (17.0%) | 30.0dB | 191.43/191.43 (0.0%) | 193.9/219.83 (13.0%) | 32.0dB | 188.96/182.78 (-3.0%) | 193.9/206.25 (6.0%) |
| Rx EVM=0.5% | 28.75dB | 192.53/197.2 (2.0%) | 193.9/230.13 (19.0%) | 29.34dB | 192.53/195.7 (2.0%) | 193.9/225.19 (16.0%) | 30.11dB | 191.29/190.49 (-0.0%) | 193.9/217.37 (12.0%) | 32.18dB | 188.69/181.98 (-4.0%) | 193.48/204.6 (6.0%) |
| Rx EVM=1.0% | 29.0dB | 192.14/196.05 (2.0%) | 193.9/224.53 (16.0%) | 29.63dB | 192.14/193.79 (1.0%) | 193.9/220.33 (14.0%) | 30.46dB | 190.91/187.82 (-2.0%) | 193.65/211.44 (9.0%) | 32.75dB | 187.92/179.69 (-4.0%) | 192.66/200.32 (4.0%) |
| Rx EVM=1.5% | 29.46dB | 191.57/194.32 (1.0%) | 193.46/215.98 (12.0%) | 30.17dB | 191.57/190.9 (-0.0%) | 193.46/214.31 (11.0%) | 31.11dB | 190.33/183.77 (-3.0%) | 192.66/205.74 (7.0%) | 33.92dB | 186.77/176.22 (-6.0%) | 192.66/195.06 (1.0%) |
| Rx EVM=2.0% | 30.2dB | 190.84/189.79 (-1.0%) | 192.66/206.82 (7.0%) | 31.05dB | 190.25/187.26 (-2.0%) | 192.66/205.08 (6.0%) | 32.22dB | 188.43/179.85 (-5.0%) | 192.66/198.17 (3.0%) | 36.37dB | 184.14/174.78 (-5.0%) | 191.93/189.02 (-2.0%) |

TDL-B Nominal delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | -dB | 159.32/145.73 (-9.0%) | 187.72/171.67 (-9.0%) | -dB | 158.08/146.97 (-7.0%) | 186.49/169.2 (-9.0%) | -dB | 155.61/143.26 (-8.0%) | 180.31/167.96 (-7.0%) | -dB | 151.91/140.79 (-7.0%) | 172.9/165.49 (-4.0%) |
| Rx EVM=0.5% | -dB | 158.65/145.33 (-8.0%) | 186.08/170.43 (-8.0%) | -dB | 157.42/146.43 (-7.0%) | 184.43/168.37 (-9.0%) | -dB | 154.95/142.99 (-8.0%) | 178.26/166.73 (-6.0%) | -dB | 151.24/140.52 (-7.0%) | 171.67/163.44 (-5.0%) |
| Rx EVM=1.0% | -dB | 156.74/144.18 (-8.0%) | 181.55/167.47 (-8.0%) | -dB | 155.5/144.9 (-7.0%) | 179.08/165.74 (-7.0%) | -dB | 153.03/142.23 (-7.0%) | 173.4/163.27 (-6.0%) | -dB | 149.33/139.76 (-6.0%) | 168.46/158.33 (-6.0%) |
| Rx EVM=1.5% | -dB | 153.85/142.45 (-7.0%) | 174.43/162.88 (-7.0%) | -dB | 152.61/142.59 (-7.0%) | 171.96/160.04 (-7.0%) | -dB | 150.14/141.07 (-6.0%) | 168.69/157.57 (-7.0%) | -dB | 146.44/138.6 (-5.0%) | 164.18/153.07 (-7.0%) |
| Rx EVM=2.0% | -dB | 150.21/139.1 (-7.0%) | 166.79/156.62 (-6.0%) | -dB | 148.98/139.1 (-7.0%) | 165.05/155.39 (-6.0%) | -dB | 145.92/137.86 (-6.0%) | 162.58/151.46 (-7.0%) | -dB | 142.8/135.39 (-5.0%) | 158.87/149.21 (-6.0%) |

TDL-B Nominal delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 29.37dB | 193.9/200.07 (3.0%) | 193.9/237.12 (22.0%) | 29.62dB | 193.9/197.6 (2.0%) | 193.9/237.12 (22.0%) | 30.71dB | 193.9/187.72 (-3.0%) | 193.9/226.01 (17.0%) | 33.67dB | 192.66/176.61 (-8.0%) | 193.9/200.07 (3.0%) |
| Rx EVM=0.5% | 29.47dB | 193.9/199.01 (3.0%) | 193.9/237.12 (22.0%) | 29.73dB | 193.9/196.54 (1.0%) | 193.9/237.12 (22.0%) | 30.85dB | 193.76/187.19 (-3.0%) | 193.9/223.95 (16.0%) | 33.93dB | 192.66/176.34 (-8.0%) | 193.9/198.43 (2.0%) |
| Rx EVM=1.0% | 29.77dB | 193.9/195.95 (1.0%) | 193.9/236.14 (22.0%) | 30.05dB | 193.9/193.48 (-0.0%) | 193.9/234.66 (21.0%) | 31.26dB | 193.38/185.66 (-4.0%) | 193.9/218.35 (13.0%) | 34.83dB | 192.66/175.57 (-9.0%) | 193.9/194.39 (0.0%) |
| Rx EVM=1.5% | 30.33dB | 193.9/191.32 (-1.0%) | 193.9/227.83 (18.0%) | 30.64dB | 193.9/188.85 (-3.0%) | 193.9/221.29 (14.0%) | 32.07dB | 192.8/183.35 (-5.0%) | 193.9/207.63 (7.0%) | 36.94dB | 192.66/174.42 (-9.0%) | 193.9/190.55 (-2.0%) |
| Rx EVM=2.0% | 31.25dB | 193.31/186.09 (-4.0%) | 193.9/212.55 (10.0%) | 31.64dB | 193.31/184.79 (-4.0%) | 193.9/206.16 (6.0%) | 33.52dB | 192.66/179.26 (-7.0%) | 193.9/197.38 (2.0%) | -dB | 191.49/171.21 (-11.0%) | 193.9/187.5 (-3.0%) |

TDL-B Very short delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 37.0dB | 151.91/145.73 (-4.0%) | 177.84/175.37 (-1.0%) | 37.5dB | 150.67/144.5 (-4.0%) | 175.37/169.2 (-4.0%) | 38.0dB | 148.2/144.5 (-2.0%) | 171.67/167.96 (-2.0%) | -dB | 145.73/140.79 (-3.0%) | 167.96/163.02 (-3.0%) |
| Rx EVM=0.5% | 37.58dB | 151.24/144.8 (-4.0%) | 176.2/173.32 (-2.0%) | 38.16dB | 150.01/143.96 (-4.0%) | 174.14/169.2 (-3.0%) | 38.75dB | 147.67/143.7 (-3.0%) | 170.84/165.91 (-3.0%) | -dB | 145.07/139.99 (-3.0%) | 167.14/161.79 (-3.0%) |
| Rx EVM=1.0% | 40.02dB | 149.33/142.12 (-5.0%) | 172.16/167.97 (-2.0%) | 41.18dB | 148.09/142.43 (-4.0%) | 170.93/167.72 (-2.0%) | 42.33dB | 146.14/141.4 (-3.0%) | 168.21/161.05 (-4.0%) | 44.69dB | 143.15/137.7 (-4.0%) | 164.75/158.58 (-4.0%) |
| Rx EVM=1.5% | -dB | 146.44/138.08 (-6.0%) | 167.45/160.41 (-4.0%) | -dB | 145.2/140.12 (-3.0%) | 166.22/159.17 (-4.0%) | -dB | 143.83/137.94 (-4.0%) | 162.95/156.34 (-4.0%) | -dB | 140.26/134.23 (-4.0%) | 159.61/153.43 (-4.0%) |
| Rx EVM=2.0% | -dB | 142.21/135.33 (-5.0%) | 160.62/153.43 (-4.0%) | -dB | 141.57/134.87 (-5.0%) | 159.38/152.92 (-4.0%) | -dB | 139.74/133.57 (-4.0%) | 156.91/149.5 (-5.0%) | -dB | 137.21/130.45 (-5.0%) | 152.7/147.98 (-3.0%) |

TDL-B Very short delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 27.75dB | 192.66/202.54 (5.0%) | 193.9/233.42 (20.0%) | 29.0dB | 192.66/198.84 (3.0%) | 193.9/232.18 (20.0%) | 30.0dB | 192.66/192.66 (0.0%) | 193.9/224.77 (16.0%) | 31.75dB | 190.19/182.78 (-4.0%) | 193.9/206.25 (6.0%) |
| Rx EVM=0.5% | 27.82dB | 192.66/201.88 (5.0%) | 193.9/232.18 (20.0%) | 29.09dB | 192.53/198.04 (3.0%) | 193.9/230.95 (19.0%) | 30.11dB | 192.53/192.0 (-0.0%) | 193.9/224.77 (16.0%) | 31.92dB | 189.92/182.25 (-4.0%) | 193.9/204.6 (6.0%) |
| Rx EVM=1.0% | 28.02dB | 192.66/199.96 (4.0%) | 193.9/228.48 (18.0%) | 29.36dB | 192.14/195.74 (2.0%) | 193.9/226.75 (17.0%) | 30.46dB | 192.14/190.08 (-1.0%) | 193.9/222.56 (15.0%) | 32.46dB | 189.16/180.72 (-4.0%) | 193.9/200.32 (3.0%) |
| Rx EVM=1.5% | 28.38dB | 192.66/197.07 (2.0%) | 193.9/221.36 (14.0%) | 29.86dB | 191.57/192.28 (0.0%) | 193.9/217.22 (12.0%) | 31.11dB | 191.57/187.19 (-2.0%) | 193.9/210.61 (9.0%) | 33.55dB | 188.0/178.41 (-5.0%) | 193.46/195.49 (1.0%) |
| Rx EVM=2.0% | 28.94dB | 191.49/194.02 (1.0%) | 193.9/213.72 (10.0%) | 30.66dB | 190.84/189.67 (-1.0%) | 193.9/209.51 (8.0%) | 32.22dB | 189.67/184.14 (-3.0%) | 193.17/202.83 (5.0%) | 35.79dB | 185.96/176.67 (-5.0%) | 192.66/190.98 (-1.0%) |

TDL-D Nominal delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 28.64dB | 128.44/132.15 (3.0%) | 145.73/159.32 (9.0%) | 29.0dB | 128.44/130.91 (2.0%) | 144.5/156.85 (9.0%) | 29.62dB | 125.97/127.21 (1.0%) | 140.79/151.91 (8.0%) | 32.0dB | 123.5/120.54 (-2.0%) | 139.56/145.73 (4.0%) |
| Rx EVM=0.5% | 28.72dB | 128.04/131.48 (3.0%) | 144.91/157.67 (9.0%) | 29.09dB | 128.04/130.25 (2.0%) | 143.26/155.2 (8.0%) | 29.72dB | 125.6/126.49 (1.0%) | 139.97/150.26 (7.0%) | 32.18dB | 123.14/120.22 (-2.0%) | 138.32/144.09 (4.0%) |
| Rx EVM=1.0% | 28.98dB | 126.89/129.57 (2.0%) | 142.52/153.39 (8.0%) | 29.36dB | 126.89/128.33 (1.0%) | 140.3/150.92 (8.0%) | 30.04dB | 124.53/124.42 (-0.0%) | 137.83/146.23 (6.0%) | 32.75dB | 122.11/119.3 (-2.0%) | 135.36/139.81 (3.0%) |
| Rx EVM=1.5% | 29.43dB | 125.16/126.68 (1.0%) | 138.68/147.26 (6.0%) | 29.86dB | 125.16/125.44 (0.0%) | 137.45/145.22 (6.0%) | 30.63dB | 122.91/121.3 (-1.0%) | 134.54/141.52 (5.0%) | 33.92dB | 120.55/117.91 (-2.0%) | 132.07/133.67 (1.0%) |
| Rx EVM=2.0% | 30.16dB | 123.33/122.92 (-0.0%) | 134.9/140.35 (4.0%) | 30.66dB | 122.86/121.69 (-1.0%) | 133.67/138.39 (4.0%) | 31.62dB | 120.93/118.31 (-2.0%) | 130.69/134.68 (3.0%) | 36.37dB | 118.58/115.87 (-2.0%) | 128.22/127.49 (-1.0%) |

TDL-D Nominal delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 22.58dB | 150.67/170.43 (13.0%) | 167.96/186.49 (11.0%) | 23.0dB | 149.44/169.2 (13.0%) | 166.73/185.25 (11.0%) | 23.5dB | 148.2/166.73 (13.0%) | 164.26/182.78 (11.0%) | 24.5dB | 146.97/159.32 (8.0%) | 161.79/181.55 (12.0%) |
| Rx EVM=0.5% | 22.6dB | 150.27/169.9 (13.0%) | 166.73/185.66 (11.0%) | 23.02dB | 149.04/168.66 (13.0%) | 165.49/184.02 (11.0%) | 23.52dB | 147.8/166.19 (12.0%) | 163.02/181.96 (12.0%) | 24.53dB | 146.57/158.65 (8.0%) | 160.55/180.31 (12.0%) |
| Rx EVM=1.0% | 22.66dB | 149.12/168.37 (13.0%) | 163.76/183.52 (12.0%) | 23.09dB | 147.89/167.13 (13.0%) | 162.53/181.3 (12.0%) | 23.6dB | 146.65/164.66 (12.0%) | 159.81/179.82 (13.0%) | 24.63dB | 145.42/156.74 (8.0%) | 157.59/177.1 (12.0%) |
| Rx EVM=1.5% | 22.77dB | 147.39/166.06 (13.0%) | 160.48/180.24 (12.0%) | 23.2dB | 146.16/164.82 (13.0%) | 159.24/179.0 (12.0%) | 23.73dB | 144.92/162.35 (12.0%) | 155.97/176.53 (13.0%) | 24.79dB | 143.69/153.85 (7.0%) | 154.3/172.39 (12.0%) |
| Rx EVM=2.0% | 22.91dB | 145.21/163.73 (13.0%) | 155.17/175.66 (13.0%) | 23.36dB | 144.56/162.5 (12.0%) | 154.66/174.42 (13.0%) | 23.91dB | 143.32/159.44 (11.0%) | 152.19/171.95 (13.0%) | 25.03dB | 142.09/149.62 (5.0%) | 150.45/167.01 (11.0%) |

TDL-D Very short delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 29.0dB | 128.44/130.91 (2.0%) | 145.73/158.08 (8.0%) | 29.33dB | 127.21/129.68 (2.0%) | 144.5/155.61 (8.0%) | 30.0dB | 125.97/125.97 (0.0%) | 140.79/150.67 (7.0%) | 32.0dB | 122.64/121.03 (-1.0%) | 138.32/144.5 (4.0%) |
| Rx EVM=0.5% | 29.09dB | 128.04/130.25 (2.0%) | 144.5/156.44 (8.0%) | 29.43dB | 126.94/129.01 (2.0%) | 143.26/153.97 (7.0%) | 30.11dB | 125.53/125.46 (-0.0%) | 139.97/149.03 (6.0%) | 32.18dB | 122.25/120.58 (-1.0%) | 137.09/142.85 (4.0%) |
| Rx EVM=1.0% | 29.36dB | 126.89/128.33 (1.0%) | 141.53/152.16 (8.0%) | 29.73dB | 126.17/127.1 (1.0%) | 140.3/149.69 (7.0%) | 30.46dB | 124.27/124.01 (-0.0%) | 137.58/144.99 (5.0%) | 32.75dB | 121.14/119.28 (-2.0%) | 134.12/138.57 (3.0%) |
| Rx EVM=1.5% | 29.86dB | 125.16/125.44 (0.0%) | 138.25/146.46 (6.0%) | 30.27dB | 125.02/124.21 (-1.0%) | 137.01/143.99 (5.0%) | 31.11dB | 122.36/121.82 (-0.0%) | 133.74/139.85 (5.0%) | 33.92dB | 119.47/117.31 (-2.0%) | 131.27/132.87 (1.0%) |
| Rx EVM=2.0% | 30.66dB | 123.04/122.27 (-1.0%) | 133.67/139.62 (4.0%) | 31.18dB | 122.63/121.16 (-1.0%) | 132.43/137.15 (4.0%) | 32.22dB | 120.14/118.35 (-1.0%) | 129.96/133.67 (3.0%) | 36.37dB | 117.35/114.78 (-2.0%) | 127.49/127.49 (0.0%) |

TDL-D Very short delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 23.0dB | 150.67/170.43 (13.0%) | 167.96/186.49 (11.0%) | 23.0dB | 149.44/169.2 (13.0%) | 166.73/185.25 (11.0%) | 23.0dB | 148.2/165.49 (12.0%) | 163.64/182.78 (12.0%) | 24.5dB | 146.97/159.32 (8.0%) | 161.79/181.55 (12.0%) |
| Rx EVM=0.5% | 23.02dB | 150.27/169.9 (13.0%) | 166.73/185.66 (11.0%) | 23.02dB | 149.04/168.66 (13.0%) | 165.49/184.43 (11.0%) | 23.02dB | 147.8/165.09 (12.0%) | 162.59/181.96 (12.0%) | 24.53dB | 146.57/158.52 (8.0%) | 160.96/180.72 (12.0%) |
| Rx EVM=1.0% | 23.09dB | 149.12/168.37 (13.0%) | 163.76/183.52 (12.0%) | 23.09dB | 147.89/167.13 (13.0%) | 162.53/182.29 (12.0%) | 23.09dB | 146.65/163.94 (12.0%) | 160.06/179.82 (12.0%) | 24.63dB | 145.42/156.22 (7.0%) | 158.58/178.09 (12.0%) |
| Rx EVM=1.5% | 23.2dB | 147.39/166.06 (13.0%) | 160.48/180.24 (12.0%) | 23.2dB | 146.16/164.82 (13.0%) | 159.24/179.0 (12.0%) | 23.2dB | 144.92/162.21 (12.0%) | 156.77/176.53 (13.0%) | 24.79dB | 143.69/152.76 (6.0%) | 154.3/172.39 (12.0%) |
| Rx EVM=2.0% | 23.36dB | 145.21/163.15 (12.0%) | 155.17/175.66 (13.0%) | 23.36dB | 144.56/162.5 (12.0%) | 154.66/174.42 (13.0%) | 23.36dB | 143.32/159.44 (11.0%) | 152.19/171.95 (13.0%) | 25.03dB | 142.09/148.98 (5.0%) | 150.45/167.01 (11.0%) |

TDL-E Nominal delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 28.03dB | 115.35/122.39 (6.0%) | 125.97/143.26 (14.0%) | 28.19dB | 114.73/120.91 (5.0%) | 124.74/142.03 (14.0%) | 29.1dB | 113.25/116.46 (3.0%) | 123.01/137.09 (11.0%) | 31.47dB | 111.89/108.93 (-3.0%) | 121.4/130.91 (8.0%) |
| Rx EVM=0.5% | 28.1dB | 115.11/121.79 (6.0%) | 125.15/142.03 (13.0%) | 28.26dB | 114.49/120.3 (5.0%) | 124.2/140.38 (13.0%) | 29.19dB | 113.04/115.86 (2.0%) | 122.39/135.85 (11.0%) | 31.63dB | 111.64/108.53 (-3.0%) | 120.78/129.27 (7.0%) |
| Rx EVM=1.0% | 28.32dB | 114.42/120.07 (5.0%) | 123.23/138.82 (13.0%) | 28.48dB | 113.8/118.54 (4.0%) | 122.74/136.35 (11.0%) | 29.47dB | 112.43/114.14 (2.0%) | 120.74/132.4 (10.0%) | 32.14dB | 110.91/107.38 (-3.0%) | 119.13/125.16 (5.0%) |
| Rx EVM=1.5% | 28.7dB | 113.38/117.47 (4.0%) | 121.36/134.11 (11.0%) | 28.89dB | 112.76/115.88 (3.0%) | 120.33/131.64 (9.0%) | 29.99dB | 111.5/111.54 (0.0%) | 118.4/127.57 (8.0%) | 33.14dB | 109.81/105.65 (-4.0%) | 116.8/120.11 (3.0%) |
| Rx EVM=2.0% | 29.31dB | 112.13/114.02 (2.0%) | 118.39/128.0 (8.0%) | 29.52dB | 111.51/112.65 (1.0%) | 117.57/126.26 (7.0%) | 30.82dB | 110.34/107.91 (-2.0%) | 116.04/123.28 (6.0%) | 35.15dB | 108.26/100.83 (-7.0%) | 114.5/114.36 (-0.0%) |

TDL-E Nominal delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 22.09dB | 123.5/144.5 (17.0%) | 135.85/158.08 (16.0%) | 22.24dB | 122.88/143.26 (17.0%) | 134.62/156.85 (17.0%) | 22.75dB | 121.89/140.79 (16.0%) | 134.62/155.61 (16.0%) | 23.77dB | 121.28/135.85 (12.0%) | 133.38/154.38 (16.0%) |
| Rx EVM=0.5% | 22.1dB | 123.23/144.1 (17.0%) | 135.03/157.26 (16.0%) | 22.26dB | 122.62/142.86 (17.0%) | 133.79/156.02 (17.0%) | 22.77dB | 121.67/140.52 (15.0%) | 133.79/154.79 (16.0%) | 23.8dB | 121.04/135.05 (12.0%) | 132.56/153.55 (16.0%) |
| Rx EVM=1.0% | 22.16dB | 122.47/142.95 (17.0%) | 132.89/154.87 (17.0%) | 22.32dB | 121.85/141.71 (16.0%) | 131.65/153.64 (17.0%) | 22.83dB | 121.02/139.76 (15.0%) | 131.65/152.4 (16.0%) | 23.88dB | 120.35/132.76 (10.0%) | 130.42/151.17 (16.0%) |
| Rx EVM=1.5% | 22.25dB | 121.31/141.22 (16.0%) | 130.04/151.03 (16.0%) | 22.41dB | 120.7/139.98 (16.0%) | 129.24/149.8 (16.0%) | 22.94dB | 120.04/138.6 (15.0%) | 128.8/148.56 (15.0%) | 24.01dB | 119.31/129.29 (8.0%) | 127.57/146.89 (15.0%) |
| Rx EVM=2.0% | 22.38dB | 119.98/139.62 (16.0%) | 126.98/147.98 (17.0%) | 22.55dB | 119.42/138.97 (16.0%) | 126.26/146.74 (16.0%) | 23.09dB | 118.8/137.15 (15.0%) | 125.75/144.78 (15.0%) | 24.21dB | 118.12/125.51 (6.0%) | 124.51/142.31 (14.0%) |

TDL-E Very short delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 28.5dB | 114.48/117.82 (3.0%) | 124.74/140.79 (13.0%) | 28.82dB | 113.87/116.71 (2.0%) | 123.5/138.32 (12.0%) | 29.69dB | 112.26/113.5 (1.0%) | 122.14/132.15 (8.0%) | 32.0dB | 110.66/103.37 (-7.0%) | 120.41/128.44 (7.0%) |
| Rx EVM=0.5% | 28.58dB | 114.22/117.35 (3.0%) | 124.16/139.56 (12.0%) | 28.9dB | 113.64/116.23 (2.0%) | 122.88/136.68 (11.0%) | 29.79dB | 112.04/112.85 (1.0%) | 121.4/130.91 (8.0%) | 32.18dB | 110.47/102.51 (-7.0%) | 119.71/126.8 (6.0%) |
| Rx EVM=1.0% | 28.82dB | 113.45/116.02 (2.0%) | 122.64/136.1 (11.0%) | 29.16dB | 112.99/114.85 (2.0%) | 121.33/132.64 (9.0%) | 30.11dB | 111.39/110.97 (-0.0%) | 119.6/127.95 (7.0%) | 32.75dB | 109.93/100.02 (-9.0%) | 117.94/122.69 (4.0%) |
| Rx EVM=1.5% | 29.26dB | 112.3/113.99 (2.0%) | 120.33/130.4 (8.0%) | 29.64dB | 112.01/112.77 (1.0%) | 119.35/127.93 (7.0%) | 30.72dB | 110.4/108.14 (-2.0%) | 117.53/124.4 (6.0%) | 33.92dB | 109.13/96.26 (-12.0%) | 115.72/117.64 (2.0%) |
| Rx EVM=2.0% | 29.96dB | 110.9/110.98 (0.0%) | 117.5/124.8 (6.0%) | 30.41dB | 110.6/109.8 (-1.0%) | 116.76/122.7 (5.0%) | 31.73dB | 109.17/102.76 (-6.0%) | 115.07/119.49 (4.0%) | 36.37dB | 107.81/89.83 (-17.0%) | 113.27/111.81 (-1.0%) |

TDL-E Very short delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 22.11dB | 123.5/144.5 (17.0%) | 135.85/158.08 (16.0%) | 22.27dB | 123.13/143.26 (16.0%) | 135.85/156.85 (15.0%) | 22.77dB | 122.39/142.03 (16.0%) | 134.62/155.61 (16.0%) | 23.76dB | 121.77/137.09 (13.0%) | 134.62/154.38 (15.0%) |
| Rx EVM=0.5% | 22.12dB | 123.27/144.23 (17.0%) | 135.03/157.26 (16.0%) | 22.29dB | 122.88/142.99 (16.0%) | 135.03/156.02 (16.0%) | 22.79dB | 122.14/141.76 (16.0%) | 133.79/154.79 (16.0%) | 23.79dB | 121.52/136.29 (12.0%) | 133.79/153.55 (15.0%) |
| Rx EVM=1.0% | 22.18dB | 122.62/143.46 (17.0%) | 132.89/155.12 (17.0%) | 22.35dB | 122.15/142.23 (16.0%) | 132.89/153.88 (16.0%) | 22.85dB | 121.41/140.99 (16.0%) | 131.65/152.65 (16.0%) | 23.87dB | 120.79/133.99 (11.0%) | 131.65/151.17 (15.0%) |
| Rx EVM=1.5% | 22.27dB | 121.64/142.31 (17.0%) | 130.04/151.83 (17.0%) | 22.44dB | 121.05/141.07 (17.0%) | 130.04/151.03 (16.0%) | 22.96dB | 120.31/139.84 (16.0%) | 128.8/149.36 (16.0%) | 24.0dB | 119.69/130.53 (9.0%) | 128.8/146.89 (14.0%) |
| Rx EVM=2.0% | 22.4dB | 120.23/140.27 (17.0%) | 126.98/147.98 (17.0%) | 22.58dB | 119.73/139.03 (16.0%) | 126.98/147.25 (16.0%) | 23.11dB | 118.99/137.21 (15.0%) | 125.75/145.51 (16.0%) | 24.2dB | 118.31/126.51 (7.0%) | 125.75/142.31 (13.0%) |

TDL-D Hadamard Nominal delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 28.37dB | 193.9/217.36 (12.0%) | 193.9/237.12 (22.0%) | 28.8dB | 193.9/213.66 (10.0%) | 193.9/235.89 (22.0%) | 30.07dB | 193.9/192.66 (-1.0%) | 193.9/235.89 (22.0%) | 33.11dB | 193.9/188.96 (-3.0%) | 193.9/214.89 (11.0%) |
| Rx EVM=0.5% | 28.45dB | 193.9/215.5 (11.0%) | 193.9/237.12 (22.0%) | 28.88dB | 193.9/211.66 (9.0%) | 193.9/236.3 (22.0%) | 30.18dB | 193.9/192.39 (-1.0%) | 193.9/235.89 (22.0%) | 33.34dB | 193.9/188.69 (-3.0%) | 193.9/211.19 (9.0%) |
| Rx EVM=1.0% | 28.69dB | 193.9/210.15 (8.0%) | 193.9/237.12 (22.0%) | 29.14dB | 193.9/205.93 (6.0%) | 193.9/236.87 (22.0%) | 30.54dB | 193.9/191.63 (-1.0%) | 193.9/235.39 (21.0%) | 34.11dB | 193.9/187.92 (-3.0%) | 193.9/201.56 (4.0%) |
| Rx EVM=1.5% | 29.11dB | 193.9/202.05 (4.0%) | 193.9/236.68 (22.0%) | 29.62dB | 193.9/197.26 (2.0%) | 193.9/235.89 (22.0%) | 31.2dB | 193.9/190.47 (-2.0%) | 193.9/229.06 (18.0%) | 35.82dB | 193.9/186.77 (-4.0%) | 193.9/191.79 (-1.0%) |
| Rx EVM=2.0% | 29.79dB | 193.9/195.38 (1.0%) | 193.9/232.97 (20.0%) | 30.38dB | 193.9/192.2 (-1.0%) | 193.9/230.06 (19.0%) | 32.35dB | 193.31/189.02 (-2.0%) | 193.9/214.52 (11.0%) | -dB | 193.31/183.56 (-5.0%) | 193.9/190.19 (-2.0%) |

TDL-D Hadamard Nominal delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 24.0dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 25.0dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 26.45dB | 193.9/232.18 (20.0%) | 193.9/235.89 (22.0%) | 29.17dB | 193.9/200.07 (3.0%) | 193.9/233.42 (20.0%) |
| Rx EVM=0.5% | 24.03dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 25.03dB | 193.9/235.75 (22.0%) | 193.9/237.12 (22.0%) | 26.5dB | 193.9/229.39 (18.0%) | 193.9/235.89 (22.0%) | 29.26dB | 193.9/199.27 (3.0%) | 193.9/231.77 (20.0%) |
| Rx EVM=1.0% | 24.11dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 25.14dB | 193.9/235.37 (21.0%) | 193.9/237.12 (22.0%) | 26.65dB | 193.9/221.36 (14.0%) | 193.9/235.89 (22.0%) | 29.54dB | 193.9/196.98 (2.0%) | 193.9/224.29 (16.0%) |
| Rx EVM=1.5% | 24.25dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 25.32dB | 193.9/234.79 (21.0%) | 193.9/236.68 (22.0%) | 26.91dB | 193.9/209.22 (8.0%) | 193.9/235.89 (22.0%) | 30.06dB | 193.9/193.51 (-0.0%) | 193.9/206.61 (7.0%) |
| Rx EVM=2.0% | 24.46dB | 193.9/230.61 (19.0%) | 193.9/236.39 (22.0%) | 25.59dB | 193.9/224.1 (16.0%) | 193.9/236.61 (22.0%) | 27.31dB | 193.9/205.66 (6.0%) | 193.9/235.89 (22.0%) | 30.92dB | 193.9/191.49 (-1.0%) | 193.9/204.28 (5.0%) |

TDL-D Hadamard Very short delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 28.5dB | 193.9/211.19 (9.0%) | 193.9/237.12 (22.0%) | 29.0dB | 193.9/206.25 (6.0%) | 193.9/237.12 (22.0%) | 30.11dB | 193.9/192.66 (-1.0%) | 193.9/235.89 (22.0%) | 33.12dB | 193.9/188.96 (-3.0%) | 193.9/216.13 (11.0%) |
| Rx EVM=0.5% | 28.58dB | 193.9/209.86 (8.0%) | 193.9/237.12 (22.0%) | 29.09dB | 193.9/204.92 (6.0%) | 193.9/237.12 (22.0%) | 30.22dB | 193.9/192.26 (-1.0%) | 193.9/235.47 (21.0%) | 33.35dB | 193.9/188.56 (-3.0%) | 193.9/211.61 (9.0%) |
| Rx EVM=1.0% | 28.82dB | 193.9/206.03 (6.0%) | 193.9/236.87 (22.0%) | 29.36dB | 193.9/201.09 (4.0%) | 193.9/236.87 (22.0%) | 30.58dB | 193.9/191.11 (-1.0%) | 193.9/233.67 (21.0%) | 34.13dB | 193.9/187.41 (-3.0%) | 193.9/200.57 (3.0%) |
| Rx EVM=1.5% | 29.26dB | 193.9/200.25 (3.0%) | 193.9/235.45 (21.0%) | 29.86dB | 193.9/195.31 (1.0%) | 193.9/235.01 (21.0%) | 31.26dB | 193.9/189.38 (-2.0%) | 193.9/225.36 (16.0%) | 35.84dB | 193.9/185.68 (-4.0%) | 193.9/191.79 (-1.0%) |
| Rx EVM=2.0% | 29.96dB | 193.9/194.15 (0.0%) | 193.9/231.01 (19.0%) | 30.66dB | 193.9/190.97 (-2.0%) | 193.9/226.87 (17.0%) | 32.42dB | 193.31/187.78 (-3.0%) | 193.9/209.36 (8.0%) | -dB | 192.72/182.91 (-5.0%) | 193.9/190.19 (-2.0%) |

TDL-D Hadamard Very short delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 24.0dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 25.2dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 26.5dB | 193.9/228.48 (18.0%) | 193.9/235.89 (22.0%) | 29.29dB | 193.9/200.07 (3.0%) | 193.9/233.42 (20.0%) |
| Rx EVM=0.5% | 24.03dB | 193.9/235.75 (22.0%) | 193.9/237.12 (22.0%) | 25.24dB | 193.9/235.49 (21.0%) | 193.9/237.12 (22.0%) | 26.55dB | 193.9/226.08 (17.0%) | 193.9/235.89 (22.0%) | 29.38dB | 193.9/199.14 (3.0%) | 193.9/231.36 (19.0%) |
| Rx EVM=1.0% | 24.11dB | 193.9/235.37 (21.0%) | 193.9/237.12 (22.0%) | 25.35dB | 193.9/234.34 (21.0%) | 193.9/237.12 (22.0%) | 26.7dB | 193.9/219.2 (13.0%) | 193.9/235.89 (22.0%) | 29.67dB | 193.9/196.46 (1.0%) | 193.9/223.3 (15.0%) |
| Rx EVM=1.5% | 24.25dB | 193.9/234.79 (21.0%) | 193.9/237.12 (22.0%) | 25.54dB | 193.9/232.61 (20.0%) | 193.9/236.68 (22.0%) | 26.96dB | 193.9/208.8 (8.0%) | 193.9/235.89 (22.0%) | 30.21dB | 193.9/192.42 (-1.0%) | 193.9/206.61 (7.0%) |
| Rx EVM=2.0% | 24.46dB | 193.9/227.03 (17.0%) | 193.9/236.39 (22.0%) | 25.82dB | 193.9/222.22 (15.0%) | 193.9/235.89 (22.0%) | 27.36dB | 193.9/205.07 (6.0%) | 193.9/235.16 (21.0%) | 31.11dB | 193.9/190.25 (-2.0%) | 193.9/204.28 (5.0%) |

TDL-E Hadamard Nominal delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 28.75dB | 193.9/230.95 (19.0%) | 193.9/237.12 (22.0%) | 29.08dB | 193.9/223.54 (15.0%) | 193.9/237.12 (22.0%) | 30.12dB | 193.9/191.43 (-1.0%) | 193.9/235.89 (22.0%) | 33.17dB | 193.9/191.43 (-1.0%) | 193.9/229.71 (18.0%) |
| Rx EVM=0.5% | 28.83dB | 193.9/227.09 (17.0%) | 193.9/237.12 (22.0%) | 29.17dB | 193.9/220.08 (14.0%) | 193.9/236.71 (22.0%) | 30.24dB | 193.9/191.29 (-1.0%) | 193.9/235.89 (22.0%) | 33.4dB | 193.9/191.29 (-1.0%) | 193.9/221.91 (14.0%) |
| Rx EVM=1.0% | 29.09dB | 193.9/216.01 (11.0%) | 193.9/237.12 (22.0%) | 29.44dB | 193.9/210.14 (8.0%) | 193.9/236.13 (22.0%) | 30.6dB | 193.9/190.91 (-2.0%) | 193.9/235.64 (22.0%) | 34.18dB | 193.9/190.91 (-2.0%) | 193.9/203.29 (5.0%) |
| Rx EVM=1.5% | 29.56dB | 193.9/199.24 (3.0%) | 193.9/236.68 (22.0%) | 29.95dB | 193.9/195.11 (1.0%) | 193.9/236.68 (22.0%) | 31.28dB | 193.9/190.33 (-2.0%) | 193.9/234.21 (21.0%) | 35.93dB | 193.9/190.33 (-2.0%) | 193.9/190.99 (-1.0%) |
| Rx EVM=2.0% | 30.31dB | 193.9/192.79 (-1.0%) | 193.9/235.89 (22.0%) | 30.78dB | 193.9/190.84 (-2.0%) | 193.9/235.16 (21.0%) | 32.45dB | 193.9/190.19 (-2.0%) | 193.9/220.32 (14.0%) | -dB | 193.9/184.92 (-5.0%) | 193.9/190.19 (-2.0%) |

TDL-E Hadamard Nominal delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 27.03dB | 193.9/237.12 (22.0%) | 193.9/237.12 (22.0%) | 27.09dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 26.45dB | 193.9/233.42 (20.0%) | 193.9/235.89 (22.0%) | 29.12dB | 193.9/202.54 (4.0%) | 193.9/233.42 (20.0%) |
| Rx EVM=0.5% | 27.09dB | 193.9/236.99 (22.0%) | 193.9/237.12 (22.0%) | 27.15dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 26.5dB | 193.9/230.49 (19.0%) | 193.9/235.89 (22.0%) | 29.22dB | 193.9/201.48 (4.0%) | 193.9/233.42 (20.0%) |
| Rx EVM=1.0% | 27.26dB | 193.9/236.6 (22.0%) | 193.9/237.12 (22.0%) | 27.32dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 26.65dB | 193.9/222.08 (15.0%) | 193.9/235.89 (22.0%) | 29.5dB | 193.9/198.42 (2.0%) | 193.9/228.25 (18.0%) |
| Rx EVM=1.5% | 27.56dB | 193.9/236.03 (22.0%) | 193.9/237.12 (22.0%) | 27.62dB | 193.9/235.89 (22.0%) | 193.9/236.68 (22.0%) | 26.91dB | 193.9/209.36 (8.0%) | 193.9/235.89 (22.0%) | 30.01dB | 193.9/193.79 (-0.0%) | 193.9/206.61 (7.0%) |
| Rx EVM=2.0% | 28.01dB | 193.9/234.71 (21.0%) | 193.9/237.12 (22.0%) | 28.09dB | 193.9/227.68 (17.0%) | 193.9/235.89 (22.0%) | 27.31dB | 193.9/205.66 (6.0%) | 193.9/235.89 (22.0%) | 30.86dB | 193.9/192.07 (-1.0%) | 193.9/204.28 (5.0%) |

TDL-E Hadamard Very short delay spread 2T2R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 28.6dB | 193.9/230.95 (19.0%) | 193.9/237.12 (22.0%) | 29.08dB | 193.9/221.07 (14.0%) | 193.9/237.12 (22.0%) | 30.12dB | 193.9/191.43 (-1.0%) | 193.9/235.89 (22.0%) | 33.18dB | 193.9/190.19 (-2.0%) | 193.9/230.95 (19.0%) |
| Rx EVM=0.5% | 28.68dB | 193.9/227.22 (17.0%) | 193.9/237.12 (22.0%) | 29.17dB | 193.9/217.88 (12.0%) | 193.9/237.12 (22.0%) | 30.23dB | 193.9/191.29 (-1.0%) | 193.9/235.89 (22.0%) | 33.41dB | 193.9/190.06 (-2.0%) | 193.9/222.32 (15.0%) |
| Rx EVM=1.0% | 28.93dB | 193.9/216.52 (12.0%) | 193.9/237.12 (22.0%) | 29.45dB | 193.9/208.7 (8.0%) | 193.9/236.87 (22.0%) | 30.59dB | 193.9/190.91 (-2.0%) | 193.9/235.64 (22.0%) | 34.2dB | 193.9/189.67 (-2.0%) | 193.9/202.3 (4.0%) |
| Rx EVM=1.5% | 29.38dB | 193.9/200.33 (3.0%) | 193.9/236.68 (22.0%) | 29.96dB | 193.9/194.83 (0.0%) | 193.9/235.89 (22.0%) | 31.27dB | 193.9/190.33 (-2.0%) | 193.9/234.65 (21.0%) | 35.96dB | 193.9/189.1 (-2.0%) | 193.9/190.99 (-1.0%) |
| Rx EVM=2.0% | 30.1dB | 193.9/193.44 (-0.0%) | 193.9/235.89 (22.0%) | 30.79dB | 193.9/190.84 (-2.0%) | 193.9/235.16 (21.0%) | 32.44dB | 193.9/190.19 (-2.0%) | 193.9/221.55 (14.0%) | -dB | 193.9/184.27 (-5.0%) | 193.9/190.19 (-2.0%) |

TDL-E Hadamard Very short delay spread 2T4R

| Rx EVM / Tx EVM | Tx EVM= 0.0 |  |  | Tx EVM= 1.0 |  |  | Tx EVM= 2.0 |  |  | Tx EVM= 3.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB | Crossover SNR | SNR = 30dB | SNR = 35dB |
| Rx EVM=0% | 27.03dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 27.11dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 26.6dB | 193.9/233.42 (20.0%) | 193.9/235.89 (22.0%) | 29.22dB | 193.9/202.54 (4.0%) | 193.9/233.42 (20.0%) |
| Rx EVM=0.5% | 27.09dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 27.17dB | 193.9/235.75 (22.0%) | 193.9/237.12 (22.0%) | 26.65dB | 193.9/230.23 (19.0%) | 193.9/235.89 (22.0%) | 29.31dB | 193.9/201.34 (4.0%) | 193.9/232.59 (20.0%) |
| Rx EVM=1.0% | 27.26dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 27.34dB | 193.9/235.37 (21.0%) | 193.9/237.12 (22.0%) | 26.81dB | 193.9/221.05 (14.0%) | 193.9/235.89 (22.0%) | 29.6dB | 193.9/197.9 (2.0%) | 193.9/226.03 (17.0%) |
| Rx EVM=1.5% | 27.56dB | 193.9/235.89 (22.0%) | 193.9/237.12 (22.0%) | 27.65dB | 193.9/234.79 (21.0%) | 193.9/236.68 (22.0%) | 27.08dB | 193.9/207.18 (7.0%) | 193.9/235.89 (22.0%) | 30.13dB | 193.9/192.7 (-1.0%) | 193.9/205.37 (6.0%) |
| Rx EVM=2.0% | 28.02dB | 193.9/233.54 (20.0%) | 193.9/237.12 (22.0%) | 28.12dB | 193.9/224.69 (16.0%) | 193.9/235.89 (22.0%) | 27.49dB | 193.9/203.78 (5.0%) | 193.9/235.16 (21.0%) | 31.01dB | 193.9/190.25 (-2.0%) | 193.9/203.78 (5.0%) |