3GPP TSG RAN WG1 Meeting #89	 R1-1709324

Hangzhou, China, 15-19 May 2017

Agenda Item:	5.2.3

Source:	Huawei, HiSilicon

Title:	TP on impact on network implementation

Document for:	Discussion and decision

Introduction

This text proposal is to provide a short discussion on the impact on network implementation for the RAN WGs on the study item, a simplified HS-SCCH for UMTS [1].

Text proposal

------------------------------------------------------------- Text start -----------------------------------------------------

- 8.X.X		Capability reporting

In this Section we discuss potential impact to network implementation for the identified solution. UE capability reporting is needed to enable network to save power consumption on simplified HS-SCCH.

If the UE may use an advanced receiver to enhance the HS-SCCH detection performance, it would be good for the network to know that the UE is capable of doing it. The NodeB can either extend the coverage of HS-SCCH or can trade-off for power savings. If network knows that the target UE receiving HS-SCCH is capable of the ‘known in advance’ reception, network can choose a smaller power on HS-SCCH when choosing the “known in advance” sub-set within the original set of codewords, saving downlink power resource. Otherwise, it is only possible to enhance HS-SCCH reliability or coverage, but not saving HS-SCCH transmit power. Hence, network knowing UE’s capability for advanced HS-SCCH part 1 reception can save HS-SCCH part 1 DL transmit power when the “known in advance” subset of the HS-SCCH part 1 codewords was selected.

------------------------------------------------------------- Text end -----------------------------------------------------

Conclusion

It is proposed:

Proposal: Agree and capture the text proposal on the TR for the study on simplified HS-SCCH for UMTS.

References

- [1] RP-170725, Study on a simplified HS-SCCH for UMTS