3GPP TSG RAN WG1 89			R1- 1709501

Hangzhou, China, May 15th-19th

Agenda item:	7.1.2.1.3

Source: 	Intel

Title: 	Summary of Issues and WFs on Codebook based transmission for UL

Document for:	Discussion

# Introduction

In RAN1#88bis, the following agreements were made [1]:

Agreements: - Codebook based transmission for UL is supported at least by following signaling in UL grant: - SRI+TPMI+TRI, where - The TPMI is used to indicate preferred precoder over the SRS ports in the selected SRS resource by the SRI. - No SRI when a single SRS resource is configured - The TPMI is used to indicate preferred precoder over the SRS ports in the configured single SRS resource. - Support indication on selection of multiple SRS resources - FFS details Agreements: - For codebook based transmission for CP-OFDM based UL, when a UE is configured with UL frequency selective precoding and if subband TPMI signaling is supported, support one of the following alternatives: - Alt 1:Subband TPMIs are signaled via DCI to the UE only for allocated PRBs for a given PUSCH transmission - Alt 2:Subband TPMIs are signaled via DCI to the UE for all PRBs in UL, regardless of the actual RA for a given PUSCH transmission - Other alternatives are not precluded - Note: Subband TPMI may correspond to W2 if dual-stage codebook is supported - FFS whether or not wideband TPMI is always signaled along with subband TPMI

Based on the submitted contributions in RAN1#89 for this agenda item, the following issues are identified and summarized in the following sections.

- UL codebook structure

- Number of APs for frequency selective precoding

- DL control signaling

# UL codebook structure

There two options for the UL codebook structure:

- Single-stage codebook: Qualcomm, Ericsson, Nokia(wideband transmission), Intel

- Two-stage codebook: LG, Samsung, Nokia(frequency selective transmission)

# Number of APs for frequency selective precoding

For the number of APs X where the frequency selective precoding can be applied, the following options.

- X=2: Huawei

- X>4: Qualcomm, Nokia, MTK, CATT

There is also another proposal from Huawei that whether to use frequency-selective precoding is a configuration, and the limitation of X ports in the specification is unnecessary.

# DL Control Signaling

In terms of the DL subband TPMI indication, there are the following options:

- Couple TPMI & RA: CATT, Ericsson

- Decouple TPMI & RA: Intel

- Wide-band and mid-band indication: IDC

# List of Current WF

The following related WFs are listed below:

WF on UL codebook design principle  		LG

- UL codebook is designed by assuming a single panel.

  - Codebook details are FFS

  - Existing LTE codebooks should be considered as baseline

WF on UL Codebook Structure			Nokia, CATT

- UL single-stage or dual-stage codebook support is a UE capability.

- If single-stage codebook is supported, the codebook selection is based on gNB wideband and/or subband indication.

- If dual-stage codebook is supported, the codebook structure is based on W=W1*W2 structure

  - At least the selection of W1 is based on wideband indication from gNB.

    - FFS partial-band indication of W1.

  - The selection of W2 can be based on wideband feedback or subband feedback.

NR UL MIMO Signaling Mechanism	Ericsson, Nokia, Alcatel-Lucent Shanghai Bell, Qualcomm, CATT, ZTE

- TPMI, SRI, and TRI are carried in single stage DCI

- Study further the maximum size of TPMI, SRI, and TRI and their combinations

  - Note: ~10 bits can be used as an initial estimate

WF on Frequency-Selective Precoding in UL Transmission	Qualcomm, Nokia, ASB, CATT, Ericsson, MediaTek

- For both Scheme A and B, frequency selective precoding is not supported when the number of transmission ports is less than or equal to 4.

WF on frequency-selective precoding in UL transmission 	Huawei

- For UL transmissions:

  - Frequency selective precoding for CP-OFDM can be configured by gNB without the limitation of X and Y ports

WF on codebook based UL transmission		Huawei

- For UL frequency selective precoding, NR supports subband TPMI indication to the UE via DCI for the allocated PRBs only

  - Subband TPMIs are signaled in the second stage DCI if two-stage-DCI is supported

- Support at least two-stage codebook in NR for codebook based UL-MIMO

- On top of indication of multiple SRS resources selection, support signaling to indicate TPMI across the selected multiple SRS resources in the UL grant for PUSCH precoder determination

# Proposal

After reviewing the contributions and the WF, we make the following proposals:

Proposal:

- UL codebook is designed by assuming a single panel

- For CP-OFDM, single stage codebook is used at least when number of transmission port is 2

  - LTE Rel-8 DL codebook is the baseline

- For frequency selective precoding, down-select from the following options:

  - Option 1: Determine the number of transmission port which could support frequency selective precoding

    - System level simulation is encouraged to determine the number of transmission port

  - Option 2: define the frequency selective precoding regardless the number of transmission port

  - Determine the codebook for 4 transmission ports after making a decision whether frequency selective precoding can be supported.

  - If option 2 is agreed or the number of transmission port supporting frequency selective precoding is equal to or smaller than the maximum number of SRS antenna port.

    - Make a decision whether subband TPMI is indicated by single DCI to two stage DCI

    - Make a decision whether couple subband TPMI and resource allocation or not

# References

- 3GPP, RAN1#88bis, Chairman’s Notes