3GPP TSG RAN WG1 Meeting #90                                        R1-1712167

Prague, Czech Republic, 21st -25th August 2017

Agenda Item:	6.1.4.2.1

Source:	Huawei, HiSilicon

Title:	Distributed CRC for Polar code construction

Document for:	Discussion and Decision

# Introduction

For polar code construction for NR control channels, the following was agreed in RAN1 NR AH2 [1]:

Agreement:

- All companies work together to design for the DL a Single CRC polynomial + Interleaver scheme to deliver early termination benefits while achieving the FAR (in presence of AWGN, and in presence of random QPSK, and undetected errors in intended user’s codeword), and BLER targets with acceptable complexity and latency.

  - Working assumption that the CRC length is 19 bits, to be finalised as part of the design, taking into account the number of blind decodes or hypotheses to be tested.

    - Longer CRCs will be considered if required to meet the FAR target

- For DL for K+nFAR>=12, and for UL where K+nFAR>22, J+J’ = nFAR + 3

- For UL, where 12<=K+nFAR<=22, J+J’ = nFAR + 6, comprising 3 parity bits and nFAR + 3 additional CRC bits

In this contribution, the design of a single CRC polynomial and the corresponding interleaver is provided for both DL and UL.

Notations:

Kmax:	Maximum information bit size (not including CRC bit)

K:		Information bit size (not including CRC bit)

K': 		Information bit size including CRC bit

Lcrc:		CRC bit size

KΠ:		Interleaving pattern size

π: 		Interleaver pattern

# Interleaver of DCRC

A polar code with distributed-CRC is proposed in [2] and its early termination (ET) gains are analyzed in [2] and [3]. Given a maximum information bit size Kmax and an Lcrc-bit CRC polynomial, an interleaving pattern of length KΠ(=Kmax+Lcrc) is generated, from which both K-bit information and Lcrc-bit CRC [2] are interleaved so that every CRC bit is placed after its relevant information bits.

As pointed out in [2], an interleaving pattern is generated in function of an Lcrc-bit CRC generator matrix. Thus, a KΠ-sized interleaver pattern is used as follows.

Assume that the bit sequence input for a given code block to channel coding is denoted by c0,c1,c2,…,cK'-1, where K'(=K+Lcrc) is the number of bits to be encoded (including CRC), and the interleaver is denoted by pattern π.

- Extract interleaving pattern π' from pattern π by

k'=0;

for k=0: KΠ-1

if πk≥KΠ-K'

π'(k')= πk-(KΠ-K');

k'=k'+1;

end if

end for

- Interleave c0,c1,c2,…,cK'-1 by interleaving pattern π' to have the interleaved output bit stream c0',c1',c2',…,cK'-1': ck'=cπ'(k).

Alternatively, the operation of interleaving can be expressed as follows, generating the same output.

Assume that the bit sequence input for a given code block to channel coding is denoted by c0,c1,c2,…,cK'-1, where K'(=K+Lcrc) is the number of bits to be encoded (including CRC), and the interleaver is denoted by sequence π.

- The bit sequence input c0,c1,c2,…,cK-1 is extended to x0,x1,x2,…,xKΠ-1 by:

xk=NULL			for k=0,1,…,KΠ-K'-1,

xk=ck-KΠ-K' 		for k=KΠ-K',KΠ-K'+1,…,KΠ-1.

- Interleave x0,x1,x2,…,xKΠ-1 by an interleaving pattern π: yk=xπ(k).

- Remove the (KΠ-K') NULL-bit from y0,y1,y2,…,yKΠ-1 to have the interleaved output bit stream c0',c1',c2',…,cK'-1'

The generation of CRC bits based on the information bits is in accordance to the TS38.212 draft specification. Note that all the CRC polynomials herein have a bit-reversed and explicit +1 notation. For example, 0x11d has a binary representation of 100011101, and the polynomial representation is D8+D6+D5+D4+1.

In Section 2.1, for different nFAR in DL and UL, we list the proposed CRC polynomial and corresponding interleaver. The evaluation results of FAR and ET gain for these interleavers can be found in [4].

## DL for K+nFAR>=12

For DL with K+nFAR>=12, assuming the maximum information bit length of 200, the interleavers corresponding to different nFAR levels is listed in Table 1. If a maximum information bit length Kmax is less than 200, the corresponding interleaver can be extracted from the one given in Table 1, by excluding the values i for i<200-Kmax, and replacing the values i by i-(200-Kmax) for i>=200-Kmax .

Table 1. CRC polynomial and corresponding interleaver for DL with K+nFAR>=12

| nFAR | Polynomial | Interleaver |
| --- | --- | --- |
| 16 | D19+D16+D15+D14+D13+D11+D10+D8+D6+D2+1 (0xA2B79) | 3, 4, 6, 7, 8, 9, 11, 16, 17, 18, 19, 20, 21, 24, 25, 27, 29, 31, 33, 34, 35, 36, 37, 38, 39, 40, 44, 48, 50, 51, 52, 54, 56, 57, 59, 62, 63, 65, 69, 75, 79, 85, 86, 88, 90, 92, 94, 96, 97, 100, 104, 105, 106, 108, 109, 115, 116, 125, 127, 129, 132, 136, 143, 144, 145, 147, 149, 151, 155, 159, 161, 163, 164, 165, 172, 173, 175, 176, 181, 182, 183, 185, 187, 190, 192, 195, 196, 197, 198, 199, 203, 0, 1, 2, 12, 13, 14, 23, 28, 30, 32, 41, 42, 47, 49, 53, 61, 66, 67, 72, 73, 80, 87, 91, 95, 98, 101, 103, 112, 113, 117, 126, 131, 134, 140, 146, 150, 156, 160, 169, 174, 177, 178, 184, 188, 189, 191, 193, 204, 15, 43, 55, 58, 60, 68, 74, 76, 81, 89, 93, 99, 102, 110, 114, 118, 133, 135, 137, 141, 148, 152, 157, 170, 179, 186, 194, 205, 5, 22, 77, 82, 107, 111, 119, 128, 138, 142, 153, 158, 162, 171, 180, 206, 10, 26, 45, 64, 70, 78, 84, 120, 121, 139, 167, 208, 71, 122, 123, 130, 166, 210, 46, 83, 213, 124, 154, 168, 200, 201, 202, 207, 209, 211, 212, 214, 215, 216, 217, 218 |
| 17 | D20+D19+D16+D12+D10+D8+D4+D+1 (0x191513) | 2, 3, 6, 10, 11, 15, 17, 18, 19, 21, 26, 27, 31, 32, 33, 36, 38, 40, 42, 44, 45, 48, 49, 50, 51, 53, 54, 58, 61, 62, 68, 69, 72, 73, 78, 80, 81, 83, 85, 88, 89, 90, 93, 94, 95, 96, 100, 101, 103, 105, 107, 108, 110, 111, 112, 114, 115, 116, 118, 121, 124, 126, 127, 130, 132, 133, 134, 139, 140, 141, 142, 143, 144, 146, 148, 149, 153, 154, 155, 156, 158, 160, 161, 164, 169, 176, 178, 179, 180, 181, 184, 185, 186, 188, 189, 193, 195, 197, 198, 199, 200, 0, 4, 7, 12, 16, 20, 22, 28, 34, 37, 39, 41, 43, 46, 52, 55, 59, 63, 70, 74, 79, 82, 84, 86, 91, 97, 102, 104, 106, 109, 113, 117, 119, 122, 125, 128, 131, 135, 145, 147, 150, 157, 159, 162, 165, 170, 177, 182, 187, 190, 194, 196, 201, 1, 5, 8, 13, 23, 29, 35, 47, 56, 60, 64, 71, 75, 87, 92, 98, 120, 123, 129, 136, 151, 163, 166, 171, 183, 191, 202, 9, 14, 24, 30, 57, 65, 76, 99, 137, 152, 167, 172, 192, 203, 25, 66, 77, 138, 168, 173, 204, 67, 174, 205, 175, 206, 207, 208, 209, 210, 211, 212, 213, 214, 215, 216, 217, 218, 219 |
| 18 | D21+D18+D16+D15+D12+D10+D8+D4+D3+D2+1 (0x2E2A69) | 1, 5, 6, 8, 9, 10, 12, 14, 15, 17, 19, 21, 23, 26, 27, 28, 29, 30, 32, 33, 35, 39, 41, 47, 49, 51, 52, 53, 55, 58, 59, 60, 61, 63, 66, 69, 72, 74, 76, 78, 79, 83, 87, 88, 89, 94, 96, 97, 101, 103, 105, 108, 109, 110, 111, 113, 114, 118, 125, 127, 131, 133, 134, 135, 136, 137, 138, 141, 142, 147, 150, 151, 156, 159, 160, 162, 163, 164, 168, 170, 171, 172, 173, 174, 177, 180, 181, 182, 184, 188, 190, 195, 197, 200, 0, 2, 7, 11, 13, 16, 18, 20, 22, 24, 31, 34, 36, 40, 42, 48, 50, 54, 56, 62, 64, 67, 70, 73, 75, 77, 80, 84, 90, 95, 98, 102, 104, 106, 112, 115, 119, 126, 128, 132, 139, 143, 148, 152, 157, 161, 165, 169, 175, 178, 183, 185, 189, 191, 196, 198, 201, 3, 25, 37, 43, 57, 65, 68, 71, 81, 85, 91, 99, 107, 116, 120, 129, 140, 144, 149, 153, 158, 166, 176, 179, 186, 192, 199, 202, 4, 38, 44, 82, 86, 92, 100, 117, 121, 130, 145, 154, 167, 187, 193, 203, 45, 93, 122, 146, 155, 194, 204, 46, 123, 205, 124, 206, 207, 208, 209, 210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220 |
| 19 | D22+D20+D18+D16+D13+D11+D9+D6+D4+D2+1 (0x552A55) | 1, 2, 3, 4, 12, 14, 15, 17, 19, 20, 22, 24, 27, 30, 34, 35, 36, 37, 38, 39, 40, 41, 44, 45, 48, 49, 50, 51, 52, 56, 57, 58, 60, 61, 62, 65, 67, 68, 69, 72, 74, 75, 76, 80, 81, 82, 84, 85, 86, 87, 92, 93, 95, 102, 105, 106, 107, 108, 110, 111, 113, 116, 117, 119, 121, 122, 125, 129, 130, 131, 132, 134, 136, 138, 141, 142, 147, 148, 149, 150, 152, 159, 161, 165, 169, 170, 171, 172, 174, 175, 180, 182, 183, 185, 189, 190, 191, 192, 198, 200, 5, 13, 16, 18, 21, 23, 25, 28, 31, 42, 46, 53, 59, 63, 66, 70, 73, 77, 83, 88, 94, 96, 103, 109, 112, 114, 118, 120, 123, 126, 133, 135, 137, 139, 143, 151, 153, 160, 162, 166, 173, 176, 181, 184, 186, 193, 199, 201, 6, 26, 29, 32, 43, 47, 54, 64, 71, 78, 89, 97, 104, 115, 124, 127, 140, 144, 154, 163, 167, 177, 187, 194, 202, 0, 7, 33, 55, 79, 90, 98, 128, 145, 155, 164, 168, 178, 188, 195, 203, 8, 91, 99, 146, 156, 179, 196, 204, 9, 100, 157, 197, 205, 10, 101, 158, 206, 11, 207, 208, 209, 210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220, 221 |
| 20 | D23+D18+D16+D13+D11+D10+D9+D8+D6+D5+1 (0x86F4A1) | 1, 3, 4, 5, 11, 12, 13, 14, 18, 20, 23, 24, 25, 28, 30, 31, 32, 33, 37, 39, 42, 45, 46, 47, 49, 50, 51, 53, 55, 57, 59, 60, 61, 64, 65, 67, 69, 70, 71, 72, 73, 74, 75, 77, 80, 83, 84, 85, 86, 87, 88, 89, 90, 93, 94, 99, 101, 102, 104, 106, 109, 111, 112, 113, 115, 116, 119, 121, 123, 124, 133, 134, 135, 137, 139, 140, 141, 142, 144, 145, 148, 149, 152, 153, 154, 157, 159, 162, 164, 166, 168, 169, 175, 176, 178, 179, 180, 181, 182, 187, 188, 193, 195, 200, 0, 2, 6, 15, 19, 21, 26, 29, 34, 38, 40, 43, 48, 52, 54, 56, 58, 62, 66, 68, 76, 78, 81, 91, 95, 100, 103, 105, 107, 110, 114, 117, 120, 122, 125, 136, 138, 143, 146, 150, 155, 158, 160, 163, 165, 167, 170, 177, 183, 189, 194, 196, 201, 7, 16, 22, 27, 35, 41, 44, 63, 79, 82, 92, 96, 108, 118, 126, 147, 151, 156, 161, 171, 184, 190, 197, 202, 8, 17, 36, 97, 127, 172, 185, 191, 198, 203, 9, 98, 128, 173, 186, 192, 199, 204, 10, 129, 174, 205, 130, 206, 131, 207, 132, 208, 209, 210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220, 221, 222 |
| 21 | D24+D23+D21+D20+D17+D15+D13+D12+D8+D4+D2+D+1 (0x1D11A9B) | 0, 2, 3, 5, 6, 8, 11, 12, 13, 16, 19, 20, 22, 24, 28, 32, 33, 35, 37, 38, 39, 40, 41, 42, 44, 46, 47, 49, 50, 54, 55, 57, 59, 60, 62, 64, 67, 69, 74, 79, 80, 84, 85, 86, 88, 91, 94, 102, 105, 109, 110, 111, 113, 114, 116, 118, 119, 121, 122, 125, 126, 127, 129, 130, 131, 132, 136, 137, 141, 142, 143, 147, 148, 149, 151, 153, 155, 158, 161, 164, 166, 168, 170, 171, 173, 175, 178, 179, 180, 182, 183, 186, 187, 189, 192, 194, 198, 199, 200, 1, 4, 7, 9, 14, 17, 21, 23, 25, 29, 34, 36, 43, 45, 48, 51, 56, 58, 61, 63, 65, 68, 70, 75, 81, 87, 89, 92, 95, 103, 106, 112, 115, 117, 120, 123, 128, 133, 138, 144, 150, 152, 154, 156, 159, 162, 165, 167, 169, 172, 174, 176, 181, 184, 188, 190, 193, 195, 201, 10, 15, 18, 26, 30, 52, 66, 71, 76, 82, 90, 93, 96, 104, 107, 124, 134, 139, 145, 157, 160, 163, 177, 185, 191, 196, 202, 27, 31, 53, 72, 77, 83, 97, 108, 135, 140, 146, 197, 203, 73, 78, 98, 204, 99, 205, 100, 206, 101, 207, 208, 209, 210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220, 221, 222, 223 |

## UL for 12<=K+nFAR<=22

For UL with 12<=K+nFAR<=22, assuming maximum information bit length of 22, the CRC polynomial and interleaver are given in Table 2.

Table 2. CRC polynomial and corresponding interleaver for UL with 12<=K+nFAR<=22

| nFAR | Polynomial | Interleaver |
| --- | --- | --- |
| 0 | D3+D+1 (0xD) | 1, 4, 5, 6, 12, 11, 13, 8, 0, 19, 2, 3, 15, 18, 20, 7, 9, 22, 10, 21, 16, 14, 23, 17, 24 |

## UL for K+nFAR>22

For UL with K+nFAR>22, assuming maximum information bit length of 1000, the CRC polynomial and interleaver are given in Table 3. The nFAR is 8, corresponding to the LTE system UL FAR design and the length of CRC is 11. If a maximum information bit length Kmax is less than 1000, the corresponding interleaver can be extracted from that given in Table 3, by excluding the values i for i<1000-Kmax, and replacing the values i by i-(1000-Kmax) for i>=1000-Kmax.

Table 3. CRC polynomial and corresponding interleaver for UL with K+nFAR>22

| nFAR | Polynomial | Interleaver |
| --- | --- | --- |
| 8 | D11+D10+D9+D8+D5+D3+1 (0x94F) | 0, 1, 4, 5, 6, 8, 13, 14, 15, 17, 18, 22, 24, 26, 29, 30, 32, 33, 34, 38, 39, 40, 43, 44, 47, 50, 52, 53, 58, 59, 62, 63, 65, 67, 72, 73, 75, 76, 78, 79, 82, 85, 86, 88, 89, 90, 91, 95, 97, 100, 104, 105, 108, 112, 113, 115, 116, 118, 124, 126, 127, 128, 129, 130, 132, 133, 134, 135, 141, 142, 145, 147, 148, 149, 151, 153, 154, 156, 159, 161, 164, 167, 168, 169, 170, 173, 175, 176, 177, 181, 185, 189, 191, 193, 194, 195, 196, 198, 200, 202, 203, 205, 207, 211, 212, 213, 214, 215, 217, 218, 222, 223, 224, 228, 230, 231, 232, 233, 234, 235, 236, 237, 239, 240, 241, 243, 245, 246, 247, 248, 250, 251, 252, 255, 256, 267, 270, 272, 273, 275, 279, 280, 281, 282, 285, 288, 289, 290, 292, 295, 297, 299, 300, 302, 303, 306, 307, 309, 314, 320, 322, 324, 325, 330, 335, 336, 343, 346, 347, 348, 349, 350, 351, 352, 353, 356, 357, 358, 359, 360, 362, 364, 366, 369, 370, 371, 372, 373, 376, 377, 378, 380, 385, 386, 387, 389, 390, 394, 396, 398, 401, 402, 404, 405, 406, 410, 411, 412, 415, 416, 419, 422, 424, 425, 430, 431, 434, 435, 437, 439, 444, 445, 447, 448, 450, 451, 454, 457, 458, 460, 461, 462, 463, 467, 469, 472, 476, 477, 480, 484, 485, 487, 488, 490, 496, 498, 499, 500, 501, 502, 504, 505, 506, 507, 513, 514, 517, 519, 520, 521, 523, 525, 526, 528, 531, 533, 536, 539, 540, 541, 542, 545, 547, 548, 549, 553, 557, 561, 563, 565, 566, 567, 568, 570, 572, 574, 575, 577, 579, 583, 584, 585, 586, 587, 589, 590, 594, 595, 596, 600, 602, 603, 604, 605, 606, 607, 608, 609, 611, 612, 613, 615, 617, 618, 619, 620, 622, 623, 624, 627, 628, 639, 642, 644, 645, 647, 651, 652, 653, 654, 657, 660, 661, 662, 664, 667, 669, 671, 672, 674, 675, 678, 679, 681, 686, 692, 694, 696, 697, 702, 707, 708, 715, 718, 719, 720, 721, 722, 723, 724, 725, 728, 729, 730, 731, 732, 734, 736, 738, 741, 742, 743, 744, 745, 748, 749, 750, 752, 757, 758, 759, 761, 762, 766, 768, 770, 773, 774, 776, 777, 778, 782, 783, 784, 787, 788, 791, 794, 796, 797, 802, 803, 806, 807, 809, 811, 816, 817, 819, 820, 822, 823, 826, 829, 830, 832, 833, 834, 835, 839, 841, 844, 848, 849, 852, 856, 857, 859, 860, 862, 868, 870, 871, 872, 873, 874, 876, 877, 878, 879, 885, 886, 889, 891, 892, 893, 895, 897, 898, 900, 903, 905, 908, 911, 912, 913, 914, 917, 919, 920, 921, 925, 929, 933, 935, 937, 938, 939, 940, 942, 944, 946, 947, 949, 951, 955, 956, 957, 958, 959, 961, 962, 966, 967, 968, 972, 974, 975, 976, 977, 978, 979, 980, 981, 983, 984, 985, 987, 989, 990, 991, 992, 994, 995, 996, 999, 1000, 2, 7, 9, 16, 19, 23, 25, 27, 31, 35, 41, 45, 48, 51, 54, 60, 64, 66, 68, 74, 77, 80, 83, 87, 92, 96, 98, 101, 106, 109, 114, 117, 119, 125, 131, 136, 143, 146, 150, 152, 155, 157, 160, 162, 165, 171, 174, 178, 182, 186, 190, 192, 197, 199, 201, 204, 206, 208, 216, 219, 225, 229, 238, 242, 244, 249, 253, 257, 268, 271, 274, 276, 283, 286, 291, 293, 296, 298, 301, 304, 308, 310, 315, 321, 323, 326, 331, 337, 344, 354, 361, 363, 365, 367, 374, 379, 381, 388, 391, 395, 397, 399, 403, 407, 413, 417, 420, 423, 426, 432, 436, 438, 440, 446, 449, 452, 455, 459, 464, 468, 470, 473, 478, 481, 486, 489, 491, 497, 503, 508, 515, 518, 522, 524, 527, 529, 532, 534, 537, 543, 546, 550, 554, 558, 562, 564, 569, 571, 573, 576, 578, 580, 588, 591, 597, 601, 610, 614, 616, 621, 625, 629, 640, 643, 646, 648, 655, 658, 663, 665, 668, 670, 673, 676, 680, 682, 687, 693, 695, 698, 703, 709, 716, 726, 733, 735, 737, 739, 746, 751, 753, 760, 763, 767, 769, 771, 775, 779, 785, 789, 792, 795, 798, 804, 808, 810, 812, 818, 821, 824, 827, 831, 836, 840, 842, 845, 850, 853, 858, 861, 863, 869, 875, 880, 887, 890, 894, 896, 899, 901, 904, 906, 909, 915, 918, 922, 926, 930, 934, 936, 941, 943, 945, 948, 950, 952, 960, 963, 969, 973, 982, 986, 988, 993, 997, 1001, 3, 10, 20, 28, 36, 42, 46, 49, 55, 61, 69, 81, 84, 93, 99, 102, 107, 110, 120, 137, 144, 158, 163, 166, 172, 179, 183, 187, 209, 220, 226, 254, 258, 269, 277, 284, 287, 294, 305, 311, 316, 327, 332, 338, 345, 355, 368, 375, 382, 392, 400, 408, 414, 418, 421, 427, 433, 441, 453, 456, 465, 471, 474, 479, 482, 492, 509, 516, 530, 535, 538, 544, 551, 555, 559, 581, 592, 598, 626, 630, 641, 649, 656, 659, 666, 677, 683, 688, 699, 704, 710, 717, 727, 740, 747, 754, 764, 772, 780, 786, 790, 793, 799, 805, 813, 825, 828, 837, 843, 846, 851, 854, 864, 881, 888, 902, 907, 910, 916, 923, 927, 931, 953, 964, 970, 998, 1002, 11, 21, 37, 56, 70, 94, 103, 111, 121, 138, 180, 184, 188, 210, 221, 227, 259, 278, 312, 317, 328, 333, 339, 383, 393, 409, 428, 442, 466, 475, 483, 493, 510, 552, 556, 560, 582, 593, 599, 631, 650, 684, 689, 700, 705, 711, 755, 765, 781, 800, 814, 838, 847, 855, 865, 882, 924, 928, 932, 954, 965, 971, 1003, 12, 57, 71, 122, 139, 260, 313, 318, 329, 334, 340, 384, 429, 443, 494, 511, 632, 685, 690, 701, 706, 712, 756, 801, 815, 866, 883, 1004, 123, 140, 261, 319, 341, 495, 512, 633, 691, 713, 867, 884, 1005, 262, 342, 634, 714, 1006, 263, 635, 1007, 264, 636, 1008, 265, 637, 1009, 266, 638, 1010 |

# Conclusion

In this contribution, we investigate the CRC polynomial and interleaver design for NR control channel including DL with K+nFAR>=12, UL with 12<=K+nFAR<=22, and UL with K+nFAR>22, respectively.

Proposal 1: Adopt the following interleaver procedure for DCRC.

(In accordance to the TS38.212 draft specification, the notation of K' used in section 2.1, indicating the size of information bits plus CRC bits, is replaced by K in below)

- Extract interleaving pattern π' from pattern π by

k'=0;

for k=0: KΠ-1

if πk≥KΠ-K

π'(k')= πk-(KΠ-K);

k'=k'+1;

end if

end for

- Interleave c0,c1,c2,…,cK-1 by interleaving pattern π' to have the interleaved output bit stream c0',c1',c2',…,cK-1': ck'=cπ'(k).

Proposal 2: Adopt the CRC polynomial and interleaver in Table 1 for DL with K+nFAR>=12

Proposal 3: Adopt the CRC polynomial and interleaver in Table 2 for UL with 12<=K+nFAR<=22

Proposal 4: Adopt the CRC polynomial and interleaver in Table 3 for UL with K+nFAR>22

# References

- Chairman’s notes in 3GPP TSG RAN WG1 NR Ad-Hoc#2

- R1-1708833, “Design details of Distributed CRC”, Nokia, Alcatel-Lucent Shanghai Bell, 3GPP TSG RAN WG1 #89

- R1-1709997, “Early termination for Polar code”, Huawei, HiSilicon, 3GPP TSG RAN WG1 NR Ad-Hoc#2

- R1-1713738,” Evaluations of distributed CRC for Polar code construction”, Huawei, HiSilicon, 3GPP TSG RAN WG1 #90