3GPP TSG RAN WG1 #90	R1-1714531

# Prague, Czech, 21-25 Aug, 2017

Agenda item:	6.1.2.3.8

Source: 	Samsung

Title: 	Evaluations on pre-dft PT-RS

Document for:	Discussion and Decision

# Introduction

During RAN1 Ad-hoc #2, RAN1 captured the following working assumption regarding UL phase tracking RS for DFT-s-OFDM.

Agreements:

- For PT-RS insertion for UL DFT-S-OFDM

  - Companies are encouraged to perform simulations with realistic simulation assumptions comparing pre-DFT vs. post-DFT PT-RS insertion

    - For pre-DFT, companies are encouraged to compare chunk-based distribution vs. non-chunk based distribution

Agreements:

- Support at least full symbol-level time density for time-domain PT-RS for DFT-S-OFDM (every PUSCH carrying symbol)

  - FFS: whether to support configurable symbol-level time density for time-domain PT-RS density reduction for DFT-S-OFDM

    - Note: If supported, the configuration can be implicit (associated with scheduled MCS and/or BW and/or DM-RS port(s)/position) or explicit, which is to be decided in next meeting

This contribution considers UL pre-DFT PT-RS insertion for DFT-s-OFDM.

# Discussion on pre-DFT PT-RS insertion for DFT-s-OFDM

## Chunk based distribution vs. non-chunk based distribution

In previous meeting, it is agreed to compare chunk-based distribution with non-chunk based distribution for pre-DFT PT-RS insertion as shown Figure 1.

Figure 1. Example on non-chunk-based and chunk-based distribution in pre-DFT region

Figure 2 shows the BLER performance for pre-DFT PT-RS insertion with various chunk size and non-chunk based distribution. We use the 8RB or 32RB, 64QAM, 120kHz sub-carrier spacing, and TDLC channel. ‘Pre C2L2’ means 2 chunks with 2 localized PT-RS. We assume that all chunks are uniformly distributed in total DFT-s-OFDM samples. In 8RB case, the total number of PT-RS symbol is 4 in our assumption. So, ‘Pre C4L1’ means the non-chunk based distribution. We show that chunk based distribution and non-chunk based distribution have similar BLER performance. Also, the localized PT-RS insertion, ‘Pre C1L4’, has worse performance. Since pre-DFT PT-RS insertion is performed time-domain, the localized PT-RS insertion do not obtain the CPE for overall DFT-s-OFDM samples.

Observation 1: Chunk based distribution shows similar BLER performance to non-chunk based distribution

Figure 2. BLER performance comparison of chunk-based distribution and non-chunk-based distribution

Figure 3. Example on chunk-based extensively uniform distribution in pre-DFT region

# Conclusions

This contribution discusses on UL PT-RS in CP-OFDM and DFT-s-OFDM. The observations and proposals are as follows:

Observation 1: Chunk based distribution shows similar BLER performance to non-chunk based distribution

# References

- Chairman’s Note of 3GPP TSG RAN WG1 Ad-hoc #2