3GPP TSG RAN WG1 Meeting NR #90bis 			      R1-1717979

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	7.4.2.2

Source: 	LG Electronics

Title: 	Performance evaluation of DL channel interleaver

Document for:	Discussion and decision

- Introduction

The following working assumptions on channel interleavers for control channel have been achieved in the RAN1 #90 [1] and RAN1 NR AH3 [2].

Working Assumption [1]: - Channel interleaver: - Uplink: Triangular interleaver (e.g. as in R1-1713474) - Downlink: Parallel rectangular interleaver (e.g. as in R1-1714691) - To be confirmed at NR AH#3 unless it is shown that there are no meaningful benefits of including the downlink channel interleaver, using evaluation assumptions in R1-1714983

Working Assumption [2]: - If a DL bit-level channel interleaver is adopted: - Its span is equal to the number of coded bits corresponding to 1 CCE - The span can be increased to the number of coded bits corresponding to 2 CCEs if there is a benefit of doing so - FFS whether the interleaver is not used at higher ALs - Companies are encouraged to assess by RAN1#90bis the implementation impacts of using or not using the interleaver at higher ALs Conclusions and next steps to help towards a decision on the Working Assumption from RAN1#90: - From the cases evaluated so far, gains of DL channel interleaver are not significant for AL >2 - Continue evaluations until RAN1#90bis, according to the above working assumption - Focus on AL=1,2 cases, with and without REG bundle interleaver - Include evaluations with up to 3 OFDM symbols for the control channel - Companies are also encouraged to compare block parallel interleaver with low-complexity block interleavers, e.g. single block interleaver.

In this contribution, we evaluate the parallel rectangular interleaver in [3] under the evaluation assumptions in Appendix.

- Interleaver design for DL

In [3], after separating the rate matched coded bits into two, each separated bit passes through a rectangular interleaver with different depths. The output bits of each interleaver are interlaced to perform modulation mapping.

- Numerical results

In this section, we present the performance comparison between using and not using channel interleaver. The evaluation assumptions are given in the appendix. When the aggregation level is equal to 1, there are meaningful gains of including the parallel rectangular interleaver compared to the case of not using it in figure 1 to figure 4.

Observation 1: When CCE aggregation level is 1 and bundle size is 2, channel interleaver can provide ~1 dB @ 1% BLER and ~1.5 dB @ 0.1% BLER.

Figure 1. Performance of channel interleaver (AL=1, REG bundle size=2, Payload=32, list size=1)

Figure 2. Performance of channel interleaver (AL=1, REG bundle size=2, Payload=32, list size=8)

Figure 3. Performance of channel interleaver (AL=1, REG bundle size=2, Payload=60, list size=1)

Figure 4. Performance of channel interleaver (AL=1, REG bundle size=2, Payload=60, list size=8)

Proposal 1: Apply the channel interleaver in working assumption from RAN1#90 to DL control channel with aggregation level 1.

- Conclusion

In this contribution, , we evaluated the channel interleaver design for downlink control. The following are provided:

Observation 1: When CCE aggregation level is 1 and bundle size is 2, channel bit interleaver can provide ~1 dB @ 1% BLER and ~1.5 dB @ 0.1% BLER.

Proposal 1: Apply the channel interleaver in working assumption from RAN1#90 to DL control channel with aggregation level 1.

- Reference

RAN1 chairman’s notes, RAN1 #90, August 2017

RAN1 chairman’s notes, RAN1 NR Ad-Hoc#3, September 2017.

R1-1714691, “Channel Interleaver for Polar Codes”, Ericsson

R1-1714983, “Proposed Evaluation assumptions for Polar Channel Interleaver for DL”, Intel Corporation, Fujitsu Inc.

Appendix. Evaluation assumptions [4]

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| Waveform | OFDMA |
| Numerology | 15 kHz |
| Payload (not including CRC) | 32, 60 bits |
| FEC type and Modulation | Polar with CRC size =24, QPSK CRC polynomial [1]: gCRC24(D) = [D24+D23+D21+D20+D17+D15+D13+D12+D8+D4+D2+D+1] |
| Tx-Rx antenna configuration | 2x2 |
| Transmit diversity scheme | 1-port per REGB precoder cycling |
| Channel estimation | 1/3 1/4 DM-RS density, practical channel estimation (MMSE) |
| Channel model | TDL-C 300ns |
| Number of REGs per CCE | 6 |
| Aggregation levels | 1, 2 |
| REG bundle size | 2 REGs, 6 REGs |
| CORESET configuration | 1 symbol (up to 3 symbols), 48 PRBs (i.e. PRB0,PRB1…PRB47) |
| CCE-to-REG mapping | Frequency first |
| Interleaving for CCE-to-REG mapping | For evaluation only, Sub-block interleaver operating on REG bundles |