3GPP TSG-RAN WG1#91	R1-1719366

Reno, Nevada, USA, 27th November – 1st December 2017

Source:	Ericsson

Title:	Narrowband measurement accuracy improvements for NB-IoT

Agenda Item:	6.2.6.4

Document for:	Discussion and Decision

# Introduction

The objective of the work item on further enhanced NB-IoT is to introduce further support of enhanced features for NB-IoT, supporting standalone, guard-band, and in-band operation modes. One objective is to study, and if found necessary improve, NPRACH reliability and range enhancements as listed below.

Narrowband measurement accuracy improvements [RAN1, RAN4, RAN2]

- Evaluate and if appropriate specify use of additional existing signals than NRS for RRM measurements, with associated RAN4 core requirements taking into account e.g. UE complexity, power consumption, system capacity.

In RAN1#89, the following agreements were made with regards to measurement accuracy improvements:

- For connected mode in the serving cell and idle mode in camped cell and neighbour cells, it is feasible from RAN1 point of view to use NSSS on an anchor carrier for RRM measurement

  - RAN4 are requested to consider if NSSS is a suitable transmission

  - How to use, i.e. if and/or under what circumstances to combine NRS with NSSS depend on RAN4 study

  - Send an LS containing the above to RAN4, including the agreement from RAN1#88bis that “At least NSSS is used additionally to NRS for in-band, guard-band, standalone”

- Continue to study

  - CRS for in-band operation mode

  - NPBCH, NPDCCH, NPDSCH on an anchor carrier in the serving cell

In this contribution, we present our views on the suitability to use CRS for measurements and on the gains from and ability to improve connected mode measurements using the NPBCH, NPDCCH and NPDSCH. This paper is a resubmission of R1-1717015.

# Discussion

## On CRS based measurements

In the basic design of NB-IoT Release 13 it was decided to base radio resource management (RRM) measurement on NRS transmissions. In cased of idle mode, this was, in our opinion, an unfortunate decision since the NB-IoT downlink is already carrying a large overhead in terms of NPSS, NSSS and NPBCH transmissions dimensioned to support idle mode operations even in extended coverage. These transmission were already in Release 13 valuable candidates for supporting RRM.

Hence, we would not like to introduce new mandatory idle mode measurement signal in the NB-IoT downlink, as this would further increase the requirement on the base station transmitter to be on with negative implications in terms of base station power consumption and downlink interference levels. It is also unwanted to create strong couplings between NB-IoT and LTE as NB-IoT in a few years’ time may be operating within a NR carrier and not within a LTE carrier.

- In case of inband mode with same PCI set in LTE and NB-IoT, there is already a requirement on the base station transmitter to deliver the CRS. In this specific case CRS may be used, if proven beneficial, to measure NRSRP/NRSRQ for connected mode procedures. However, when there are 4 TX antenna ports at the eNB used for LTE, the CRS cannot be used by the NB-IoT UE. Therefore, there are lots of restrictions to use CRS for idle mode RRM measurement.

- CRS is not a candidate for improving NB-IoT idle mode RRM measurements.

## On the use of NPBCH, NPDCCH and NPDSCH for improving connected mode measurements

### Connected mode power control tolerance

For connected mode improved measurement accuracy can be expected to positively influence e.g. the power control accuracy. TS 36.101 section 6.3.5.1 specifies an absolute power tolerance of ± 9 dB (under normal conditions), which sets the initial accuracy by which a device configures its output power. In addition, one needs to consider the aggregated power control tolerance specified in TS 36.101 section 6.3.5F.3 which specifies the ability of the device to maintain the initial selected power level during a NPUSCH transmission.  The aggregated power control tolerance is set to ± 3.5 dB. This implies that a device may have a power inaccuracy spanning ± 12.5 dB under normal conditions.

Such high inaccuracy has several drawbacks e.g. in terms of unpredictable link level performance, loss of orthogonality between subcarriers and reduced device battery life. As the allowed inaccuracy to a large extent can be related to the NRSRP measurement inaccuracy, an improved measurement accuracy will allow a tightened power control tolerance.

#### Impact on subcarrier orthogonality

The orthogonality between NB-IoT subcarriers are in theory determined by the transmitter Fourier transformation but are in practice set by the inband spectrum emission requirements as specified in TS 36.101 section 6.5.2F.3.

Consider two devices which are located next to each other and experiencing the same received NRSRP, e.g. the same coupling loss. Ideally their uplink transmission should use the same power, resulting in both transmissions reaching the base station at the same receive power level. However, in the extreme case, if the first UE underestimates the NRSRP with 12.5 dB while the second UE overestimates the NRSRP with 12.5 dB, then the system need to cope with a power difference of 25 dB between the two devices. Figure 1 illustrates this situation for two devices assigned NPUSCH transmissions on subcarriers 4 and 5. Assuming 0 dB is the ideal configured power, the two UEs are 12.5 dB above and 12.5 dB below the ideal level. The NB-IoT inband emission requirements require an UE to supress its emission with 18 dBc. The figure clearly shows that this is far from sufficient, as the second UE is 7 dB below the inband emission level allowed for the first UE. It should be noted that the inband emissions shown for UE1 and UE2 are only examples used in this illustration, and not examples of expected emission levels seen in a real UE implementation.

Notice that only open-loop power control is not used by NB-IoT. Therefore, there are only limited tools for the eNB to limit the powers from the UEs, which are not sufficient is this case. Based on this it is clear that an improved NRSRP and power control accuracy will improve the orthogonality between adjacent subcarriers and result in an overall more robust system design.

- Improved connected mode NRSRP and power control accuracy will improve the orthogonality between adjacent subcarriers and result in an overall more robust system design.

To secure that UE2 is never below the inband emission levels of UE1 times the sum of the absolute power tolerance and the aggregated power control tolerance should not be allowed to exceed 9 dB.

- RAN1 should strive to limit the sum of the absolute power tolerance and the aggregated power control tolerance to ±9 dB.

Figure 1 Illustration of impact on subcarrier orthogonality due to power control inaccuracy of ± 12.5 dB.

#### Impact on UE power consumption

It can be expected that an inaccurate power control will lead to a suboptimal power consumption. Since NB-IoT is a new technology limited/no public material describing the power consumption profile for a range of output power levels exists. For LTE, there are however published articles in this area that can be used to exemplify the negative impact on UE power consumption from using an incorrect power level. In [1] a model to characterize the average power consumption of a 23 dBm smart phone transmitting at 800 MHz is described as:

PINPTX=4.8PTX+1600 mW,  for PTX≤12 dBm68PTX+790 mW,  for PTX>12 dBm

Figure 2 Smart phone power consumption model [1].

Based on this simple model the excessive power consumption for a given ideal power level PIDEAL and a power configuration error PERR can be calculated as:

PEX=PINmin⁡(23, PIDEAL+PERR)-PIN(PIDEAL)PIN(PIDEAL)

Figure 3 gives the excessive power consumption for the worst case power error of +12.5 dB which gives a peak excessive power consumption of ~42% at an ideal output power of 11 dB which results in an configured actual output power of 23 dBm due to the large error of +12.5 dB.

Figure 3 Excessive power consumption due to an power error of 12.5 dB [1].

It is clear to this is a very simple model that only applies in worst case for a smart phone device, but it serves its purpose to illustrate that incorrectly configured output power levels may cause waste of battery power.

- Improved connected mode NRSRP and power control accuracy will improve the UE power consumption characteristics.

### Connected mode measurements

In connected mode, a UE is currently required to base NRSRP on NRS transmissions. In our view it is reasonable to also consider making general use of the NPDCCH for improving the measurements. This since an uplink transmission per definition is scheduled by a NPDCCH, so a device should be able to decode the NPDCCH and measure its power at the same time.

For a device configured on the anchor carrier the NSSS and NPBCH transmissions may in addition to the NPDCCH be used for connected mode measurements.

The NRS is at most transmitted on 8 resource elements per antenna port, i.e. at most on 16 out of the 168 resource elements (REs) in a subframe. This can be compared to the NPBCH which is transmitted on 100 REs and the NPDCCH that is transmitted on between 100 and 160 REs depending on the mode of operation and the NRS/CRS antenna port configurations. The power carried by the NPBCH and NPDCCH is hence in the order of 8 to 13 dB higher than what is carried by the NRSs. The densification offered by the NPBCH and NPDCCH should have the ability to improve the reliability of NRSRP measurements.

The use of the NPDSCH for measuring NRSRP is somewhat limited due to NB-IoT not supporting simultaneous uplink and downlink data transmission. It is therefore proposed to focus on improving connected mode anchor and non-anchor RRM measurements based on NPDCCH and NPBCH transmission.

- RAN1 should consider connected mode anchor and non-anchor RRM measurements based on NPDCCH and NPBCH transmission.

# Conclusions

In this contribution, we have discussed the support for NPRACH false alarm reduction. We made the following observations and proposals:

- CRS is not a candidate for improving NB-IoT idle mode RRM measurements.

- RAN1 should strive to limit the sum of the absolute power tolerance and the aggregated power control tolerance to ±9 dB.

- RAN1 should consider connected mode anchor and non-anchor RRM measurements based on NPDCCH and NPBCH transmission.

- Improved connected mode NRSRP and power control accuracy will improve the orthogonality between adjacent subcarriers and result in an overall more robust system design.

- Improved connected mode NRSRP and power control accuracy will improve the UE power consumption characteristics.

# References

An accurate measurement-based power consumption model for LTE uplink transmissions, IEEE Infocom 2013 Demo/Poster session, Dusza et al

R1-1706890, Narrowband measurement accuracy improvements for NB-IoT, RAN1#89, Ericsson

R4-78AH-0129, “Further RRM measurements simulation results in stationary scenario”, RAN4#78-NB-IoT, source Ericsson