3GPP TSG RAN WG1 Meeting 91		R1-1719789

Reno, USA, November 27th – December 1st 2017

Source:	vivo

Title:	Design of long-PUCCH for UCI of more than 2 bits

Agenda Item:	7.3.2.2.2

Document for:	Discussion and Decision

- Introduction

In the previous meetings, long-PUCCH format with more than 2 bits were discussed and some agreements were achieved [1] [2].

For DM-RS aspect

Agreements:

For long PUCCH for UCI of more than 2 bits, if frequency hopping is disabled,

  - The number of DMRS symbols is  1 for long PUCCH duration <= X

  - The number of DMRS symbols is 2 for long PUCCH duration >X  and <= 2X+1

  - The number of DMRS symbols (either 2 or 4 symbols)  for the long PUCCH with a duration  >2X+1  is configurable by RRC UE-specific signaling

    - RRC Bit-width is 1 bit (reuse the same RRC for long PUCCH with enabled frequency hopping)

    - X is fixed in the spec w/o RRC impact and is the same as specified for long PUCCH with enabled frequency hopping

      - Note: As agreed, if frequency hopping is enabled, the number of DMRS symbols (between 1-2 symbol) per hop for long PUCCH with duration per hop > X symbols is configurable by RRC UE-specific signaling

Agreements:

For long PUCCH for UCI of more than 2 bits

  - For a long PUCCH with duration > X,  the DMRS location within the long PUCCH with frequency hopping disabled, is the same as the DMRS location within long PUCCH (of the same duration) with frequency hopping enabled.

  - For a long PUCCH with duration <=X,  the DMRS location within the long PUCCH with frequency hopping disabled is located in (near) the middle of the long PUCCH.

    - FFS exact locations

  - For a long PUCCH with frequency hopping enabled,

    - If there is 1 DMRS symbol in a hop, the DMRS symbol is located in (near) the middle of the hop.

      - FFS exact locations

    - If there is 2 DMRS symbols in a hop, the first DMRS is in the second symbol of the hop and the second DMRS symbol is in the penultimate symbol of the hop

Agreements:

- For long PUCCH for UCI of more than 2 bits

  - If frequency hopping is enabled, the number of DMRS symbols (between 1-2 symbol) per hop for long PUCCH duration per hop > X is configurable by RRC UE-specifically

    - RRC Bit-width is 1 bit

    - X is fixed in the spec w/o RRC impact

    - FFS the value X

  - The location of DMRS is pre-defined within long PUCCH in each hop based on number of DMRS per hop and duration of the hop.

Agreements:

For long PUCCH for UCI of more than 2 bits with enabled frequency hopping,

  - the number of DMRS symbols per hop is 1 for long PUCCH with duration of at least one hop <= X symbols

  - the number of DMRS symbols (between 1-2 symbol) per hop for long PUCCH with duration of both hops > X symbols is configurable by RRC UE-specific signalling

    - Down select between X=4 and 5

For PUCCH with moderate payload size

Agreements:

- Support Pre-DFT-OCC as the UCI structure for long PUCCH for UCI of more than 2 bits with moderate payload

  - FFS:  DMRS structure between CDM and IFDM

    - Considering the impact on channel estimation and power imbalance among UEs

  - It will be denoted as a new format

- Support multiplexing capacity of 2 and 4 users for long PUCCH for UCI of more than 2 bits with moderate payload using one PRB in Rel-15

  - FFS design of OCC

    - No RRC signalling is necessary

- DMRS for Long PUCCH with more than 2 bits and without multiplexing

For a PUCCH format for UCI with large payload, no multiplexing capacity is required. A LTE PUSCH-like structure can be considered for such design, e.g., LTE PUCCH format 4. In this section, we evaluate the performance for the different number of DMRS symbols with and without frequency hopping.

  - Simulation assumptions

In order to determine the DMRS density per each frequency hop, the following aspects are considered in the simulation,

- With Frequency hopping and without frequency hopping

- In case frequency hopping is disabled, the DMRS are equally distributed in the first half and second half of PUCCH.

- Different payload size and UE speed

- 20-bit, 60-bit and 100-bit payloads are considered. For high UE speed, e.g. 120-km/h, the UCI payload with 100-bit may not be necessary and only 20-bit and 60-bit are considered.

- Length of the DMRS

- For simplicity, only 8/10/12/14 symbols length PUCCH are considered.

- BLER target

- The UCI BLER target for simulation is set to 1%, which is aligned with TS36.101

Other simulation assumptions are included in Annex A.

  - Performance of different DMRS configurations

With frequency hopping

Table 1-3 provide the required SNR at 1% BLER for different DMRS configurations.

Table 1 Required SNR for 1% BLER with 20bits UCI with frequency hopping

| Required SNR [dB] |  | 8 symbols | 10 symbols | 12 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
| 3km 300ns | 1RS/hop | -4.8279 | -5.2889 | -6.0081 | -6.2292 |
|  | 2RS/hop | -2.0908 | -4.2725 | -6.1419 | -7.5752 |
| 120km 300ns | 1RS/hop | -1.2029 | -2.9456 | -2.6116 | -3.7404 |
|  | 2RS/hop | 4.6018 | -2.0391 | -2.971 | -3.8706 |
| 3km 1000ns | 1RS/hop | -3.2598 | -4.165 | -4.9252 | -6.0373 |
|  | 2RS/hop | 4.2877 | -3.0576 | -4.6598 | -6.0586 |
| 120km 1000ns | 1RS/hop | -0.2471 | -1.1513 | -1.4345 | -2.4253 |
|  | 2RS/hop | 5.8663 | 0.5768 | -2.2176 | -2.4345 |

For UCI payload size with 20bits, 1 RS per frequency hop shows the better performance for long PUCCH format with less than 12 symbols. 2 RS per frequency hop shows the better performance for long PUCCH format with 12-14 symbols.

Observation 1: when FH is enabled and UCI payload size is 20-bit,

- When PUCCH duration is less than 12 symbols, one-strip DMRS is always better than two-strip DM-RS.

- When PUCCH duration is not less than 12 symbols, when DS us large, two-strip DMRS is better for payload 20-bit.

Table 2  The required SNR for 1% BLER with 60bits UCI with frequency hopping

| Required SNR [dB] |  | 8 symbols | 10 symbols | 12 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
| 3km 300ns | 1RS/hop | 2.8609 | 0.4261 | -0.4396 | -1.8358 |
|  | 2RS/hop | 5.7972 | 2.6252 | -0.7909 | -1.4982 |
| 120km 300ns | 1RS/hop | 10.0845 | 4.5858 | 6.8438 | 10.2209 |
|  | 2RS/hop | - | - | 6.7795 | 3.9795 |
| 3km 1000ns | 1RS/hop | 9.9163 | 5.3451 | 2.4229 | -0.3993 |
|  | 2RS/hop | - | - | 3.6628 | 1.2726 |
| 120km 1000ns | 1RS/hop | - | - | 8.5703 | 9.7974 |
|  | 2RS/hop | - | - | 9.9397 | 8.8464 |

For UCI payload size with 60-bit, 1 RS per frequency hop shows the better performance when DS is lager. However, for medium DS, similar performance is observed compared to payload 20-bit.

Observation 2: when FH is enabled and UCI payload size is 60-bit,

- When PUCCH duration is less than 12 symbols, one-strip DMRS is always better than two-strip DM-RS.

- When PUCCH duration is not less than 12 symbols, when DS us large, two-strip DMRS is better for payload 60-bit.

Table 3  The required SNR for 1% BLER with 100bits UCI with frequency hopping

| Required SNR [dB] |  | 8 symbols | 10 symbols | 12 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
| 1-RS/hop 300ns | 3km/h | 6.9832 | 4.4239 | 2.8288 | 1.138 |
| 2-RS/hop 300ns | 3km/h | - | 6.8323 | 4.276 | 1.5606 |
| 1-RS/hop 1000ns | 3km/h | - | - | 8.7728 | 7.3326 |
| 2-RS/hop 1000ns | 3km/h | - | - | 12.079 | 8.8962 |

For UCI payload size with 100-bit, UCI without frequency hopping does not provide sufficient performance gain. And for all cases, 1 RS per frequency hop shows the better performance.

Observation 3: one DMRS strip for each frequency hop provides better performance for payload 100-bit,

For 1 DMRS per frequency hop, DMRS can be located around the middle of the frequency hop. For 2 DMRS symbols per frequency hop, the almost symmetrical location can be considered for DMRS.

NR PUCCH supports flexible number of symbols from 4 to 14 symbols. It is not efficient to fix the DMRS location relative to the slot for different long PUCCH length, since no UE multiplexing is required. Alternatively, fixed DMRS location relative to the PUCCH can be considered.

To sum up, the following proposals is made by considering the agreements made in the last meeting, which actually means X=5.

Proposal 1: For long PUCCH for UCI of more than 2 bits with enabled frequency hopping,

  - the number of DMRS symbols per hop is 1 for long PUCCH with duration of at least one hop <= 5 symbols

  - the number of DMRS symbols (between 1-2 symbol) per hop for long PUCCH with duration of both hops > 5 symbols is configurable by RRC UE-specific signalling

Without frequency hopping

Table 4 provides the PUCCH performance without frequency hopping. It can be seen that compared to PUCCH with frequency hopping, the optimal X value varies according to the different DS. But the differences are relative small. It can be observed,

Observation 4: when FH is disabled and UCI payload size is 20-bit,

- When PUCCH duration is less than 12 symbols, 2-strip DMRS is always better than 4-strip DM-RS.

- When PUCCH duration is not less than 12 symbols, depending on the DS, 2-strip may be better than 4-strip if DS is small or may be worse than 4-strip if DS is large.

Table 4 20-bit, 3km/h, w/o FH

| Required SNR [dB] |  | 8symbols | 10 symbols | 12 symbols | 14symbols |
| --- | --- | --- | --- | --- | --- |
| 3km 300ns w/o FH | 4 RS | -1.6878 | -3.5033 | -4.8729 | -5.4243 |
|  | 2 RS | -2.9230 | -4.3086 | -5.0681 | -5.5831 |
| 3km 1000ns w/o FH | 4 RS | 2.4180 | -0.7038 | -2.3059 | -3.2431 |
|  | 2 RS | 0.0074 | -1.4415 | -2.2307 | -3.0114 |

In summary, considering both large payload and small payload, large DS and small DS, with and without hopping, It is proposed that X = 5 as the same for frequency hopping case.

Proposal 2: For long PUCCH for UCI of more than 2 bits, if frequency hopping is disabled,

  - The number of DMRS symbols is  1 for long PUCCH duration <= 5

  - The number of DMRS symbols is 2 for long PUCCH duration >6  and <= 11

  - The number of DMRS symbols (either 2 or 4 symbols)  for the long PUCCH with a duration  >11  is configurable by RRC UE-specific signaling

    - RRC Bit-width is 1 bit (reuse the same RRC for long PUCCH with enabled frequency hopping)

- PUCCH structure for supporting multiplexing of users

In RAN1 NR Adhoc#3, it is agreed that for the format of long PUCCH supporting multiplexing of users, target to select one from:

- Alt.2: User multiplexing is realized by pre-DFT-OCC.

  - Similar to LTE PUCCH format 5, Alt 2 applies orthogonal code before DFT operation. [1 1] and [1 -1] is used for two users as orthogonal code.

- Alt.3: User multiplexing is realized by FDM within the PRB.

  - IFDMA structure with comb size=2 is used for DMRS and UCI parts in the simulation.

Some contribution [4] has proved that for UCI part, Pre-DFT-OCC and IFDMA are equivalent. Therefore in RAN1#90bis, it is agreed that

- Support Pre-DFT-OCC as the UCI structure for long PUCCH for UCI of more than 2 bits with moderate payload

  - FFS:  DMRS structure between CDM and IFDM

    - Considering the impact on channel estimation and power imbalance among UEs

  - It will be denoted as a new format

The rest of this section further compares the differences between different DMRS options, i.e., CDM and IFDM.

For alt 2, it is important to model the power imbalance between two users. 2dB and 6dB are considered respectively in order to model power imbalance introduced by non-ideal power control. For alt 3, because different users are multiplexed in different subcarriers, the power imbalance may not have the big impact. The timing differences between two users are not modeled for these alternatives considering the cell radius is not large. For 10-bit payload, RM code is used, while for 40-bit payload, polar code with 8-bit CRC is used. Simulation results are listed Annex D.

Figure 1 BLER vs SNR for long PUCCH format. 2-users, 10-bits, TDL-C, 300ns/1000ns, 3km/h

For 3km/h case, it can be observed that when the DS is 300ns, alt 2 and alt 3 have the similar performance. Even if power imbalance is considered, the performance degradation is not significant. However, when the DS is 1000ns, alt 2 (pre-DFT OCC) are sensitive to power imbalance, alt 3(IFDMA) is not sensitive to power imbalance due to the orthogonality in frequency domain.

Observation 5: pre-DFT OCC with CDM DMRS is sensitive to power imbalance.

Figure 2 BLER vs SNR for long PUCCH format. 2-users, 40-bits, TDL-C, 300ns, 3km/h

For both 40-bit and 10-bit UCI payload, alt 3 (IFDMA) and alt 2 (pre-DFT OCC) have the similar performance when DS with 300ns, even when power imbalance is considered. And for 10-bit UCI payload with 3km/h, it can be observed that when the DS is 300ns, alt 2 and alt 3 also have the similar performance, even power imbalance is considered.

Observation 6: For small/medium DS and low UE speed, CDM and IFDM DMRS have the similar performance.

Table 5  The required SNR for 1% BLER with 10bits UCI

|  |  | Alt 2: PreDFT |  | Alt 3: IFDMA |
| --- | --- | --- | --- | --- |
|  |  | 2dB | 6dB |  |
| 1000ns | 3km/h | -0.8672 | 5.4583 | -0.1866 |
|  | 120km/h | 1.5368 | 3.9517 | 3.3205 |
| 300ns | 3km/h | -3.7843 | -3.1709 | -4.0171 |
|  | 120km/h | -0.3335 | 0.2076 | -0.6900 |

Table 6 The required SNR for 1% BLER with 40bits UCI

|  |  | Alt 2: Pre-DFT |  | Alt 3: IFDMA |
| --- | --- | --- | --- | --- |
|  |  | 2db | 6db |  |
| 1000ns | 3km/h | 4.2799 | 10.1877 | 8.3106 |
| 300ns | 3km/h | 0.7042 | 1.0827 | 1.1268 |

Considering the link performance of alt 2 and alt 3, other realistic factors, e.g. power imbalance, it is proposed to adopt alt 3 as the NR long PUCCH structure with multiplexing capacity.

For SRS, the position can be around the last symbol(s) of a slot. Based on the comb structure, long PUCCH format with SRS can be easily multiplexed with less impact due to imbalanced power among UEs. Thus, the multiplexing between long PUCCH format and SRS based on IFDMA is suggested in NR.

Proposal 3: IFDMA DMRS is considered for the NR long PUCCH structure with multiplexing capacity.

Proposal 4: The multiplexing between long PUCCH format and SRS based on IFDMA is supported in NR.

- Conclusion

In this document, we discuss long PUCCH format with more than 2 bits with the following observations and proposals:

For DMRS evaluation, it is observed and proposed as follows,

Observation 1: when FH is enabled and UCI payload size is 20-bit,

- When PUCCH duration is less than 12 symbols, one-strip DMRS is always better than two-strip DM-RS.

- When PUCCH duration is not less than 12 symbols, when DS us large, two-strip DMRS is better for payload 20-bit.

Observation 2: when FH is enabled and UCI payload size is 60-bit,

- When PUCCH duration is less than 12 symbols, one-strip DMRS is always better than two-strip DM-RS.

- When PUCCH duration is not less than 12 symbols, when DS us large, two-strip DMRS is better for payload 60-bit.

Observation 3: one DMRS strip for each frequency hop provides better performance for payload 100-bit,

Observation 4: when FH is disabled and UCI payload size is 20-bit,

- When PUCCH duration is less than 12 symbols, 2-strip DMRS is always better than 4-strip DM-RS.

- When PUCCH duration is not less than 12 symbols, depending on the DS, 2-strip may be better than 4-strip if DS is small or may be worse than 4-strip if DS is large.

Proposal 1: For long PUCCH for UCI of more than 2 bits with enabled frequency hopping,

  - the number of DMRS symbols per hop is 1 for long PUCCH with duration of at least one hop <= 5 symbols

  - the number of DMRS symbols (between 1-2 symbol) per hop for long PUCCH with duration of both hops > 5 symbols is configurable by RRC UE-specific signalling

Proposal 2: For long PUCCH for UCI of more than 2 bits, if frequency hopping is disabled,

  - The number of DMRS symbols is  1 for long PUCCH duration <= 5

  - The number of DMRS symbols is 2 for long PUCCH duration >6  and <= 11

  - The number of DMRS symbols (either 2 or 4 symbols)  for the long PUCCH with a duration  >11  is configurable by RRC UE-specific signaling

    - RRC Bit-width is 1 bit (reuse the same RRC for long PUCCH with enabled frequency hopping)

For PUCCH structure for supporting multiplexing of user

Observation 5: pre-DFT OCC with CDM DMRS is sensitive to power imbalance.

Observation 6: For small/medium DS and low UE speed, CDM and IFDM DMRS have the similar performance.

Proposal 3: IFDMA DMRS is considered for the NR long PUCCH structure with multiplexing capacity.

Proposal 4: The multiplexing between long PUCCH format and SRS based on IFDMA is supported in NR.

References

- Chairman's Notes RAN1# AH3.

- Chairman's Notes RAN1#90bis.

- R1-1717493	, Design of long-PUCCH for UCI of more than 2 bits	vivo

- R1-1718805	, “Long PUCCH design with more than 2 bits UCI payload” Qualcomm Incorporated

Appendix A : simulation assumpitions for DMRS

Table A-1 : Simulation parameters for DMRS

| System bandwidth | 20Mhz |
| --- | --- |
| Subcarrier spacing | {15kHz } |
| Channel modeling | TDL-C channel delay spread ={300nS, 1000nS} |
| Antenna modeling | # UE Tx =1, # gNB Rx =2 |
| Channel coding | Payload sizes without CRC: 20 bits, 60 bits, 100 bits |
| CRC | Polar code with 8CRC |
| Number of RB | {1 RB} |
| Carrier frequency | 4Ghz |
| Number of UEs | {1} |
| UE speed: | 3km/h, 120km/h, 500km/h |
| channel estimation | Practical channel estimation and ideal noise estimation |

Table A-1 : DMRS structure (‘R’ for RS and ‘U’ for UCI)

|  | 1RS | 2RS |
| --- | --- | --- |
| 8 symbols | URUU URUU | RURU RURU |
| 10symbols | UURUU UURUU | URURU URURU |
| 12 symbols | UURUUU UURUUU | URUURU URUURU |
| 14 symbols | UUURUUU UUURUUU | URUUURU URUUURU |

Appendix B : Simulation assumptions for PUCCH with multiplexing

Table B-1 : Simulation parameters for DMRS

| System bandwidth | 20Mhz |
| --- | --- |
| Subcarrier spacing | {15kHz } |
| Channel modeling | TDL-C channel delay spread ={300nS, 1000nS} |
| Antenna modeling | # UE Tx =1, # gNB Rx =2 |
| Channel coding | For 10-bits, RM code is used. For 40-bits, polar code is used |
| CRC | For 40-bit, polar code with 8-bit CRC is used |
| Number of RB | {1 RB} |
| Carrier frequency | 4Ghz |
| Number of UEs | {2} |
| UE speed: | 3km/h, 120km/h |
| channel estimation | Practical channel estimation and ideal noise estimation |
| Power imbalance | 2dB and 6dB |

Appendix C : DMRS simulation results

Figure C-1 BLER vs SNR for long PUCCH format w/ FH and w/o FH. TDL-C channel model with 300ns delay spread at 3 km/h. Payload size is 20bits with 2 DMRS in PUCCH.

Figure C-2 BLER vs SNR for long PUCCH format w/ FH and w/o FH .TDL-C channel model with 1000ns delay spread at 3 km/h. Payload size is 20bits with 2 DMRS in PUCCH.

Figure C-3 BLER vs SNR for long PUCCH format w/ FH and w/o FH. TDL-C channel model with 300ns delay spread at 3 km/h. Payload size is 60bits with 2 DMRS in PUCCH.

Figure C-4 BLER vs SNR for long PUCCH format w/ FH and w/o FH .TDL-C channel model with 1000ns delay spread at 3 km/h. Payload size is 60bits with 2 DMRS in PUCCH.

Figure C-5 BLER vs SNR for long PUCCH format w/ FH and w/o FH .TDL-C channel model with 300ns delay spread at 3 km/h. Payload size is 100bits with 2 DMRS in PUCCH.

Figure C-6 BLER vs SNR for long PUCCH format w/ FH and w/o FH. TDL-C channel model with 1000ns delay spread at 3 km/h. Payload size is 100bits with 2 DMRS in PUCCH.

Figure C-7 BLER vs SNR for long PUCCH format w/ FH and w/o FH. TDL-C channel model with 300ns delay spread at 120 km/h. Payload size is 20bits with 2 DMRS in PUCCH.

Figure C-8 BLER vs SNR for long PUCCH format w/ FH and w/o FH. TDL-C channel model with 1000ns delay spread at 120 km/h. Payload size is 20bits with 2 DMRS in PUCCH.

Figure C-9 BLER vs SNR for long PUCCH format w/ FH and w/o FH. TDL-C channel model with 300ns delay spread at 120 km/h. Payload size is 60bits with 2 DMRS in PUCCH.

Figure C-10 BLER vs SNR for long PUCCH format w/ FH and w/o FH. TDL-C channel model with 1000ns delay spread at 120 km/h. Payload size is 60bits with 2 DMRS in PUCCH.

Figure C-11 BLER vs SNR for long PUCCH format w/ FH. TDL-C channel model with 300ns delay spread at 120 km/h. Payload size is 100bits with 2 DMRS in PUCCH.

Figure C-12 BLER vs SNR for long PUCCH format w/ FH and w/o FH. TDL-C channel model with 1000ns delay spread at 120 km/h. Payload size is 100bits with 2 DMRS in PUCCH.

Figure C-13 BLER vs SNR for long PUCCH format w/ FH. TDL-C channel model with 300ns and 1000ns delay spread at 3 km/h. Payload size is 20bits with 4 DMRS in PUCCH.

Figure C-14 BLER vs SNR for long PUCCH format w/ FH. TDL-C channel model with 300ns and 1000ns delay spread at 3 km/h. Payload size is 60bits with 4 DMRS in PUCCH.

Figure C-15 BLER vs SNR for long PUCCH format w/ FH. TDL-C channel model with 300ns and 1000ns delay spread at 3 km/h. Payload size is 100bits with 4 DMRS in PUCCH.

Figure C-16 BLER vs SNR for long PUCCH format w/ FH. TDL-C channel model with 300ns and 1000ns delay spread at 120 km/h. Payload size is 20bits with 4 DMRS in PUCCH.

Figure C-17 BLER vs SNR for long PUCCH format w/ FH. TDL-C channel model with 300ns and 1000ns delay spread at 120 km/h. Payload size is 60bits with 4 DMRS in PUCCH.

Figure C-18 BLER vs SNR for long PUCCH format w/ FH. TDL-C channel model with 300ns and 1000ns delay spread at 120 km/h. Payload size is 100bits with 4 DMRS in PUCCH.

Appendix D : simulation results for PUCCH with multiplexing