3GPP TSG RAN WG1 Meeting #91		                                                      R1-1720162

Reno, USA, November 27th – December 1st, 2017

Source:	CATT

Title:	Evaluations for Tx diversity schemes in PC5

Agenda Item:	6.2.3.3.2

Document for:	Discussion and Decision

Introduction

In RAN1 #90 meeting, the potential Tx diversity schemes for PC5 are discussed and evaluated, particularly with following achieved working assumption [1].

Working Assumption (may be revisited based on RAN4 response):

- For designing PSSCH, RAN1 assumes the use of two-port non-transparent transmit diversity

  - The use of non-transparent transmit diversity is configured

  - Details, including diversity scheme, are FFS

  - Support of transmission and/or reception up to UE capability

    - Note: It is RAN1 understanding that requirements on capabilities can be set at regional level and are outside 3GPP scope

  - Send LS to RAN4 to ask their opinion about when non-transparent scheme for transmit diversity is used by Rel-15 UEs:

    - Impact on Rel-14 UEs of PSSCH-RSRP measurement accuracy

    - MPR for Rel-15 UEs

- Non-transparent Transmit diversity is not used in the following cases:

  - When communicating with Rel-14 UEs

  - When there is a high probability of resource collision with Rel-14 UEs

- Note: Some companies observe that the performance of MMSE-IRC receiver degrades when a non-transparent Transmit diversity scheme is used in interference limited scenarios with a dominant interferer

Furthermore, RAN1 has the following list of candidate two-port non-transparent diversity schemes [2]:

- STBC (including half symbol STBC proposal in R1-1705002)

- SFBC

- PVS in time domain

Note: other schemes are not precluded

In RAN1 #90bis meeting, the following agreement is achieved:

Agreement:

- For PSCCH, small delay CDD can be used on PSCCH

  - FFS whether the cyclic delay value is specified or left for UE implementation

In this contribution, we will provide the evaluation results about DMRS design, and the evaluation results on the candidate Tx diversity schemes based on MMSE-IRC receiver in interference limited scenario.

This is a revision of R1-1717793 [3], some additional evaluation results were added in this contribution.

DMRS Evaluations

In two-port non-transparent Tx diversity schemes, there are 4 DMRS design options raised in RAN1 #89 meeting, the details of DMRS design options are described in company’s contribution [2]. In this section, the evaluation results about CDM manner (option 1) and FDM manner (option 3) are provided.

PSSCH BLER performance between CDM and FDM are provided in Figure 1, it can be observed that PSSCH BLER performance of both options are very close, and the CDM manner is slightly better than FDM in medium an high speed scenarios.

The CM evaluations are further investigated between CDM and FDM manner. The CM evaluation results are provided in Figure 2, where the initial ID for DMRS sequence is traversed from 0 to 216-1. It can be observed that the FDM manner has significantly higher CM statistics than that of CDM manner.

Observation 1: In two-port non-transparent Tx diversity scheme:

- Both DMRS design options (FDM and CDM manners) has the similar PSSCH BLER performance, and the CDM manner is slightly better than FDM in medium and high speed scenarios.

- The DMRS sequence of FDM manner has significantly higher CM statistics than that of CDM manner.

Figure 1: BLER evaluation for DMRS design options (FDM vs. CDM)

Figure 2: CM evaluation for DMRS design options (FDM vs. CDM)

BLER performance with MMSE-IRC Receiver in Interference Limited Scenario

In this section, the sensitivity on Rel-14 UE PSSCH performance due to different candidate Tx diversity schemes is further evaluated. The interfering UE(s) is either a Rel-14 UE or Rel-15 UE, and candidate Tx diversity schemes for Rel-15 interfering UE are evaluated, including: small-delay CDD and SFBC. Since the BLER performance of Rel-14 UE based on MMSE-MRC receiver has already been provided in our last contribution [4], we will mainly focus on the impact of MMSE-IRC on different candidate Tx diversity schemes. The evaluation assumptions are provided in Table 1.

Table 1: Evaluation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 6 GHz |
| System bandwidth | 10 MHz |
| Channel Model | NLOS in TR36.843 |
| Vehicle speed (absolute) | 15 km/h, 70km/h, 140km/h |
| Tx-Rx antenna configuration | 2x2, 1x2 |
| TxD schemes | SFBC, Small Delay CDD (with 1.1us delay) |
| Receiver type | MMSE-IRC |
| Interference type | Rel-14, SFBC, Small Delay CDD |
| Num of interfering UE(s) | 1, 2, 5, 10 |
| SNR | 25dB, 20dB, 15dB, 10dB |
| TBS | 300Bytes |
| Modulation | 16QAM |
| Number of PRBs | 12 |

The BLER vs. SIR analysis is conducted for the following three scenarios:

- Different speed of UEs

- Different number of interferers

- Different SNR of system

The evaluation results of different speed of UEs are provided in Figure 3. The number of interfering UE is 1, and the SNR of system is 25dB in the first scenario. It can be observed that the performance gets very closed with the increase of the speed whether the interference is a Rel-14 UE or Rel-15 UE with different Tx diversity schemes. When the speed is 140km/h, their performance is almost same as that in MMSE-MRC receiver. MMSE-IRC receiver can only bring significant performance gain at very low speed scenario when the interfering UE is Rel-14 or transmission with small CDD.

Figure 3: Rel-14 UE PSSCH BLER performance with different UE speed

Figure 4 compares the evaluation results of different number of interfering UE(s). According to the evaluation assumptions of the second scenario, the UE speed is 15km/h and the SNR of system is 25dB. It shows that the performance gap of three interference types becomes smaller with the increasing of the number of interfering UEs. This kind of change is already obvious when the number of interfering UEs increase to 2. So the number of interfering UEs also has impacts on the performance of MMSE-IRC.

Figure 4: Rel-14 UE PSSCH BLER performance with different interfering UE number.

The evaluation results of different system SNR are shown in Figure 5. Only one interfering UE exists in this scenario, where UE speed is 15km/h, and the SNR of system is variable. It can be observed that the performance gain of MMSE-IRC receiver will degrade with the decreasing of the system SNR, and when the system SNR is 10dB, there is no performance gain for MMSE-IRC receiver. Therefore, the system SNR also has impacts on the performance of MMSE-IRC.

Figure 5: Rel-14 UE PSSCH BLER performance with different system SNR

Observation 2:

- MMSE-IRC receiver can only bring performance gain at very low speed scenario when the interfering UE is Rel-14 UE or Small delay CDD.

- The number of interfering UEs also has impacts on the performance of MMSE-IRC receiver.

- The system SNR also has impacts on the performance of MMSE-IRC

Proposal 1: Keep the current work assumption on the baseline receiver (MMSE-MRC) for evaluation of transmit diversity schemes.

Conclusion

In this contribution, the Tx diversity schemes and DMRS design options are evaluated. Particularly, we have following observations:

Observation 1: In two-port non-transparent Tx diversity scheme:

- Both DMRS design options (FDM and CDM manners) has the similar PSSCH BLER performance, and the CDM manner is slightly better than FDM in medium and high speed scenarios.

- The DMRS sequence of FDM manner has significantly higher CM statistics than that of CDM manner.

Observation 2:

- MMSE-IRC receiver can only bring performance gain at very low speed scenario when the interfering UE is Rel-14 UE or Small delay CDD.

- The number of interfering UEs also has impacts on the performance of MMSE-IRC receiver.

- The system SNR also has impacts on the performance of MMSE-IRC.

Proposal 1: Keep the current work assumption on the baseline receiver (MMSE-MRC) for evaluation of transmit diversity schemes.

References

  - RAN1 #90 chairman notes

  - R1-1717792, “Discussion on Tx diversity schemes in PC5”, CATT

  - R1-1717793, “Evaluations for Tx diversity schemes in PC5”, CATT

  - R1-1712343, “Evaluations for Tx diversity schemes in PC5”, CATT