3GPP TSG-RAN WG1 Meeting #91	R1-1720535

Reno, U.S.A., 27th Nov- 1st Dec 2017	Update of R1-1717454

Agenda Item:	6.2.8.3

Source:	Ericsson

Title:	Evaluation of latency in LTE

Document for:	Discussion

# Introduction

In this paper, we will go through the various methods for reducing latency in LTE and evaluate the latency performance.

The document is a revision of R1-1717454.

# Discussion

In the following we will analyse the worst-case latency in the user plane. We will follow the ITU definition of UP latency as being defined from L2/L3 ingress to L2/L3 egress.

## Assumptions

Assumption are listed in Table 1 and depicted in Figure 1.

Table 1: Assumptions for latency analysis

| Type | Comment |
| --- | --- |
| L1/L2 processing delay, TL1/L2 | For the L1/L2 processing performed in the eNB we have assumed 1 TTI for both TX and RX. The same is assumed in the UE. Also, it is assumed that data can be delivered to higher layers after processing but before ACK feedback is transmitted |
| Alignment delay, TAlign | The alignment delay is the time required after being ready to transmit until transmission can start. We assume the worst-case latency meaning the alignment delay is assumed to the longest possible. A 2-symbol PDCCH is assumed |
| UE/eNB processing, TProc | The delay between SR and UL grant, and between DL HARQ and retransmission, is assumed to be the same as the UE timing, i.e. the same as the time between DL data and DL HARQ and UL grant and UL data |
| UL scheduling, TSched | For UL data, the scheduling can either be based on SR or SPS UL. In both cases it is assumed that the period is set to 1TTI |

Figure 1: illustration of latency components for DL and UL data.

## Rel-14 baseline

In LTE Rel-14, FastUL was introduced, enabling SPS UL with a period down to 1TTI, effectively reducing the UL latency for first data down to the same level as DL.

## Rel-15

In LTE Rel-15, two improvements were made, reduced processing time for 1ms TTI, and short TTI.

### Reduced processing time

With reduced processing time, the UE response time from DL data to DL HARQ and from UL grant and UL data is reduced from n+4 TTI to n+3 TTI. This means that the HARQ RTT is reduced from n+8 to n+6 for both DL and UL.

### Short TTI

With short TTI the 1ms subframe is divided into either 2 parts (7-symbol sTTI) or 5-6 parts (2/3-symbol sTTI). For the 7-symbol TTI the latencies are calculated on the assumption that the TTI is 7 symbols, and for the 2/3-symbol TTI the latencies are calculated on the assumption that the TTI is 3 symbols. For 7-symbol TTI the UL timing is n+4 TTI, and for 2/3-symbol TTI the UL timing is assumed (still FFS) to be n+6 TTI. For 2/3os the HARQ RTT is set such that the n+6 timing is realized with the agreed sequence of 2 and 3os TTI, i.e. such that the average TTI length is 2.33os giving an UL timing of 14os.

## HARQ-based retransmissions

Retransmission based on HARQ feedback is very spectral efficient. However, it requires the transmitting side to wait for feedback to be received, and thus the delay increases with one HARQ RTT for every retransmission.

With k retransmissions, the DL data delay can be expressed as:

TTot=2TL1/L2+TAlign+2kTProc+(1+2k)TTx

For 1 retransmission and Rel-14 n+4 timing, and taking the assumptions in Table 1, 
TL1/L2=TAlign=TTx=1 TTI, and that TProc=3 TTI for n+4 timing, TTot becomes:
TTot,TTI=4+8k

That is, with k=1, the latency equals 12 TTI or 12ms. For the 2/3os TTI it is assumed that TProc=2.33 TTI.

Furthermore, it can be noted that the delay for ‘DL data’ for 2/3os includes the worst case allocation of latency when 2 PDCCH symbols are used resulting in a delay of 2 extra symbols before the first DL sTTI allocation, see Figure 2.

Figure 2: Worst case alignment time for 2/3os sTTI

Table 2: FDD UP one-way latency for data transmission with HARQ-based retransmission.

| Latency (ms) | HARQ | Rel-14 | Rel-15 Reduced processing (n+3) | Rel-15 7-symbol sTTI | Rel-15 2/3-symbol sTTI |
| --- | --- | --- | --- | --- | --- |
| DL data | 1st transmission | 4.0 | 4.0 | 2.0 | 1.0 |
|  | 1 retx | 12 | 10 | 6.0 | 3.0 |
|  | 2 retx | 20 | 16 | 10 | 5.0 |
|  | 3 retx | 28 | 22 | 14 | 7.0 |
| UL data (SR) | 1st transmission | 12 | 10 | 6.0 | 3.4 |
|  | 1 retx | 20 | 16 | 10 | 5.4 |
|  | 2 retx | 28 | 22 | 14 | 7.4 |
|  | 3 retx | 36 | 28 | 18 | 9.4 |
| UL data (SPS) | 1st transmission | 4.0 | 4.0 | 2.0 | 0.86 |
|  | 1 retx | 12 | 10 | 6.0 | 2.9 |
|  | 2 retx | 20 | 16 | 10 | 4.9 |
|  | 3 retx | 28 | 22 | 14 | 6.9 |

Only the 2/3-symbol TTI in can fulfil a 1ms target, while all configurations can fulfil a 5ms target in FDD DL.

SPS UL is required to fulfil a 1ms target, while SR can be used for a 5ms target.

HARQ retransmissions can’t be done within 1ms target but possible for 5ms target.

## HARQ-less retransmissions

An alternative to HARQ-based retransmission is “HARQ-less”, “blind”, or “automatic” retransmission or repetitions. Since these do not require feedback they can be executed back-to-back without any further delay. The spectral efficiency is lower since the retransmissions may not be necessary.

Table 2: FDD UP one-way latency for data transmission with HARQ-less retransmission.

| Latency (ms) | HARQ | Rel-14 | Rel-15 Reduced processing (n+3) | Rel-15 7-symbol sTTI | Rel-15 2/3-symbol sTTI |
| --- | --- | --- | --- | --- | --- |
| DL data | 1st transmission | 4.0 | 4.0 | 2.0 | 1.0 |
|  | 1 repetition | 5.0 | 5.0 | 2.5 | 1.2 |
|  | 2 repetitions | 6.0 | 6.0 | 3.0 | 1.4 |
|  | 3 repetitions | 7.0 | 7.0 | 3.5 | 1.6 |
| UL data (SR) | 1st transmission | 12 | 10 | 6.0 | 3.4 |
|  | 1 repetition | 13 | 11 | 6.5 | 3.6 |
|  | 2 repetitions | 14 | 12 | 7.0 | 3.9 |
|  | 3 repetitions | 15 | 13 | 7.5 | 4.1 |
| UL data (SPS) | 1st transmission | 4.0 | 4.0 | 2.0 | 0.86 |
|  | 1 repetition | 5.0 | 5.0 | 2.5 | 1.1 |
|  | 2 repetitions | 6.0 | 6.0 | 3.0 | 1.3 |
|  | 3 repetitions | 7.0 | 7.0 | 3.5 | 1.5 |

With a 5ms target 3 repetitions can be made with sTTI.

## TDD aspects

With TDD there are additional alignment delays caused by the sequence of subframes. Depending on when the data arrives in the transmit buffer the latency may be similar or much longer than the FDD latency. Looking at the TDD configuration with most equal distribution of DL and UL subframes and with the highest frequency of switching points, configuration 1 (DSUUD) should be the most latency optimal choice. Assuming that the special subframe is completely DL, the resulting latency is as indicated in Table 3. As can be seen in the table, no TDD configuration can reach a 1ms target, but 5ms is within reach.

Table 3. TDD UP one-way latency for data transmission with TDD configuration 1.

| Latency (ms) | HARQ | Rel-14 | Rel-15 Reduced processing (n+3) | Rel-15 7-symbol sTTI |
| --- | --- | --- | --- | --- |
| DL data | 1st transmission | 6.0 | 6.0 | 4.0 |
|  | 1 retx | 17 | 15 | 11 |
|  | 2 retx | 28 | 24 | 17 |
|  | 3 retx | 39 | 33 | 24 |
| UL data (SR) | 1st transmission | 18 | 16 | 12 |
|  | 1 retx | 29 | 25 | 18 |
|  | 2 retx | 40 | 34 | 25 |
|  | 3 retx | 51 | 43 | 31 |
| UL data (SPS) | 1st transmission | 7.0 | 7.0 | 5.0 |
|  | 1 retx | 18 | 16 | 12 |
|  | 2 retx | 29 | 25 | 18 |
|  | 3 retx | 40 | 34 | 25 |

With TDD a latency below 5ms can be achieved.

# Conclusion

In Section 2 we made the following observations

Observation 1	Only the 2/3-symbol TTI in can fulfil a 1ms target, while all configurations can fulfil a 5ms target in FDD DL.

Observation 2	SPS UL is required to fulfil a 1ms target, while SR can be used for a 5ms target.

Observation 3	HARQ retransmissions can’t be done within 1ms target but possible for 5ms target.

Observation 4	With a 5ms target 3 repetitions can be made with sTTI.

Observation 5	With TDD a latency below 5ms can be achieved.

# References

RAN1, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #88bis, April 2017.

TR 36.881	Study on latency reduction techniques for LTE.