3GPP TSG-RAN WG1 Meeting RAN1#91		R1-1721004

Reno, Nevada, United States, 27th November – 1st December 2017

Source:	Ericsson

Title:	On Support of Long PUCCH Over Multiple Slots

Agenda Item:	7.3.2.2.3

Document for:	Discussion and Decision

# Introduction

The following agreements have been made in previous meetings.

Agreements:

In RAN1#87 the following was agreed [1]

- A UCI carried by long duration UL control channel at least with low PAPR design can be transmitted in one slot or multiple slots

  - Transmission across multiple slots should allow a total duration of [1] ms at least for some cases

    - FFS: more than [1] ms at least for some cases

  - FFS the numbers of the slots

In RAN1#88 the following was agreed

- For PUCCH in long duration,

  - At least for 1 or 2 UCI bits, the UCI can be repeated within N slots (N>1)

    - The N slots may or may not be adjacent in slots where PUCCH in long duration is allowed

    - Details are FFS, including repetition scheme including same or different formats, the possible value(s) N, the mechanism to determine the value of N, etc.

  - FFS for >2 UCI bits

  - FFS the case of within a slot

In RAN1#90bis the following was agreed

- For long PUCCH over multiple slots, at least support the case that the duration of long PUCCH in each slot is the same

  - FFS the case of different durations in different slots

- For long PUCCH over multiple slots, inter-slot hopping is supported by configuration

  - FFS details

- For long PUCCH over multiple slots, the intra-slot hopping and inter-slot hopping are not enabled at the same time for a UE

- Each slot in the multiple slots for long-PUCCH over multiple slots is always contained with a slot

- For long PUCCH with more than 2 bits over multiple slots, all UCI bits are encoded and transmitted in each slot

- For long PUCCH over multi-slots, for the case duration of long PUCCH in each slot is the same, the number of slots with long PUCCH transmission is configurable in a UE-specific manner

  - Up to 4 possible RRC configured numbers, detailed values FFS

In this document, which is a revision of R1-1718637, we provide our view on long PUCCH transmission over multiple slots.

# Discussion

In LTE, HARQ-ACK repetition was introduced for UL to improve coverage. For NR, especially with sub-carrier spacing (SCS) higher than 15kHz long PUCCH duration will be smaller than that of LTE due to which HARQ-ACK transmission on the UL can become a coverage liming factor. Also, even for the 15kHz SCS case, supporting long PUCCH repetition will enable NR to operate with the same link budget (i.e. MCL) as LTE uplink, under the same usage scenarios and similar deployment configurations, and should be supported.

The most natural way to enable or disable long PUCCH repetition is over RRC.

NR long PUCCH repetition is enabled/disabled via RRC configuration.

Because of the importance of ACK/NACK reporting, resources should be focused on conveying HARQ-ACK and SR. CSI reports are not as critical and a few missed CSI opportunities for a UE in bad coverage is not a big problem.

Long PUCCH repetition is supported for HARQ-ACK.

When a UE is in bad coverage all resources, as far as possible, should be used to convey the long PUCCH so that the number of repetitions can be kept to a minimum to minimize latency and control signaling overhead. Further, if there are unused parts of the affected slots they will most likely not be enough to convey any additional information in the current coverage situation.

UE is not expected to transmit other physical channels/signals in slots where it performs a repetition-based long PUCCH transmission.

Once the long PUCCH repetition transmission has started it should not be interrupted or modified. If for example signaling could interrupt the repetitions or cancel specific slots this would increase the complexity of scheduling and introduce the problem of falsely detecting such interrupt requests. Also, if new downlink data, which needs to be ACKed, is received during the repetition of a previously started long PUCCH transmission these new HARQ-ACKs should not be jointly coded in the already ongoing transmission. If that was allowed it would complicate scheduling and HARQ codebook a lot and make coding/decoding of the message troublesome.

An ongoing long PUCCH repetition cannot be interrupted or reconfigured.

## Minimalistic design

A design with a very small footprint could be to exactly repeat the long PUCCH transmitted in the first slot. This would include repeating existing intra-slot frequency hopping resources, slot layout and data for the long PUCCH. The main benefit of such a scheme is the very low extra signaling needed with only the number of repetitions to configure. In RAN1#90bis it was agreed that as a baseline the number of symbols used in each slot should be the same and the number of repetition slots shall, in this case, be configured via RRC. The number of repetitions was left for further study. The PUCCH length in normal operation of LTE, using a sub carrier spacing of 15 kHz, is 1 ms. The maximum length of an NR long PUCCH, using 14 symbols and a subcarrier spacing of 120 kHz, without repetition is 1/8 ms, i.e. 8 times shorter than the LTE PUCCH. Even though the coverage for equally long PUCCHs for LTE and NR will generally not be the same because of the different carrier frequency, it still makes sense to aim for a maximum length for the NR PUCCH of 1ms since excessive lengths of the long PUCCH will increase latency and complicate scheduling. The coverage differences because of the carrier frequencies can be compensated for with other methods. Hence the maximum number of slots used for transmission of long PUCCH over multiple slots is proposed to be 8.

The number of slots used for transmission of long PUCCH over multiple slots is signalled with 2 bits and the possible choices are 1 (i.e. no repetitions), 2, 4 and 8 slots.

The repetitions use the same resources as the first repetition of the same transmission.

UE determines the PUCCH resource for first repeated slot from corresponding DL DCI (as discussed in [1]), and the same PUCCH resource is used for all the subsequent repeated slots of the same transmission.

UE uses the long PUCCH configuration (i.e., starting position) determined for first repeated slot for all the subsequent repeated slots of the same transmission.

If intra-slot frequency hopping is configured for the first repeated slot, intra-slot frequency hopping is used in all the subsequent repeated slots of the same transmission using the same resources.

In RAN1#90bis it was agreed to allow inter-slot frequency hopping as a configurable alternative to intra-slot frequency hopping. It would be preferred to transmit the same amount of information before and after the inter-slot frequency hop, but the hop should take place between two slots. The already discussed split after either ceil(N/2) or floor(N/2) slots makes sense. Which of these to choose is not regarded as a big problem.

A configured inter-slot frequency hop shall take place after slot floor(N/2) (or ceil(N/2)). Where, in this context, the slots are numbered 1…N.

For latency reasons and to save buffer space in gNB, it is good to transmit all repetitions as quickly as possible. Hence the repetitions should be performed over N consecutive UL slots. For FDD that is every slot and for TDD that is every configured UL slot, until N repetitions have been transmitted. For TDD when higher layer indication for UL/DL slots is not present the long PUCCH repetitions should be performed over N consecutive slots and the network should ensure that relevant resources are reserved for this. Considering that the UL/DL structure of a slot is signalled by the slot frame indicator (SFI), the N repetitions could be selected on suitable slots indicated by SFI. However, considering that the link between the UE and gNB is coverage limited, relying on SFI to configure in which slots to transmit repetitions is risky since it is not certain that the SFI is reliable.

Long PUCCH repetitions is performed over N consecutive UL slots. For FDD every slot and for TDD every UL slot until N repetitions have been transmitted.

For TDD when higher layer indication for UL/DL slots is not present, long PUCCH is performed over N consecutive slots and the network ensures that relevant resources are reserved for this.

## Alternative designs

Adjusting the design to add more flexibility comes at the cost of more control signaling. Natural alternative designs could be based on not using the same slot layout in all repetitions. This could span between a few different layouts spread across the slots to different layouts in every slot. Typically, the control signaling in this case would scale with the number of unique slot layouts. Either one massive DL-DCI could configure all slots at once or there could be one DL-DCI per slot. Using one DL-DCI per slot raises the question if this could be achieved when a UE is in bad coverage since that could require repetitions also of the DL-DCI message which would spread out the PUCCH repetitions in time and both complicate scheduling and increase latency. Further spreading out repetitions in time would require more buffer space in gNB since the PUCCH repetitions would be interlaced with other received messages in gNB. If instead one massive DL-DCI was used, that would put further stress on the already coverage limited link between gNB and the UE.

# Conclusions

In this document, we discuss long PUCCH transmission over multiple slots and propose the following

Proposal 1	NR long PUCCH repetition is enabled/disabled via RRC configuration.

Proposal 2	Long PUCCH repetition is supported for HARQ-ACK.

Proposal 3	UE is not expected to transmit other physical channels/signals in slots where it performs a repetition-based long PUCCH transmission.

Proposal 4	An ongoing long PUCCH repetition cannot be interrupted or reconfigured.

Proposal 5	The number of slots used for transmission of long PUCCH over multiple slots is signalled with 2 bits and the possible choices are 1 (i.e. no repetitions), 2, 4 and 8 slots.

Proposal 6	UE determines the PUCCH resource for first repeated slot from corresponding DL DCI (as discussed in [1]), and the same PUCCH resource is used for all the subsequent repeated slots of the same transmission.

Proposal 7	UE uses the long PUCCH configuration (i.e., starting position) determined for first repeated slot for all the subsequent repeated slots of the same transmission.

Proposal 8	If intra-slot frequency hopping is configured for the first repeated slot, intra-slot frequency hopping is used in all the subsequent repeated slots of the same transmission using the same resources.

Proposal 9	A configured inter-slot frequency hop shall take place after slot floor(N/2) (or ceil(N/2)). Where, in this context, the slots are numbered 1…N.

Proposal 10	Long PUCCH repetitions is performed over N consecutive UL slots. For FDD every slot and for TDD every UL slot until N repetitions have been transmitted.

Proposal 11	For TDD when higher layer indication for UL/DL slots is not present, long PUCCH is performed over N consecutive slots and the network ensures that relevant resources are reserved for this.

# References

Chairman Note’s 3GPP RAN WG1 Meeting#87