3GPP TSG RAN WG1 Meeting #92bis	   R1-1804258

Sanya, China, April 16th – 20th, 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussions on support of 64QAM for LTE V2X sidelink

Agenda item:	6.2.5.2

Document for:	Discussion and Decision

# 1	Introduction

In RAN1-92, the candidate schemes of TBS scaling and MCS update and their evaluation results were discussed and the following working assumption was reached [1].

Working assumption

  - TBS scaling (<1) is applied with additional MCS indices in ‘Modulation and TBS index table’

    - Number of additional MCS indices is three

    - Additional TBS values which will be down-selected from Table 7.1.7.2.1-1 in 36.213

      - FFS downselected TBS values

      - Select the scaling factor <1 so as to avoid reducing the peak SE (after adding additional MCS values above 28) compared to MCS 28 with scaling factor 1

        - FFS the exact scaling factor.

In this contribution, we make further discussions on the TBS scaling and additional MCS indices based on the working assumption.

# 2	TBS scaling and additional MCSs

As stated in the working assumption, three additional MCSs shall be added to the MCS table with TBS indices which will be selected from Table 7.1.7.2.1-1 in TS 36.213. Assume that a constant scaling factor is used across all the 32 MCSs, which implies that the added three MCSs correspond to TBS indices for  larger TBS to avoid reduction of peak SE in comparison with the MCS 28 with scaling factor 1.

On the other hand, considering that the TBS scaling is mainly motivated by the increase in DMRS symbol density, the reservation of the last symbol as guard time and the potentially puncturing of the first symbol for AGC settling, it is appropriate to keep the TBS scaling factor in the range 0.667 to 0.750, respectively corresponding to the cases the first SC-FDMA symbol is punctured (i.e. for AGC handling) and is used for data detection without distortion (i.e. not punctured) at the receiver.

With the above taking into account, the three added MCSs could correspond to TBS indices of 30, 32 and 34, as defined in Table 7.1.7.2.1-1 in TS 36.213. The TBS scaling factor could be 0.725, which guarantees the non-reduction of the peak spectral efficiency (assume TBS table element-wise scaling) and in the preferred scaling factor range. The updated MCS table is shown in Table 1 and the spectral efficiency of the updated MCS table with scaling and the legacy MCS table (with scaling factor 1) is shown in Figure 1, taking the transmission bandwidth NPRB=8 as an example. Note that in Table 1, for MCS index 20, it is preferred to change the modulation from 16QAM to 64QAM to optimize the link performance (especially for RV2/first symbol puncturing).

Regarding the implementation of the TBS scaling, the TBS table element-wise scaling method (discussed in [2]) and TBS table column-wise scaling method (discussed in [3]) could be considered. Due to the better accuracy than the column-wise scaling especially for relatively small number of PRBs (e.g. assume the nominal scaling factor is 0.7, actual scaling factor of 1 and 0.5 are used for 1-PRB and 2-PRB transmissions), the TBS table element-wise scaling is preferred.

For convenience of readability, the TBS table element-wise scaling method is duplicated as follows.

(1)

(2)

where

denotes the TBS scaling factor less than or equal to one

TBS denotes the original transport block size

TBSnew denotes TBS after the scaling

[.] denotes operations rounding the input value to nearest integer

Table 1: Legacy and new mappings between MCS index, TBS index, and modulation order

| MCS Index | Legacy Mapping |  | New Mapping |  | Coding rate** |
| --- | --- | --- | --- | --- | --- |
|  | Modulation Order | TBS Index | Modulation Order | TBS Index |  |
| 0 | 2 | 0 | 2 | 0 | 0.0880 |
| 1 | 2 | 1 | 2 | 1 | 0.1065 |
| 2 | 2 | 2 | 2 | 2 | 0.1389 |
| 3 | 2 | 3 | 2 | 3 | 0.1852 |
| 4 | 2 | 4 | 2 | 4 | 0.2315 |
| 5 | 2 | 5 | 2 | 5 | 0.2824 |
| 6 | 2 | 6 | 2 | 6 | 0.3380 |
| 7 | 2 | 7 | 2 | 7 | 0.4028 |
| 8 | 2 | 8 | 2 | 8 | 0.4583 |
| 9 | 2 | 9 | 2 | 9 | 0.5231 |
| 10 | 2 | 10 | 2 | 10 | 0.5787 |
| 11 | 4 | 10 | 4 | 10 | 0.2894 |
| 12 | 4 | 11 | 4 | 11 | 0.3356 |
| 13 | 4 | 12 | 4 | 12 | 0.3819 |
| 14 | 4 | 13 | 4 | 13 | 0.4282 |
| 15 | 4 | 14 | 4 | 14 | 0.4745 |
| 16 | 4 | 15 | 4 | 15 | 0.5208 |
| 17 | 4 | 16 | 4 | 16 | 0.5486 |
| 18 | 4 | 17 | 4 | 17 | 0.6042 |
| 19 | 4 | 18 | 4 | 18 | 0.6597 |
| 20 | 4 | 19 | 6 | 18 | 0.4398/0.6597* |
| 21 | 6 | 19 | 6 | 19 | 0.4892 |
| 22 | 6 | 20 | 6 | 20 | 0.5262 |
| 23 | 6 | 21 | 6 | 21 | 0.5633 |
| 24 | 6 | 22 | 6 | 22 | 0.6003 |
| 25 | 6 | 23 | 6 | 23 | 0.6373 |
| 26 | 6 | 24 | 6 | 24 | 0.6991 |
| 27 | 6 | 25 | 6 | 25 | 0.7238 |
| 28 | 6 | 26 | 6 | 26 | 0.8349 |
| 29 |  |  | 6 | 30 | 0.8719 |
| 30 |  |  | 6 | 32 | 0.9336 |
| 31 |  |  | 6 | 34 | 1.1836 |

Note**: the coding rate depends on transmission bandwidth to some extent. The coding rates shown here are obtained for 8 PRBs.

Note*: the two coding rates correspond to 64QAM and 16QAM respectively.

Figure 1: Spectral efficiency for all MCS for NPRB=8

# 3	Evaluation results

Some link level evaluations were made for the updated MCS table as shown in Table 1 with TBS scaling. The TBS scaling factor is set to 0.725, as discussed in previous section. In the simulations, AWGN channel is used with SNR fixed to 30dB. The detailed simulation conditions are listed in appendix. From the simulation results, we can get the following observation

Observation 1: For the proposed updated MCS table with TBS scaling, V2X sidelink works well for all the MCSs except the one with peak SE for RV0 transmission without puncturing for the first symbol. The RV2 transmission and/or the puncturing for first symbol will bring some performance loss.

- RV0, first symbol not punctured

- RV0, first symbol punctured

- RV2, first symbol not punctured

- RV2, first symbol punctured

Figure 2: Evaluations of the updated MCS table with scaling (green: BLER=0; red: BLER=1; blue:0<BLER<1)

# 5	Conclusion

This paper made further discussions on the MCS table update with three additional MCSs and TBS scaling. In particular, it was proposed that the three added MCSs correspond to TBS indices of 30, 32 and 34 and the TBS scaling factor is 0.725. Some link evaluations were made for the updated MCSs with TBS scaling from which the following observation is obtained.

Observation 1: For the proposed updated MCS table with TBS scaling, V2X sidelink works well for all the MCSs except the one with peak SE for RV0 transmission without puncturing for the first symbol. The RV2 transmission and/or the puncturing for first symbol will bring some performance loss.

Based on the discussions and evaluations, we propose the following proposals

Proposal 1: Confirm the working assumptions on TBS scaling reached in RAN1-92.

Proposal 2: The MCS update shown in Table 1 together with TBS scaling is used for V2X PSSCH. The TBS table element-wise TBS scaling with the scaling factor of 0.725 is preferred.

# References

- RAN1#92 chairman notes.

- R1-1802582 Support of 64QAM for LTE V2V sidelink, Nokia, Nokia Shanghai Bell, RAN1-92.

- R1-1720033, “Support of 64QAM for LTE V2V sidelink communication”, Intel Corporation, RAN1-91.

# Appendix A

In this section, we provide simulation parameter setting of the link level simulations in this paper.

| Parameters | Values |
| --- | --- |
| Carrier frequency | 6.0 GHz |
| Resource allocation | All possible PRB numbers |
| MCS table | Shown in Table 1 |
| TBS scaling | TBS table element-wise TBS scaling shown in (1)(2) with the scaling factor of 0.725. |
| Antenna numbers | 1 TX single port 2 RX antennas with MMSE-MRC for detection |
| Number of TTI | 1 TTI with RV0 or RV2 |
| Puncturing assumption for the 1st symbol | Being punctured or not |
| Channel model | AWGN channel |
| Timing/frequency offset | No time and frequency offset for TX, but RX assumes its estimation and compensation |
| Channel estimation method | Practical, including Half-symbol based timing/frequency synchronization, LMMSE channel estimation in frequency and linear interpolation in time |